#### FACULTY OF ARCHITECTURE, COMPUTING AND ENGINEERING #### SCHOOL OF APPLIED COMPUTING #### EXAMINATION SOLUTIONS: SESSION 2016/17

| Programme: | BEng Computer Systems and Electronics | Year/Level: | Year 2, Level 5 |
| --- | --- | --- | --- |
| Module/Unit: | Digital Signal Processing and Digital Communications M2X8518 |  |  |
| Author: | Prof Ian Wells | Page: | 1 of 14 |

1.

- Draw a constellation diagram for a quadrature phase shift keying (QPSK) system and allocate binary codes to each phase. Using your allocation of codes indicate the phases used to transmit the following sequence of digits.

(6 marks)

QPSK

Q

I

11

01

00

10

QPSK

Q

I

11

01

00

10

Code allocation to phases is as follows:

11 = pi/4

01 = 3pi/4

00 = 5pi/4

10 = 7pi/4

The sequence x(k) would therefore be translated into the following set of phases:

Constellation diagram 2 marks, Gray code allocation 2 marks, correct phase allocation 2 marks

TOTAL = 6 marks

##### Continuation Sheet

| Module/Unit: | Digital Signal Processing and Digital Communications | Page: | 2 of 14 |
| --- | --- | --- | --- |

1.

- State 'Shannon's Noisy Channel Coding Theorem' and use your answer to calculate the required bandwidths necessary to achieve a 1 x 103 bits/sec data rate when the signal to noise ratio (S/N) is 1, 3, 15, 255 and 1023 respectively.

(4 marks)

##### 'Shannon's Noisy Channel Coding Theorem' is:

##### C=Blog21+SN bits / sec

##### Where S is average signal power, N is average Noise power, B is channel bandwidth and C is capacity of channel:

##### For S/N = 1:

##### 1000=Blog21+1 bits / sec

B = 1kHz

##### For S/N = 3:

##### 1000=Blog21+3

B = 500Hz

##### For S/N = 15:

##### 1000=Blog21+15

B = 250Hz

##### For S/N = 255:

##### 1000=Blog21+255

B = 125Hz

##### For S/N = 1023:

##### 1000=Blog21+1023

B = 100Hz

##### 1mark for stating the theorem and 3 marks for the calculations

##### TOTAL  = 4 MARKS
Continuation Sheet

| Module/Unit: | Digital Signal Processing and Digital Communications | Page: | 3 of 14 |
| --- | --- | --- | --- |

1.

- A set of transmitted data is to be encoded using a (7,4) Hamming code with the parity check matrix,  given by equation Q1b.

H=110011101111100010001	Equation Q1b

Find the generator matrix,  to produce the coded data and calculate the codes 				produced by the sequences .

(10 marks)

The generator matrix  is given by:

G=1000010000100001101111010111

- For the uncoded word  the transmitted code is given by:

T=0101.1000010000100001101111010111

T=0101,001,011,101

T=0101100

- For the uncoded word  the transmitted code is given by:

T=1101.1000010000100001101111010111

T=1101,101,111,101

T=0101010

- For the uncoded word  the transmitted code is given by:

T=1010.1000010000100001101111010111

T=1101,110,100,010

T=0101011

1 mark for generator matrix and 3 marks each for the codes

TOTAL = 10 Marks

##### Continuation Sheet

| Module/Unit: | Digital Signal Processing and Digital Communications | Page: | 4 of 14 |
| --- | --- | --- | --- |

2.

- A number of characters x1, x2,...,x8 are to be transmitted with the following transmission probabilities p(x1)=0.1, p(x2)=0.05, p(x3)=0.09, p(x4)=0.07, p(x5)=0.35, p(x6)=0.01, p(x7)=0.25 and p(x8)=0.08 .

Design a Shannon-Fano code for the transmission of these characters and calculate:

- the entropy of the source

- the average length of the code

Comment upon the optimality of your code using the 'Source Coding Theorem' (Shannon's      	      First Theorem)

(14 marks)

| Message | Probability | I | II | III | IV | V |  |  | Code | Bits/message |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X5 | 0.35 | 0 |  |  |  |  |  |  | 0 | 1 |
| X7 | 0.25 | 1 | 0 |  |  |  |  |  | 10 | 2 |
| X1 | 0.1 | 1 | 1 | 0 |  |  |  |  | 110 | 3 |
| X3 | 0.09 | 1 | 1 | 1 | 0 |  |  |  | 1110 | 4 |
| X8 | 0.08 | 1 | 1 | 1 | 1 | 0 |  |  | 11110 | 5 |
| X4 | 0.07 | 1 | 1 | 1 | 1 | 1 | 0 |  | 111110 | 6 |
| X2 | 0.05 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1111110 | 7 |
| X6 | 0.01 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1111111 | 7 |

4 marks for code design

Entropy of the source is given by:

H=i=1∞pilog21pi

=0.35log210.35+0.25log210.25+0.1log210.1+0.09log210.09

+0.08log210.08+0.07log210.07+0.05log210.05+0.01log210.01

= 2.5175 bits/symbol

6 marks for Entropy calculation

Average Length of the code (average bits per message) is given by:

L= (1x0.35)+(2x0.25)+(3x0.1)+(4x0.09)+(5x0.08)+(6x0.07)+(7x0.05)+(7x0.01)

=0.35+0.5+0.3+0.36+0.48+0.42+0.35+0.07

=2.83 bits per message on average

Since L and H are similar in size then this code is fairly efficient since the maximum is where each bit on average carries one bit of information.

4 marks for length calculation

TOTAL = 14 Marks

##### Continuation Sheet

| Module/Unit: | Digital Signal Processing and Digital Communications | Page: | 5 of 14 |
| --- | --- | --- | --- |

2.

- Determine the bandwidth needed for the channel when using 'Time Division Multiplexing (TDM)' to multiplex the signals shown in figure Q2b.

Figure Q2b

(6 marks)

For a TDM signal it is the highest bandwidth amongst a group of signals that dictates the rate of sampling via;

1τ≥2NsWb

Where 1/ is the bandwidth needed by the channel, Wb is the highest frequency signal of the group  and Ns is the number of signals

Therefore for this set of signals the highest frequency is the same for the 2nd and 5th waveforms and is given by 1/0.005 =200Hz. The number of signals Ns=6

Substitute in the equation gives:

1τ≥2.6.200≥2400Hz

2 marks for the equation its description and 4 marks for the calculation

TOTAL = 6 MARKS

Continuation Sheet

| Module/Unit: | Digital Signal Processing and Digital Communications | Page: | 6 of 14 |
| --- | --- | --- | --- |

3.

- Figure Q3a shows the magnitude and phase response of a digital filter. What can you say about the characteristics of this filter?

Figure Q3a

(4 marks)

##### The filter has the following characteristics:

- The sample frequency was at 28000Hz

- The linear phase response in the pass band indicates that it’s probably FIR

- It is a low pass filter

- Its cut off using a -3dB cut off is a little less than 2000Hz

1 mark each

Total=4 marks

##### Continuation Sheet

| Module/Unit: | Digital Signal Processing and Digital Communications | Page: | 7 of 14 |
| --- | --- | --- | --- |

3.

- Design an N=7 finite impulse response (FIR) filter (use the frequency transformation method) that is band pass between  Hz and  Hz and has a sampling frequency of  Hz.

Use a Hamming window (equation Q3b) in your design.

Equation Q3b

(16 marks)

Answer

Step 1:

Draw a sketch of the analogue filter response

Magnitude

Magnitude

1

1

Frequency (Hz)

Frequency (Hz)

6000Hz

6000Hz

4000Hz

4000Hz

0.5 marks

Step 2:

Convert any frequencies to normalised frequency

υl=flfs=400028000=0.1429

υu=fufs=600028000=0.2143

1 marks

##### Continuation Sheet

| Module/Unit: | Digital Signal Processing and Digital Communications | Page: | 8 of 14 |
| --- | --- | --- | --- |

Step 3:

Redraw the sketch using normalised frequencies

Magnitude

Magnitude

1

1

Normalised Frequency

Normalised Frequency

0.2143

0.2143

0.5

0.5

-0.2143

-0.2143

-0.5

-0.5

-0.1429

-0.1429

0

0

0.1429

0.1429

0.5 marks

Step 4:

Refer to Frequency Transformation Method Tables and insert values for filter design

l

0

1

u

-u

-0

-l

l

0

1

u

-u

-0

-l

Calculate the Low Pass Prototype response:

Calculate the centre frequency:

Calculate Alpha

2 marks

##### Continuation Sheet

| Module/Unit: | Digital Signal Processing and Digital Communications | Page: | 9 of 14 |
| --- | --- | --- | --- |

Step 5:

Using data from step 4 draw a sketch of the prototype low pass filter

Magnitude

Magnitude

1

1

B=0.0714

B=0.0714

Normalised Frequency

Normalised Frequency

0.0357

0.0357

-0.0357

-0.0357

-0.5

-0.5

0.5

0.5

0

0

Determine the filters total bandwidth B from the rectangle width

1 mark

Step 6

Calculate the set of filter values for the low pass prototype

Substitute calculated values:

| k |  |
| --- | --- |
| 0 | 0.0661 |
| 1 | 0.0690 |
| 2 | 0.0708 |
| 3 | 0.0714 |
| 4 | 0.0708 |
| 5 | 0.0690 |
| 6 | 0.0661 |

3 marks

##### Continuation Sheet

| Module/Unit: | Digital Signal Processing and Digital Communications | Page: | 10 of 14 |
| --- | --- | --- | --- |

Step 7

Calculate the transformation equation values and apply to the low pass prototype

| k |  |  |  |
| --- | --- | --- | --- |
| 0 | 0.0661 | -0.2245 | -0.0148 |
| 1 | 0.0690 | 0.8674 | 0.0599 |
| 2 | 0.0708 | 1.6934 | 0.1199 |
| 3 | 0.0714 | 2.0000 | 0.1428 |
| 4 | 0.0708 | 1.6934 | 0.1199 |
| 5 | 0.0690 | 0.8674 | 0.0599 |
| 6 | 0.0661 | -0.2245 | -0.0148 |

4 marks

Step 8

Calculate the window coefficients and apply to the filter by multiplying the window coefficient column with the filter column.

| k |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 0 | 0.0661 | -0.2245 | -0.0148 | 0.1256 | -0.0019 |
| 1 | 0.0690 | 0.8674 | 0.0599 | 0.4376 | 0.0262 |
| 2 | 0.0708 | 1.6934 | 0.1199 | 0.8268 | 0.0991 |
| 3 | 0.0714 | 2.0000 | 0.1428 | 1.0000 | 0.1428 |
| 4 | 0.0708 | 1.6934 | 0.1199 | 0.8268 | 0.0991 |
| 5 | 0.0690 | 0.8674 | 0.0599 | 0.4376 | 0.0262 |
| 6 | 0.0661 | -0.2245 | -0.0148 | 0.1256 | -0.0019 |

The design is now complete!

4 marks

TOTAL = 16 marks

##### Continuation Sheet

| Module/Unit: | Digital Signal Processing and Digital Communications | Page: | 11 of 14 |
| --- | --- | --- | --- |

4.

- For a linear time invariant system find and sketch the resultant output  for an input. The systems impulse response is given by.

(4 marks)

This is a convolution between the two functions:

For setting up the convolution correctly

0.5 marks

| x(k) |  |  |  | 2 | 3 | 2 | 1 |  |  |  | y(k) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3 | 2 | 1 | 2 |  |  |  |  |  |  | y0=2×2=4 | 0.5 marks |
|  |  | 3 | 2 | 1 | 2 |  |  |  |  |  | y1=1×2+2×3=8 | 0.5 marks |
|  |  |  | 3 | 2 | 1 | 2 |  |  |  |  | y2=2×2+1×3+2×2=11 | 0.5 marks |
|  |  |  |  | 3 | 2 | 1 | 2 |  |  |  | y3=3×2+2×3+1×2+2×1=16 | 0.5 marks |
|  |  |  |  |  | 3 | 2 | 1 | 2 |  |  | y4=3×3+2×2+1×1=14 | 0.5 marks |
|  |  |  |  |  |  | 3 | 2 | 1 | 2 |  | y5=3×2+2×1=8 | 0.5 marks |
|  |  |  |  |  |  |  | 3 | 2 | 1 | 2 | y6=3×1=3 | 0.5 marks |

TOTAL = 4 Marks

##### Continuation Sheet

| Module/Unit: | Digital Signal Processing and Digital Communications | Page: | 12 of 14 |
| --- | --- | --- | --- |

4.

- Figure Q4b shows the butterfly diagram of a radix 2, decimation in time (DIT) FFT, calculate the final equations for the spectral outputs, X(0) to X(3), in terms of the time domain inputs, x(0) to x(3). Find the spectrum of the input sequence  where there is no overlap between frames.

| x(0) |  |  |  |  |  |  |  | X(0) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x(2) | W40 |  |  |  |  |  |  | X(1) |
|  |  |  |  |  | W40 |  |  |  |
| x(1) |  |  |  |  |  |  |  | X(2) |
|  |  |  |  |  | W41 |  |  |  |
| x(3) | W40 |  |  |  |  |  |  | X(3) |

# Figure Q4b

(16 marks)

Firstly we need to realise that the data to be analysed needs to split into 2 parts in order to analyse it:

Frame 1:

Frame 2:

1 mark

Frame 1:

| x(0)=1 |  |  |  | 1+4=5 |  |  |  | X(0) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | =5+5=10 |
| x(2)=4 | W40 |  |  | 1-4=-3 |  |  |  | X(1) |
|  |  |  |  |  | W40 |  |  | =-3- W41 |
| x(1)=2 |  |  |  | 2+3=5 |  |  |  | X(2) |
|  |  |  |  |  | W41 |  |  | =5-5=0 |
| x(3)=3 | W40 |  |  | 2-3=-1 |  |  |  | X(3) =-3+ W41 |

4 marks

##### Continuation Sheet

| Module/Unit: | Digital Signal Processing and Digital Communications | Page: | 13 of 14 |
| --- | --- | --- | --- |

Frame 2:

| x(0)=3 |  |  |  | 3+1=4 |  |  |  | X(0) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | =4+7=11 |
| x(2)=1 | W40 |  |  | 3-1=2 |  |  |  | X(1) |
|  |  |  |  |  | W40 |  |  | =2+3 W41 |
| x(1)=5 |  |  |  | 5+2=7 |  |  |  | X(2) |
|  |  |  |  |  | W41 |  |  | =4-7=-2 |
| x(3)=2 | W40 |  |  | 5-2=3 |  |  |  | X(3) =2-3 W41 |

4 marks

Calculating W41 to interpret the results:

The definition of the term W is as follows

In our case N=4 and so the term will give us:

This can be expanded by the Euler formula

2 marks

So the W41 terms can now be substituted for in the answers developed for frames1 and frame 2. We can also calculate the magnitude of  the numbers (some of which are complex) and plot the graphs of the relting specta for frame 1 and frame 2.

To calculate the magnitude of a complex number we use:

| Frame 1 | Magnitude Frame 1 | Frame 2 | Magnitude Frame 2 |
| --- | --- | --- | --- |
| X(0) |  | X(0) |  |
| =5+5=10 | 10 | =4+7=11 | 11 |
| X(1) |  | X(1) |  |
| =-3- W41= -3+j |  | =2+3 W41=2-3j |  |
| X(2) |  | X(2) |  |
| =5-5=0 | 0 | =4-7=-2 | 2 |
| X(3) =-3+ W41= -3-j |  | X(3) =2-3 W41=2+3j |  |

2 marks

##### Continuation Sheet

| Module/Unit: | Digital Signal Processing and Digital Communications | Page: | 14 of 14 |
| --- | --- | --- | --- |

We can then plot the spectra separately:

1.5 marks

1.5 marks

TOTAL = 16 marks