a)

(i)

Candidates are required to calculate the planning factors for direct labour hours and machine time using the equations below: -

Planning factor for direct labour = Direct labour hoursNo of units

Planning factor for machine time = Machine time hoursNo of units

Planning factors for Wiper System 1

Planning factor for direct labour =   180,000500,000=0.36 hours per unit

Planning factor for machine time = 50,000500,000=0.10 hours per unit

Planning factors for Wiper System 2

Planning factor for direct labour =   220,000400,000=0.55 hours per unit

Planning factor for machine time = 20,000400,000=0.05 hours per unit

|  | Planning factors |  |
| --- | --- | --- |
|  | Direct Labour | Machine Time |
| System 1 | 0.36 | 0.10 |
| System 2 | 0.55 | 0.05 |

(ii)

Candidates need to calculate the workload generated and total capacity needs for direct labour and machine time.  For example Q1 direct labour system 1 multiply 100,000 units by planning factor 0.36 direct labour hours gives 36,000 hours.  This process is now repeated for each quarter.

|  | Direct Labour hours required |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Q1 | Q2 | Q3 | Q4 | Totals |
| System 1 | 36,000 | 54,000 | 54,000 | 46,800 | 190,800 |
| System 2 | 82,500 | 55,000 | 49,500 | 27,500 | 214,500 |
| Totals | 118,500 | 109,000 | 103,500 | 74,300 | 405,300 |

|  | Machine Time hours required |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Q1 | Q2 | Q3 | Q4 | Totals |
| System 1 | 10,000 | 15,000 | 15,000 | 13,000 | 53,000 |
| System 2 | 7,500 | 5,000 | 4,500 | 2,500 | 19,500 |
| Totals | 17,500 | 20,000 | 19,500 | 15,500 | 72,500 |

(iii)

Work centre capacity needs are calculated as follows:

Multiply total quarter 1 labour needs by 65% (118,500 x 0.65 = 77,025 hours).  This process is repeated for each quarter remembering to multiply values for department B by 0.35.

Direct labour hours required by departments

|  | Historical Allocation | Q1 | Q2 | Q3 | Q4 | Totals |
| --- | --- | --- | --- | --- | --- | --- |
| Dept A | 65% | 77,025 | 70,850 | 67,275 | 48,295 | 263,445 |
| Dept B | 35% | 41,478 | 38,150 | 36,225 | 26,005 | 141,855 |
| Totals |  | 118,500 | 109,000 | 103,500 | 74,300 | 405,300 |

Machine time hours required by departments

|  | Historical Allocation | Q1 | Q2 | Q3 | Q4 | Totals |
| --- | --- | --- | --- | --- | --- | --- |
| Dept A | 65% | 11,375 | 13,000 | 12,675 | 10,075 | 47,125 |
| Dept B | 35% | 6,125 | 7,000 | 6,825 | 5,425 | 25,375 |
| Totals |  | 17,500 | 20,000 | 19,500 | 15,500 | 72,500 |