Contents

1.	Introduction and Research	2

2.	Calculation and Analysis	3

Section 1	4

Section 2	4

Section 3	4

Section 4	5

Section 5	5

Section 6	5

Section 7	6

Section 8	6

Section 9	6

Section 10	7

Section 11	7

Section 12	10

Section 13	12

Section 14	13

Section 15	13

Section 16	14

3.	Results	15

4.	References	15

Series Hybrid Vehicle

By Mingxuan Guo

Student ID : 1704789

# Introduction and Research

Before analyze the typical duty cycle about the buggy, we should select one of the available buddies from the website. Comparing the buggies from the website, I finally choose Polaris RZR170 (The following Figure 1 ).

Figure 1

The vehicle is suitable for Children 10 years and over, and will also accommodate an adult. The off road buggy is more dangerous to drive, so I think this one, which is not designed for children, is much better. The buggy has a four stroke 169cc air cooled engine with an electric start and automatic transmission. It is two wheel drive, driven off the rear axle, and also has full suspension. Polaris have built into the RZR170 several key safety features including a standard removable ignition key, electronic throttle safety control, daytime running lights, side nets, full cab frame, 3 point seat belts, color coded foot controls, parking brake, hydraulic disc brakes, and speed controls to enable adults to set a maximum speed control limit. The complete equipment allows us to drive more confident and comfortable. The Blitzworld offers a upgraded 4 strap safety harness to replace the existing shoulder cross strap supplied with the buggy. And the advanced seat belt can keep our safety. So, these are the reasons why I choose this buggy.

The second step is to design the off road track lap. The following Figure 2 is the track which I designed. Figure 3 is the front view of the track.

Figure 2

Figure 3

Figure 4

# Calculation and Analysis

In Figure 2, we can see that there is a blue point at the top left corner. The blue point is the start of the track. The buggy follows the track and drives to the right. And in Figure 4, we find dimensions and capacities of the buggy, so that we can do the duty cycle analysis. The duty cycle analysis is complicated, so I divide the track into several sections to analyze. But at first, we should do some calculations for basic data. We know that the equation of aerodynamic resistance is : Faero = 0.5 * ρair * Cd * Width * Height * v2. And ρair = 1.4 kg / m3, Cd =0.8, Width = 1194mm, Height = 1397mm, so Faero = 0.934 * v2. Because it is an off road track, so the rolling resistance coefficient is similar with the coefficient on the field. We find that the rolling resistance coefficient on the field is 0.1-0.35, so we choose 0.2 as the rolling resistance coefficient. From Figure 4, we know that the weight of the buggy is 227kg. There will be 2 persons on the buggy when it will be driven. Assuming that one person weighs 70kg, the total weight is 367kg. For the vehicle performance is adequate and entertaining, the top speed must be lower than 10m / s, the acceleration must be lower than 2m / s2, the gradient is lower than 15°, and the maximum power must be lower than 15kW. Based on those calculations, we can analyze the track by sections.

Section 1 : the buggy starts on the field

Because the buggy just starts, the velocity of the buggy is 0. From the equation of aerodynamic resistance, we know that Faero = 0. For there is no angle, the climbing resistance is 0 too. And the acceleration is 0, so the accelerate resistance is 0. But the rolling resistance always exists, and it is : μ * m * g = 0.2 * 367 * 9.81 = 720.054N. The total Force equals the rolling resistance = 720.054N. The total power = the total force * v = 0.

v = 0

a = 0

Faero = 0

Froll = 720.054N

Fgred = 0

Fa = 0

FT = 720.054N

PT = 0

Section 2 : the buggy starts to accelerate on the ground

The velocity of the buggy will increase from 0 to 8m / s. The acceleration will be 1m / s2. Because the speed is not a fixed value, the aerodynamic resistance = 0.934 * v2. The rolling resistance =μ * m * g = 0.2 * 367 * 9.81 = 720.054N. For there is no angle, the climbing resistance is 0. And the acceleration is 1m / s, so the accelerate resistance = m * a = 367 * 1 = 367N. The total force = 0.934 * v2 + 720.054N + 367N = 0.934 * v2 + 1087.054N. The total power = the total force * v = 0.934 * v3 + 1087.054v W. The total time of this section = vf / a = 8 / 1 = 8sec. The distance of this section = v2f – v2s / 2a = 82 – 02 / 2 * 1 = 32m.

v = 0-8m / s

a = 1m / s2

Faero = 0.934 * v2

Froll = 720.054N

Fgred = 0

Fa = 367N

FT = 0.934 * v2 + 1087.054N

PT = 0.934 * v3 + 1087.054v W

FTmin = 1087.054N

PTmin = 0

FTmax = 1146.83N

PTmax = 9174.64W

Total time = 8sec

L = 32m

Section 3 : the buggy drives straight at a uniform speed

The velocity of the buggy is 8m / s. The acceleration is 0m / s2. Because the speed is a fixed value, the aerodynamic resistance = 0.934 * v2 = 0.934 * 82 =59.776N. The rolling resistance =μ * m * g = 0.2 * 367 * 9.81 = 720.054N. For there is no angle, the climbing resistance is 0. And the acceleration is 0, so the accelerate resistance is 0. The total force = 59.776N + 720.054N = 779.83N. The total power = the total force * v = 779.83 * 8 = 6238.64W. The total time of this section = 5sec. The distance of this section = 40m.

v = 8m / s

a = 0m / s2

Faero = 59.776N

Froll = 720.054N

Fgred = 0

Fa = 0

FT = 779.83N

PT = 6238.64W

Total time = 5sec

L = 40m

Section 4 : the buggy climbs at a uniform speed

The velocity of the buggy is 8m / s. The acceleration is 0m / s2. Because the speed is a fixed value, the aerodynamic resistance = 0.934 * v2 = 0.934 * 82 =59.776N. The rolling resistance =μ * m * g * cosα = 0.2 * 367 * 9.81 * cos10° = 709.25319N. For the angle is 10°, the climbing resistance = m * g * sinα = 367 * 9.81 * sin10° = 626.44698N. And the acceleration is 0, so the accelerate resistance is 0. The total force = 59.776N + 709.25319N + 626.44698N = 1395.47617N. The total power = the total force * v = 1395.47617 * 8 = 11163.8094W. The total time of this section = 5sec. The distance of this section = 40m.

v = 8m / s

a = 0m / s2

Faero = 59.776N

Froll = 709.25319N

Fgred = 626.22698N

Fa = 0

FT = 1395.47617N

PT = 11163.8094W

Total time = 5sec

L = 40m

Section 5 is the same as the section 3.

Section 6 : the buggy goes downhill at a uniform speed

The velocity of the buggy is 8m / s. The acceleration is 0m / s2. Because the speed is a fixed value, the aerodynamic resistance = 0.934 * v2 = 0.934 * 82 =59.776N. The rolling resistance =μ * m * g * cosα = 0.2 * 367 * 9.81 * cos10° = 709.25319N. For the angle is 10°, the climbing resistance = m * g * sinα = 367 * 9.81 * -sin10° = -626.44698N. And the acceleration is 0, so the accelerate resistance is 0. The total force = 59.776N + 709.25319N - 626.44698N = 142.58221N. The total power = the total force * v = 142.58221 * 8 = 1140.65768W. The total time of this section = 5sec. The distance of this section = 40m.

v = 8m / s

a = 0m / s2

Faero = 59.776N

Froll = 709.25319N

Fgred = -626.22698N

Fa = 0

FT = 142.58221N

PT = 1140.65768W

Total time = 5sec

L = 40m

Section 7 is the same as the section 3.

Section 8 : continuous uphill and then continuous downhill

- Uphill 1

The analysis is the same as section 4, but the total time = 3sec. And the distance of this section = 24m.

- Straight at a uniform speed 1

The analysis is the same as section 3, but the total time = 1sec. And the distance of this section = 8m.

- Uphill 2

The analysis is the same as section 8 (1), but the total time = 7sec. And the distance of this section = 56m.

- Straight at a uniform speed 2

Straight at a uniform speed 2 is the same as the section 8 (2) .

- Downhill 1

The analysis is the same as section 6, but the total time = 7sec. And the distance of this section = 56m.

- Straight at a uniform speed 3

Straight at a uniform speed 2 is the same as the section 8 (2) .

- Downhill 2

The analysis is the same as section 6, but the total time = 3sec. And the distance of this section = 24m.

Section 9 : the buggy decelerates uniformly before it enters the corner

The velocity of the buggy will decrease from 8 to 4m / s. The acceleration will be -1m / s2. Because the speed is not a fixed value, the aerodynamic resistance = 0.934 * v2. The rolling resistance =μ * m * g = 0.2 * 367 * 9.81 = 720.054N. For there is no angle, the climbing resistance is 0. And the acceleration is 1m / s, so the accelerate resistance = m * a = 367 * (-1) = -367N. The total force = 0.934 * v2 + 720.054N - 367N = 0.934 * v2 + 353.054N. The total power = the total force * v = 0.934 * v3 + 353.054v W. The total time of this section = vf - vs / a = (4-8) / (-1)= 4sec. The distance of this section = v2f – v2s / 2a = 42 – 82 / 2 * (-1) = 24m.

v = 8-4m / s

a = -1m / s2

Faero = 0.934 * v2

Froll = 720.054N

Fgred = 0

Fa = -367N

FT = 0.934 * v2 + 353.054N

PT = 0.934 * v3 + 353.054v W

FTmin = 367.998N

PTmin = 1471.992W

FTmax = 412.83N

PTmax = 3302.64W

Total time = 4sec

L = 24m

Section 10 : corner

The velocity of the buggy is 4m / s. The acceleration is 0m / s2. Because the speed is a fixed value, the aerodynamic resistance = 0.934 * v2 = 0.934 * 42 =14.944N. The rolling resistance =μ * m * g = 0.2 * 367 * 9.81 = 720.054N. For there is no angle, the climbing resistance is 0. And the acceleration is 0, so the accelerate resistance is 0. The total force = 14.944N + 720.054N = 734.998N. The total power = the total force * v = 734.998 * 4 = 2939.992W. The total time of this section = 5sec. The distance of this section = 20m.

v = 8m / s

a = 0m / s2

Faero = 14.944N

Froll = 720.054N

Fgred = 0

Fa = 0

FT = 734.998N

PT = 2939.992W

Total time = 5sec

L = 20m

Section 11 : uphill and then continuous downhill

- Uniform deceleration uphill

The velocity of the buggy will decrease from 4 to 1m / s. The acceleration will be -1m / s2. Because the speed is not a fixed value, the aerodynamic resistance = 0.934 * v2. The rolling resistance =μ * m * g * cosα = 0.2 * 367 * 9.81 * cos10° = 709.25319N. For the angle is 10°, the climbing resistance = m * g * sinα = 367 * 9.81 * sin10° = 626.44698N. And the acceleration is 1m / s, so the accelerate resistance = m * a = 367 * -1 = -367N. The total force = 0.934 * v2 + 709.25319N + 626.44698N - 367N = 0.934 * v2 + 968.70017N. The total power = the total force * v = 0.934 * v3 + 968.70017v W. The total time of this section = vf - vs / a = (1-4) / -1 = 3sec. The distance of this section = v2f – v2s / 2a = 12 – 42 / 2 * (-1) = 7.5m.

v = 4-1m / s

a = -1m / s2

Faero = 0.934 * v2

Froll = 709.25319N

Fgred = 626.44698N

Fa = -367N

FT = 0.934 * v2 + 968.70017N

PT = 0.934 * v3 + 968.70017v W

FTmin = 969.63417N

PTmin = 969.63417N

FTmax = 983.64417N

PTmax = 3934.57668W

Total time = 3sec

L = 7.5m

- Straight at a uniform speed 1

The analysis is the same as section 3, but the total time = 4sec. And the distance of this section = 4m. Because v = 1m / s, Faero = 0.934N, the total force = 720.988N, the total power = 720.988N.

- Uniform acceleration downhill 1

The velocity of the buggy will increase from 1 to 8m / s. The acceleration will be 1m / s2. Because the speed is not a fixed value, the aerodynamic resistance = 0.934 * v2. The rolling resistance =μ * m * g * cosα = 0.2 * 367 * 9.81 * cos10°= 709.25319N. For the angle is 10°, the climbing resistance = m * g * sinα = 367 * 9.81 * -sin10° = -626.44698N.  And the acceleration is 1m / s, so the accelerate resistance = m * a = 367 * 1 = 367N. The total force = 0.934 * v2 + 709.25319N -626.44698N + 367N = 0.934 * v2 + 449.80621N. The total power = the total force * v = 0.934 * v3 + 449.80621v W. The total time of this section = vf - vs / a = (8-1) / 1 = 7sec. The distance of this section = v2f – v2s / 2a = 82 – 12 / 2 * 1 = 31.5m.

v = 1-8m / s

a = 1m / s2

Faero = 0.934 * v2

Froll = 709.25319N

Fgred = -626.44698N

Fa = 367N

FT = 0.934 * v2 + 449.80621N

PT = 0.934 * v3 + 449.80621v W

FTmin = 450.74021N

PTmin = 450.74021W

FTmax = 509.58221N

PTmax = 4076.65768W

Total time = 7sec

L = 31.5m

- Uniform deceleration linear motion 1

The velocity of the buggy will decrease from 8 to 1m / s. The acceleration will be -1m / s2. Because the speed is not a fixed value, the aerodynamic resistance = 0.934 * v2. The rolling resistance =μ * m * g = 0.2 * 367 * 9.81 = 720.054N. For there is no angle, the climbing resistance is 0. And the acceleration is 1m / s, so the accelerate resistance = m * a = 367 * (-1) = -367N. The total force = 0.934 * v2 + 720.054N - 367N = 0.934 * v2 + 353.054N. The total power = the total force * v = 0.934 * v3 + 353.054v W. The total time of this section = vf - vs / a = (1-8) / -1 = 7sec. The distance of this section = v2f – v2s / 2a = 12 – 82 / 2 * (-1) = 31.5m.

v = 8-1m / s

a = -1m / s2

Faero = 0.934 * v2

Froll = 720.054N

Fgred = 0

Fa = -367N

FT = 0.934 * v2 + 353.054N

PT = 0.934 * v3 + 353.054v W

FTmin = 353.988N

PTmin = 353.988N

FTmax = 412.83N

PTmax = 3302.64W

Total time = 7sec

L = 31.5m

- Uniform acceleration downhill 2

Uniform acceleration downhill 2 is the same as the section 11 (3) .

- Uniform deceleration linear motion 2

The velocity of the buggy will decrease from 8 to 3m / s. The acceleration will be -1m / s2. Because the speed is not a fixed value, the aerodynamic resistance = 0.934 * v2. The rolling resistance =μ * m * g = 0.2 * 367 * 9.81 = 720.054N. For there is no angle, the climbing resistance is 0. And the acceleration is 1m / s, so the accelerate resistance = m * a = 367 * (-1) = -367N. The total force = 0.934 * v2 + 720.054N - 367N = 0.934 * v2 + 353.054N. The total power = the total force * v = 0.934 * v3 + 353.054v W. The total time of this section = vf - vs / a = (3-8) / -1 = 5sec. The distance of this section = v2f – v2s / 2a = 32 – 82 / 2 * (-1) = 27.5m.

v = 8-3m / s

a = -1m / s2

Faero = 0.934 * v2

Froll = 720.054N

Fgred = 0

Fa = -367N

FT = 0.934 * v2 + 353.054N

PT = 0.934 * v3 + 353.054v W

FTmin = 361.46N

PTmin = 1084.38N

FTmax = 412.83N

PTmax = 3302.64W

Total time = 7sec

L = 27.5m

- Downhill

The velocity of the buggy is 3m / s. The acceleration is 0m / s2. Because the speed is a fixed value, the aerodynamic resistance = 0.934 * v2 = 0.934 * 32 =8.406N. The rolling resistance =μ * m * g * cosα = 0.2 * 367 * 9.81 * cos10° = 709.25319N. For the angle is 10°, the climbing resistance = m * g * sinα = 367 * 9.81 * -sin10° = -626.44698N. And the acceleration is 0, so the accelerate resistance is 0. The total force = 8.406N + 709.25319N - 626.44698N = 91.21221N. The total power = the total force * v = 91.21221 * 3 = 273.63663W. The total time of this section = 19sec. The distance of this section = 57m.

v = 3m / s

a = 0m / s2

Faero = 8.406N

Froll = 709.25319N

Fgred = -626.22698N

Fa = 0

FT = 91.21221N

PT = 273.63663W

Total time = 19sec

L = 57m

Section 12 : continuous uphill

- Uniform acceleration linear motion 1

The velocity of the buggy will increase from 3 to 8m / s. The acceleration will be 1m / s2. Because the speed is not a fixed value, the aerodynamic resistance = 0.934 * v2. The rolling resistance =μ * m * g = 0.2 * 367 * 9.81 = 720.054N. For there is no angle, the climbing resistance is 0. And the acceleration is 1m / s, so the accelerate resistance = m * a = 367 * 1 = 367N. The total force = 0.934 * v2 + 720.054N + 367N = 0.934 * v2 + 1087.054N. The total power = the total force * v = 0.934 * v3 + 1087.054v W. The total time of this section = vf - vs / a = (8-3) / 1 = 5sec. The distance of this section = v2f – v2s / 2a = 82 – 32 / 2 * 1 = 27.5m.

v = 3-8m / s

a = 1m / s2

Faero = 0.934 * v2

Froll = 720.054N

Fgred = 0

Fa = 367N

FT = 0.934 * v2 + 1087.054N

PT = 0.934 * v3 + 1087.054v W

FTmin = 1095.464N

PTmin = 3286.38N

FTmax = 1146.83N

PTmax = 9174.64W

Total time = 5sec

L = 27.5m

- Uniform deceleration uphill

The velocity of the buggy will decrease from 8 to 1m / s. The acceleration will be -1m / s2. Because the speed is not a fixed value, the aerodynamic resistance = 0.934 * v2. The rolling resistance =μ * m * g * cosα = 0.2 * 367 * 9.81 * cos10° = 709.25319N. For the angle is 10°, the climbing resistance = m * g * sinα = 367 * 9.81 * sin10° = 626.44698N. And the acceleration is 1m / s, so the accelerate resistance = m * a = 367 * -1 = -367N. The total force = 0.934 * v2 + 709.25319N + 626.44698N - 367N = 0.934 * v2 + 968.70017N. The total power = the total force * v = 0.934 * v3 + 968.70017v W. The total time of this section = vf - vs / a = (1-8) / -1 = 7sec. The distance of this section = v2f – v2s / 2a = 12 – 82 / 2 * (-1) = 31.5m.

v = 8-1m / s

a = -1m / s2

Faero = 0.934 * v2

Froll = 709.25319N

Fgred = 626.44698N

Fa = -367N

FT = 0.934 * v2 + 968.70017N

PT = 0.934 * v3 + 968.70017v W

FTmin = 969.63417N

PTmin = 969.63417N

FTmax = 1028.47617N

PTmax = 8227.80936W

Total time = 7sec

L = 31.5m

- Uniform acceleration linear motion 2

The velocity of the buggy will increase from 1 to 9m / s. The acceleration will be 1m / s2. Because the speed is not a fixed value, the aerodynamic resistance = 0.934 * v2. The rolling resistance =μ * m * g = 0.2 * 367 * 9.81 = 720.054N. For there is no angle, the climbing resistance is 0. And the acceleration is 1m / s, so the accelerate resistance = m * a = 367 * 1 = 367N. The total force = 0.934 * v2 + 720.054N + 367N = 0.934 * v2 + 1087.054N. The total power = the total force * v = 0.934 * v3 + 1087.054v W. The total time of this section = vf - vs / a = (9-1) / 1 = 8sec. The distance of this section = v2f – v2s / 2a = 92 – 12 / 2 * 1 = 40m.

v = 1-9m / s

a = 1m / s2

Faero = 0.934 * v2

Froll = 720.054N

Fgred = 0

Fa = 367N

FT = 0.934 * v2 + 1087.054N

PT = 0.934 * v3 + 1087.054v W

FTmin = 1087.988N

PTmin = 1087.988N

FTmax = 1162.708N

PTmax = 10464.372W

Total time = 8sec

L = 40m

- Uphill

The velocity of the buggy is 9m / s. The acceleration is 0m / s2. Because the speed is a fixed value, the aerodynamic resistance = 0.934 * v2 = 0.934 * 92 = 75.654N. The rolling resistance =μ * m * g * cosα = 0.2 * 367 * 9.81 * cos10° = 709.25319N. For the angle is 10°, the climbing resistance = m * g * sinα = 367 * 9.81 * sin10° = 626.44698N. And the acceleration is 0, so the accelerate resistance is 0. The total force = 75.654N + 709.25319N + 626.44698N = 1411.35417N. The total power = the total force * v = 1411.35417 * 9 = 12702.1875W. The total time of this section = 5sec. The distance of this section = 40m.

v = 8m / s

a = 0m / s2

Faero = 75.654N

Froll = 709.25319N

Fgred = 626.22698N

Fa = 0

FT = 1411.35417N

PT = 12702.1875W

Total time = 9sec

L = 81m

Section 13 : straight

- Uniform deceleration linear motion

The velocity of the buggy will decrease from 9 to 8m / s. The acceleration will be -0.5m / s2. Because the speed is not a fixed value, the aerodynamic resistance = 0.934 * v2. The rolling resistance =μ * m * g = 0.2 * 367 * 9.81 = 720.054N. For there is no angle, the climbing resistance is 0. And the acceleration is 1m / s, so the accelerate resistance = m * a = 367 * (-0.5) = -183.5N. The total force = 0.934 * v2 + 720.054N – 183.5N = 0.934 * v2 + 536.554N. The total power = the total force * v = 0.934 * v3 + 536.554v W. The total time of this section = vf - vs / a = (8-9) / (-0.5) = 2sec. The distance of this section = v2f – v2s / 2a = 82 – 92 / 2 * (-0.5) = 17m.

v = 9-8m / s

a = -0.5m / s2

Faero = 0.934 * v2

Froll = 720.054N

Fgred = 0

Fa = -183.5N

FT = 0.934 * v2 + 536.554N

PT = 0.934 * v3 + 536.554v W

FTmin = 596.33N

PTmin = 4770.64W

FTmax = 612.208N

PTmax = 5509.872W

Total time = 2sec

L = 17m

- Uniform linear motion

The analysis is the same as section 3, but the total time = 4sec. And the distance of this section = 32m.

Section 14 : the buggy decelerates uniformly before it enters the corner

The velocity of the buggy will decrease from 9 to 4m / s. The acceleration will be -1m / s2. Because the speed is not a fixed value, the aerodynamic resistance = 0.934 * v2. The rolling resistance =μ * m * g = 0.2 * 367 * 9.81 = 720.054N. For there is no angle, the climbing resistance is 0. And the acceleration is 1m / s, so the accelerate resistance = m * a = 367 * (-1) = -367N. The total force = 0.934 * v2 + 720.054N - 367N = 0.934 * v2 + 353.054N. The total power = the total force * v = 0.934 * v3 + 353.054v W. The total time of this section = vf - vs / a = (4-9) / (-1)= 5sec. The distance of this section = v2f – v2s / 2a = 42 – 92 / 2 * (-1) = 32.5m.

v = 9-4m / s

a = -1m / s2

Faero = 0.934 * v2

Froll = 720.054N

Fgred = 0

Fa = -367N

FT = 0.934 * v2 + 353.054N

PT = 0.934 * v3 + 353.054v W

FTmin = 367.998N

PTmin = 1471.992W

FTmax = 428.708N

PTmax = 3858.372W

Total time = 5sec

L = 32.5m

Section 15 is the same as the section 10.

Section 16 : straight

- Uniform linear motion

The velocity of the buggy is 4m / s. The acceleration is 0m / s2. Because the speed is a fixed value, the aerodynamic resistance = 0.934 * v2 = 0.934 * 42 = 14.944N. The rolling resistance =μ * m * g = 0.2 * 367 * 9.81 = 720.054N. For there is no angle, the climbing resistance is 0. And the acceleration is 0, so the accelerate resistance is 0. The total force = 14.944N + 720.054N = 734.988N. The total power = the total force * v = 734.988 * 4 = 2939.952W. The total time of this section = 1sec. The distance of this section = 4m.

v = 4m / s

a = 0m / s2

Faero = 14.944N

Froll = 720.054N

Fgred = 0

Fa = 0

FT = 734.988N

PT = 2939.952W

Total time = 1sec

L = 4m

(2)Uniform deceleration linear motion to the end

The velocity of the buggy will decrease from 4 to 0m / s. The acceleration will be -1m / s2. Because the speed is not a fixed value, the aerodynamic resistance = 0.934 * v2. The rolling resistance =μ * m * g = 0.2 * 367 * 9.81 = 720.054N. For there is no angle, the climbing resistance is 0. And the acceleration is 1m / s, so the accelerate resistance = m * a = 367 * (-1) = -367N. The total force = 0.934 * v2 + 720.054N – 367N = 0.934 * v2 + 353.054N. The total power = the total force * v = 0.934 * v3 + 353.054v W. The total time of this section = vf - vs / a = (0-4) / (-1) = 4sec. The distance of this section = v2f – v2s / 2a = 02 – 42 / 2 * (-1) = 8m.

v = 4-0m / s

a = -1m / s2

Faero = 0.934 * v2

Froll = 720.054N

Fgred = 0

Fa = -367N

FT = 0.934 * v2 + 353.054N

PT = 0.934 * v3 + 353.054v W

FTmin = 353.054N

PTmin = 0

FTmax = 367.988N

PTmax = 1471.992W

Total time = 4sec

L = 8m

# Results

That is the total duty cycle analysis. From the analysis, we can know that :

Total time : 167sec

Total length : 944m

Total climb : 240m

Total decline : 240m

Total straight : 424m

Total corner : 40m

The maximum power : 12702.1875W

The average power : 4506.011055W

Because all the data in Word is calculated by hand, several data may be different from excel spread sheet.

# References

- ZONG Changfu, GUO Konghui, LI Bo. Theoretical prediction and comprehensive evaluation for vehicle handling stability [J]. Automobile Engineering, 2001, 23(1):5.

- ZHAO Youqun, GUO Konghui. A research on active safety evaluation of vehicle [J]. Automobile Engineering, 2001, 23(1):1.

- YU Zhisheng. Automobile theory [M]. 3rd ed. Beijing: China Machine Press, 1998.

- YU Zhisheng. Automobile theory [M]. 5th ed. Beijing: China Machine Press, 2009.

- ZHENG Anwen, Song Jian, WU Kaihui. Mechaninal parameter and steering characteristics of vehicle [J]. Automobile Engineering, 1999, 21(5):269.