APPENDIX GA36b

#### LEVEL 5 ASSESSMENT SPECIFICATION

| Student name: |  | Student ID number: |  |
| --- | --- | --- | --- |
| Programme: | HND / BSc (Hons) Electrical and Electronic Engineering |  |  |
| Module: | POWER AND MACHINES |  |  |
| Module code: | SGEE5006 | ## Contribution to Overall Module Assessment (%): | 50% |
| Lecturer: | Robert Davies | Internal Verifier: | Michael Hall |
| Assignment Title: | Single-Phase Transformers and Three-Phase Induction Motors | Word count (or equivalent): | 2500 |
| Submission deadline: | 22nd December 2022 | Return date of provisional marks & written feedback: | 22nd January 2023 |
| Submission method: | All written assessments, where practical and possible, must be submitted via Turnitin unless otherwise instructed by the Lecturer. (Please DO NOT put this assessment specification into Turnitin or it will match many similarities with other students’ submissions.) Alternative submission method (if applicable): Late submission of the assessment will result in a late penalty mark. Penalties for late submission: Up to one week late, maximum mark of 40%. Over one week late, 0%. Only the Extenuating Circumstances Panel may approve a change to submission dates. |  |  |
| Academic honesty / referencing: | Academic honesty is required. In the main body of your submission you must give credit to authors on whose research and ideas your work is based. Append to your submission a reference list that indicates the books, articles, etc. that you have used, cited or quoted in order to complete this assessment. |  |  |

Module Learning Outcomes (from module syllabus)

- Demonstrate a systematic understanding of the theory and operating principles of electrical machines, electronic power converters and basic power systems. - Analyse the performance of an electrical machine (or power system device) and an electronic power converter through practical experiment, comparing theoretical predictions with actual measured values.

TASK DESCRIPTION

The aims of this assignment are to investigate the performance of AC power system components.

Task 1 - Work Plan for Lifelong Learning and Risk Assessment

Write a short plan of how you will approach this assignment. When the assignment is complete, record your learning in this section. Produce a brief risk assessment as a table and place in the appendix of the report.

Task 2 – Research

Carry out individual research into the following areas using credible and appropriate sources:

- Four induction machine speed control methods using power electronic converters and the most suited industrial application for each.

- Power Systems required to deliver a three-phase AC supply to a typical factory where induction machines are employed and the most suitable supply voltage for induction motor applications.

Critically evaluate the information, ensure its relevance and organise in a logical format. You must write in a succinct way and avoid repetition. Use a good range of sources, insert in-text citations and list your sources using the Harvard method. See further guidance later in this brief. You must summarise your research and produce some conclusions.

Task 3 – Single-Phase Transformer

- For the given single-phase transformer, use suitable measuring equipment to carry out open and short circuit tests to determine the approximate equivalent circuit parameters of the transformer (observing health and safety precautions). Provide an explanation of your experimental setup and explain how you worked safely in a group to achieve the results. Note that the practical results can be shared in the group but all analysis must be done individually.

- Use this circuit to predict the voltage regulation and efficiency of the transformer under suitable loads. As a group, carry out load tests and record results.

- Using simulation software such as Multisim, draw a schematic diagram model of the equivalent circuit of the transformer. Perform the open and short circuit tests on the model using voltmeters, ammeters and watt-meters to prove your equivalent circuit is correct. Apply similar loads as for part (b) and measure voltage regulation and efficiency, and compare with calculated values.

Task 4 – Three-Phase Induction Motor

- For the given three-phase induction motor, use suitable measuring equipment to carry out stator resistance, no-load and blocked-rotor tests (observing health and safety precautions), to determine the approximate per-phase equivalent circuit parameters for the motor. Provide an explanation of your experimental setup and explain how you worked safely in a group to achieve the results. Note that the practical results can be shared in the group but all analysis must be done individually.

- From the circuit parameter, produce a spreadsheet that simulates the performance of the motor in terms of torque and stator current over a range of speeds showing the main regions of operation including motoring, generating and braking.

- Demonstrate the usefulness of the equivalent circuit as a means of predicting the performance of the motor by testing the motor under load for motoring and by driving it faster than synchronous speed using a VSD for generating. Compare predicted and actual values.

- Connect a 4-pole induction motor to an inverter and a Prony brake. Taking all necessary health and safety precautions, measure stator voltage, stator frequency and rotor speed under a small friction load for a range of speeds demonstrating the inverter’s performance. Compare with expected values from theory obtained by research.

Task 5 – Overall Conclusions and Recommendations

Critically analyse all that has been learnt in this assignment through research, simulation and practical investigation. How do the simulation and practical results compare with research and theory? What have you gained from this assignment? Produce some overall conclusions and recommendations of what else you would like to do to further your understanding of this subject.

Task 6 – Team Working

Write a full explanation of how working as a team has enabled you all to produce a good outcome and give a clear indication of your inclusive approach to engineering practice and the responsibilities, benefits and importance of supporting equality, diversity and inclusion when working as a team.

Task 7 – Citation List

Using the built-in reference system within MS Word or equivalent, provide a citation list of all your sources using the Harvard method. The larger and wider the range of sources, the higher will be your mark. Ensure you have correctly cited from this list in your report. If you wish to include a bibliography, make sure it does not include any sources already listed.

Task 8 – Appendix

Your risk assessment and most of your test results, images and calculations should be placed in the appendix to avoid cluttering up the main the report. Remember that the main report should be readable with well organised paragraphs and references should be made to material in the appendix.

Task 9 – Executive Summary

Write an executive summary of all that has been done in this assignment. It must be written in the past tense but placed at the beginning of the report. It must summarise all that was done and MUST NOT be simply a table of contents written as a paragraph. Be specific! What exactly did you do? Use no more than 300 words and do not include in your word count.

GUIDANCE FOR STUDENTS IN THE COMPLETION OF TASKS

You are required to present an electronic report containing detailed observational notes of your work. The report must contain 2500 words and should be preceded by a title page, table of contents and list of figures (which are not part of the word count):

- Executive summary

- Work plan for Life-long Learning

- Research

- Transformer Investigation

- Three-phase Induction Motor Investigation

- Conclusions and Recommendations

- Team Working

- Citation List

- Appendix

Your title page for the report should contain the following information:

Course:			HND / BSc (Hons) Electrical and Electronic Engineering

Module:			SGEE5006 Power and Machines

Assignment: 		Single-Phase Transformers and Three-Phase Induction motors

Name:			?????? ??????

University number:	XXXXXXX

Date submitted:		DD/MM/YYYY

Word count:		?????

Further guidance:

- Do not use a styled document (template) and don’t add your company logo.

- Do not include photographs of course notes or formative classroom work.

- All pages and figures must be numbered and listed in the Table of Contents and List of Figures.

- DO NOT include datasheets; simply quote from them and cite their source.

- Write all sections in the third person.

- Note which sections of your report have been identified above as places where the marks will be awarded and use your word allocation wisely.

- Save your report as a .pdf with your university number as the file name.

- The TurnIt-In rubric will be used to record your marks and provide you with tutor feedback.

- Use Arial font size 11pt, with full justification of paragraphs and 1.5 line spacing with pages numbering set to bottom centred. This all makes your report easier to mark and allows your tutor room to add comments. It will also incur more marks for transferrable skills.

- To reduce your word count, avoid telling a story of how you carried out your work, simply focus on the end product and explain it. This is not a journal!

Going Over the Word Limit

If you are found to have exceeded the specified word limit for an assessment, your work may be subject to a penalty:

- Under length		- No penalty, but your work is probably missing key information

- Up to 10% over length	- No deduction off final mark

- 11% or more over length	- Deduction of 5 marks off final mark (but cannot reduce mark below 40%)

To measure the word count, DO NOT use Turnit-In and do not include the following:

- Executive Summary

- Life-long Learning Plan

- Risk Assessment

- Assignment brief

- Index, Table of Contents and Table of Figures

- Formula and calculations (use MSEquation or text boxes to contain these)

- Reference list or bibliography

- Appendices used for: large results tables, computer code listings, data sheet extracts and any other 3rd party information

Spot checking of declared word counts will be carried out by your lecturer and second marker.

NOTE: The guidance offered below is linked to the five generic assessment criteria overleaf. Significant differences from Level 4 are highlighted in red.

- Engagement with Literature Skills

Your work must be informed and supported by scholarly material that is relevant to and focused on the task(s) set. You should provide evidence that you have accessed an appropriate range of sources, which may be academic, governmental and industrial; these sources may include academic journal articles, textbooks, current news articles, organisational documents, and websites.  You should consider the credibility of your sources; academic journals are normally highly credible sources while websites require careful consideration/selection and should be used sparingly.   Any sources you use should be current and up-to-date, mostly published within the last five years or so, though seminal/important works in the field may be older.  You must provide evidence of your research/own reading throughout your work, using a suitable referencing system, including in-text citations in the main body of your work and a reference list at the end of your work.

Guidance specific to this assessment: This will be assessed by the extent of your reading, relevance of credible sources and the Harvard method used.

- Knowledge and Understanding Skills

At level 5, you should be able to demonstrate: sound knowledge and critical understanding of the well-established concepts and principles of the subject area and the way in which those principles have developed; knowledge of the main methods of enquiry in the discipline.  Knowledge relates to the facts, information and skills you have acquired through your learning.  You demonstrate your understanding by interpreting the meaning of the facts and information (knowledge). This means that you need to select and include in your work the concepts, techniques, models, theories, etc. appropriate to the task(s) set.  You should be able to explain the theories, concepts, etc. meaningfully to show your understanding.  Your mark/grade will also depend upon the extent to which you demonstrate your knowledge and understanding.

Guidance specific to this assessment: This will be assessed by the knowledge displayed in your research section and your explanations of the simulation.

- Cognitive and Intellectual Skills

You should be able to critically analyse information, and propose solutions to problems arising from that analysis, including the critical evaluation of the appropriateness of different approaches to solving problems. Your work must contain evidence of logical, analytical thinking, evaluation and synthesis. For example, to examine and break information down into parts, make inferences, compile, compare and contrast information.  This means not just describing what! But also justifying: Why? How? When? Who? Where? At what cost? At all times, you must provide justification/evidence for your arguments and judgements.  Evidence that you have reflected upon the ideas of others within the subject area is crucial to you providing a reasoned and informed debate within your work.  Furthermore, you should provide evidence that you are able to make sound judgements and convincing arguments using data and concepts, with an understanding of the limits of knowledge, and how this influences analyses and interpretations. Sound, valid conclusions are necessary and must be derived from the content of your work.   Where relevant, alternative solutions and recommendations may be proposed.

Guidance specific to this assessment: This will be assessed by your calculations and your analysis of simulation results together with your conclusions and recommendations.

- Practical Skills

At level 5, you should be able to use/deploy a range of established techniques within the discipline, and apply underlying concepts and principles outside the context in which they were first studied, including, where appropriate, the application of those principles in an employment context. You should be able to demonstrate how the subject-related concepts and ideas relate to real world situations and/or a particular context.  How do they work in practice?  You will deploy models, methods, techniques, and/or theories, in that context, to assess current situations, perhaps to formulate plans or solutions to solve problems, or to create artefacts, some of which may be creative.  This is likely to involve, for instance, the use of real world artefacts, examples and cases, the application of a model within an organisation and/or benchmarking one theory or organisation against others based on stated criteria.  You should show awareness of the limitations of concepts and theories when applied in particular contexts.

Guidance specific to this assessment: This will be assessed by your use of equipment and the spreadsheet in the simulation section and the quality of the presentation of your test results.

- Transferable Skills for Life and Professional Practice

Your work must provide evidence of the qualities and transferable skills necessary for employment requiring the exercise of personal responsibility and decision-making.  This includes demonstrating: professional development to advance existing skills and acquire new competences that will enable you to assume significant responsibility within organisations; that you can initiate and complete tasks and procedures, whether individually and/or collaboratively; that you can use appropriate media to effectively communicate information, arguments and analysis in a variety of forms to specialist and non-specialist audiences; fluency of expression; clarity and effectiveness in presentation and organisation. Work should be coherent and well-structured in presentation and organisation.

Guidance specific to this assessment: This will be assessed by the written and presentational quality of your report including page numbering, figure quality, figure numbering and listing, writing style, punctuation, grammar and spelling and your word count will also be included here.

Achieving the PLOs for IET Accreditation at AHEP 4

The following PLOs have been targeted to be satisfied by this assignment. Details are given below. The marking scheme on the next page will also be used to record your PLO achievements and comments will be found in the Turnit-in rubric.

C3 - Analytical tools and techniques:

Select and apply appropriate computational and analytical techniques to model complex problems, recognising the limitations of the techniques employed.

This will be achieved by your analysis of results and induction motor modelling using the spreadsheet.

C4 - Technical literature:

Select and evaluate technical literature and other sources of information to address complex problems.

This will be achieved by your research and citation list.

C9 - Risk

Use a risk management process to identify, evaluate and mitigate risks (the effects of uncertainty) associated with a particular project or activity.

This will be achieved by your risk assessment found in the appendix.

C11 - Equality, diversity and inclusion

Adopt an inclusive approach to engineering practice and recognise the responsibilities, benefits and importance of supporting equality, diversity and inclusion.

This will be achieved by your explanation of team working.

C12 - Practical and workshop skills

Use practical laboratory and workshop skills to investigate complex problems.

This will be achieved by your testing evidence

C16 - Teamwork

Function effectively as an individual, and as a member or leader of a team.

This will be achieved by your explanation of team working.

C17 - Communication

Communicate effectively on complex engineering matters with technical and non-technical audiences.

This will be achieved by your overall report and executive summary.

C18 - Lifelong learning

Plan and record self-learning and development as the foundation for lifelong learning/CPD.

This will be achieved by your work plan.

STUDENT FEEDBACK FORM

This section details the extent to which the assessment criteria are demonstrated by you, which in turn determines your mark. The marks available for each category of skill are shown. Lecturers will use the space provided to comment on the achievement of the task(s), including those areas in which you have performed well and areas that would benefit from development/improvement.

| Generic Assessment Criteria | Marks available |
| --- | --- |
| 1. Engagement with Literature Skills |  |
| Report sections 3 and 8 | 20 |
| 2. Knowledge and Understanding Skills |  |
| Report sections 3, 4 and 5 | 20 |
| 3. Cognitive and Intellectual Skills |  |
| Report sections 4, 5 and 6 | 20 |
| 4. Practical Application Skills |  |
| Report sections 4 and 5 | 20 |
| 5. Transferable Skills for Life and Professional Practice |  |
| All report sections, but particularly sections 1, 2, 7 and word count penalty. | 20 |

| Assessment Mark (Assessment marks are subject to ratification at the Exam Board. These comments and marks are to give feedback on module work and are for guidance only until they are confirmed. ) | Late Submission Penalties (tick if appropriate) |  | % |
| --- | --- | --- | --- |
|  | Up to 1 week late (40% Max) |  |  |
|  | Over 1 week late (0%) |  |  |

GENERIC ASSESSMENT CRITERIA

Level 5

In accordance with the Framework for Higher Education Qualifications, at the end of Level 5 students will be expected to have developed sound knowledge and critical understanding of the well-established concepts and principles in their field of study, and of the way in which those principles have developed. They will have learned to apply those concepts and principles more widely outside the context in which they were first studied, including, where appropriate, the application of those principles in an employment context. They will have knowledge of the main methods of enquiry in the subject area, and ability to evaluate critically different approaches to problem solving. They will possess an understanding of the limits of their knowledge, and how this influences their analyses and interpretations. They will be able to use a range of established techniques to initiate and undertake critical analysis of information, and to propose solutions to problems arising from that analysis. They will be able to effectively communicate information, arguments and analysis in a variety of forms to specialist and non-specialist audiences, and deploy key techniques of the discipline effectively. They will be able to undertake further training, develop existing skills and acquire new competences that will enable them to assume significant responsibility within organisations. They will have the qualities and transferable skills necessary for employment requiring the exercise of personal responsibility and decision-making.

| Level 5 | FAIL | MARGINAL FAIL | SATISFACTORY (3rd / Pass) | GOOD (2.2 / Pass) | VERY GOOD (2.1 / Merit) | EXCELLENT (1st / Distinction) | EXCEPTIONAL (1st / Distinction) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Category | 0-29% | 30-39% | 40-49% | 50-59% | 60-69% | 70-84% | 85-100% |
| 1. Engagement with literature (including reading, referencing, academic conventions and academic honesty) | Little or no evidence of reading and/or reliance on inappropriate sources. Views and findings mostly unsupported and non-authoritative. Referencing conventions used incoherently or largely absent. | Poor engagement with essential reading. No evidence of wider reading. Reliance on inappropriate sources, and/or indiscriminate use of sources. Heavily reliant on information gained through class contact. Inconsistent and weak use of referencing. | Engagement with a limited range of mostly relevant and credible sources. Some omissions and minor errors. Referencing conventions evident though not always applied accurately or consistently. | Engagement with an appropriate range of literature, including sources retrieved independently. Some over-reliance on texts. Referencing may show minor inaccuracies or inconsistencies. | Engagement with a wide range of literature, including sources retrieved independently. Selection of relevant and credible sources. Very good use of referencing, with no/very few inaccuracies or inconsistencies. | Engagement with an extensive range of relevant and credible literature. Consistently accurate application of referencing. | Exceptional engagement with an extensive range of relevant and credible literature. High-level referencing skills consistently applied. |
| 2. Knowledge and understanding (Sound knowledge and critical understanding of the well-established concepts and principles in their field of study; knowledge of the main methods of enquiry in the discipline.) | Major gaps in knowledge with unsatisfactory, uncritical understanding of the subject matter. Much irrelevant material. Substantial inaccuracies. Significantly flawed understanding of the main methods of enquiry in the discipline. | Fragmentary knowledge, with only superficial critical understanding. Some significant inaccuracies and/or irrelevant material. Incomplete or partially flawed understanding of the main methods of enquiry in the discipline. | Limited but adequate knowledge and critical understanding of the well-established concepts and principles within the subject area, with a few gaps in the selection of material. A narrow critical understanding of the main methods of enquiry. | Knowledge is reasonably detailed and accurate. A good critical understanding of the well-established concepts and principles and the main methods of enquiry, with minor gaps in the selection of material. | Knowledge is reasonably extensive. Exhibits very competent critical understanding of the well-established concepts and principles of the subject and the main methods of enquiry. Breadth and depth of knowledge. | Excellent, detailed knowledge and highly critical understanding of the well-established concepts and principles of the subject and the main methods of enquiry. | Exceptionally detailed knowledge and outstanding critical understanding of the well-established concepts and principles of the subject and the main methods of enquiry. May go beyond established theories. |
| 3. Cognitive and intellectual skills (Critical evaluation and analysis of concepts and principles; argument and judgement; the limits of their knowledge, and how this influences their analyses and interpretations.) | Wholly or almost wholly descriptive work. Little or no critical evaluation and analysis of concepts and principles. Failure to develop arguments, leading to illogical or invalid judgements. Unsubstantiated generalisations or opinion, made without use of any credible evidence. | Largely descriptive work, with superficial use of critical evaluation and analysis of concepts and principles. Weak development of arguments and judgements. Information accepted uncritically, uses generalised statements made with scant evidence and unsubstantiated opinions. Ideas sometimes illogical and contradictory. | Limited attempt at critical evaluation and analysis of concepts and principles, tending towards description. Some evidence to support arguments and judgements but these may be underdeveloped, with a little inconsistency / mis-interpretation or failure to fully recognise limits of knowledge. | Some critical evaluation and analysis of concepts and principles, though descriptive in parts. An emerging awareness the limits of their knowledge and ability to use evidence to support the argument though with some tendency to assert/state opinion rather than argue on the basis of reason and evidence. Mostly valid arguments and logical judgements. | Sound critical evaluation and analysis of concepts. Is selective in the range of evidence used and synthesises rather than describes. Ability to devise arguments that show awareness of different stances, and use evidence convincingly, to support appropriate and valid judgements. | Excellent critical evaluation and analysis of concepts and principles leading to logical, evidence-based, reasoned arguments and judgements. Explicit recognition of other stances and a strong awareness of the limits of their knowledge. A capacity for independent thought and ability to ‘see beyond the question’, suggesting some grasp of the broader field and wider concepts. | Outstanding critical evaluation and analysis of concepts and principles. Uses evidence exceptionally well to connect ideas, and support highly logical and persuasive, arguments and judgements. Evidence of independent thought and ability to ‘see beyond the question’, suggesting a clear grasp of the broader field and wider concepts. Perceptive recognition of the limits of their knowledge, and how this influences their analyses and interpretations. |
| 4. Practical skills (Apply underlying concepts and principles more widely outside the context in which they were first studied; use a range of established techniques; propose solutions to problems arising from analysis.) | Limited or no use of established methods, materials, tools and/or techniques. Little or no appreciation of the context of the application. Limited understanding of the application of theory to practice or making appropriate links between the two. Very weak problem-solving skills outside the context in which they were first studied. | Rudimentary application of established methods, materials, tools and/or techniques but without consideration and competence. Flawed appreciation of the context of the application. Weak understanding of the application of theory to practice, with only occasional evidence of making appropriate links between the two. Weak problem-solving skills outside the context in which they were first studied. | An adequate awareness and mostly appropriate application of established methods, materials, tools and/or techniques. Basic appreciation of the context of the application. Theoretical knowledge and understanding applied in practice, but not always making logical links between the two. Can identify problems and propose basic solutions outside the context in which they were first studied. | A good and appropriate application of established methods, materials, tools and/or techniques. Clear appreciation of the context of the application. Mainly consistent, accurate and logical application of theory to practice, making appropriate links between the two. Can identify problems and propose mostly appropriate solutions outside the context in which they were first studied. | A very good application of a range of established methods, materials, tools and/or techniques. Very good consideration of the context of the application, with perceptive insights. Consistent, accurate and logical application of theory to practice, making appropriate links between the two. Can identify problems and propose appropriate solutions outside the context in which they were first studied. Evidence of some creativity. | An advanced application of a range of established methods, materials, tools and/or techniques. The context of the application is well considered, and insightful. Consistent, accurate and logical application of theory to practice, making well-developed links between the two. Can identify problems and propose excellent, creative solutions outside the context in which they were first studied. | Exceptional levels of application and deployment skills using established methods, materials, tools and/or techniques. Consistent, accurate and logical application of theory to practice, making highly developed links between the two. Can identify routine and non-routine problems and propose quite sophisticated, creative solutions outside the context in which they were first studied. |
| 5. Transferable skills for life and professional practice (Effectively communicate in a variety of forms to specialist and non-specialist audiences; the qualities and transferable skills necessary for employment requiring the exercise of personal responsibility and decision-making.) | Communication medium is inappropriate or misapplied. Work is poorly structured, disorganised and/or confusingly expressed. Very weak use of language and/or very inappropriate style. Failure to work effectively as part of a group. Little or no evidence of the skills for employment requiring the exercise of personal responsibility and decision-making. | Communication medium is poorly designed and/or not suitable for the audience. Work is poorly presented in a disjointed manner. It is loosely, and at times incoherently, structured, with information and ideas often poorly expressed. Weak use of language and/or inappropriate style. Flawed approach to group work, meeting only partial obligations to others. Limited evidence of the skills for employment requiring the exercise of personal responsibility and decision-making. | Can communicate in a suitable medium for the audience but with some room for improvement. Mostly ordered presentation and structure in which relevant ideas / concepts are reasonably expressed. Work may lack coherence in places. Can work as part of a group, meeting most obligations to others but perhaps with limited involvement in group activities. Demonstrates the basic skills for employment requiring the exercise of personal responsibility and decision-making, with some areas of minor weakness. | Can communicate effectively in a suitable medium for the audience, but may have minor errors. Mostly coherent, organised work, in a suitable structure and is for the most part clearly expressed. Can work effectively independently and/or as part of a team, with clear contribution to group activities. Demonstrates the skills for employment requiring the exercise of personal responsibility and decision-making, with some areas of strength and some of minor weakness. | Can communicate well, confidently and consistently in a suitable medium for the audience. Work is coherent, fluent, well-structured and organised. Can work very well autonomously and/or as part of a team, with very good contribution to group activities. Demonstrates very good skills for employment requiring the exercise of personal responsibility and decision-making, with just occasional minor weakness. | Can communicate professionally confidently and consistently in a suitable medium for the audience. Work is coherent, very fluent and is presented proficiently. Can work autonomously with initiative. Where relevant can work professionally within a team, showing leadership skills as appropriate, and meeting obligations. Demonstrates excellent skills for employment requiring the exercise of personal responsibility and decision-making and an appetite for further development. | Can communicate with an exceptionally high level of professionalism, highly suitable for the audience. Work is exceptionally coherent, very fluent and is presented professionally. Can work exceptionally well within a team, showing leadership skills. Demonstrates exceptional skills for employment requiring the exercise of personal responsibility and decision-making and an appetite for further development. |

THIS DOCUMENT IS ALSO AVAILABLE IN WELSH