Problem 1 (No friction)

A 2 Kg box is put on the surface of an inclined plane at 27 ° with the horizontal. The surface of the inclined plane is assumed to be frictionless.

a) Draw a free body diagram of the box on the inclined plane and label all forces acting on the box.

b) Determine the magnitude of the force exerted by the inclined plane on the box.

c) Determine the force down the slope.

Problem 2 A particle of mass 5 Kg rests on a 30° inclined plane with the horizontal. A force Fa of magnitude 30 N acts on the particle in the direction parallel and up the inclined plane. a) Draw a Free Body Diagram including the particle, the inclined plane and all forces acting on the particle with their labels. b) Find the force of friction acting on the particle. c) Find the normal force exerted by the inclined on the particle.

Problem 3

A box of mass M = 10 Kg rests on a 35° inclined plane with the horizontal. A string is used to keep the box in equilibrium. The string makes an angle of 25 ° with the inclined plane. The coefficient of friction between the box and the inclined plane is 0.3.

a) Draw a Free Body Diagram including all forces acting on the particle with their labels.

b) Find the magnitude of the tension T in the string.

c) Find the magnitude of the force of friction acting on the particle.

Problem 4

A 100 Kg box is to be lowered at constant speed down an inclined plane 4 meters long from the back of a lorry 2 meters above the ground. The coefficient of kinetic friction is equal to 0.45. What is the magnitude of the force Fa to be applied parallel to the inclined plane to hold back the box so that it is lowered at constant speed?

Problem 5

A box of mass M = 7 Kg is held at rest on a 25° inclined plane by force Fa acting horizontally as shown in the figure below. The box is on the point of sliding down the inclined plane. The static coefficient of friction between the box and the inclined plane is ?s = 0.3. Find the magnitude of force Fa.