L4 – Chemistry (OME Degree – Sem 2) ACDB4011

Calorimetry Lab Report

Charlotte McCombe

Student Number: 2411524

Contents

Introduction	3

Results:	4

Experiment 1: Heat capacity of the calorimeter	4

Experiment 2: Heat capacity of a metal.	5

Experiment 3:	7

Discussion	8

Significance of results	8

Relevance to OME and future work	8

Limitations or potential sources of error in the experiment.	9

Conclusion	9

Bibliography	10

Appendix	11

Introduction

The aim of the experiment was to measure the specific heat capacity of aluminium metal and the molar heat of a reaction of hydrochloric acid and sodium hydroxide. The experiment showed how energy transfer occurs between two variables of different energies and temperatures. The specific heat capacity can be defined as the quantity of heat required to raise the temperature of unit mass, in this case 1g of metal and 1 mole of a substance by one degree [1]. The hypothesis for these experiments was that heat transfer can be measured using a calorimeter in a closed system. Within the experiments the independent variable was the the substances. The dependant variable was the temperature and time, and the controlled variables was the calorimeter.

# Results:

Within the three experiments it was assumed that no energy was transferred to surroundings and all energy remained in the reactions as an isolated system.

## Experiment 1: Heat capacity of the calorimeter

Higher temperature water mixed with a lower temperature:

|  | Initial Temp (°C) | Mass (KG) | Change in Temperature (°C) |
| --- | --- | --- | --- |
| Hot Water | 60.0 | 0.050 | -13.4 |
| Cold Water | 24.9 | 0.050 | 21.7 |
| Calorimeter | 24.9 | --------------------------- | 13.4 |

Maximum Temperature = 46.6°C

Specific heat capacity of hot and cold waters:

Q=mh×cw×△Th

Q=0.05×4186×13.4

Q=2804.62 J/Kg°C

Q=ml×cw×△Tl

0.05×4186×21.7

Q=4541.8 J/Kg°C

Heat capacity of calorimeter:

QH-QC=Qcal

4541.81-2804.62=1737.19

Heat capacity=Qcal△Tcal

1737.1913.4=129.64

HC of calorimeter=129.64 J/Kg°C

The hot water had a higher energy with more spread out and higher velocity molecules, this created collisions and energy transfers with the cold molecules. The hotter molecules then had a lower energy and the colder molecules had a higher energy. These interactions took place until the particles mostly were at the equilibrium temperature

## Experiment 2: Heat capacity of a metal.

Higher temperature metal placed in room temperature water:

|  | Mass (KG) | Initial Temperature(°C) | Final Temperature(°C) | Change in Temperature (°C) |
| --- | --- | --- | --- | --- |
| Metal | 0.0185 | 75 | 29.5 | -45.5 |
| Water | 0.030 | 21.7 | 29.5 | 7.8 |
| Calorimeter | --------------- | 21.7 | 29.5 | 7.8 |

| Time(s) | Twater(°C) | Time(s) | Twater(°C) | Time(s) | Twater(°C) |
| --- | --- | --- | --- | --- | --- |
| 0 | 28.2 | 120 | 29.3 | 240 | 29.0 |
| 10 | 28.5 | 130 | 29.3 | 250 | 29.0 |
| 20 | 28.8 | 140 | 29.3 | 260 | 29.0 |
| 30 | 29.1 | 150 | 29.3 | 270 | 29.0 |
| 40 | 29.5 | 160 | 29.3 | 280 | 29.0 |
| 50 | 29.5 | 170 | 29.3 | 290 | 29.0 |
| 60 | 29.5 | 180 | 29.3 | 300 | 29.0 |
| 70 | 29.4 | 190 | 29.3 | 310 | 29.0 |
| 80 | 29.4 | 200 | 29.1 | 320 | 29.0 |
| 90 | 29.4 | 210 | 29.1 | 330 | 28.8 |
| 100 | 29.4 | 220 | 29.1 | 340 | 28.8 |
| 110 | 29.3 | 230 | 29.1 | 350 | 28.7 |

The specific heat capacity of metal:

Mm×Cpm×△Tm=-(mw×cpw×△Tmw+(HCcal×Tmcal))

0.0185×Cpm×-45.5=(0.03×4186×7.8+129.64×7.8)

Cpm=2364.97 J/Kg°C

The actual value for the specific heat capacity of aluminium has been determined as 900J/Kg.K [2]. This was not close to the calculated value in the experiment however the purity of the metal is not certain and the calorimeter was not a perfect system.

Assumed all the energy transferred from the metal went to the water the specific heat capacity was calculated as:

mw ×c×△Tw=mm×c×△Tm

0.0185×c×45.5=0.030×4186×7.8

specfic heat capacity of metal=1163.68 J/Kg°C

This value was much closer to the actual value and suggests that energy was being transferred outside of the system

The graph of time against temperature showed that the water initially heated up at a steady rate until it reached the maximum temperature where it remained for a short period. The water then cooled at a much slower rate. As the water cooled the kinetic energy decreased leading to slower collisions. The metal cooled much more than the temperature of the water increased due to its lower specific heat capacity. As water required more energy to raise the temperature it did this at a much slower rate

## Experiment 3:

The reaction of the two chemicals caused an exothermic neutralisation reaction and two products which were H2O (water) and sodium chloride (salt). The limiting reactant in the experiment was the Hydrochloric acid as it was all used up in the reaction and left 0.002L of sodium Hydroxide unreacted.

HCl aq+NaOHaq--→NaCl(aq)+ H2O

moles=massmolar mass

mass=moles×molar mass

|  | Volume (L) | No moles | Initial Temp (°C) | Final Temp (°C) | Temp Change (°C) |
| --- | --- | --- | --- | --- | --- |
| HCl | 0.050 | 0.050 | 20.3 | 26.9 | 6.6 |
| NaOH | 0.052 | 0.052 | 20.3 | 26.9 | 6.6 |

Where q is molar heat, n is number of moles and Q is the specific heat capacity

q=Qn

n=Vreactant ×Creactant

n=0.050×1=0.050

Assuming c=4186 J/Kg°C and 1ml solution=1g

Q=mreaction×cwater×△Treactant

Q=0.102×4186×6.6

Q=2818 J/Kg°C

q=28180.050=-56360 J/Mol

Q is negative as heat was given out in the neutralisation. This value was close to the range of  -57 and -58kJ/mol  [3] for a reaction between a strong acid and alkali.

# Discussion

## Significance of results

Experiments 1 and 2 showed that energy is transferred from a hotter object/liquid to a cooler one until the point of equilibrium. Substances with a higher specific heat capacity require more energy to increase in temperature therefore do not heat up as quickly as a substance with a lower heat capacity. Within a chemical reaction bonds are broken and new ones formed which can cause the reactions to be endothermic or exothermic. These changes in energy have been successfully measured in a calorimeter.

## Relevance to OME and future work

Thermal analysis of explosive materials including heat capacity, activation energy and thermal conductivity are all important factors relating to explosives. These materials must be stored and handled in a safe manor therefore it if critical to know how they act under different circumstances including temperature. Techniques such as Speyer 1993 and differential scanning calorimetry are used to determine the specific heat capacity of explosives and energetics [4]. The activation enthalpies of explosion are calculated using conditions including molar heats and specific heats shown in  (Appendix A [5]. Potential for use of calorimeter to detect explosives and energetic materials due to the threat of non-traditional explosives [6] as it can be difficult to identify non nitro/nitrate explosives. This method would allow thermal analysis to determine explosive behaviour [6].

## Limitations or potential sources of error in the experiment.

The experiment relied on precise and accurate measurements of weights and volumes which can be vulnerable to human error. Substances were also transferred to the calorimeter with the temperature recorded before transfer, therefore any change in temperature during transfer was not accounted for.

# Conclusion

In conclusion the specific heat capacity of a substance can be calculated using a calorimeter if it is a closed system. The hypothesis was successful and showed the relation between thermal qualities required in heat transfer including heat capacity and enthalpy. Calorimetry was a successful method for determining these values and the hypothesis was correct. More accurate values could be produced using a calorimeter with less heat transfer.

# Bibliography

| [1] | R. Edwards, “CHAPTER 7 - The Measurement of Heat Quantity,” Physics for O.N.C Course, pp. 103-124, 1970. |
| --- | --- |
| [2] | P. Evans, “Specific heat capacity of materials,” The Engineering Mindset, 16 October 2016. [Online]. Available: https://theengineeringmindset.com/specific-heat-capacity-of-materials/. [Accessed 23 April 2025]. |
| [3] | J. Clark, “ENTHALPY CHANGE OF NEUTRALISATION,” Chem Guide, July 2013. [Online]. Available: https://www.chemguide.co.uk/physical/energetics/neutralisation.html. [Accessed 25th April 2025]. |
| [4] | J. C. Gois and F. R. Chaves, “Research on the Specific Heat Capacity of PBX Formulations Based on RDX,” Journal of Aerospace Technology and Management, vol. 8, no. 3, pp. 352-356, 2026. |
| [5] | M. J. Cawkwell, J. Davis, N. Lease, F. W. Marrs, A. Burch, S. Ferreira and V. W. Manner, “Understanding Explosive Sensitivity with Effective Trigger Linkage Kinetics,” ACS physical chemistry, vol. 2, no. 5, pp. 448-458, 2022. |
| [6] | Y.-S. Liu, V. Ugaz, S. North, W. Rogers and S. Mannan, “Development of a miniature calorimeter for identification and detection of explosives and other energetic compounds,” Journal of Hazardous Materials, vol. 142, no. 3, pp. 662-668, 2007. |

# Appendix

Appendix A – Molar heat and specific heat of explosives [5].

Appendix B – Methods and equipment

Experiment 1 – heat capacity od calorimeter

Equipment – Two stacked Styrofoam cups and fitted lid, thermometer, stirring rod, clamp, stand, measuring cylinder, beaker, Bunsen burner.

- Calorimeter set up with thermometer on clamp and stand and stirring rod in the Styrofoam lid.

- 50ml water measured out using measuring cylinder and added to calorimeter and temperature reading taken as cold measurement.

- 50 ml water measured using measuring cylinder and added into a beaker this was heated up to ……….. degrees and left to cool to ………… degrees.

- Hot water swiftly added into the calorimeter and mixed thoroughly, maximum temperature recorded at equilibrium point.

Experiment 2 – specific heat capacity of titanium metal

Equipment – Two stacked Styrofoam cups with fitted lid, thermometer, stirring rod, clamp, stand, measuring cylinder, Bunsen burner, titanium metal, tongs, weighing scales

- Titanium metal used of weigh ……………g

- Water bath set up with a beaker and heated until boiling point using Bunsen burner.

- 30ml water measure using measuring cylinder and added to calorimeter and temperature measured as initial cold temp of water.

- Metal placed in water bath for 5 minutes then rested in water bath for 2 minutes t reach equilibrium temperature (hot temperature).

- Metal swiftly added into calorimeter using tongs and water stirred.

- Temperature recorded every 10 seconds to see the rise and drop of temperature in the calorimeter. Maximum temperature recorded at T maximum.

Experiment 3 – Molar heat of reaction of HCl with NaOH

Equipment – 1.0M NaOH, 1.0M HCl, calorimeter, beaker, measuring cylinder, thermometer.

- 52ml 1.0M NaOH measured using a measuring cylinder and placed in colorimeter

- 50ml 1.0M HCl measured using a measuring cylinder and placed in beaker

- Temperatures monitored using 2 thermometers until both reached same temperature and temperature recorded.

- HCl swiftly added into calorimeter and temperature recorded at maximum.

Appendix C – Original Data Sheets