Lab 1 - Building a Simple Network

Reflection

Network:
Switches connected with cross-over cable (similar equipment).
Each PC connected to a switch with straight-through cable (dissimilar equipment).
PCs configured.
No default gateway required as no router on network.
Checked the network visually – green arrows.
Ping from PC to PC – successful.

Switch configuration:
Prevent DNS lookups – no ip domain-lookup – stops switch looking for IP typos that don’t exist.
Set an encrypted login – enable secret class (class is password)
Configure console port 0 with non encrypted password – line con 0, then password cisco.

Create ‘message of the day’ banner – a warning to anyone attempting to access switch.

Save running config to file - copy running-config startup-config – saves the current settings to a file that can be used to immediately recreate the switch should it have a fail or need resetting.

To display current config – show running-config 
The ‘secret’ is encrypted so shows as a long string – real password is not displayed.

- Step 13.

- Record the interface status for the following interfaces.

| Interface | S1 |  | S2 |  |
| --- | --- | --- | --- | --- |
|  | Status | Protocol | Status | Protocol |
| F0/1 | Up | up | Up | Up |
| F0/6 | Up | up | Down | Down |
| F0/18 | Down | Down | Up | Up |
| VLAN 1 | Admin down | Down | Admin down | Down |

Why some FastEthernet ports on the switches are up and others are down?
At beginning we set the ports by cabling them. Ports are down by default. When we connect a cable they automatically come up.

What could prevent a ping from being sent between the PCs?
Pinging wrong address
Incorrect config of any of the equipment
wrong cable connections