EXERCISES

- Vector quantities

- If  p  is a force of 40 N acting in the direction due east and  q  is a force of 30 N acting in the direction due north, what is the magnitude r and the direction of the vector sum r=p+q ?

- If  p  is an horizontal force acting to the right and  q  is a force of 61.8 N acting in the direction north-west so it makes an angle of 72° with  q .

What is the magnitude r of the vector sum r=p+q given the following direction for r ?

r

q

p

q=61.8 N

72°

42°

θ

- If  p  is an horizontal force of 40 N acting to the right and  q  is a force of 30 N acting in the direction north-east so it makes an angle of 135° with  p  such as:

r

q

p

135°

What is the magnitude r of the vector sum r=p+q ?

- Stress and Strain

- A wrought-iron bar with a diameter of 4 cm is used in a roof truss. If the tensile force supported is 90 kN, what is the direct stress?

- A steel girder having the cross-section shown below is used to support a compressive force of 500 kN. What is the compressive stress in the steel girder?

40 mm

15 mm

15 mm

60 mm

- The forces P, acting on the plates shown in the following figure are 25 kN each and the rivet has a diameter of 1.8 cm. What is the shear stress in the rivet?

P

P

- The following figure shows a component which is attached to a vertical surface by means of adhesive. The component sags by 0.03 mm under the action of a shear force P of 30 N. If the strain is found to be 210-3, what is the initial thickness of the adhesive?

P

P

Adhesive

0.03 mm

- Elasticity, Plasticity and Hooke’s Law

- Calculate the strain that will be produced by a tensile stress of 10 N/mm2 extending a bar of aluminium alloy with a Young’s modulus of 70 kN/mm2.

- A tie bar has 2 holes, a distance of 4 m apart. By how much does this distance increase when a tensile load of 20 kN is applied to the tie bar? The tie bar has a rectangular section of 40 mm  10 mm and the material of which the tie bar is made has a modulus of Elasticity of 210 kN/mm2.

4 m

40 mm

10 mm

20 kN

20 kN

- In a tensile test on a steel sample, an extensometer recorded an increase in length of 0.117 mm for an applied load of 60 kN. If the diameter and the original gauge length of the sample were 15.77 mm and 80 mm respectively, determine the modulus of elasticity for the steel.

- A steel bar ABCD consists of three sections: AB is of 20 mm diameter and 200 mm long, BC is 25 mm square and 400 mm long, and CD is of 12 mm diameter and 200 mm long. The bar is subjected to an axial compressive load which induces a stress of 30 MN/m2 on the largest cross-section.

When the load is applied, determine:

- The stress in the other two sections of the bar.

- The total decrease in the length of the bar.

For steel E = 210 GN/m2.