| LOCATION: /Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 6 | LEFEL 6 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| FeB 2022 | 2022 |

MODULE CODE: ACMA6009

ADVANCED PROCESSES AND MATERIALS

TIME:  2 HOURS

answer Four QUESTIONS OUT OF FOUR

Equipment or materials required – None

Adnoddau neu ddefnyddiau sydd eu hangen - Dim

1 (a) Critically appraise the resistance solid state welding processes, which includes: Spot,

Seam and Projection welding.

Your answer should consider surface preparation, weld integrity,

exclusion of the atmosphere, potential defects, and distortion.

(10 marks)

1 (b) Discuss the significantly different modes of operation which are available in the

Metal Arc Gas Shielding process (namely: Dip, Spray and Pulse

Transfer mechanisms) Your answer should include reference to the

following:

i)   The quality of the welds produced in each case

ii) The overall efficiency” of each of the transfer mechanisms.

(10 marks)

2 (a)

Distinguish between the two generic types of thermal spaying processes which are

based on temperature/heat produced by combustion and electricity in terms of:

i)   The adhesion/bonding of the coating.

ii)  The overall integrity of the coating (i.e. porosity, inclusion and oxide

content).

iii)  The Health and Safety/COSHH implications.

(10 marks)

2 (b) Discuss the three stages of creep which occur in metals and suggest four ways by

which creep may be minimized in practice.

(10 marks)

3 (a) Critically appraise the production of steel using the electric arc process which used

steel scrap as its primary source of material.

Your answer should consider:

i)   The energy required for the process to be carried out

ii)  The effects that contamination has on the resultant properties of this recycled

steel.

(10 marks)

3 (b) Discuss the limitations of using stainless steel as a corrosion resistant alloy.

Your answer should refer to:

i) Typical corrosion mechanisms operating with these alloys

ii) The significance of the two major alloying additions made to nominal steel which

are chromium and nickel.

l

(10 marks)

Question 4 :

A mountain  bike frame is to be made from natural materials.

- Based on what you know about the lifecycle and customer  desired features of the frame, synthesise a list of qualitative requirements .  (5 Marks)

- How do these translate to desired material properties   for the frame material. (5 marks)

- With reference to the material selection charts below,  which natural materials might be interesting to investigate for this application.   (4 Marks)

- Which of these potential choices would you reject immediately and on what grounds? (3 Marks)

- How does your choice compare with welded aluminium T6 tube section?  (5 Marks)

Hint : look up the pertinent properties on the internet

- What other aspects of your natural material choice would you need to check out before you decided to make a mountain bike frame from it? (3 marks)

Diwedd/End