Transforming from world space to normalized device co-ordinates

Example 1.  Imagine you wanted to place the camera at (0,5,5) looking at point (0,0,0).

This is equivalent to:-

1.	A rotation around  the world x-axis by -45 degrees followed by

2.	A translation of +5 along  the y axis and +5 along the world* z axis

y

1.                                                                                  2.

y

+z

+z

But really we leave the camera where it is and move the world in the reverse way which is

- A translation of -5 along  z axis and -5 along the y axis

y

A rotation around the world** x-axis by +45 degrees

y

1.                                                                                               2.

+z

+z

That is, our transformation is:-  RxT  and a vertex, v will be transformed by the multiplication v’= RxTv

The matrix multiplication, RxT  is shown here.

1   0   0     0          1   0   0    0            1   0     0           0

0  12  - 12  0         0    1  0    -5     =    0  12  - 12    (-52  -  -52)

0  12   12   0         0    0   1   -5           0   12   12     (-52  +  -52)

0    0   0    1         0    0   0    1            0    0   0             1

1    0     0       0

0    12  - 12     0

=      0    12   12    -102

0    0    0       1

* Note that if we are thinking in OpenGL local co-ordinate systems then this is equivalent to translation of (0,+5,+5) then a rotation about the camera’s local co-ordinates.

** and equally, if we are still thinking in OpenGL local co-ordinate systems then this is equivalent to rotation about the object’s local co-ordinates then a translation of (0,-5,-5) along the object’s local co-ordinate system.

Example 2.

Assuming the camera has been positioned as in example 1, show the matrix operations that will result in a further translation of a point v=(1,1,1) by (3,0,-1) in the object’s local co-ordinate system followed by a rotation of 45 degrees about the object’s local x axis (i.e. use the matrix multiplication order as used in OpenGL).

Assuming our transformation is already RxT  as before, then a further translation of a point is achieved by composing the transform RxT T1. A subsequent rotation about the local co-ordinate x axis is then achieved with the transformation RxT T1 R1 where:

1     0      0     3

0    1      0     0

T1=   0    0      1    -1

0    0      0     1

and

1    0      0      0

0    12  - 12     0

R1=   0    12    12     0

0    0     0       1

Let A=T1R1.

1    0      0      3                                                  1    0      0       0          1    0      0      3

0    12  - 12     0                                                  0    12  - 12     0          0    12  - 12     0

A =   0    12    12    -1            and let M = RxTA.  M=     0   12    12   -102     .     0    12   12    -1

0    0     0       1                                                   0    0     0      1           0    0    0       1

1    0      0       3

0    0     -1      12

M=   0    1      0      -112

0    0     0       1

The point  v will be transformed by the multiplication Mv. Therefore the transformed (including the camera transformation of example one is) point is at:-

1+3           1    0      0       3         1

-1+12         0    0     -1      12        1

1+ -112  =   0    1      0      -112       1

1           0    0     0       1          1

Example 3.

Apply the perspective projection used by OpenGL’s  gluPerspective() function to the point calculated in example 2. Use the following parameters: field of view = 40 degrees, aspect ratio =1.0, near = 1, far = 25. Now apply the perspective divide to the result.

gluPerspective uses the same projection matrix as the one we developed in class. It is repeated here for convenience.

Where:-

near specifies the distance from the viewer to the near clipping plane (always positive).

far specifies the distance from the viewer to the far clipping plane (always positive).

f/a   0     0      0                          A=(far+near)/(near-far).

P=     0    f      0       0                          B=(2*far*near)/(near-far)

0    0      A       B                          f=cotangent(field of view/2)

0    0     -1      0                           a= aspect ratio

f =  cotan(40/2) = 2.75,  A = 26/24;   B =50/24

V’=Pv

11                   2.75   0     0       0           4

-0.81              0    2.75    0       0       -1+12

-5.26     =         0       0      2624     5024    1+ -112

6.78             0        0       -1      0         1

And now divide x,y, z and w by 6.78

1.623

-0.118   =   V’

-0.776

1

Q: Is this point, V’ going to be displayed after transforming to the viewport window?

A: Well, since the normalized device co-ordinates range from -1 to +1 for x and -1 to +1for y and z is not used for the  moment, then it will NOT since the x value is > +1.