Title Page

University of Wales Trinity Saint David

School of Engineering

Title of the coursework: Combustion and Emission Study: Modelling Fuels and NOx.

Your Name: Loaghan Slade

Your Student Number: P151151

Date: 25/11/19

Module Name and Code: Powertrain and sustainability, M6X01206

Lecturer’s name: Owen Williams

Contents

1	Introduction	1

2	Question 1	2

2.1	Summary	2

2.2	Part A	2

2.3	Part B	2

2.4	Part C	3

2.5	Part D	5

2.6	Part E	5

2.7	Part F	5

2.8	Part G	6

3	Question 2	7

3.1	Summary	7

3.2	Part A	7

3.3	Part B	14

4	Bibliography	17

4.1	Figures	17

4.2	Graphs	17

# Introduction

The aim of this assignment is to complete a series of calculations outlining the emissions of specific fuels and demonstrate an understanding of the combustion process taking place when fuel is burned.

# Question 1

## Summary

Part A is a study of a specific blend of the fuels Methanol and Iso-Octane having an equal 50% split of the total molar mass. (50% Methanol and 50% Iso-Octane)

## Part A

Calculate the mass percentages of each fuel in this blend.

Iso-Octane (C8H18) has a molar mass of 114.22850 Kg/Kmol, and Methanol (CH4O) has a molar mass of 32.04186 Kg/Kmol (EBI, 2016)

114.22850+32.04186=146.27036 Kg/Kmol total.

114.22850/146.27037=0.78094

0.78094x100=78.0941%

100-78.0941=21.90592%

Iso-Octane accounts for 78% of the total molar mass while Methanol accounts for 22%.

## Part B

Calculate the air-fuel mass ratio for stoichiometric combustion in air.

Assuming atmospheric air is comprised of 79% Nitrogen (N2) and 21% oxygen (O2). (excluding the 0.96% of Carbon dioxide (CO2), Argon (Ar), and other trace gasses found in atmospheric air) 79/21=3.762 atmospheric air = O2+3.762N2

Atomic weights

Hydrogen = 1

Carbon = 12

Oxygen = 16

Nitrogen = 14

Stoichiometric burning is calculated by combining both chemical equations for iso-octane and methanol to form C8+1H18+4O1+0 or C9H22O

Using the equation

CaHbOc+122a+b2-cO2+3.762N2→aCO2+b2H2O+122a+b2-c×3.762N2

(MIT, 2014)

A= 9

B=22

C=1

C9H22O1+1229+222-1O2+3.762N2→9CO2+222H2O+12(29+222-1)×3.762N2

C9H22O+14(O2+3.762N2)→9CO2+11H2O+52.668N2

AFR by mass = Air massFuel mass

AFR by mass = 14(2×16+3.762×2×14)9×12+22×1+1×16

AFR by mass =1922.704146

AFR by mass = 13.169:1

## Part C

Calculate the enthalpy of formations and the lower heating values in MJ/Kg

Specific enthalpy for Iso-Octane = -259.2 MJ/Kmol

Specific enthalpy for Methanol = -201.5 MJ/Kmol

Specific enthalpy of Carbon Dioxide = -393.5 MJ/Kmol

Specific enthalpy of water = -241.8 MJ/Kmol

Equation for enthalpy of formation

ΔHreaction=∑ΔHof(products)−∑ΔHof(Reactants)

(Texts, 2019)

Methanol

Enthalpy needed for products

201.5+1.5(0+3.762x0)

201.5 MJ/Kmol

Enthalpy released

(1x393.5)+(2x241.8)+(5.643x0)

393.5+483.6+0=877.1

Enthalpy of formation = 877.1-201.5

Enthalpy of formation for methanol = 675.6 MJ

LHV (Lower Heating Value) = EnthalpyMolar mass

LHV=675.632.04

LHV=21.086 MJ/Kg

Iso-Octane

Enthalpy needed for products

259.2+12.5(0+3.762x0)

259.2 MJ/Kmol

Enthalpy released

(8x393.5)+(9x241.8)+(47x0)

3148+2176.2+0=5324.2

Enthalpy of formation = 5324.2-259.2

Enthalpy of formation for Iso-Octane = 5065 MJ

LHV=5065114

LHV=44.430 MJ/Kg

Blend

Enthalpy of formation for blend

0.78(5065)+0.22(675.6)=4099.332MJ/Kg

LHV of blend = 0.78(44.430)+0.22(21.086)

LHV of blend = 39.294 MJ/Kg

## Part D

Explain enthalpy of formation.

Enthalpy of formation  is the value given when 1 mol of a substance is made from its elemental component chemicals; the value can be impacted by a range of physical or chemical factors such as heat, pressure and the state of matter in which the component parts are when they are reacted. To calculate enthalpy of formation it is best to assume the reaction is carried out under ideal circumstances, these are 1 BAR (or 1 atmosphere) of pressure and at 25 degrees Celsius (298k). Values calculated under these circumstances are considered standard.

## Part E

Explain the difference between higher and lower heating values.

Higher heating value (HHV) or gross calorific value, is used when water is present in its liquid form in the reacted agents where the heat released can be re-captured and used, whereas lower heating value (LHV) or net calorific value, is used when water is present in its gaseous form (steam), this assumes that the heat released by the steam is entirely wasted. (HydrogenTools, 2019)

## Part F

Calculate the mass of CO2 produced per Kg of fuel consumed.

Iso-Octane = 114 Kg/Kmol

Methanol = 32.04 Kg/Kmol

Kmol of CO2 produced per Kmol of Methanol = 1

Kmol of CO2 produced per Kmol of Iso-Octane = 8

0.78(8)+0.22(1)=Kmol of CO2 produced per Kmol of blended fuel

6.24+0.22=6.46 Kmol of CO2 produced per Kmol of blended fuel.

Molar mass of CO2 = 44.01 Kg/Kmol

6.46x44.01=284.305 Kg/Kmol of CO2 per Kmol of blended fuel burned

0.78(114)+0.22(32.04)=95.969 Kg/Kmol for blended fuel

284.30595.969=Kg of CO2 produced per Kg of blended fuel

2.962 Kg of CO2 is produced per Kg of blended fuel burned.

## Part G

Describe what properties an “ideal” fuel may have.

An ideal fuel should have a high calorific value and burn at a reasonable rate, produce no harmful emissions after burning, thus causing the least amount of air pollution possible. The fuel should also be stable at room temperature but not volatile when in transit, thus making transportation and storage easy. The fuel should also be cheap, available and renewable. A good example of a fuel currently available that is close to being ideal is E85 or bioethanol, though combustion of bioethanol still produces too many harmful by-products to be considered ideal. (Malaysia, 2019)

# Question 2

## Summary

The aim of part B is to use either Excel or Matlab to perform calculations which can then be used as an exhaust gas simulation tool; the information produced by this simulation can then be used to calculate components of the exhaust gas as a function of fuel equivalence ratio.

## Part A

Create a NOx model of Iso-octane using lean stoichiometric values.

A NOx simulation can be produced by first calculating adiabatic flame temperatures.

Adiabatic flame temperatures are calculated by first calculating the enthalpy of formation for each stoichiometric value used. This is done by multiplying the initial enthalpy of formation by the new stoichiometric value.

| Enthalpy of formation by fuel equivalence ratio in MJ |  |  |
| --- | --- | --- |
| Φ | Methanol (CH₄O) | Iso-octane (C₈H₁₈) |
| 0.6 | 405.36 | 3039 |
| 0.65 | 439.14 | 3292.25 |
| 0.7 | 472.92 | 3545.5 |
| 0.75 | 506.7 | 3798.75 |
| 0.8 | 540.48 | 4052 |
| 0.85 | 574.26 | 4305.25 |
| 0.9 | 608.04 | 4558.5 |
| 0.95 | 641.82 | 4811.75 |
| 0.96 | 648.576 | 4862.4 |
| 0.97 | 655.332 | 4913.05 |
| 0.98 | 662.088 | 4963.7 |
| 0.99 | 668.844 | 5014.35 |
| 1 | 675.6 | 5065 |

Table 1 Enthalpy of formation by FER

This can also be achieved by completing the chemical equation previously used to calculate the air fuel ratio multiplied by the new fuel equivalence ratio for lean combustion.

n1=φ×C

n2=φ×H2

2n1+n2+n3=2x

n4=3.762x

| n Values for Iso-octane |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Φ | n₁ | n₂ | n₃ | n₄ | EX Kmols |
| 0.6 | 4.8 | 5.4 | 10 | 47 | 2.35E-05 |
| 0.65 | 5.2 | 5.85 | 8.75 | 47 | 2.34E-05 |
| 0.7 | 5.6 | 6.3 | 7.5 | 47 | 2.32E-05 |
| 0.75 | 6 | 6.75 | 6.25 | 47 | 2.30E-05 |
| 0.8 | 6.4 | 7.2 | 5 | 47 | 2.29E-05 |
| 0.85 | 6.8 | 7.65 | 3.75 | 47 | 2.27E-05 |
| 0.9 | 7.2 | 8.1 | 2.5 | 47 | 2.26E-05 |
| 0.95 | 7.6 | 8.55 | 1.25 | 47 | 2.24E-05 |
| 0.96 | 7.68 | 8.64 | 1 | 47 | 2.24E-05 |
| 0.97 | 7.76 | 8.73 | 0.75 | 47 | 2.23E-05 |
| 0.98 | 7.84 | 8.82 | 0.5 | 47 | 2.23E-05 |
| 0.99 | 7.92 | 8.91 | 0.25 | 47 | 2.23E-05 |
| 1 | 8 | 9 | 0 | 47 | 2.23E-05 |

Table 2 n Values for Iso-octane

| n Values for Methanol |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Φ | n₁ | n₂ | n₃ | n₄ | EX Kmols |
| 0.6 | 0.6 | 1.2 | 1.2 | 5.643 | 7.33E-07 |
| 0.65 | 0.65 | 1.3 | 1.3 | 5.643 | 7.50E-07 |
| 0.7 | 0.7 | 1.4 | 1.4 | 5.643 | 7.66E-07 |
| 0.75 | 0.75 | 1.5 | 1.5 | 5.643 | 7.82E-07 |
| 0.8 | 0.8 | 1.6 | 1.6 | 5.643 | 7.97E-07 |
| 0.85 | 0.85 | 1.7 | 1.7 | 5.643 | 8.13E-07 |
| 0.9 | 0.9 | 1.8 | 1.8 | 5.643 | 8.28E-07 |
| 0.95 | 0.95 | 1.9 | 1.9 | 5.643 | 8.43E-07 |
| 0.96 | 0.96 | 1.92 | 1.92 | 5.643 | 8.46E-07 |
| 0.97 | 0.97 | 1.94 | 1.94 | 5.643 | 8.49E-07 |
| 0.98 | 0.98 | 1.96 | 1.96 | 5.643 | 8.52E-07 |
| 0.99 | 0.99 | 1.98 | 1.98 | 5.643 | 8.55E-07 |
| 1 | 1 | 2 | 2 | 5.643 | 8.58E-07 |

Table 3 n Values for Methanol

| Chemical reactants by fuel equivalence ratio |  |  |
| --- | --- | --- |
| Φ | Methanol (CH₄O+1.5(O₂+3.762N₂)) | Iso-octane (C₈H₁₈+12.5(O₂+3.762N₂)) |
| 0.6 | 0.6CO₂+1.2H₂O+1.2O₂+5.643N₂ | 4.8CO₂+5.4H₂O+10O₂+47N₂ |
| 0.65 | 0.65CO₂+1.3H₂O+1.05O₂+5.643N₂ | 5.2CO₂+5.85H₂O+8.75O₂+47N₂ |
| 0.7 | 0.7CO₂+1.4H₂O+0.9O₂+5.643N₂ | 5.6CO₂+6.3H₂O+7.5O₂+47N₂ |
| 0.75 | 0.75CO₂+1.5H₂O+0.75O₂+5.643N₂ | 6CO₂+6.75H₂O+6.25O₂+47N₂ |
| 0.8 | 0.8CO₂+1.6H₂O+0.6O₂+5.643N₂ | 6.4CO₂+7.2H₂O+5O₂+47N₂ |
| 0.85 | 0.85CO₂+1.7H₂O+0.45O₂+5.643N₂ | 6.8CO₂+7.65H₂O+3.75O₂+47N₂ |
| 0.9 | 0.9CO₂+1.8H₂O+0.3O₂+5.643N₂ | 7.2CO₂+8.1H₂O+2.5O₂+47N₂ |
| 0.95 | 0.95CO₂+1.9H₂O+0.15O₂+5.643N₂ | 7.6CO₂+8.55H₂O+1.25O₂+47N₂ |
| 0.96 | 0.96CO₂+1.92H₂O+0.12O₂+5.643N₂ | 7.68CO₂+8.64H₂O+1O₂+47N₂ |
| 0.97 | 0.97CO₂+1.94H₂O+0.09O₂+5.643N₂ | 7.76CO₂+8.73H₂O+0.75O₂+47N₂ |
| 0.98 | 0.98CO₂+1.96H₂O+0.06O₂+5.643N₂ | 7.84CO₂+8.82H₂O+0.5O₂+47N₂ |
| 0.99 | 0.99CO₂+1.98H₂O+0.03O₂+5.643N₂ | 7.92CO₂+8.91H₂O+0.25O₂+47N₂ |
| 1 | 1.0CO₂+2.0H₂O+0.0O₂+5.643N₂ | 8CO₂+9H₂O+0.0O₂+47N₂ |

Table 4 Chemical reactants by FER

The new enthalpy of formation is then used to calculate the initial flame temperatures used to calculate flame temperatures for each degree of crank angle for the duration of combustion. The initial flame temperature is calculated through trial and error, this was completed by completing an initial calculation for iso-ethanol at a “Φ” value of 1, and an initial arbitrary temperature of 2400K. Enthalpy for each component of the resultant chemicals is found by consulting the tables provided by the university. (Williams, 2019)

For 2400k  Hf= 5065MJ

8(115.79)+9(93.74)+47(70.64)=5090.06MJ this value is too high (A)

For 2350K

8(112.73)+9(91.07)+47(68.82)=4956.01MJ this value is too low (B)

The correct value must lie between 2350k and 2400k. the actual temperature can be calculated using the formula;

T-TBTA-TB=hf-hB hA-hB

T=TB+TA-TB×hf-hBha-hB

This results in;

T-23502400-2350=5065-4956.015090.06-4965.01

T=2350+50×108.99125.05=2393.579K

This is then repeated for each of the required fuel equivalence ratios.

| Carbon Dioxide (CO₂) |  |  |  | Water (H₂O) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Temp | Spec heat | Enthalpy |  | Temp | Spec heat | Enthalpy |
| K | KJ/Kmol-K | MJ/Kmol |  | K | KJ/Kmol-K | MJ/Kmol |
| 1000 | 54.36 | 33.42 |  | 1000 | 41.31 | 25.99 |
| 1050 | 54.86 | 36.15 |  | 1050 | 41.99 | 28.07 |
| 1100 | 55.33 | 38.91 |  | 1100 | 42.64 | 30.19 |
| 1150 | 55.78 | 41.69 |  | 1150 | 43.26 | 32.34 |
| 1200 | 56.2 | 44.49 |  | 1200 | 43.87 | 34.52 |
| 1250 | 56.6 | 47.31 |  | 1250 | 44.46 | 36.72 |
| 1300 | 56.98 | 50.15 |  | 1300 | 45.02 | 38.96 |
| 1350 | 57.34 | 53 |  | 1350 | 45.57 | 41.23 |
| 1400 | 57.67 | 55.88 |  | 1400 | 46.1 | 43.52 |
| 1450 | 57.99 | 58.77 |  | 1450 | 46.61 | 45.84 |
| 1500 | 58.29 | 61.68 |  | 1500 | 47.1 | 48.18 |
| 1550 | 58.57 | 64.6 |  | 1550 | 47.58 | 50.55 |
| 1600 | 58.83 | 67.54 |  | 1600 | 48.03 | 52.94 |
| 1650 | 59.08 | 70.48 |  | 1650 | 48.47 | 55.35 |
| 1700 | 59.31 | 73.44 |  | 1700 | 48.9 | 57.78 |
| 1750 | 59.53 | 76.41 |  | 1750 | 49.31 | 60.24 |
| 1800 | 59.73 | 79.4 |  | 1800 | 49.7 | 62.71 |
| 1850 | 59.93 | 82.39 |  | 1850 | 50.08 | 65.21 |
| 1900 | 60.11 | 85.39 |  | 1900 | 50.45 | 67.72 |
| 1950 | 60.27 | 88.4 |  | 1950 | 50.8 | 70.25 |
| 2000 | 60.43 | 91.42 |  | 2000 | 51.14 | 72.8 |
| 2050 | 60.58 | 94.44 |  | 2050 | 51.47 | 75.3 |
| 2100 | 60.71 | 97.47 |  | 2100 | 51.78 | 77.95 |
| 2150 | 60.84 | 100.51 |  | 2150 | 52.08 | 80.54 |
| 2200 | 60.96 | 103.56 |  | 2200 | 52.38 | 83.16 |
| 2250 | 61.08 | 106.61 |  | 2250 | 52.66 | 85.78 |
| 2300 | 61.18 | 109.66 |  | 2300 | 52.92 | 88.42 |
| 2350 | 61.28 | 112.73 |  | 2350 | 53.18 | 91.07 |
| 2400 | 61.37 | 115.79 |  | 2400 | 53.43 | 93.74 |
| 2450 | 61.46 | 118.86 |  | 2450 | 53.67 | 96.42 |
| 2500 | 61.55 | 121.94 |  | 2500 | 53.9 | 99.11 |
| 2550 | 61.62 | 125.02 |  | 2550 | 54.12 | 101.81 |
| 2600 | 61.7 | 128.1 |  | 2600 | 54.34 | 104.52 |
| 2650 | 61.77 | 131.19 |  | 2650 | 54.54 | 107.24 |
| 2700 | 61.84 | 134.28 |  | 2700 | 54.74 | 109.97 |
| 2750 | 61.9 | 137.37 |  | 2750 | 54.93 | 112.72 |
| 2800 | 61.96 | 140.47 |  | 2800 | 55.11 | 115.47 |
| 2850 | 62.02 | 143.57 |  | 2850 | 55.29 | 118.23 |
| 2900 | 62.08 | 246.67 |  | 2900 | 55.46 | 120.99 |
| 2950 | 62.14 | 149.78 |  | 2950 | 55.62 | 123.77 |
| 3000 | 62.19 | 152.88 |  | 3000 | 55.78 | 126.56 |

Table 5 Enthalpy table for Carbon Dioxide          Table 6 Enthalpy table for Water

| Nitrogen (N₂) |  |  |  | Oxygen(O₂) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Temp | Spec heat | Enthalpy |  | Temp | Spec heat | Enthalpy |
| K | KJ/Kmol-K | MJ/Kmol |  | K | KJ/Kmol-K | MJ/Kmol |
| 1000 | 32.76 | 21.47 |  | 1000 | 34.93 | 22.72 |
| 1050 | 33.01 | 23.11 |  | 1050 | 35.1 | 24.47 |
| 1100 | 33.26 | 24.77 |  | 1100 | 35.27 | 26.23 |
| 1150 | 33.49 | 26.44 |  | 1150 | 35.43 | 28 |
| 1200 | 33.71 | 28.12 |  | 1200 | 35.59 | 29.77 |
| 1250 | 33.91 | 29.81 |  | 1250 | 35.75 | 31.56 |
| 1300 | 34.11 | 31.51 |  | 1300 | 35.9 | 33.35 |
| 1350 | 34.3 | 33.22 |  | 1350 | 36.05 | 35.15 |
| 1400 | 34.48 | 34.94 |  | 1400 | 36.2 | 36.95 |
| 1450 | 34.64 | 36.67 |  | 1450 | 36.35 | 38.77 |
| 1500 | 34.8 | 38.4 |  | 1500 | 36.49 | 40.59 |
| 1550 | 34.95 | 40.15 |  | 1550 | 36.63 | 42.42 |
| 1600 | 35.1 | 41.9 |  | 1600 | 36.77 | 44.25 |
| 1650 | 35.23 | 43.66 |  | 1650 | 36.9 | 46.09 |
| 1700 | 35.36 | 45.42 |  | 1700 | 37.03 | 47.94 |
| 1750 | 35.48 | 47.19 |  | 1750 | 37.17 | 49.8 |
| 1800 | 35.59 | 48.97 |  | 1800 | 37.29 | 51.66 |
| 1850 | 35.7 | 50.75 |  | 1850 | 37.42 | 53.53 |
| 1900 | 35.8 | 52.54 |  | 1900 | 37.54 | 55.4 |
| 1950 | 35.9 | 54.33 |  | 1950 | 37.67 | 57.28 |
| 2000 | 35.99 | 56.13 |  | 2000 | 37.79 | 59.17 |
| 2050 | 36.07 | 57.93 |  | 2050 | 37.9 | 61.06 |
| 2100 | 36.15 | 59.73 |  | 2100 | 38.02 | 62.96 |
| 2150 | 36.23 | 61.54 |  | 2150 | 38.14 | 64.86 |
| 2200 | 36.3 | 63.36 |  | 2200 | 38.25 | 66.77 |
| 2250 | 36.36 | 65.17 |  | 2250 | 38.36 | 68.69 |
| 2300 | 36.43 | 66.99 |  | 2300 | 38.47 | 70.61 |
| 2350 | 36.49 | 68.82 |  | 2350 | 38.58 | 72.53 |
| 2400 | 36.54 | 70.64 |  | 2400 | 38.68 | 74.46 |
| 2450 | 36.59 | 72.47 |  | 2450 | 38.79 | 76.4 |
| 2500 | 36.64 | 74.3 |  | 2500 | 38.89 | 78.34 |
| 2550 | 36.69 | 76.13 |  | 2550 | 38.99 | 80.29 |
| 2600 | 36.74 | 77.97 |  | 2600 | 39.09 | 82.24 |
| 2650 | 36.78 | 79.81 |  | 2650 | 39.19 | 84.2 |
| 2700 | 36.82 | 81.65 |  | 2700 | 39.29 | 86.16 |
| 2750 | 36.86 | 83.49 |  | 2750 | 39.38 | 88.13 |
| 2800 | 36.89 | 85.33 |  | 2800 | 39.48 | 90.1 |
| 2850 | 36.93 | 87.18 |  | 2850 | 39.57 | 92.07 |
| 2900 | 36.96 | 89.03 |  | 2900 | 39.66 | 94.06 |
| 2950 | 36.99 | 90.88 |  | 2950 | 39.75 | 96.04 |
| 3000 | 37.03 | 92.73 |  | 3000 | 39.84 | 98.03 |

Table 7 Enthalpy table for Nitrogen                     Table 8 Enthalpy table for Oxygen

When the initial temperatures have been found for each of the ratios, the increase can be calculated using the equation;

T2nT1n=P2n-1P1n-1

Which is then transposed to make T2 the subject.

T2=nP2n-1P1n-1×T1n

From the temperatures found it is possible to calculate the NOx emitted at each of the fuel equivalence ratios, the equation;

4.7×1013×N2[O2]12EXP(-67837T)RPM×36060×40×10-6=KMol°

Where the volume used is 40cc and RPM is 1800.

Table 9 Temperature and NOx for  fuel equivalence ratios of 0.6 to 0.7

This is repeated for all ratios from 0.6 to 1, at 0.05 intervals between 0.6 and 0.95, between 0.96 and 1.0 an interval of 0.01 is used. (full excel worksheets can be found in the appendix after this report) The total NOx released at each ratio can then be graphed to show how a fuel equivalence ratio impacts emissions. The number of oxygen and nitrogen atoms must also be found by calculating the volume present in the exhaust gas as a proportion of total emissions. This value is given the name alpha.

Exhaust KMols=Pressure×VolumeGas constant×Temperature

α=Exhaust KMolsφ+12.5(O2+3.762N2)

N2=α×n4Volume  O2=α×n3Volume

Graph 1 NOx PPM vs FER for Iso-octane

Graph 2 NOx PPM vs FER for Methanol

## Part B

At the FER of 0.97 model the results of adding exhaust gas recirculation up to 20%

The process of calculating the impact of EGR is almost identical to the way NOx is found without the addition of EGR. The steps must be repeated with a modified chemical equation to account for the additional gasses added to the combustion chamber before the ignition point.

This is done using the equation;

yn1CO2+n2H2O+n3O2+n4N2+φ(C8H18+12.5(O2+3.762N2)→y(n1CO2+n2H2O+n3O2+n4N2)

Where y=EGR% and φ = Fuel equivalence ratio.

This is completed at 2% intervals between 0% and 20%.

The new chemical equations are then used to find the new enthalpy and flame temperatures.

Table 10 2% EGR Temp and NOx

These values can also be graphed to show the difference between how much NOx is evolved at each percentile of EGR.

Graph 3 NOx production across EGR% against crank angle

The average NOx can also be graphed showing a clear relationship between NOx production and EGR.

Graph 4 Average NOx vs EGR%

# Bibliography

EBI. (2016, December 15). CHEBI:62805 - isooctane. Retrieved from ChEBI: https://www.ebi.ac.uk/chebi/searchId.do?chebiId=CHEBI:62805

HydrogenTools. (2019, November 25). Lower and Higher Heating Values of Fuels. Retrieved from h2tools.org: https://h2tools.org/hyarc/calculator-tools/lower-and-higher-heating-values-fuels

Malaysia, G. (2019, November 25). Ideal Fuel. Retrieved from GasMalaysia.com: https://www.gasmalaysia.com/index.php/gas-fundamentals/ideal-fuel

MIT. (2014). Lec. 04 Comb & thermochemistry. Massachusetts: Massachusetts Institute of Technology.

Texts, C. L. (2019, June 5). Thermochemistry. Retrieved from chem.libretexts.org: chem.libretexts.org/Bookshelves/Physical_and_Theoretical_Chemistry_Textbook_Maps/Map%3A_Physical_Chemistry_for_the_Biosciences_(Chang)/03%3A_The_First_Law_of_Thermodynamics/3.6%3A_Thermochemistry

Williams, D. O. (2019, November 10). Fuels Database/JANAF Tables. Swansea, Swansea, Wales: University of wales trinity saint david.

## Figures

Table 1 Enthalpy of formation by FER	7

Table 2 n Values for Iso-octane	8

Table 3 n Values for Methanol	8

Table 4 Chemical reactants by FER	9

Table 5 Enthalpy table for Carbon Dioxide          Table 6 Enthalpy table for Water	10

Table 7 Enthalpy table for Nitrogen                     Table 8 Enthalpy table for Oxygen	11

Table 9 Temperature and NOx for a fuel equivalence ratio of 0.6	12

Table 10 2% EGR Temp and NOx	15

## Graphs

Graph 1 NOx PPM vs FER for Iso-octane	13

Graph 2 NOx PPM vs FER for Methanol	14

Graph 3 NOx production across EGR% against crank angle	15

Graph 4 Average NOx vs EGR%	16

# Appendix

Table 11 Flame temperature and NOx production for Iso-octane without EGR

Table 12 Flame temperature and NOx production for Methanol without EGR

Table 13 Flame temperature and NOx production for Iso-octane with EGR

Table 14 Reference tables used for calculations