LEVEL/ LEfEL 4

SEMESTER 1/2

MODULE CODE/ Cod Y modIwl: PDSE4005  - Programmable Logic Controllers

aSSESSMENT TYPE/ mATH O ASESU: aSSIGNMENT

Assessment/Component/ AsESIAD/CYDRAN: 1 of 3

COMPONENT WEIGHTING/ PWYSAU’R CYDRAN: 30%

WORD COUNT/ NIFER Y GEIRIAU: 2000 WORDS/O EIRIAU

MODULE TUTOR / TIWTOR Y MODIWL: Richard Wray

SUBMISSION DETAILS/ MANYLION CYFLWYNO:

- Make sure that Your Name , Your Student P Number, Your Module Title, Assignment Title and Your Module Lecturer’s Name are clearly shown on the front page of your assignment

- All assignments must be submitted electronically to Turnitin unless indicated by the module lecturer here: …………………………………………...

- If a paper submission is required then it will be indicated here: ………………….

you will then be required to submit the assignment parts in a suitable report folder– not in poly-pockets - to the library.

- For paper submissions this form (ALL PAGES) MUST be printed out, signed, attached to the front of your assignment and submitted to the library separately from your electronic assignment submission.

- Assignments will not be accepted without this form as it is a requirement that you sign the disclosure regarding the referencing convention.

- DO NOT put this form into Turnitin or it will match many similarities with other students’ submissions.

###### RESULT

#### School of Applied Computing ASSIGNMENT SPECIFICATION

| Student name: |  | Student P number: |  |
| --- | --- | --- | --- |
| Programme: | HND/HNC Electrical & Electronic Engineering, HNC Instrumentation Engineering, HNC Power Systems Engineering. |  |  |
| Lecturer: | Richard Wray | Internal Verifier: | Jason Pointer |
| Assignment Title: | PLC Program design using FBD |  |  |
| Hand Out Date: | 08/11/17 | Submission deadline: | 13/12/17 |

| Referencing: | In the main body of your submission you must give credit to authors on whose research your work is based. Append to your submission a reference list that indicates the books, articles, etc. that you have read or quoted in order to complete this assignment (e.g. for books: surname of author and initials, year of publication, title of book, edition, publisher: place of publication). |  |  |
| --- | --- | --- | --- |
| Disclosure: | I declare that this assignment is all my own work and that I have acknowledged all materials used from the published or unpublished works of other people. All references have been duly cited. |  |  |
| Student’s Signature: | Hardcopy assignments will not be accepted without a signature here | Date: |  |

Late submission will result in a late penalty mark, as follows: Up to one week late, maximum mark of 40% for first attempts and 0% for resubmissions. (Academic Quality Handbook 2017/18 7.5 (5))

Learning Outcomes tested (from module syllabus)

LO1, LO2.

TASK DESCRIPTION

Part A. Introduction

Programmable Logic Controllers are available in three design types, which are termed:

- Unitary PLC

- Modular PLC

- Rack mounted PLC

Each PLC type has been developed to meet specific applications with respect to operational characteristics, system capacity, I/O functionality and potential expansion.

Task 1 – Programmable Logic Controller types

- Through your own research, describe in detail the three types of PLCs available. Include in your answer the specifications that define each type, possible applications and any considerations that need to be assessed when selecting a specific PLC type.

- Explain with the aid of a diagram the main components within a PLC and show the appropriate connections to and from the outside world. In addition, identify methods of protecting a PLC input/output interface from potential electrical overloads or fault conditions that may occur in the field devices.

Task 2 – Logic diagrams and truth tables

There are many control situations that require actions to be initiated when a certain combination of conditions are realised. Such situations can involve the use of logic gates to produce an output when certain input conditions are met.

a)	Assuming two inputs (A & B) and one output (Q), produce truth tables for each of the following logic gates:

- AND

- OR

- NAND

- NOR

- XOR

b) 	Produce a truth table for the logic circuit shown below (figure 1). Comment on the circuit output with respect to the inputs, noting any issues that may arise when using this configuration of logic gates.

Figure 1. Logic Gate Circuit

Task 3 - Timers and Counters

In many control tasks there is a need to apply timing constraints to a process operation. For example, a motor might need to be controlled to operate for a particular interval of time, or perhaps be switched on after some time interval. PLCs thus have timers as built-in devices.

a) 	With the aid of suitable timing diagrams explain the operation of each timer type listed below:

- On-delay timer

- Off-delay timer

- Pulse timer.

b)	In addition there may be a need to count the number of events before allowing an action to take place. Explain the two types of counters that could be used by PLCs for this purpose.

Part B Introduction

Unitary PLCs are widely used in industry where sequential control of small processes or manufacturing operations are required. Typically these systems use development software employing graphic symbols to aid in the construction of a PLC program. For this part of the assignment you will investigate the use of this type of development software to design a range of PLC programs.

Task 4 – Programming problems

Use the Mitsubishi Alpha software to develop programs to meet the following sequential operations:

a) 	For part of a washing machine cycle, a water supply pump is required to run for 50 seconds. It is then to be switched off and a heater switched on for 20 seconds. The heater should then be turned off and a drain pump run for 15 seconds to remove the water. The cycle should be started using a momentary ‘on’ switch, run automatically and terminate once the drain pump has stopped. Indicator lights should be used to inform the user of each stage of the cycle.

b)	A photocell sensor is being used to detect objects moving on a conveyor belt. The system is required to give an output after receiving a 10 pulse input from the sensor as a result of detecting 10 objects passing in front of it. The system should have a manual and automatic counter reset.

c)		A detection system is required to monitor the number of people entering and leaving a store and thereby calculate the total number of customers in the store at any point in time. The system should be able to monitor customers entering and leaving the store and display a warning if this number reaches 50. The system should have a manual counter reset to clear the stored value at the end of the working day.

To complete the tasks you are required to demonstrate the operation of your programs and attach a Tutor verified copy of the program to your report. Briefly discuss the I/O, flow charts and operation of each program with respect to your circuit design.

Task 5 – Traffic Light programming problem

For this task you are required to develop a PLC program using the Mitsubishi Alpha development software that will operate a set of two way traffic lights (see figure 2) in the standard sequence shown in table 1.

To start the program a momentary push button (press and release) will be used to initiate the sequence, after which it should continue automatically.

| Figure 2. Traffic light system. The traffic light control system will operate the two sets of traffic lights in the following timed sequence: Table 1 Timing (s); 5; 10; 15; 20; 25; 30; 35; 40; 45; 50; 55; 60; R E T U R N. North / South Traffic Light Set Sequence. Set 1; Red. Amber. Green. East / West Traffic Light Set Sequence. Set 2; Red. Amber. Green. |  |
| --- | --- |

To aid the design process consider the use of timing diagrams and flow charts, which should be included in your report. To complete this task you are required to demonstrate the operation of your program and attach a Tutor verified copy of the program to your report.

To complete this assignment you are required to generate an individual report of 2000 words (±10%).

Your report should critically present the research and experimental work undertaken in the various tasks, including the work of others where appropriate. Present and discuss the results obtained with reference to any background material and draw appropriate conclusions with respect to your results.

GUIDANCE FOR Students IN THE COMPLETION OF TASKS

1. Research-informed Literature: You must provide evidence of individual research with respect to PLC and systems theory and operation, and provide relevant referencing.

2. Knowledge and Understanding of Subject: Your report should show you gained knowledge of the PLCs and associated systems used in this assignment.

3. Analysis: The report should show analysis of the research and experimental programming work.

4. Practical Application and Deployment: Verify program design and operation against expected theoretical and calculated outcomes. Although the practical exercises may be completed as part of a group exercise the resulting report and analysis of data must be an independent piece of work. You will need a verified copy of your program design (screen shot) to confirm the PLC program has been demonstrated and witnessed.

5. Skills for Professional Practice: Your work should be well organised and presented in a professional manner.

marking criteria and Student FEEDBACK

This section details the assessment criteria. The extent to which these are demonstrated by you determines your mark. The marks available for each criterion are shown. Lecturers will use the space provided to comment on the achievement of the task(s), including those areas in which you have performed well and areas that would benefit from development/improvement.

| Common Assessment Criteria Applied | Marks available | Marks awarded |
| --- | --- | --- |
| 1. Research-informed Literature Extent of research and/or own reading, selection of credible sources, application of appropriate referencing conventions. | 10 |  |
| 2. Knowledge and Understanding of Subject Extent of knowledge and understanding of concepts and underlying principles associated with the discipline. | 25 |  |
| 3. Analysis Analysis, evaluation and synthesis; logic, argument and judgement; analytical reflection; organisation of ideas and evidence | 25 |  |
| 4. Practical Application and Deployment Deployment of methods, materials, tools and techniques; application of concepts; formulation of innovative and creative solutions to solve problems. | 30 |  |
| 5. Skills for Professional Practice Attributes in professional practice: individual and collaborative working; deployment of appropriate media; presentation and organisation. | 10 |  |

| Assignment Mark (Assessment marks are subject to ratification at the Exam Board. These comments and marks are to give feedback on module work and are for guidance only until they are confirmed. ) | Late Submission Penalties (tick if appropriate) |  | % |
| --- | --- | --- | --- |
|  | Up to one weeks late capped at 40%) |  |  |
|  | First Attempt? |  |  |

### FdSc & HN COMMON ASSESSMENT AND MARKING CRITERIA

|  | OUTRIGHT FAIL | UNSATISFACTORY | SATISFACTORY | GOOD | VERY GOOD | EXCELLENT | EXCEPTIONAL |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Assessment Criteria | REFER (equivalent to 0-29% | REFER (equivalent to 30-39%* | PASS (equivalent to 40-49%) | PASS (equivalent to 50-59%) | MERIT (equivalent to 60-69% | DISTINCTION (equivalent to 70-79% | DISTINCTION (equivalent to 80-100% |
| 1. Research-informed Literature Extent of research and/or own reading, selection of credible sources, application of appropriate referencing conventions | Little or no evidence of reading. Views and findings unsupported and non-authoritative. Referencing conventions largely ignored. | Poor evidence of reading and/or of reliance on inappropriate sources, and/or indiscriminate use of sources. Referencing conventions used inconsistently. | References to a limited range of mostly relevant sources. Some omissions and minor errors. Referencing conventions evident though not always applied consistently. | Inclusion of a range of research-informed literature, including sources retrieved independently. Referencing conventions mostly consistently applied. | Inclusion of a wide range of research-informed literature, including sources retrieved independently. Selection of relevant and credible sources. Very good use of referencing conventions, consistently applied. | A comprehensive range of research informed literature embedded in the work. Excellent selection of relevant and credible sources. High-level referencing skills, consistently applied. | Outstanding knowledge of research-informed literature embedded in the work. Outstanding selection of relevant and credible sources. High-level referencing skills consistently and professionally applied. |
| 2. Knowledge and Understanding of Subject Extent of knowledge and understanding of concepts and underlying principles associated with the discipline. | Major gaps in knowledge and understanding of material at this level. Substantial inaccuracies. | Gaps in knowledge, with only superficial understanding. Some significant inaccuracies. | Evidence of basic knowledge and understanding of the relevant concepts and underlying principles. | Knowledge is accurate with a good understanding of the field of study. | Knowledge is extensive. Exhibits understanding of the breadth and depth of established views. | Excellent knowledge and understanding of the main concepts and key theories. Clear awareness of challenges to established views and the limitations of the knowledge base. | Highly detailed knowledge and understanding of the main theories/concepts, and a critical awareness of the ambiguities and limitations of knowledge. |
| 3. Analysis Analysis, evaluation and synthesis; logic, argument and judgement; analytical reflection; organisation of ideas and evidence | Unsubstantiated generalisations, made without use of any credible evidence. Lack of logic, leading to unsupportable/ missing conclusions. Lack of any attempt to analyse, synthesise or evaluate. | Some evidence of analytical intellectual skills, but for the most part descriptive. Ideas/findings sometimes illogical and contradictory. Generalised statements made with scant evidence. Conclusions lack relevance. | Evidence of some logical, analytical thinking and some attempts to synthesise, albeit with some weaknesses. Some evidence to support findings/ views, but evidence not consistently interpreted. Some relevant conclusions and recommendations, where relevant | Evidence of some logical, analytical thinking and synthesis. Can analyse new and/or abstract data and situations without guidance. An emerging awareness of different stances and ability to use evidence to support the argument. Valid conclusions and recommendations, where relevant | Sound, logical, analytical thinking; synthesis and evaluation. Ability to devise and sustain persuasive arguments, and to review the reliability, validity & significance of evidence. Ability to communicate ideas and evidence accurately and convincingly. Sound, convincing conclusions / recommendations. | Thoroughly logical work, supported by evaluated evidence. High quality analysis, developed independently or through effective collaboration. Ability to investigate contradictory information and identify reasons for contradictions. Strong, persuasive, conclusions, justifiable recommendations. | Exceptional work; judiciously selected and evaluated evidence. Very high quality analysis, developed independently or through effective collaboration. Ability to investigate contradictory information and identify reasons for contradictions. Highly persuasive conclusions |
| 4. Practical Application and Deployment Effective deployment of appropriate methods, materials, tools and techniques; extent of skill demonstrated in the application of concepts to a variety of processes and/or contexts; formulation of innovative and creative solutions to solve problems. | Limited or no use of methods, materials, tools and/or techniques. Little or no appreciation of the context of the application. | Rudimentary application of methods, materials, tools and/or techniques but without consideration and competence. Flawed appreciation of the context of the application. | An adequate awareness and mostly appropriate application of well-established methods, materials, tools and/or techniques. Basic appreciation of the context of the application. | A good and appropriate application of standard methods, materials, tools and/or techniques. Good appreciation of the context of the application, with some use of examples, where relevant. | A very good application of a range of methods, materials, tools and/or techniques. Very good consideration of the context of the application, with perceptive use of examples, where relevant. Evidence of some innovation and creativity. | An advanced application of a range of methods, materials, tools and/or techniques. The context of the application is well considered, with extensive use of relevant examples. Application and deployment extend beyond established conventions. Innovation and creativity evident throughout. | Outstanding levels of application and deployment skills. Assimilation and development of cutting edge processes and techniques. |
| 5. Skills for Professional Practice Demonstrates attributes expected in professional practice including: individual initiative and collaborative working; deployment of appropriate media to communicate (including written and oral); clarity and effectiveness in presentation and organisation. | Communication media is inappropriate or misapplied. Little or no evidence of autonomy in the completion of tasks. Work is poorly structured and/or largely incoherent. | Media is poorly designed and/or not suitable for the audience. Poor independent or collaborative initiative. Work lacks structure, organisation, and/or coherence | Can communicate in a suitable format but with some room for improvement. Can work as part of a team, but with limited involvement in group activities. Work lacks coherence in places and could be better structured. | Can communicate effectively in a suitable format, but may have minor errors. Can work effectively as part of a team, with clear contribution to group activities. Mostly coherent work and is in a suitable structure. | Can communicate well, confidently and consistently in a suitable format. Can work very well as part of a team, with very good contribution to group activities. Work is coherent and fluent and is well structured and organised. | Can communicate professionally and, confidently in a suitable format. Can work professionally within a team, showing leadership skills as appropriate, managing conflict and meeting obligations. Work is coherent, very fluent and is presented professionally. | Can communicate with an exceptionally high level of professionalism. Can work exceptionally well and professionally within a team, showing advanced leadership skills. Work is exceptionally coherent, very fluent and is presented professionally. |

Cyfadran Dylunio Cymhwysol a Pheirianneg

Faculty of Applied Design and Engineering

Taflen Cyflwyno Aseiniad

Assignment Hand in Sheet

| MANYLION Y MYFYRIWR STUDENT’S DETAILS |  |  |  |  |
| --- | --- | --- | --- | --- |
| ENW: NAME: |  | RHIF P Y MYFYRIWR: STUDENT P NUMBER: | P |  |
| CWRS: COURSE: | Foundation Year/HNC/HND/FdSc/BEng/BSc/MEng/MComp/MSc * dileer fel y bo'n berthnasol / delete as appropriate |  | LEFEL Y CWRS: LEVEL OF COURSE: |  |
| MODIWL: MODULE: |  |  |  |  |
| TEITL YR ASEINIAD: ASSIGNMENT TITLE: |  |  |  |  |
| DARLITHIWR/WYR: LECTURER/S: |  |  | DYDDIAD: DATE: |  |
| Staff Llyfrgell YN UNIG / Library Staff ONLY Derbyniwyd gan / Accepted by: _______ Dyddiad Cyflwyno: Date Submitted: ________________________ Amser / Time: _____ |  |  |  |  |

----------------------------------------------------------------------------------------------------------------------------------------------------------

| DERBYNNEB / RECEIPT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ENW / NAME: |  | RHIF P Y MYFYRIWR: STUDENT P NUMBER: |  | P |  |
| CWRS / COURSE: | Foundation Year/HNC/HND/FdSc/BEng/BSc/MEng/MComp/MSc * dileer fel y bo'n berthnasol / delete as appropriate |  | LEFEL Y CWRS / LEVEL OF COURSE: |  |  |
| MODIWL / MODULE: |  |  |  |  |  |
| TEITL YR ASEINIAD/ ASSIGNMENT TITLE: |  |  |  |  |  |
| DARLITHIWR/WYR: LECTURER/S: |  |  | DYDDIAD / DATE: |  |  |
| Staff Llyfrgell YN UNIG / Library Staff ONLY Derbyniwyd gan / Accepted by: _______ Dyddiad Cyflwyno: Date Submitted: ________________________ Amser / Time: _____ |  |  |  |  |  |