Plot E9 Rotary Drilling – SA1 Swansea Waterfront

Ground Investigation Factual Report

Report Number 12345

November 2019

| Document Control |  |  |
| --- | --- | --- |
| Client | Novus Construction |  |
| Site | SA1 Swansea |  |
| Date | November 2019 |  |
| Report Reference | 12345 |  |
| Issue Number | 1 |  |
| Revision | 0 |  |
| Status | Draft |  |

| Parties |  |  |  |  |
| --- | --- | --- | --- | --- |
| Client |  | Novus Construction |  |  |
| Drilling Contractor |  | Apex Drilling Services Limited |  |  |
|  |  | Sturmi Way |  |  |
|  |  | Village Farm Industrial Estate |  |  |
|  |  | Pyle |  |  |
|  |  | Bridgend |  |  |
|  |  | CF33 6BZ |  |  |
| Approval |  |  |  |  |
| Prepared | Anonymous |  |  |  |
| Reviewed |  |  |  |  |
| Approved |  |  |  |  |

Note

The title to this report is vested in the Employer named but title to copyright is retained. The Contracts (Rights of Third Parties) Act 1999 does not apply to the contract with the Employer and the provisions of the said Act are hereby excluded

Contents

Introduction	1

General	1

Purpose of Investigation	1

Scope of Work	1

Site Details	2

Description	2

Predicted Geology	2

Surface	2

Published Geology	2

Site Work	3

General	3

Exploratory Hole Locations	3

Rotary Boreholes	3

Encountered Geology	4

Made Ground	4

Superficial	4

Bedrock ..	4

SPT	4

References	5

Document Ref: 12345	November 2019

# Introduction

## General

Upon the instruction of the employer Novus Construction, Apex Drilling Services Limited were commissioned to undertake a ground investigation for the SA1 Swansea Project Located on plot E9.

## Purpose of Investigation

The intrusive investigation was designed to determine the geotechnical parameters of the superficial soil deposits and underlying bedrock of the wharf area, this was for the purposes of further foundation design for a residential building.

As part of this investigation the following goals were to be achieved:

- Determine the underlying geology of the ground;

- Determine the mechanical properties the underlying soils in the proposed area of development.

## Scope of Work

The scope of work instructed by Novus Construction was as follows:

- Using rotary drilling techniques develop a borehole to 30 m bgl;

- Identify and record the composition of the underling geology;

- Perform SPTs every 1.5 m when in superficial geology;

- Collect core samples from bedrock geology.

General notes on the techniques employed by Apex Drilling Services Limited are described in the foreword.

Document Ref: 12345	1	November 2019

# Site Details

## Description

The site is located at to the north of Kings Dock, Port Tennant in Swansea, to the east of Kings road and Roberts Road and to the west of Langdon Road. The grid coordinates for the site are Easting 266921 Northing 192775 with a Latitude of 51.618023 and Longitude of -3.9236104.

# Predicted Geology

## Surface

The surface of BH001 was overlain with granite paving slabs, this was above a layer of concrete over made ground fill, the nearby vicinity of the borehole included a “waste” area of small vegetation and mixed made ground gravels, cobbles, sand and silt.

The surface of BH002 comprised of small vegetation and grasses over a brown made ground of gravelly silt with occasional cobbles.

The topography of both locations was predominantly level.

## Published Geology

The British Geological Survey (BGS) published sheet number 247 indicates that the superficial geology in the area comprises of large amounts of made ground, likely comprised of colliery and mineral smelting spoil

The made ground is indicated to overly post glacial beach and tidal flat deposits, these deposits generally comprise of sand silt and clay.

The BGS indicate that the underling bedrock at this site comprises of the South Wales Middle Coal Measures Formation, this can comprise of grey mudstone, siltstone and sandstone.

Document Ref: 12345	2	November 2019

# Site Work

## General

Apex Drilling Services Limited attended site on 28th October 2019 with demobilisation completed on 7th November 2019.

Site supervision and attendance by a qualified engineer from Apex Drilling was provided for all drilling activities during this time period in general accordance with BS 5930+A2, BS EN 197-2 (2007), BS EN ISO 22475-1 and BS 10175 (2011).

## Exploratory Hole Locations

Two exploratory hole locations were set out by Novus Construction who determined areas which were clear of services.

Both locations were hand pitted and CAT scanned prior to drilling.

## Rotary Boreholes

Drilling was completed using a Fraste Multidrill XL Max rotary drilling rig and air as a flushing medium, due to the anticipated excessive water on the site due to the location of the boreholes it can be noted that water was present in BH001 from a depth of 2 m bgl and BH002 from a depth of 14 m bgl

Overburden / superficial deposits were drilled using a temporary simultaneous Odex 115 mm casing system.

Drilling of bedrock was completed using 423 F rotary coring methodsproducing a 76 mm bore to the completed base depth of 30 m bgl in line with the specification of works.

Throughout the drilling operation the borehole was logged by the engineer on site noting:

- Depths, description, colour, identification of stratum;

- Depths at which groundwater was encountered;

- Loss of flushing medium.

Upon completion the linear length of the boreholes was backfilled using a bentonite concrete mix in areas of bedrock and bentonite pelts in areas of superficial strata, this was to better emulate the natural stratum.

Details of the rotary boreholes, including final depths in metres below ground level (m bgl) are provided below in Table 1.

| Borehole ID | Overburden |  | Termination Depth |  | Terminating Strata |  |  | Void |  | Terminate Reason |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | m bgl |  | m bgl |  |  |  |  |  |  |  |  |
| BH001 | 20.0 |  | 30.2 |  |  | Middle Coal |  | None Observed |  | Specified depth |  |
|  |  |  |  |  |  | Measures |  |  |  |  |  |
| BH002 | 25.2 |  | 30.5 |  |  | Middle Coal |  | None Observed |  | Specified depth |  |
|  |  |  |  |  |  | Measures |  |  |  |  |  |

Table 1- Summary of Borehole Details

Document Ref: 12345	3	November 2019

# Encountered Geology

## Made Ground

Both boreholes were found to have made ground of significant depth, this largely comprised of black gravelly silt, the gravels within this were of mixed material including, slag concrete, sandstone and some areas of bituminous surfacing.

## Superficial

Underlying the made ground deposits consistent with the predicted beach and tidal flat deposits were identified; these deposits comprised generally of grey and greyish silt with occasional gravels and sands. At .50 m bgl within BH002 peat was identified this is likely associated with the nearby Crymlyn Bog, this persisted t 8.00 m bgl.

## Bedrock

Bedrock geology was identified to comprise of light grey fine to medium grain micaceous sandstone. Laminations of carbonaceous siltstone and coal were occasionally observed. This is consistent with the published geology of Middle Coal Measures.

Table 2 below details the depths and thickness of the encountered geology.

|  |  |  | Made Ground |  |  |  |  |  | Superficial |  |  |  |  |  | Bedrock |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Borehole ID |  | Depth to top |  |  | Thickness |  |  | Depth to top |  |  | Thickness |  |  | Depth to top |  | Thickness |  |
|  |  |  | of Strata |  |  |  |  |  | of Strata |  |  |  |  |  | of Strata |  |  |  |
|  |  |  |  |  |  | (m) |  |  |  |  |  | (m) |  |  |  |  | (m) |  |
|  |  |  | (m) |  |  |  |  |  | (m) |  |  |  |  |  | (m) |  |  |  |
| BH001 |  | 0 |  |  | 2.00 |  |  | 2.00 |  |  | 18.00 |  |  | 20.00 |  |  | > 10.20 |  |
| BH002 |  | 0 |  |  | 5.50 |  |  | 5.50 |  |  | 19.80 |  |  | 25.30 |  |  | > 5.50 |  |

Table 2- Geological Details

## SPT

SPT testing as described in ISO 22476-3 was conducted throughout both boreholes at 1.5m intervals. It should be noted that there was limited granular lithology the ‘N’ value does not correlate exactly to the relative density of the soil.

SPT values within the made ground in BH001 ranged from 11 to 15 and in BH002 were 10 to 23, this indicates a medium dense material within BH002.

Within the beach and tidal flat deposits in BH001 the SPT value ranged between 9 and 21 between 3.50 m bgl and 8.00 m bgl, this was determined to be representative of a soft to firm silt material. A band of very dense gravel was observed between 11.40 and 12.40 m bgl that no SPT test was conducted in due to the limited thickness of the strata. At depths of 9.50 m bgl to 18.50 m bgl the values ranged between 27 and 50, this was associated with a firm to stiff silt.

This was similarly reflected in shallow soils of BH002 where the silty peat had ‘N’ values of 10 and 11 and was observed to be soft. This was above a soft grey silty clay with ‘N’ values of 15 and 18 this clay was observed to a depth of 11.00 m bgl. Underlying the soft clay was a band of medium dense gravel with ‘N’ values of 21 and 27 below which was grey silt which graded from firm to stiff between 14.50 m bgl and 18 m bgl. Between 18.00 and 25.3 m bgl many boulders and gravels were observed within the silt, SPT values were recorded as between 38 and 50 and the silt was observed to be very stiff.

Document Ref: 12345	4	November 2019

# References

AGS : 2005	Electronic transfer of geotechnical and geoenvironmental data (Edition 3.1 including

addendum May 2005). Association of Geotechnical and Geoenvironmental Specialists.

BRE Special Digest 1 : 2005	Concrete in aggressive ground. Building Research Establishment.

BS 1377-2: 1990	Methods of test for soils for civil engineering purposes. Classification tests. British

Standards Institution.

BS 5930+A2 : 2010	Code of practice for site investigations (Amendment 2). British Standards Institution.

BS 10175 : 2011	Code of practice for investigation of potentially contaminated sites. British Standards

Institution.

BS 22475-3 : 2011	Geotechnical	investigation	and	testing.	Sampling	methods	and	groundwater

measurements. British Standards Institution.

BS EN 1997-2 : 2007	Eurocode 7 - Geotechnical design - Part 2 Ground investigation and testing.	British

Standards Institution.

BS EN ISO 14688-1 : 2002	Geotechnical investigation and testing - Identification and classification of soil - Part 1

Identification and description. British Standards Institution.

BS EN ISO 14689-1 : 2003 :	Geotechnical investigation and testing - Identification and classification of rock - Part 1

Identification and description. British Standards Institution.

UK Specification for Ground investigation 2nd Edition 2012

Document Ref: 12345	5	November 2019

Rotary Log

| Project Name: Plot E9 Residential Building |  |  |  |  |  | Client: Novus Construction |  |  |  |  |  |  |  | Date: 28/10/2019 - 31/10/2019 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location: SA1 Swansea |  |  |  |  |  | Contractor: Apex Drilling Services Ltd. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Project No. : 12345 |  |  |  |  |  | Crew Name: Gary Williams / Owen Jones |  |  |  |  |  |  |  | Drilling Equipment: Fraste |  |  |  |  |  |
| Borehole Number |  |  |  | Hole Type |  |  |  | Level |  |  | Logged By |  |  | Scale |  | Page Number |  |  |  |
|  | BH001 |  |  | OH/RC |  |  |  |  |  | Anonymous |  |  |  | 1:35 |  | Sheet 1 of 5 |  |  |  |
|  |  | (m) |  | /FI | TCR SCR RQDDiameterRecovery(SPT) |  |  | (m) | (m) |  |  |  |  |  |  |  |  |  |  |
| Well Water |  | Depth |  | Type | Coring |  |  | Depth | Level Legend |  |  |  |  | Stratum Description |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 0.10 |  |  |  |  | MADE GROUND comprising granite paving |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 0.20 |  |  |  |  | slab. |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | MADE GROUND comprising concrete. |  |  |  |  |  |  |
|  |  |  |  |  |  |  | (N=15) |  |  |  |  |  | MADE GROUND comprising of black |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | clayey gravelly medium SAND. Gravels |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | fine to coarse sub angular to sub rounded. |  |  |  |  |  |  |

1

| (N=11) | 2.10 | 2 |  |  |
| --- | --- | --- | --- | --- |
|  |  | Soft to firm black very sandy SILT. (BEACH |  |  |
|  |  | AND TIDAL FLAT DEPOSITS) |  |  |

3

(N=9)

4

| (N=21) | 5 |
| --- | --- |

|  |  |  |  |  |  |  |  | (N=12) |  |  |  |  |  |  |  |  |  |  |  | 6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |
|  |  |  |  |  | Type/FI TCR |  | SCR RQD D/R/(SPT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hole Diameter |  |  | Casing Diameter |  |  |  | Chiselling |  |  | Inclination and Orientation |  |  |  |  |  |  |  | Drilling Flush |  |  |  |  |  |  |
| Depth Base |  | Diameter | Depth Base | Diameter |  | Depth Top | Depth Base | Duration | Tool | Depth Top | Depth Base |  | Inclination |  | Orientation | Depth Top | Depth Base | Type | Colour | Min (%) | Max (%) |  |  |  |
| 30.20m |  | 412F | 21.50m | ODEX |  |  |  |  |  |  |  |  | Vertical |  |  |  |  | Air |  |  |  |  |  |  |
|  |  |  |  | 115 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Remarks

Location CAT scanned and hand pitted prior to drilling.

Rotary Log

| Project Name: SA1 Bridge Investigation |  |  |  |  | Client: Novus Construction |  |  |  |  | Date: 28/10/2019 - 31/10/2019 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location: SA1 Swansea |  |  |  |  | Contractor: Apex Drilling Services Ltd. |  |  |  |  |  |  |  |  |  |
| Project No. : 12345 |  |  |  |  | Crew Name: Gary Williams / Owen Jones |  |  |  |  | Drilling Equipment: Fraste |  |  |  |  |
| Borehole Number |  |  | Hole Type |  |  |  | Level |  | Logged By | Scale |  | Page Number |  |  |
| BH001 |  |  | OH/RC |  |  |  |  |  | Anonymous | 1:35 |  | Sheet 2 of 5 |  |  |
|  | (m) |  | /FI | TCR SCR RQDDiameterRecovery(SPT) |  |  | (m) | (m) |  |  |  |  |  |  |
| Well Water | Depth |  | Type | Coring |  |  | Depth | Level Legend |  | Stratum Description |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Soft to firm black very sandy SILT. (BEACH |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | AND TIDAL FLAT DEPOSITS) |  |  |  |  |  |

| (N=13) |  | 8 |  |  |
| --- | --- | --- | --- | --- |
|  | 8.50 |  |  |  |
|  |  | Firm to stiff black gravelly slightly clayey |  |  |
|  |  | SILT. Gravels multicoloured fine to medium |  |  |
|  |  | sub angular to sub rounded. (BEACH AND |  |  |
|  |  | TIDAL FLAT DEPOSITS) |  |  |
|  |  | 9 |  |  |
| (N=42) |  |  |  |  |
|  |  | 10 |  |  |

| (N=31) |  | 11 |  |  |
| --- | --- | --- | --- | --- |
|  | 11.40 |  |  |  |
|  |  | Very dense multicoloured fine to medium |  |  |
|  |  | with rare coarse sub angular GRAVEL. |  |  |
|  |  | (BEACH AND TIDAL FLAT DEPOSITS) |  |  |
|  |  | 12 |  |  |
|  | 12.40 |  |  |  |
| (N=34) |  | Firm to stiff greyish red slightly silty CLAY |  |  |
|  |  | (BEACH AND TIDAL FLAT DEPOSITS) |  |  |
|  |  | 13 |  |  |

|  |  |  |  |  |  |  |  | (N=27) |  |  |  |  |  |  |  |  |  |  |  | 14 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Type/FI TCR |  | SCR RQD D/R/(SPT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hole Diameter |  |  | Casing Diameter |  |  |  | Chiselling |  |  | Inclination and Orientation |  |  |  |  |  |  |  | Drilling Flush |  |  |  |  |  |
| Depth Base |  | Diameter | Depth Base | Diameter |  | Depth Top | Depth Base | Duration | Tool | Depth Top | Depth Base |  | Inclination |  | Orientation | Depth Top | Depth Base | Type | Colour | Min (%) | Max (%) |  |  |
| 30.20m |  | 412F | 21.50m | ODEX |  |  |  |  |  |  |  |  | Vertical |  |  |  |  | Air |  |  |  |  |  |
|  |  |  |  | 115 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Remarks

Location CAT scanned and hand pitted prior to drilling.

Rotary Log

| Project Name: SA1 Bridge Investigation |  |  |  |  | Client: Novus Construction |  |  |  |  | Date: 28/10/2019 - 31/10/2019 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location: SA1 Swansea |  |  |  |  | Contractor: Apex Drilling Services Ltd. |  |  |  |  |  |  |  |  |  |
| Project No. : 12345 |  |  |  |  | Crew Name: Gary Williams / Owen Jones |  |  |  |  | Drilling Equipment: Fraste |  |  |  |  |
| Borehole Number |  |  | Hole Type |  |  |  | Level |  | Logged By | Scale |  | Page Number |  |  |
| BH001 |  |  | OH/RC |  |  |  |  |  | Anonymous | 1:35 |  | Sheet 3 of 5 |  |  |
|  | (m) |  | /FI | TCR SCR RQDDiameterRecovery(SPT) |  |  | (m) | (m) |  |  |  |  |  |  |
| Well Water | Depth |  | Type | Coring |  |  | Depth | Level Legend |  | Stratum Description |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Firm to stiff greyish red slightly silty CLAY |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | (BEACH AND TIDAL FLAT DEPOSITS) |  |  |  |  |  |

| 15.00 |  | 15 |  |
| --- | --- | --- | --- |
|  | Firm to stiff greyish red slightly silty slightly |  |  |
|  | gravelly CLAY. Gravels fine to medium |  |  |
|  | multicoloured sub angular to sub rounded. |  |  |
|  | (BEACH AND TIDAL FLAT DEPOSITS) |  |  |

(N=32)

16

| (N=50) | 17 |
| --- | --- |

18

(N=50)

19

|  |  |  |  |  |  |  |  |  |  |  | 50 (25 for 65mm/50 for 30mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | 20.00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Medium strong light micaceous grey fine to |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | medium grained sandstone with rare coal |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | and carbonaceous silt lamination. |  |  |  |  |  |  |  |  |  |  |
|  |  | 20.00 - 21.50 |  |  |  |  | 95 | 84 | 23 |  |  |  |  |  |  |  |  |  |  |  | Fracture set 1 - extremely close closely |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | spaced sub vertical undulating rough with |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1mm quartz of fracture face. |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Fracture set 2 - Very close to closely |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | spaced sub horizontal planar rough |  |  |  |  |  |  | 21 |  |  |  |
|  |  |  |  |  |  | Type/FI TCR |  | SCR | RQD D/R/(SPT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hole Diameter |  |  |  | Casing Diameter |  |  |  |  | Chiselling |  |  |  |  | Inclination and Orientation |  |  |  |  |  |  |  |  |  | Drilling Flush |  |  |  |  |  |  |  |
| Depth Base |  | Diameter |  | Depth Base | Diameter |  | Depth Top | Depth Base |  | Duration |  | Tool |  | Depth Top | Depth Base |  | Inclination |  |  |  | Orientation | Depth Top | Depth Base | Type | Colour | Min (%) | Max (%) |  |  |  |  |
| 30.20m |  | 412F |  | 21.50m | ODEX |  |  |  |  |  |  |  |  |  |  |  | Vertical |  |  |  |  |  |  | Air |  |  |  |  |  |  |  |
|  |  |  |  |  | 115 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Remarks

Location CAT scanned and hand pitted prior to drilling.

Rotary Log

| Project Name: SA1 Bridge Investigation |  |  |  |  |  |  |  |  |  |  |  | Client: Novus Construction |  |  |  |  |  |  |  |  |  |  |  |  | Date: 28/10/2019 - 31/10/2019 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location: SA1 Swansea |  |  |  |  |  |  |  |  |  |  |  | Contractor: Apex Drilling Services Ltd. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Project No. : 12345 |  |  |  |  |  |  |  |  |  |  |  | Crew Name: Gary Williams / Owen Jones |  |  |  |  |  |  |  |  |  |  |  |  | Drilling Equipment: Fraste |  |  |  |  |  |  |  |  |  |  |
| Borehole Number |  |  |  |  |  | Hole Type |  |  |  |  |  |  |  | Level |  |  |  |  | Logged By |  |  |  |  |  |  | Scale |  |  | Page Number |  |  |  |  |  |  |
|  |  | BH001 |  |  |  |  | OH/RC |  |  |  |  |  |  |  |  |  |  | Anonymous |  |  |  |  |  |  | 1:35 |  |  |  | Sheet 4 of 5 |  |  |  |  |  |  |
|  |  |  |  | (m) |  | /FI |  |  | TCR SCR RQD |  |  |  | DiameterRecovery(SPT) | (m) |  |  | (m) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Well | Water |  |  | Depth |  | Type |  |  | Coring |  |  |  |  | Depth |  |  | Level |  | Legend |  |  |  |  |  | Stratum Description |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Medium strong light micaceous grey fine to |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | medium grained sandstone with rare coal |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | and carbonaceous silt lamination. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Fracture set 1 - Extremely close closely |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | spaced sub vertical undulating rough with |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1mm quartz of fracture face. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Fracture set 2 - Very close to closely |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | spaced sub horizontal planar rough |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 21.50 - 22.50 |  |  |  |  |  | 100 | 44 | 0 |  |  |  |  |  |  |  |  |  |  |  |  | bedding. Fracturing commonly seen on |  |  |  |  |  |  |  | 22 |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | coal laminations. Dipping at 25 degrees. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Anticipated zone of core |  |  |  |  | loss |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | laminations |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Frequent carbonaceous |  |  |  |  |  |  |  |  |  | 23 |  |  |  |
|  |  |  | 22.50 - 24.00 |  |  |  |  |  | 100 | 83 | 23 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 24 |  |  |  |
|  |  |  | 24.00 - 24.30 |  |  |  |  |  | 90 | 57 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 25 |  |  |  |
|  |  |  | 24.30 - 25.80 |  |  |  |  |  | 93 | 93 | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 26 |  |  |  |
|  |  |  | 25.80 - 26.70 |  |  |  |  |  | 89 | 44 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 27 |  |  |  |
|  |  |  | 26.70 - 28.20 |  |  |  |  |  | 100 | 100 | 32 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | planar rough fractures |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Three 45 degree close |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | with 1mm quartz infill. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |  |  |
|  |  |  |  |  |  | Type/FI |  |  | TCR | SCR | RQD |  | D/R/(SPT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hole | Diameter |  |  | Casing Diameter |  |  |  |  |  | Chiselling |  |  |  |  |  |  | Inclination and Orientation |  |  |  |  |  |  |  |  |  |  | Drilling Flush |  |  |  |  |  |  |  |
| Depth Base |  | Diameter |  | Depth Base | Diameter |  | Depth Top |  |  | Depth Base |  | Duration |  | Tool | Depth Top |  |  | Depth Base |  | Inclination |  |  |  | Orientation | Depth Top | Depth Base |  | Type | Colour | Min (%) | Max (%) |  |  |  |  |
| 30.20m |  | 412F |  | 21.50m | ODEX |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Vertical |  |  |  |  |  |  |  | Air |  |  |  |  |  |  |  |
|  |  |  |  |  | 115 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Remarks

Location CAT scanned and hand pitted prior to drilling.

Rotary Log

| Project Name: SA1 Bridge Investigation |  |  |  |  |  |  |  | Client: Novus Construction |  |  |  |  |  |  |  | Date: 28/10/2019 - 31/10/2019 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location: SA1 Swansea |  |  |  |  |  |  |  | Contractor: Apex Drilling Services Ltd. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Project No. : 12345 |  |  |  |  |  |  |  | Crew Name: Gary Williams / Owen Jones |  |  |  |  |  |  |  | Drilling Equipment: Fraste |  |  |  |  |  |  |
| Borehole Number |  |  |  | Hole Type |  |  |  |  |  | Level |  |  | Logged By |  |  | Scale |  |  | Page Number |  |  |  |
|  | BH001 |  |  | OH/RC |  |  |  |  |  |  |  | Anonymous |  |  |  | 1:35 |  |  | Sheet 5 of 5 |  |  |  |
|  |  | (m) |  | /FI | TCR SCR RQDDiameterRecovery(SPT) |  |  |  |  | (m) | (m) |  |  |  |  |  |  |  |  |  |  |  |
| Well Water |  | Depth |  | Type | Coring |  |  |  |  | Depth | Level Legend |  |  |  |  | Stratum Description |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Medium strong light micaceous grey fine to |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | medium grained sandstone with rare coal |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | and carbonaceous silt lamination. |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Fracture set 1 - extremely close closely |  |  |  |  |  |  |  |
|  |  | 28.20 - 29.00 |  |  | 88 | 75 | 39 |  |  |  |  |  |  |  | spaced sub vertical undulating rough with |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1mm quartz of fracture face. |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Fracture set 2 - Very close to closely |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | spaced sub horizontal planar rough |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | bedding. Fracturing commonly seen on |  |  |  |  |  | 29 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | coal laminations. Dipping at 25 degrees. |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ns |  |  |  |  |  |
|  |  | 29.00 - 30.20 |  |  | 100 | 81 | 35 |  |  |  |  |  |  |  | Frequent coal laminatio |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Frequent coal laminations |  |  |  |  |  | 30 |  |
|  |  |  |  |  |  |  |  |  |  | 30.20 |  |  |  |  | Frequent coal laminations |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | End of Borehole at 30.200m |  |  |  |  |  |  |

31

32

33

34

|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 35 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Type/FI TCR |  | SCR RQD D/R/(SPT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hole Diameter |  | Casing Diameter |  |  |  | Chiselling |  |  | Inclination and Orientation |  |  |  |  |  | Drilling Flush |  |  |  |  |  |
| Depth Base | Diameter | Depth Base | Diameter |  | Depth Top | Depth Base | Duration | Tool | Depth Top | Depth Base | Inclination | Orientation | Depth Top | Depth Base | Type | Colour | Min (%) | Max (%) |  |  |
| 30.20m | 412F | 21.50m | ODEX |  |  |  |  |  |  |  | Vertical |  |  |  | Air |  |  |  |  |  |
|  |  |  | 115 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Remarks

Location CAT scanned and hand pitted prior to drilling.

Rotary Log

| Project Name: SA1 Bridge Investigation |  |  |  |  | Client: Novus Construction |  |  |  |  | Date: 01/11/2019 - 06/11/2019 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location: SA1 Swansea |  |  |  |  | Contractor: Apex Drilling Services Ltd. |  |  |  |  |  |  |  |  |  |
| Project No. : 12345 |  |  |  |  | Crew Name: Gary Williams / Owen Jones |  |  |  |  | Drilling Equipment: Fraste |  |  |  |  |
| Borehole Number |  |  | Hole Type |  |  |  | Level |  | Logged By | Scale |  | Page Number |  |  |
| BH002 |  |  | OH/RC |  |  |  |  |  | Anonymous | 1:35 |  | Sheet 1 of 5 |  |  |
|  | (m) |  | /FI | TCR SCR RQDDiameterRecovery(SPT) |  |  | (m) | (m) |  |  |  |  |  |  |
| Well Water | Depth |  | Type | Coring |  |  | Depth | Level Legend |  | Stratum Description |  |  |  |  |
|  |  |  |  |  |  |  |  |  | MADE GROUND comprising of grass over |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | dark brown slightly sandy gravelly CLAY. |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Gravels fine to coarse angular to rounded |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | slag, bituminous surfacing, concrete, tile, |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | basalt and sandstone. |  |  |  |  |  |

1

(N=10)

2

(N=17)

3

| (N=23) | 4 |
| --- | --- |

5

| (N=11)5.50 | Soft dark brown slightly silty PEAT (PEAT) |  |
| --- | --- | --- |

6

|  |  |  |  |  |  |  |  | (N=10) |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Type/FI TCR |  | SCR RQD D/R/(SPT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hole Diameter |  |  | Casing Diameter |  |  |  | Chiselling |  |  | Inclination and Orientation |  |  |  |  |  |  |  | Drilling Flush |  |  |  |  |  |
| Depth Base |  | Diameter | Depth Base | Diameter |  | Depth Top | Depth Base | Duration | Tool | Depth Top | Depth Base |  | Inclination |  | Orientation | Depth Top | Depth Base | Type | Colour | Min (%) | Max (%) |  |  |
| 30.50m |  | 412F | 25.00m | ODEX |  |  |  |  |  |  |  |  | Vertical |  |  |  |  | Air |  |  |  |  |  |
|  |  |  |  | 115 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Remarks

Location CAT scanned and hand pitted prior to drilling.

Rotary Log

| Project Name: SA1 Bridge Investigation |  |  |  |  | Client: Novus Construction |  |  |  |  | Date: 01/11/2019 - 06/11/2019 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location: SA1 Swansea |  |  |  |  | Contractor: Apex Drilling Services Ltd. |  |  |  |  |  |  |  |  |  |
| Project No. : 12345 |  |  |  |  | Crew Name: Gary Williams / Owen Jones |  |  |  |  | Drilling Equipment: Fraste |  |  |  |  |
| Borehole Number |  |  | Hole Type |  |  |  | Level |  | Logged By | Scale |  | Page Number |  |  |
| BH002 |  |  | OH/RC |  |  |  |  |  | Anonymous | 1:35 |  | Sheet 2 of 5 |  |  |
|  | (m) |  | /FI | TCR SCR RQDDiameterRecovery(SPT) |  |  | (m) | (m) |  |  |  |  |  |  |
| Well Water | Depth |  | Type | Coring |  |  | Depth | Level Legend |  | Stratum Description |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Soft dark brown slightly silty PEAT (PEAT) |  |  |  |  |  |

| 8.00 |  | 8 |  |
| --- | --- | --- | --- |
|  | Soft grey slightly silty CLAY. (BEACH AND |  |  |
|  | TIDAL FLAT DEPOSITS) |  |  |

(N=15)

9

| (N=18) | 10 |
| --- | --- |

| 11.00 |  | 11 |  |
| --- | --- | --- | --- |
|  | Medium dense grey fine to coarse angular |  |  |
|  | to sub rounded sandstone GRAVEL. |  |  |
|  | (BEACH AND TIDAL FLAT DEPOSITS) |  |  |

(N=21)

12

| (N=27) | 13 |
| --- | --- |

|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 14 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Type/FI TCR |  | SCR RQD D/R/(SPT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hole Diameter |  |  |  |  | Casing Diameter |  |  |  | Chiselling |  |  | Inclination and Orientation |  |  |  |  |  |  |  | Drilling Flush |  |  |  |  |  |
| Depth Base |  | Diameter |  |  | Depth Base | Diameter |  | Depth Top | Depth Base | Duration | Tool | Depth Top | Depth Base |  | Inclination |  | Orientation | Depth Top | Depth Base | Type | Colour | Min (%) | Max (%) |  |  |
| 30.50m |  | 412F |  |  | 25.00m | ODEX |  |  |  |  |  |  |  |  | Vertical |  |  |  |  | Air |  |  |  |  |  |
|  |  |  |  |  |  | 115 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Remarks

Location CAT scanned and hand pitted prior to drilling.

Rotary Log

| Project Name: SA1 Bridge Investigation |  |  |  |  | Client: Novus Construction |  |  |  |  |  | Date: 01/11/2019 - 06/11/2019 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location: SA1 Swansea |  |  |  |  | Contractor: Apex Drilling Services Ltd. |  |  |  |  |  |  |  |  |  |  |  |
| Project No. : 12345 |  |  |  |  | Crew Name: Gary Williams / Owen Jones |  |  |  |  |  | Drilling Equipment: Fraste |  |  |  |  |  |
| Borehole Number |  |  | Hole Type |  |  |  | Level |  |  | Logged By | Scale |  | Page Number |  |  |  |
| BH002 |  |  | OH/RC |  |  |  |  |  | Anonymous |  | 1:35 |  | Sheet 3 of 5 |  |  |  |
|  | (m) |  | /FI | TCR SCR RQDDiameterRecovery(SPT) |  |  | (m) | (m) |  |  |  |  |  |  |  |  |
| Well Water | Depth |  | Type | Coring |  |  | Depth | Level Legend |  |  | Stratum Description |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Medium dense grey fine to coarse angular |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | to sub rounded sandstone GRAVEL. |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | (BEACH AND TIDAL FLAT DEPOSITS) |  |  |  |  |  |  |
|  |  |  |  |  |  | (N=26) | 14.50 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Firm to stiff grey sandy slightly gravelly |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | SILT with occasional shell fragments. |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | (BEACH AND TIDAL FLAT DEPOSITS) |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 15 |  |  |  |

| (N=25) | 16 |
| --- | --- |

17

(N=50)

| 18.00 |  | 18 |  |
| --- | --- | --- | --- |
|  | Stiff brownish grey very sandy SILT with |  |  |
|  | rare grey sandstone boulders. Sand |  |  |
|  | brownish grey fine to medium. (BEACH |  |  |
|  | AND TIDAL FLAT DEPOSITS) |  |  |

19

|  |  |  |  |  |  |  |  | (N=38) |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 21 |  |  |  |  |
|  |  |  |  |  | Type/FI TCR |  | SCR RQD D/R/(SPT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hole Diameter |  |  | Casing Diameter |  |  |  | Chiselling |  |  | Inclination and Orientation |  |  |  |  |  |  |  | Drilling Flush |  |  |  |  |  |  |
| Depth Base |  | Diameter | Depth Base | Diameter |  | Depth Top | Depth Base | Duration | Tool | Depth Top | Depth Base |  | Inclination |  | Orientation | Depth Top | Depth Base | Type | Colour | Min (%) | Max (%) |  |  |  |
| 30.50m |  | 412F | 25.00m | ODEX |  |  |  |  |  |  |  |  | Vertical |  |  |  |  | Air |  |  |  |  |  |  |
|  |  |  |  | 115 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Remarks

Location CAT scanned and hand pitted prior to drilling.

Rotary Log

| Project Name: SA1 Bridge Investigation |  |  |  |  | Client: Novus Construction |  |  |  |  |  |  |  | Date: 01/11/2019 - 06/11/2019 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location: SA1 Swansea |  |  |  |  | Contractor: Apex Drilling Services Ltd. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Project No. : 12345 |  |  |  |  | Crew Name: Gary Williams / Owen Jones |  |  |  |  |  |  |  | Drilling Equipment: Fraste |  |  |  |  |  |
| Borehole Number |  |  | Hole Type |  |  |  | Level |  | Logged By |  |  |  | Scale |  |  | Page Number |  |  |
| BH002 |  |  | OH/RC |  |  |  |  |  | Anonymous |  |  |  | 1:35 |  |  | Sheet 4 of 5 |  |  |
|  | (m) |  | /FI | TCR SCR RQDDiameterRecovery(SPT) |  |  | (m) | (m) |  |  |  |  |  |  |  |  |  |  |
| Well Water | Depth |  | Type | Coring |  |  | Depth | Level Legend |  |  |  |  | Stratum Description |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Stiff brownish grey very sandy SILT with |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | rare grey sandstone boulders. Sand |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | brownish grey fine to medium. (BEACH |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | AND TIDAL FLAT DEPOSITS) |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Rare greyish brown sandstone boulders. |  |  |  |  |  |  |
|  |  |  |  |  |  | 50 (25 for 80mm/50 for 70mm) |  |  |  |  |  |  |  |  |  | 22 |  |  |

23

50 (25 for 20mm/50 for 35mm)

24

|  |  |  |  |  |  |  |  |  |  |  |  |  | 50 (25 for 20mm/50 for 35mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 25 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Sub-vertical |  | undulating rough fracture. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | frequent close sub |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | 25.30 |  |  |  |  |  |  |  |  |  |  |  | Sheared zone closed, |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | vertical fractures with quartz infill on surface. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Medium strong light grey micaceous fine to |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | medium grained SANDSTONE with |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 25.00 - 26.50 |  |  |  |  |  | 57 |  | 33 | 9 |  |  |  |  |  |  |  |  |  |  |  | occasional coal laminations. Dipping at 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | degrees. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Non Intact Core Zone |  |  |  |  | sub vertical planar |  |  |  |  | 26 |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Extremely closely spaced |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | rough with quartz infill on surface. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Non Intact Core Zone |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Non Intact Core Zone |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 26.50 - 27.50 |  |  |  |  |  | 55 |  | 18 | 0 |  |  |  |  |  |  |  |  |  |  |  | Non Intact Core Zone |  |  |  |  |  |  |  |  |  | 27 |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Fracture set 1 - Sub vertical (75 degrees) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | undulating rough trace of quartz on fracture |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | surface. |  |  | Fracture Set 2 - Sub |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | horizontal (25 degrees), planar smooth |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | commonly seen on the coal laminations. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Vertical planar rough fracture. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |  |  |
|  |  |  |  |  |  |  | Type/FI TCR |  |  | SCR | RQD D/R/(SPT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hole Diameter |  |  |  | Casing Diameter |  |  |  |  |  |  | Chiselling |  |  |  |  |  | Inclination and Orientation |  |  |  |  |  |  |  |  |  |  | Drilling Flush |  |  |  |  |  |  |  |  |
| Depth Base |  | Diameter |  | Depth Base | Diameter |  |  | Depth Top |  | Depth Base |  | Duration |  | Tool |  |  | Depth Top | Depth Base |  | Inclination |  |  | Orientation | Depth Top |  | Depth Base |  | Type |  | Colour | Min (%) |  | Max (%) |  |  |  |
| 30.50m |  | 412F |  | 25.00m | ODEX |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Vertical |  |  |  |  |  |  |  | Air |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 115 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Remarks

Location CAT scanned and hand pitted prior to drilling.

Rotary Log

| Project Name: SA1 Bridge Investigation |  |  |  |  | Client: Novus Construction |  |  |  |  |  |  |  | Date: 01/11/2019 - 06/11/2019 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location: SA1 Swansea |  |  |  |  | Contractor: Apex Drilling Services Ltd. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Project No. : 12345 |  |  |  |  | Crew Name: Gary Williams / Owen Jones |  |  |  |  |  |  |  | Drilling Equipment: Fraste |  |  |  |  |  |  |
| Borehole Number |  |  | Hole Type |  |  |  | Level |  | Logged By |  |  |  |  | Scale |  |  | Page Number |  |  |
| BH002 |  |  | OH/RC |  |  |  |  |  | Anonymous |  |  |  |  | 1:35 |  |  | Sheet 5 of 5 |  |  |
|  | (m) |  | /FI | TCR SCR RQDDiameterRecovery(SPT) |  |  | (m) | (m) |  |  |  |  |  |  |  |  |  |  |  |
| Well Water | Depth |  | Type | Coring |  |  | Depth | Level Legend |  |  |  |  |  | Stratum Description |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Medium strong light grey micaceous fine to |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | medium grained SANDSTONE with |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | occasional coal laminations. Dipping at 25 |  |  |  |  |  |  |  |
|  | 27.50 - 29.00 |  |  | 91 77 33 |  |  |  |  |  |  |  | degrees. |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Non Intact Core Zone |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Frequent coal laminations. |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Non Intact Core Zone |  |  |  | fractured all closed with |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Sheared zone heavily |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | quartz infill. |  |  |  |  |  | 29 |  |  |

| 29.00 - 30.50 | 87 69 33 |
| --- | --- |

Non Intact Core Zone

30

| 30.50 | End of Borehole at 30.500m |  |
| --- | --- | --- |

31

32

33

34

|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 35 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Type/FI TCR |  | SCR RQD D/R/(SPT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hole Diameter |  | Casing Diameter |  |  |  | Chiselling |  |  | Inclination and Orientation |  |  |  |  |  | Drilling Flush |  |  |  |  |  |
| Depth Base | Diameter | Depth Base | Diameter |  | Depth Top | Depth Base | Duration | Tool | Depth Top | Depth Base | Inclination | Orientation | Depth Top | Depth Base | Type | Colour | Min (%) | Max (%) |  |  |
| 30.50m | 412F | 25.00m | ODEX |  |  |  |  |  |  |  | Vertical |  |  |  | Air |  |  |  |  |  |
|  |  |  | 115 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Remarks

Location CAT scanned and hand pitted prior to drilling