#include <main.h>

#define LCD_ENABLE_PIN PIN_D2

#define LCD_RS_PIN PIN_D0

#define LCD_RW_PIN PIN_D1

#define LCD_DATA4 PIN_D4

#define LCD_DATA5 PIN_D5

#define LCD_DATA6 PIN_D6

#define LCD_DATA7 PIN_D7

#include <lcd.c>

#define delay 200	// defined delay 200ms

// define function/subroutine prototypes

void analog(void);

// void print2lcd(int);

void run_motor(void);

void button_press(void);

void lcd_lights(void);

void kitt_mode(void);

void button_off(void);

void button_on(void);

void kitt_text(void);

// declare global variables

unsigned int adc, remainder, new_adc;

unsigned int knightrider[14] = {0x01, 0x02, 0x03, 0x04, 0x05, 0x06, 0x07, 0x08, 0x07, 0x06, 0x05, 0x04, 0x03, 0x02};

static int off = 0, on = 0;

void main()

{

setup_adc_ports(sAN0);							// initialise adc channel

setup_adc(ADC_CLOCK_INTERNAL|ADC_TAD_MUL_0);				// setup adc operation

lcd_init();									// initialise the LCD

while(TRUE)

{

analog();																									// read the analog input

/*print2lcd(1);

delay_ms(1000);

print2lcd(2);

delay_ms(1000);

print2lcd(3);

delay_ms(1000);

print2lcd(4);

delay_ms(1000);*/

if(input(PIN_A2)) run_motor();		// If SW1-A set, turns motor based on pentiometer value

if(input(PIN_A3)) lcd_lights();		// If SW1-B set, light up LEDs to show ADC value

if(input(PIN_A4)) button_press();	// If SW1-C set, when button depressed run motor

if(input(PIN_A5)) {			// If SW1-D set, run kitt_mode light sequence

kitt_text();

kitt_mode();

}

}

}

void analog(void)

{

set_adc_channel(0);							//read adc channel 0

delay_us(30);								// wait 30uS

adc = read_adc();							// store value in variable ADC

}

/* void print2lcd(int line)

{

switch (line)

{

case 1: 	lcd_gotoxy(1,1);									// line 1

printf(lcd_putc," Welcome to the");

break;

case 2: 	lcd_gotoxy(1,2);									// line 2

printf(lcd_putc," Assignment for  the ");

break;

case 3: 	lcd_gotoxy(21,1);									// line 3

printf(lcd_putc," Embedded C ");

break;

case 4:		lcd_gotoxy(21,2);

printf(lcd_putc," Module       Line %d",line);

default : 	break;

}

}

*/

void run_motor(void)

{	/* Turns Motor A based on the value of RC1, this value can be seen in the LCD panel alongside the direction the motor is turning. */

{

if(adc < 75)

{ 	// If the value of the adc is less that 75, drive the motor anti-clockwise

output_low(PIN_C0);

output_high(PIN_C1);

lcd_gotoxy(4,2);				// Line 2 of the LCD - 4 to centre text

printf(lcd_putc, "Anti-clockwise");	// Print "Anti-clockwise" on the line

}

if((adc > 76) && (adc < 174))

{ 	// If the value of the adc is between 76-174, stop the motor

output_low(PIN_C0);

output_low(PIN_C1);

lcd_gotoxy(4,2);				// Line 2 of the LCD - 4 to centre text

printf(lcd_putc, "Motor Stopped ");	// Print "Clockwise" on the line

}

if(adc > 175)

{ 	// If the value of the adc is more that 175, drive the motor clockwise

output_high(PIN_C0);

output_low(PIN_C1);

lcd_gotoxy(4,2);				// Line 2 of the LCD - 4 to centre text

printf(lcd_putc, "  Clockwise   ");	// Print "Motor Stopped" on the line

}

lcd_gotoxy(6,1);				// Line 1 of the LCD - 6 to centre the text on the screen

printf(lcd_putc, "adc = %3u", adc);	       	// print the adc value on the LCD

lcd_gotoxy(22,1);				// Line 3

printf(lcd_putc, "                  ");	// Clear line 3

lcd_gotoxy(26,2);				// Line 4

printf(lcd_putc, "         ");			// Clear line 4

}

}

void lcd_lights(void) {

/* Shows the digital output of the ADC reading RV1 on the LEDs. */

lcd_gotoxy(6,1);				// Line 1 of the LCD - 6 to centre the text on the screen

printf(lcd_putc, "adc = %3u", adc);	       	// print the adc value on the LCD

lcd_gotoxy(4,2);				// Line 2

printf(lcd_putc, "              ");   	// Clear line 2

lcd_gotoxy(22,1);				// Line 3

printf(lcd_putc, "                  ");	// Clear line 3

lcd_gotoxy(26,2);				// Line 4

printf(lcd_putc, "         ");		// Clear line 4

printf("adc = %3u \n\r", adc);		// Print adc to terminal

remainder = adc % 2;				// Modulo operator - Divide the adc value by 2 and store the remainder in a new variable (will be either 1 or 0)

new_adc = adc / 2;				// Divide the adc value by 2 and store in new variable

if(remainder == 1) output_high(PIN_B0);	// If the remainder is 1 turn on LED0

if(remainder == 0) output_low(PIN_B0);	// If the remainder is 0 turn off LED0

remainder = new_adc % 2;			// Modulo operator on the new_adc value to store the remainder (1 or 0)

new_adc = new_adc / 2;			// Divide new_adc value by 2 and store into new_adc

if(remainder == 1) output_high(PIN_B1);	// If the remainder is 1 turn on LED1

if(remainder == 0) output_low(PIN_B1);	// If the remainder is 0 turn off LED1

remainder = new_adc % 2;			// Modulo operator on the new_adc value to store the remainder (1 or 0)

new_adc = new_adc / 2;			// Divide new_adc value by 2 and store into new_adc

if(remainder == 1) output_high(PIN_B2);	// If the remainder is 1 turn on LED2

if(remainder == 0) output_low(PIN_B2);	// If the remainder is 0 turn off LED2

remainder = new_adc % 2;			// Modulo operator on the new_adc value to store the remainder (1 or 0)

new_adc = new_adc / 2;			// Divide new_adc value by 2 and store into new_adc

if(remainder == 1) output_high(PIN_B3);	// If the remainder is 1 turn on LED3

if(remainder == 0) output_low(PIN_B3);	// If the remainder is 0 turn off LED3

remainder = new_adc % 2;			// Modulo operator on the new_adc value to store the remainder (1 or 0)

new_adc = new_adc / 2;			// Divide new_adc value by 2 and store into new_adc

if(remainder == 1) output_high(PIN_B4);	// If the remainder is 1 turn on LED4

if(remainder == 0) output_low(PIN_B4);	// If the remainder is 0 turn off LED4

remainder = new_adc % 2;			// Modulo operator on the new_adc value to store the remainder (1 or 0)

new_adc = new_adc / 2;			// Divide new_adc value by 2 and store into new_adc

if(remainder == 1) output_high(PIN_B5);	// If the remainder is 1 turn on LED5

if(remainder == 0) output_low(PIN_B5);	// If the remainder is 0 turn off LED5

remainder = new_adc % 2;			// Modulo operator on the new_adc value to store the remainder (1 or 0)

new_adc = new_adc / 2;			// Divide new_adc value by 2 and store into new_adc

if(remainder == 1) output_high(PIN_B6);	// If the remainder is 1 turn on LED6

if(remainder == 0) output_low(PIN_B6);	// If the remainder is 0 turn off LED6

remainder = new_adc % 2;			// Modulo operator on the new_adc value to store the remainder (1 or 0)

new_adc = new_adc / 2;			// Divide new_adc value by 2 and store into new_adc

if(remainder == 1) output_high(PIN_B7);	// If the remainder is 1 turn on LED7

if(remainder == 0) output_low(PIN_B7);	// If the remainder is 0 turn off LED7

}

void button_press(void)

{   /* Pressing the button will make Motor A rotate clockwise as long as the button is depressed, and the motor will stop when released. */

if(input(PIN_A1)) {				// If button is pressed rotate motor clockwise

output_high(PIN_C0);

output_low(PIN_C1);

lcd_gotoxy(22,1);				// Line 3 of the LCD

printf(lcd_putc, "  Button pressed  ");	// Print "Button pressed"

button_on(); 				// Run function to print "Button pressed" once to virtual terminal

}

else

{

output_low(PIN_C0);			// Stop the motor running if button not pressed

output_low(PIN_C1);

lcd_gotoxy(22,1);				// Line 3 of the LCD

printf(lcd_putc, "Button not pressed");	// Print "Button not pressed"

button_off();				// Run function to print "Button not pressed" once to virtual terminal

}

lcd_gotoxy(6,1);				// Line 1

printf(lcd_putc, "         ");		// Clear line 1

lcd_gotoxy(4,2);				// Line 2

printf(lcd_putc, "              ");   	// Clear line 2

lcd_gotoxy(26,2);				// Line 4

printf(lcd_putc, "         ");		// Clear line 4

}

void button_off(void)

{ 	/* Checks whethere the button is not pressed */

if (!off)		// Checks 'off' variable for null

{

printf("Button not pressed \n\r");	// Print to virtual terminal

off = 1;					// Change off state to 1

on = 0;					// Change on state to 0

}

}

void button_on(void)

{	/* Checks whether the button is pressed. */

if (!on)	// Checks 'on'  variable for null

{

printf("Button pressed \n\r");		// Print to virtual terminal

on = 1;					// Change on state to 1

off = 0;					// Change off state to 0

}

}

void kitt_text(void)

{	/* Prints "Kitt mode" to virtual terminal when SW1-D selected at the switch. */

static int kitt;			// Local variable to ensure "Kitt mode" only printed once

if (!kitt)				// If kitt is null, then print "Kitt mode" to virtual terminal

{

printf("Kitt mode \n\r");

kitt = 1;			// Change kitt variable to 1 so that it does not repeat until SW1-D selected once again

}

}

void kitt_mode(void)

{   /* Loops over sequence in array (knightrider) to light up LEDs Knightrider style.  */

lcd_gotoxy(6,1);				// Line 1

printf(lcd_putc, "         ");		// Clear line 1

lcd_gotoxy(4,2);				// Line 2

printf(lcd_putc, "              ");   	// Clear line 2

lcd_gotoxy(22,1);				// Line 3

printf(lcd_putc, "                  ");	// Clear line 3

lcd_gotoxy(26,2);				// Line 4, centred

printf(lcd_putc, "Kitt mode");		// Print "Kitt mode" to LCD line 4

// Turn off all LEDs (Required if SW1-B has been used first!)

output_low(PIN_B7);		// output_low = LED7 on

output_low(PIN_B6);		// output_low = LED6 off

output_low(PIN_B5);		// output_low = LED5 off

output_low(PIN_B4);		// output_low = LED4 off

output_low(PIN_B3);		// output_low = LED3 off

output_low(PIN_B2);		// output_low = LED2 off

output_low(PIN_B1);		// output_low = LED1 off

output_low(PIN_B0);		// output_low = LED0 off

for (int q = 0; q < 14; q++) 		// Loop 14 times (length of the knightrider sequence)

{

switch(knightrider[q]) 			// For each loop, look at the sequence value and run the case for each part of the sequence to light up the LEDs in sequence order.

{

case 0x01:  output_high(PIN_B7);		// output_high = LED7 on

output_low(PIN_B6);		// output_low = LED6 off

delay_ms(delay);		// 200ms delay

break;

case 0x02:  output_low(PIN_B7);		// output_low = LED7 off

output_high(PIN_B6);		// output_high = LED6 on

output_low(PIN_B5);		// output_low = LED5 off

delay_ms(delay);		// 200ms delay

break;

case 0x03:  output_low(PIN_B6);		// output_low = LED6 off

output_high(PIN_B5);		// output_high = LED5 on

output_low(PIN_B4);		// output_low = LED4 off

delay_ms(delay);		// 200ms delay

break;

case 0x04:  output_low(PIN_B5);		// output_low = LED5 off

output_high(PIN_B4);		// output_high = LED4 on

output_low(PIN_B3);		// output_low = LED3 off

delay_ms(delay);		// 200ms delay

break;

case 0x05:  output_low(PIN_B4);		// output_low = LED4 off

output_high(PIN_B3);		// output_high = LED3 on

output_low(PIN_B2);		// output_low = LED2 off

delay_ms(delay);		// 200ms delay

break;

case 0x06:  output_low(PIN_B3);		// output_low = LED3 off

output_high(PIN_B2);		// output_high = LED2 on

output_low(PIN_B1);		// output_low = LED1 off

delay_ms(delay);		// 200ms delay

break;

case 0x07:  output_low(PIN_B2);		// output_low = LED2 off

output_high(PIN_B1);		// output_high = LED1 on

output_low(PIN_B0);		// output_low = LED0 off

delay_ms(delay);		// 200ms delay

break;

case 0x08:  output_low(PIN_B1);		// output_low = LED1 off

output_high(PIN_B0);		// output_high = LED0 on

delay_ms(delay);		// 200ms delay

break;

default:	break;

}

}

}

Comments

2.. 20   3.. 30    4.. 40   5.. 10

***************************************

2.. Your code has a number of functions (subroutines) and all work. Your statements indicate what each line does 75%

3.. Your program flows nicely 75%

4.. Your program achieves the desired results. 75%

5.. Your program is nicely laid out and well indented. 75%