## Front cover

BSc (Hons.)/BEng (Hons.) Ordnance, Munitions and Explosives ACDA 5012– Level 5 Research Methods (OME): Research Proposal

Authored by: Mike Peck Student number: 1907404 Date: May 2022

## ontents

Contents

Front cover	1

Contents	2

1.	Introduction	3

2. Background to the Proposed Research	4

3.	Aims & Objectives	5

4.	Experimental Design	7

4.1.	Outline of Experimental Procedure	7

4.2.	Data Collection	7

4.3.	Data Evaluation	7

4.4.	Risks & Limitations	7

4.5.	Ethical Issues	8

5.	Resources & Planning	9

5.1.	Resources Required – Physical and Human Resources	9

5.2.	Gantt Chart	9

6.	Appendix	10

## Introduction

In any explosives train it is vitally important that each part of the train provides enough energy and the right impetus to intiate the next stage in the train. This is typically refered to as the Activation Energy (AE). However it is not just the amount of energy that is important, but also the method by which the energy is transferred or delivered.

Aircraft are often subjected to threats, which if not sucessfully countered could lead to loss of life and the air platform. For countermeasures to be safely transported, stored and utilised data needs to be obtained to ascertain pyrotechic sample AE. The project propsal as such is two fold:

- To design and commision an appartus with which to measure/calculate AE of pyrotechnic samples

- To carry out a suite of tests on said apparatus to achieve reproduceable, consistent results

## 2. Background to the Proposed Research

Similar topics have been investigated in the past which involve the use of lasers to provide the energy needed for ignition. However although there are benefits to this method, such as the the pulse width or beam ‘footprint’ can be well controlled and the duration of the pulse measured and therefore the amount of energy transmitted can be easily calculated there are a number of issues with this method. For example the beam may lead to hotspot creation in the material meaning there may not be an uneven ignition across the surface of the material. The most prohibitive aspect to this method however, would be the cost of obtaining the apparatus which is well outside the budget for this project. Then the additional costs of training or retaining staff that are Suitably Qualified and Experienced Personnel (SQEP) to calibrate, maintain and operate the apparatus.

It is therefore proposed that firstly I will design and instigate the build and commissioning of a hot bar method of ignition, in which a sacrificial metal bar is heated by means of electrical current and heat generated as work done against the metals electrical resistance. Several variables and factors will need to be considered, in order to ascertain as accurate an energy reading as possible; these will be discussed later on.

Once a working test apparatus has been built it will then need testing and as such I will also need to develop a System of Test (SoT) or suite of commissioning tests.

## Aims & Objectives

The overarching aim of this project is to design, produce and test an apparatus to determine the ignitability of pyrotechnic samples by investigating Activation Energy.  The purpose of this aim, as stated in the introduction is to allow accurate estimation of activation energy in order to produce well matched pairings of compositions within the explosives train of pyrotechnic samples. For example so the required amount of initiation energy (AE) is supplied to each part of the explosives train in turn.

To achieve this overarching aim it is necessary to break down the project into smaller aims and objectives, not only to make the project more manageable, but also to provide Key Performance Indicators (KPIs) to demonstrate progress and deliverables to the Project Team (PT) and Project Manager (PM), to ensure continued interest and funding in the project. These aims and objectives should be Specific, Measurable, Achievable, Realistic and Time bound (SMART) and are as follows:

Aim 1: Investigate suitability of different types of sample

Objectives for aim 1:

Objective A: investigate 3 types of priming and main charge to compare cost of suitable samples within the first 2 weeks of the project.

Objective B: For the cheapest 2 samples selected for both priming and main charge compositions in Objective A compare properties of samples to identify notable characteristics i.e. heat of combustion, toxic combustion products etc – factors that could affect how expensive the desired testing could be and where they need to be tested, within first 3 weeks of project.

Aim 2: Design Test Apparatus

Objectives for Aim 2:

Objective A: Find three different mechanisms for introducing the sample to the hot bar system within the first 4 weeks of project start date.

Objective B: Investigate and compare two different methods for hot bar pre-heating feedback loop/control system in the first 6 weeks of the project.

Aim 3: Identify test environment

Objectives for Aim 3:

Objective A: Within the first 8 weeks of the project and as a result of information gathered for Aims 1 & 2 locate at least possible environments in which to test selected samples

Aim 4: Collect, analyse and evaluate data

Objectives for Aim 4:

Objective A: From week 8 to week 12 data should be collected by way of thermal imaging cameras (to establish initiation start point i.e. presence of hot spot in sample), High Speed Video (HSV) to show physical changes in samples i.e. colour, smoke etc and any other data acquisition as required by customer

Objective B: 12 to 14 weeks from start of project all data sets will be compared for consistency and to highlight any data needing repartition

Objective C: Weeks 14 to 16 will comprise of data evaluation to compare to already available data, from burn tests and Small Scale Hazard Tests (SSHT)

## Experimental Design

## Outline of Experimental Procedure

The experimental procedure will initially be concerned with testing of the apparatus, with the ultimate aim being that the apparatus is able to ignite the selected sample (where ignition is defined as - a comparable relative thermal output as is achieved by the same sample ignited by any other manner). Once this has been achieved other variables and factors need to as controlled as possible to allow for comparable results, some of these factors and variables are as follows:

- Ambient temperature (Temperature of ignition)

- Pre-heating temperature

- Humidity

- Gaseous reaction products

- Conductivity of sample and hot bar

- Particle size (is sample powdered, granular, pressed pellet etc)

- Is there an aspect of confinement (i.e. set sample dimensions and set bar distance, to avoid inconsistent compression of samples)

- Intimacy of sample and hot bar i.e. effect of differences in mating surfaces and surface finish consistency

This list is not exhaustive, but the aim would be to control and record as many of these factors as possible in order to achieve reproducible and comparable results.

## Data Collection

Once the apparatus is achieving reproducible results the bulk of the data can be collected, by way of High Speed Video and thermal imaging, as well as the variable data as above. This data can then be stored to allow analysis to be carried out.

## Data Evaluation

Initially a Bruceton Staircase Method can be used to find the right range of data and other statistical tests can be applied to the data to show how reliable it is in comparison to other known data.

## Risks & Limitations

There are risks and limitations to this project. The limitations may be due to the factors and variables mentioned in section 4.1 or due to the resources, budget, time allocation and technical possibility/capability. The largest risk being that the whole process is not achievable in the time frame and therefore would give an intermediate aim of achieving a working apparatus that could be used to achieve the required testing with future recommendations.

## Ethical Issues

For ethical issues please see the attached Ethics for in section 6. Appendix.

## Resources & Planning

## Resources Required – Physical and Human Resources

Include all camera equipment (HSV and thermal), use of labs/workshops, PCs for data collection and analysis, as well as hard drives are all already available on site. However SQEP personnel will be required both for the actual fabrication and operation of the test apparatus and operation of data collection equipment, as well as explosives workers to collect/deliver samples. These personnel cost varying rates per day but are easily upward of £50 per day.

The buildings themselves will also need to be booked out and of course health and safety aspects assessed with Hazard ID and Risk Assessments being carried out and a Safe Scheme of work will all need to be written, reviewed and recorded. All of which could quite easily cost about £3000.

Then there is the added cost of the samples and the sacrificial hot bar and associated equipment and materials for manufacture of the test apparatus.

## Gantt Chart

Below is an indication of the ideal estimate for the process of the project to be carried out. I should be understood that at this time there are many unknowns, such as time required to obtain authorisation to carry out the project, lead time on materials, lead time on workshop capability access to build the required apparatus and many other unforeseen complications that will undoubtedly present themselves. To help mitigate against these unforeseen events the scale of the project is estimated as follows:

| Aim: | Objective: | Ideal duration (weeks): |
| --- | --- | --- |
| 1 | A | 2 |
|  | B | 1 |
| 2 | A | 1 |
|  | B | 2 |
| 3 | A | 2 |
| 4 | A | 4 |
|  | B | 2 |
|  | C | 2 |

Giving an ambitious total of 16 weeks or about 4 months, which then allows a 2 month contingency for the unforeseen events as mentioned above.

## Appendix

APPLICATION FOR ETHICAL APPROVAL

In order for research to result in benefit and minimise risk of harm, it must be conducted ethically.

The University follows the OECD Frascati manual definition of research activity: “creative work undertaken on a systematic basis in order to increase the stock of knowledge, including knowledge of man, culture and society, and the use of this stock of knowledge to devise new applications”. As such this covers activities undertaken by members of staff, postgraduate research students, and both taught postgraduate and undergraduate students working on dissertations/projects.

The individual undertaking the research activity is known as the “principal researcher”.

This form must be completed and approved prior to undertaking any research activity.

SECTION A: About You (Principal Researcher)

| 1 | Full Name: | Mike Peck |
| --- | --- | --- |
| 2 | Student Number: | 1907404 |
| 3 | Email address: | 1907404@student.uwtsd.ac.uk |
| 4 | Programme of Study: | OME Apprenticeship Programme |
| 5 | Director of Studies/Supervisor: | Phil Pitcher |

SECTION B:  Internal and External Ethical Guidance Materials

|  | Please list the core ethical guidance documents that have been referred to during the completion of this form (including any discipline-specific codes of research ethics, location-specific dodes of research ethics, and also any specific ethical guidance relating to the proposed methodology). Please tick to confirm that your research proposal adheres to these codes and guidelines. You may add rows to this table if needed. |  |
| --- | --- | --- |
| 1 | UWTSD Research Ethics & Integrity Code of Practice | ☒ |
| 2 | UWTSD Research Data Management Policy | ☒ |
| 3 |  | ☐ |

SECTION C: Details of Research Activity

| 1 | Indicative title: | Ignitability Study of Pyrotechnic Compositions |  |  |
| --- | --- | --- | --- | --- |
| 2 | Proposed start date: | 01/09/2023 | Proposed end date: | 01/01/2024 |
|  | Introduction to the Research (maximum 300 words in each section) Ensure that you write for a Non-Specialist Audience when outlining your response to the three points below: - Purpose of Research Activity - Proposed Research Question - Aims of Research Activity - Objectives of Research Activity Demonstrate, briefly, how Existing Research has informed the proposed activity and explain - What the research activity will add to the body of knowledge - How it addresses an area of importance. |  |  |  |
| 3 | Purpose of Research Activity To design a test apparatus to measure the ignitability of a pyrotechnic sample (this box should expand as you type) |  |  |  |
| 4 | Research Question Is it possible to measure the ignitability of a pyrotechnic sample (this box should expand as you type) |  |  |  |
| 5 | Aims of Research Activity To build and obtain reproducible/comparable results for activation energy of pyrotechnic samples from a hot bar test apparatus (this box should expand as you type) |  |  |  |
| 6 | Objectives of Research Activity Aim 1: Investigate suitability of different types of sample Objective A: investigate 3 types of priming and main charge to compare cost of suitable samples within the first 2 weeks of the project. Objective B: For the cheapest 2 samples selected for both priming and main charge compositions in Objective A compare properties of samples to identify notable characteristics i.e. heat of combustion, toxic combustion products etc – factors that could affect how expensive the desired testing could be and where they need to be tested, within first 3 weeks of project. Aim 2: Design Test Apparatus Objective A: Find three different mechanisms for introducing the sample to the hot bar system within the first 4 weeks of project start date. Objective B: Investigate and compare two different methods for hot bar pre-heating feedback loop/control system in the first 6 weeks of the project. Aim 3: Identify test environment Objective A: Within the first 8 weeks of the project and as a result of information gathered for Aims 1 & 2 locate at least possible environments in which to test selected samples Aim 4: Collect, analyse and evaluate data Objective A: From week 8 to week 12 data should be collected by way of thermal imaging cameras (to establish initiation start point i.e. presence of hot spot in sample), High Speed Video (HSV) to show physical changes in samples i.e. colour, smoke etc and any other data acquisition as required by customer Objective B: 12 to 14 weeks from start of project all data sets will be compared for consistency and to highlight any data needing repartition Objective C: Weeks 14 to 16 will comprise of data evaluation to compare to already available data, from burn tests and Small Scale Hazard Tests (SSHT) (this box should expand as you type) |  |  |  |
|  | Proposed data collection methods (maximum 600 words) Provide a brief summary of all the methods that may be used in the research activity to collect data, making it clear what specific techniques may be used. If methods other than those listed in this section are deemed appropriate later, additional ethical approval for those methods will be needed. You do not need to justify the methods here, but should instead describe how you intend to collect the data necessary for you to complete your project. |  |  |  |
| 7 | Data collection to be carried out by use of High Speed Video and IR Thermal imaging cameras (this box should expand as you type) |  |  |  |

SECTION D: Scope of Research Activity

|  | Will the research activity include: | YES | NO |
| --- | --- | --- | --- |
| 1 | Use of a questionnaire or similar research instrument? | ☐ | ☒ |
| 2 | Use of interviews? | ☐ | ☒ |
| 3 | Use of focus groups? | ☐ | ☒ |
| 4 | Use of participant diaries? | ☐ | ☒ |
| 5 | Use of video or audio recording? | ☒ | ☐ |
| 6 | Use of computer-generated log files? | ☒ | ☐ |
| 7 | Participant observation with their knowledge? | ☐ | ☒ |
| 8 | Participant observation without their knowledge? | ☐ | ☒ |
| 9 | Access to personal or confidential information without the participants’ specific consent? | ☐ | ☒ |
| 10 | Administration of any questions, test stimuli, presentation that may be experienced as physically, mentally or emotionally harmful / offensive? | ☐ | ☒ |
| 11 | Performance of any acts which may cause embarrassment or affect self-esteem? | ☐ | ☒ |
| 12 | Investigation of participants involved in illegal activities? | ☐ | ☒ |
| 13 | Use of procedures that involve deception? | ☐ | ☒ |
| 14 | Administration of any substance, agent or placebo? | ☐ | ☒ |
| 15 | Working with live vertebrate animals? | ☐ | ☒ |
| 16 | Procedures that may have a negative impact on the environment? | ☐ | ☒ |
| 17 | Other primary data collection methods. Please indicate the type of data collection method(s) below. | ☐ | ☒ |
|  | Details of any other primary data collection method: (this box should expand as you type) |  |  |

If you have ticked NO to every question then the research activity is (ethically) low risk and you may skip section E and continue to section F.

If YES to any question, then no research activity should be undertaken until full ethical approval has been obtained.

SECTION E: Intended Participants

|  | Who are the intended participants: | YES | NO |
| --- | --- | --- | --- |
| 1 | Students or staff at the University? | ☐ | ☒ |
| 2 | Adults (over the age of 18 and competent to give consent)? | ☒ | ☐ |
| 3 | Vulnerable adults? | ☐ | ☒ |
| 4 | Children and Young People under the age of 18? (Consent from Parent, Carer or Guardian will be required) | ☐ | ☒ |
| 5 | Prisoners? | ☐ | ☒ |
| 6 | Young offenders? | ☐ | ☒ |
| 7 | Those who could be considered to have a particularly dependent relationship with the investigator or a gatekeeper? | ☐ | ☒ |
| 8 | People engaged in illegal activities? | ☐ | ☒ |
| 9 | Others. Please indicate the participants below, and specifically any group who may be unable to give consent. | ☐ | ☒ |
|  | Details of any other participant groups: Complete this only if your participants cannot give consent. This includes animals (this box should expand as you type) |  |  |

|  | Participant numbers and source Provide an estimate of the expected number of participants. How will you identify participants and how will they be recruited? |  |
| --- | --- | --- |
| 10 | How many participants are expected? | 2-3 for build, and data collection (this box should expand as you type) |
| 11 | Who will the participants be? | SQEP DSTL staff (this box should expand as you type) |
| 12 | How will you identify the participants? | Via team project meeting (stakeholder meeting) (this box should expand as you type) |

|  | Information for participants: | YES | NO | N/A |
| --- | --- | --- | --- | --- |
| 13 | Will you describe the main research procedures to participants in advance, so that they are informed about what to expect? | ☒ | ☐ | ☐ |
| 14 | Will you tell participants that their participation is voluntary? | ☒ | ☐ | ☐ |
| 15 | Will you obtain written consent for participation? | ☐ | ☐ | ☒ |
| 16 | Will you explain to participants that refusal to participate in the research will not affect their treatment or education (if relevant)? | ☐ | ☐ | ☒ |
| 17 | If the research is observational, will you ask participants for their consent to being observed? | ☐ | ☐ | ☒ |
| 18 | Will you tell participants that they may withdraw from the research at any time and for any reason? | ☐ | ☐ | ☒ |
| 19 | With questionnaires, will you give participants the option of omitting questions they do not want to answer? | ☐ | ☐ | ☒ |
| 20 | Will you tell participants that their data will be treated with full confidentiality and that, if published, it will not be identifiable as theirs? | ☐ | ☐ | ☒ |
| 21 | Will you debrief participants at the end of their participation, in a way appropriate to the type of research undertaken? | ☐ | ☐ | ☒ |
| 22 | If NO to any of above questions, please give an explanation |  |  |  |
|  | You should be able to tick YES for all of these questions. If not, then explain why not in this box. Largely not required. (this box should expand as you type) |  |  |  |

|  | Information for participants: | YES | NO | N/A |
| --- | --- | --- | --- | --- |
| 24 | Will participants be paid? | ☐ | ☐ | ☒ |
| 25 | Is specialist electrical or other equipment to be used with participants? | ☐ | ☐ | ☒ |
| 26 | Are there any financial or other interests to the investigator or University arising from this study? | ☐ | ☐ | ☒ |
| 27 | Will the research activity involve deliberately misleading participants in any way, or the partial or full concealment of the specific study aims? | ☐ | ☐ | ☒ |
| 28 | If YES to any question, please provide full details |  |  |  |
|  | You should be able to tick NO for most of these questions. For any cases that you have ticked YES then provide details in this box. If you are using cameras/voice recorders to record interviews then please state that in this box. (this box should expand as you type) |  |  |  |

SECTION F: Anticipated Risks

|  | # Outline any anticipated risks that may adversely affect any of the participants, the researchers and/or the University, and the steps that will be taken to address them. |  |
| --- | --- | --- |
| 1 | Risks to participants For example: sector-specific health & safety, emotional distress, financial disclosure, physical harm, transfer of personal data, sensitive organisational information. If you have identified in section D that there are no participants then enter N/A and go skip to question 3. |  |
|  | Risk to participants: There are always risks. Do not write N/A unless you have no participants. Risk of bright lights, sound and possible burns, due to the nature of the pyrotechnic samples. (this box should expand as you type) | How you will mitigate the risk to participants: Risk will be mitigated by adhering to the RA and Safe Scheme of Work. Which would include all team members being at a safe distance if remote operation is not possible and all necessary training and PPE will be given. (this box should expand as you type) |
| 2 | If research activity may include sensitive, embarrassing or upsetting topics (e.g. sexual activity, drug use) or issues likely to disclose information requiring further action (e.g. criminal activity), give details of the procedures to deal with these issues, including any support/advice (e.g. helpline numbers) to be offered to participants. Note that where applicable, consent procedures should make it clear that if something potentially or actually illegal is discovered in the course of a project, it may need to be disclosed to the proper authorities |  |
|  | N/A (this box should expand as you type) |  |
| 3 | Risks to the investigator For example: personal health & safety, physical harm, emotional distress, risk of accusation of harm/impropriety, conflict of interest |  |
|  | Risk to the investigator: There are always risks. Do not write NA. Risk of bright lights, sound and possible burns, due to the nature of the pyrotechnic samples. (this box should expand as you type) | How you will mitigate the risk to the investigator: Risk will be mitigated by adhering to the RA and Safe Scheme of Work. Which would include all team members being at a safe distance if remote operation is not possible and all necessary training and PPE will be given. (this box should expand as you type) |
| 4 | University/institutional risks For example: adverse publicity, financial loss, data protection |  |
|  | Risk to the University: There are always risks. Do not write NA. This will be fully conducted at DSTL sites, as such will not pose a risk to the University. (this box should expand as you type) | How you will mitigate the risk to the University: Not needed (this box should expand as you type) |
| 5 | Environmental risks For example: accidental spillage of pollutants, damage to local ecosystems |  |
|  | Risk to the environment: All tests will be carried out in a controlled, independently HVAC’d specialist facility Possibility of small spills of pollutants. (this box should expand as you type) | How you will mitigate the risk to environment: All left test related materials will be properly cleared and cleaned and removed in the proper manner upon completion of tasks. (this box should expand as you type) |

SECTION G: Feedback, Consent and Confidentiality

If you have identified in section D that there are no participants then enter skip this section and continue to section H.

| 1 | Feedback What de-briefing and feedback will be provided to participants, how will this be done and when? |
| --- | --- |
|  | You don’t need to email your participants with your final report. A good alternative is to set up an email address that they will be able to contact for further details or results. The team will be notified of outcomes at de-brief meeting and via report (this box should expand as you type) |
| 2 | Informed consent Describe the arrangements to inform potential participants, before providing consent, of what is involved in participating. Describe the arrangements for participants to provide full consent before data collection begins. If gaining consent in this way is inappropriate, explain how consent will be obtained and recorded in accordance with prevailing data protection legislation. |
|  | If you are using a paper questionnaire then you should have the participants sign an appropriate consent form. These forms will count as personal data and should be noted as such in section J. If you are using an online questionnaire, then you should have a screen before the questions start that acts as a consent form, informing participants that by clicking on the NEXT button they are providing consent. N/A (this box should expand as you type) |
| 3 | Confidentiality / Anonymity Set out how anonymity of participants and confidentiality will be ensured in any outputs. If anonymity is not being offered, explain why this is the case. |
|  | Do not collect names unless you really need them. Do not name participants or organisations in any research publications (including the thesis) without their explicit permission. N/A (this box should expand as you type) |

SECTION H: Data Protection and Storage

|  | Does the research activity involve personal data (as defined by the General Data Protection Regulation 2016 “GDPR” and the Data Protection Act 2018 “DPA”)? | YES | NO |
| --- | --- | --- | --- |
| 1 | “Personal data” means any information relating to an identified or identifiable natural person (‘data subject’). An identifiable natural person is one who can be identified, directly or indirectly, in particular by reference to an identifier such as a name, an identification number, location data, an online identifier or to one or more factors specific to the physical, physiological, genetic, mental, economic, cultural or social identity of that natural person. Any video or audio recordings of participants is considered to be personal data. | ☐ | ☐ |
|  | If YES, provide a description of the data and explain why this data needs to be collected: |  |  |
| 2 | This includes audio/video data of participants, but can also include IP addresses and usernames. Names, addresses and emails also count, as do consent forms. (this box should expand as you type) |  |  |
|  | Does it involve special category data (as defined by the GDPR)? | YES | NO |
| 3 | “Special category data” means sensitive personal data consisting of information as to the data subjects’ – (a) racial or ethnic origin, (b) political opinions, (c ) religious beliefs or other beliefs of a similar nature, (d) membership of a trade union (within the meaning of the Trade Union and Labour Relations (Consolidation) Act 1992), (e) physical or mental health or condition, (f) sexual life, (g) genetics, (h) biometric data (as used for ID purposes), | ☐ | ☐ |
|  | If YES, provide a description of the special category data and explain why this data needs to be collected: |  |  |
| 4 | What counts as ‘sensitive’ will differ between cultures. Any information on behaviour that is not in accordance with cultural norms would count as sensitive personal data. (this box should expand as you type) |  |  |

|  | Will data from the research activity (collected data, drafts of the thesis, or materials for publication) be stored in any of the following ways? | YES | NO |
| --- | --- | --- | --- |
| 5 | Manual files (i.e. in paper form)? | ☒ | ☐ |
| 6 | University computers? | ☐ | ☒ |
| 7 | Private company computers? | ☐ | ☒ |
| 8 | Home or other personal computers? | ☐ | ☒ |
| 9 | Laptop computers/ CDs/ Portable disk-drives/ memory sticks? | ☒ | ☐ |
| 10 | “Cloud” storage or websites? | ☐ | ☒ |
| 11 | Other – specify: | ☐ | ☒ |
| 12 | For all stored data, explain the measures in place to ensure the security of the data collected, data confidentiality, including details of backup procedures, password protection, encryption, anonymisation and pseudonymisation: |  |  |
|  | If possible, save your data on computers that are secure and regularly backed up. Many cloud services only provide GDPR-compliant storage for business customers. An example of suitable text is given below. All manual files will be held in a secure locker appropriate to material classification, the keys or codes to which will only be help by the project lead or members of the project team, as designated. Any USB pen drives, removable hard drives and PCs used to store or transfer data, will be encrypted and password protected. (this box should expand as you type) |  |  |

|  | Data Protection |  |  |
| --- | --- | --- | --- |
|  | Will the research activity involve any of the following activities: | YES | NO |
| 13 | Electronic transfer of data in any form? | ☐ | ☒ |
| 14 | Sharing of data with others at the University outside of the immediate research team? | ☐ | ☒ |
| 15 | Sharing of data with other organisations? | ☐ | ☒ |
| 16 | Export of data outside the UK or importing of data from outside the UK? | ☐ | ☒ |
| 17 | Use of personal addresses, postcodes, faxes, emails or telephone numbers? | ☐ | ☒ |
| 18 | Publication of data that might allow identification of individuals? | ☐ | ☒ |
| 19 | If YES to any question, please provide full details, explaining how this will be conducted in accordance with the GDPR and Data Protection Act (2018) (and/or any international equivalent): |  |  |
|  | This includes data such as drafts of your thesis as well as experimental or survey data. An example of suitable text is given below. All data will be encrypted and kept in password protected cloud storage on the University Office 365 system which will not be shared. Any USB sticks used to store or transfer data will be password protected. All data transfers will be encrypted and password protected. All participants will be given a unique identifier to ensure confidentiality and this list will be kept securely in the password protected folder. The data will be stored until the completion of the project and then deleted. In accordance with the DPA2018, participants will have the right to ask to see what data is held relating to them, and this data will be deleted immediately if the participant requests this, in which case the data will not be used in the project. (this box should expand as you type) |  |  |
| 20 | List all who will have access to the data generated by the research activity: |  |  |
|  | Normally the principal researcher, possibly also the supervisor and, if the project has an industrial partner, a representative of that partner. Possibly also external examiner or second marker? Those within DSTL that require it AND hold necessary security clearance (this box should expand as you type) |  |  |
| 21 | List who will have control of, and act as custodian(s) for, data generated by the research activity: |  |  |
|  | Usually the principal researcher. Myself (this box should expand as you type) |  |  |
| 22 | Give details of data storage arrangements, including security measures in place to protect the data, where data will be stored, how long for, and in what form. |  |  |
|  | Any USB pen drives, removable hard drives and PCs used to store or transfer data, will be encrypted and password protected. These will be stored as long as required and in whatever format is preferred by the team (this box should expand as you type) |  |  |
| 22 | Confirm that you have read the UWTSD guidance on data management (see https://www.uwtsd.ac.uk/library/research-data-management/) | ☒ |  |
| 23 | Confirm that you are aware that you need to keep all data until after your research has completed or the end of your funding | ☒ |  |

SECTION I: Declaration

|  |  | The information which I have provided is correct and complete to the best of my knowledge. I have attempted to identify any risks and issues related to the research activity and acknowledge my obligations and the rights of the participants. In submitting this application I hereby confirm that I undertake to ensure that the above named research activity will meet the University’s Research Ethics and Integrity Code of Practice which is published on the website: https://www.uwtsd.ac.uk/research/research-ethics/ |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | Signature of applicant: |  | Date: 01/08/2022 |  |  |
| 2 |  | Director of Studies/Supervisor: |  |  | Date: |  |
| 3 |  | Signature: |  |  |  |  |

FOR INTERNAL USE ONLY:

|  | Ethical approval given |  |  |
| --- | --- | --- | --- |
| 1 | Signature of assessor: |  | Date: |
| 2 | Name: |  |  |
| 3 | Role: |  |  |