Steps for Eulerian Simulation

PROPERTY MODEUL:

- Create materials

- Create Eulerian section with multiple materials

- Create Assembly with all parts: Eulerian, case, liner and KE. Choose Mash on INSTANCE rather than PART

- MESH module: Mesh the Eulerian part

- In LOAD module:

  - TOOLS  DISCRETE FIELD : VOLUME FRACTION TOOL

  - Create the individual regions for the Eulerian region:  IN each case you select the Eulerian region first then the sub region. It will then create the volume fractions for each element in the Eulerian mesh.

- In LOAD Module:

  - PREDEFINED FIELD   OTHER MATERIAL ASSIGNMENT

  - Select the Eulerian Instance

  - Select DISCRETE FIELD button .

  - Select the Eulerian region(s) for the “Region”   . Hold down shift if multiple selection

  - Then from the pull-down menu for each region select the appropriate material