| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 5 | LEFEL 5 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| JanuaRy 2025 | Ionawr 2025 |

MODULE CODE: ACDA5001

Electrical Engineering

TIME:  2 HOURS

answer ALL THE questions

Equipment or materials required – Calculator

Adnoddau neu ddefnyddiau sydd eu hangen - Cyfrifiannell

ANSWER ALL THE QUESTIONS

Question 1 [25 marks]

- An Alternating voltage, used to power an AC motor is described by the equation V=100Sin (150πt-0.25). Determine the following parameters to evaluate the motor’s power supply characteristics.

- Peak to peak voltage							[2 Marks]

- RMS voltage								[2 Marks]

- Frequency									[2 Marks]

- Phase angle	 in degree							[2 Marks]

- A coil with an inductance of 0.1H and a resistance of 5Ω is used in a relay circuit. When this circuit is switched on to a 220V supply. Calculate the steady state current in the circuit and the rate of rise of current at t=0 and at t=2τ and analyse the results. [where τ=LR]	[13 Marks]

- Describe eddy current loss in a transformer. List one technique to minimise eddy current loss.	[4 Marks]

Question 2 [25 marks]

- A supply voltage of V1=25V is given to the circuit shown in fig:1. Apply Circuit Analysis to:

- Calculate the load current.							[9 Marks]

- Determine the value of load resistor for maximum power transfer across the load.										[3 Marks]

- Draw a simplified circuit diagram for the circuit shown in fig.1		[3 Marks]

- Calculate the Maximum power transferred across the load (PL).		[4 Marks]

Fig: 1

- Describe the functions of the following components used in DC machines

- Field System								[3 Marks]

- Armature									[3 Marks]

Question 3 [25 marks]

- A 400V, three phase star connected alternator supplies a delta – connected load as shown in fig: 2. Each phase of which has a resistance of 30Ω and inductive reactance of 40Ω. Analyse the system and:

- Calculate the current supplied by the alternator,				[7 Marks]

- Determine the output power of the alternator,				[4 Marks]

- Calculate the Apparent power (kVA) of the alternator.			[3 Marks]

Neglect the losses in the line between the alternator and the load.

Fig: 2

- A 415V, three phase AC motor has a power output of 12.75kW and operates at a power factor of 0.77 lagging and with an efficiency of 85%. If the motor is delta-connected, Apply you knowledge to determine the following,

- Input Power 		[3 Marks]

- Line current 		[3 Marks]

- A generator is connected to a 60Ω load and supplied with a current of 8A. If the armature resistance is 5Ω. Calculate the terminal voltage and the generated e.m.f.												[5 Marks]

Question 4 [25 marks]

- A RLC series circuit consists of R=75Ω, L=125mH and C=200μF. The circuit is excited by a sinusoidal source of voltage 115V, 60Hz. Using Complex functions analyse and determine the following.

- Impedance in the circuit 		[7 Marks]

- Current across the circuit		[2 Marks]

- Voltage across each element						[6 Marks]

- Apparent power								[2 Marks]

- Active Power								[2 Marks]

- Reactive power								[2 Marks]

- Construct a Phasor diagram to represent the circuit			[4 Marks]

Diwedd/End