University of Wales Trinity Saint David

Wales Institute of Science & Art

Kinetic Energy Recovery Report

ACHA5004

Ollie Harrison

2003607

February 2022

Contents:

- Introduction

- Vehicle Selection

- WLTP for JCB Fastrac 4220

  - Classifying the vehicle

  - Energy and Force Results

    - Aerodynamic Resistance

    - Tractive Power

    - Identifying Moments of Braking

    - Cumulative Energy – Tractive and Braking

- KERS Selection and Evaluation

  - System Selection

    - Flywheel Mechanical KERS

    - Advantages of Mechanical KERS

    - Disadvantages of Mechanical KERS

    - Variations in Vehicle Handling and Driving Characteristics

- KERS Power and Energy Return

  - Braking Energy Maximums

  - Flywheel Specification

- Appendices

- Introduction

In this report we will be analysing the efficiency of an Internal Combustion Engine along the course of a stop-start driving sequence, and the potential for a Kinetic Energy Recovery System, with the aim of reducing vehicle fuel consumption. To do this we will first select a vehicle that may benefit from such implementations, then calculate certain resistances on the vehicle.

2.0 Vehicle Selection

For the Tests we will be carrying out, a JCB Fastrac 4220 tractor is the selected vehicle for a number of reasons. Currently there are no tractors commonly manufactured with any kind of Kinetic Energy Recovery System, therefore it is worth exploring the potential in this area. Tractors are a large contributor to vehicle emissions across the world with long hours of running time, the JCB Fastrac in particular tends to be owned by farmers that have to spend a good portion of time on the roads between fields nearer urban areas, this means there would  be large amounts of wasted fuel due to stop-start driving. For this reason, it may be very effective to introduce some kind of KERS into this vehicle. It should also be noted that for this test the analysis will be carried out to show its efficiency on the road rather than in a field, however the effects of stop-start driving still occur in normal everyday use in fields.

3.0 WLTP for JCB Fastrac 4220

3.1 Classifying the vehicle

Due to the Fastrac’s high mass and high power, its power to mass ratio is small so it resides in class 1 (version 1.4) for the WLTP. The tractors specification sheet provided by the manufacturer, authored by Nick Needham (2016), states that the maximum velocity when in high range is 60km/h. Since this is <70km/h, phase medium is replaced by a repetition of phase low. This can be seen in the spreadsheet data.

3.2 Energy and Force Results

3.2.1 Aerodynamic Resistance

Below seen in Figure 1 we can view the Aero Resistance (N) on the tractor over the course of the entire test. Due to the large frontal area and un-aerodynamic shape of the tractor we see large spikes in resistance at points of acceleration, however these values do not spike to a huge magnitude since the max velocity does not exceed 60km/h.

Figure 1

3.2.2 Tractive Power

The graph below depicts Tractive Power (W) over the total elapsed time of the WLTP test. Due to relation between Power and energy, 1 Watt being 1 Joule per second, we can multiply the tractive force values by the velocity at each 1 second interval to find the tractive power across the test. As we would expect the point of maximum tractive power which breaks 50kW occurs at the point of maximum acceleration and maximum aerodynamic resistance, as seen in Figure 1.

Figure 2

3.2.3 Identifying Moments of Braking

Figure 3 below shows the time the tractor spends stopped or under braking over the course of the test. The gaps between the bars indicated the time spent accelerating.

Figure 3

3.2.4 Cumulative Energy – Tractive and Braking

As a result of the tractor weighing nearly 8 tonnes, when we compare cumulative tractive and braking energy there is a big difference between the two over the course of the test as shown below in Figure 4. The difference between the data sets accounts for the energy losses in the system such as friction, so knowing this we can see that there is a huge amount of energy wasted in this particular driving cycle of the JCB Fastrac. For this reason, it is evident that a type of Kinetic Energy Recovery System would be very beneficial to the fuel economy of the tractor.

Figure 4

4.0 Kinetic Energy Recovery System Selection and Evaluation

4.1 System Selection

After considering the possible types of KERS to decide which would result in the biggest improvement of the Fastrac’s fuel efficiency without creating a system too complicated, the option of Mechanical KERS using a Flywheel has potential in the world of tractor manufacture. Mechanical KERS will be used in parallel with the friction brakes as while it will produce more losses, due to the mass of the tractor and braking energy produced, it would not be sensible to run in series as the KERS would reach its limit very quickly.

4.1.1 Flywheel Mechanical KERS

This method of recycling lost energy was developed originally to be used in racing by a man named Jon Hilton who worked for Renault F1. In an article on Flywheel hybrid systems, Sam Collins (2009) sums up the system as a flywheel that is connected to the drivetrain by a Continuously Variable Transmission. He explains that if the CVT is moved to a gear ratio which would increase the flywheels speed, the flywheel would begin to store energy. Then if the CVT moved to a gear ratio which would slow the flywheel down, the flywheel releases the energy. There is also a clutch to control the drive to the flywheel so that in the event the RPM gets too high it can be disengaged. Collins goes on to describe a few important design specifications that must be considered when implementing a flywheel of high RPM and size. In terms of immediate safety, the flywheel structure and containment structure must be adequate enough to withstand the forces at high RPMs and protect the driver should something go wrong. When mechanical KERS was developed for F1, Jon Hilton and his team had to prevent excessive heat build-up in the system and remove losses such as air resistance due to the high RPM target of the flywheel. For this reason they decided to run it inside of a vacuum. This same idea may be beneficial when implemented on tractors too since it can remove a great deal of energy losses.

4.1.2 Advantages of Mechanical KERS

There are several reasons this option has potential, one is that due to the size and power of the tractor there are certain negative effects of the flywheel that can be ignored to a degree, such as the added weight a flywheel would add along with the weight of the housing and transmission. However, it should be noted that electrical KERS would add even more weight than the mechanical system which increases overall fuel consumption.

Another benefit of Mechanical KERS is the major difference is has to Electronic KERS and Electro-mechanical KERS. John Batalha (2009) highlights this in their description of Mechanical KERS; this option doesn’t have any kind of energy conversion, the kinetic from the tractor would store as kinetic energy in the flywheel then returned to the engine as kinetic energy via the output shaft. This removes a great deal of losses that are a result of converting energy.

An advantage that would be more significant in the success of this system in commercial tractor manufacturing is the difference in cost between the systems. Thomas Mathews (2013) stated that a Mechanical KERS is low cost in comparison to electrical systems, and it has the added benefit of reducing the brake wear since the transfer of energy from the driveline to the flywheel when the tractor is decelerating slows down the vehicle in the way engine braking works. With tractor prices already close to unaffordable, the advantage of low cost comparatively is particularly significant.

4.1.3 Disadvantages of Mechanical KERS

The disadvantages of this system mostly lie in the safety aspect. Spinning a large flywheel to high RPMS could be catastrophic if there was a failure causing the flywheel to be detached while at a high speed. This issue has been addressed however since a suitable containment chamber reduces the safety concern.

4.1.4 Variations in Vehicle Handle and Driving Characteristics

The input of the KERS while decelerating may cause the Fastrac to have the feel of harsher engine breaking.

On acceleration, the extra input from the regenerated energy coming from the flywheel would have the affect of increasing the rate of accelerating when moving off from a standstill.

5.0 KERS Power and Energy Return

5.1 Braking Energy Maximums

Below in Figure 5 can be seen the Kinetic Energy of the vehicle throughout the test, the largest drops in KE occur under braking and can be amounted to the braking energy at that moment. The maximum drop recorded was 68633.83J over 1 sec, this can be considered the largest possible power input to the flywheel, however this value has not yet had the efficiencies accounted for. Assuming 40% of the braking energy is supplied done by the KERS, and 5% mechanical losses each way into and out of the system, the maximum power the flywheel will receive is 24021.84W (24kW).

Figure 5

Once the losses have been calculated we are left with the values displayed on Figure 6 below. While a large portion of energy is lost the amount resupplied to the wheel would have a noticeable effect on overall fuel efficiency.

Figure 6

5.2 Flywheel Specification

While there isn’t a strict requirement to keep the flywheel compact in the tractors design, the JCB Fastrac 4220 has a limit of 1900RPM therefore must be of adequate radius and mass to hold enough energy.

Using a Flywheel of radius 0.2m we calculate the radius of gyration K, using the equation k2=d2/8 we get 0.14m.

From the JCB’s RPM limit we can calculate the flywheels max speed which equals 198.97rad/s.

Using the value for maximum energy the flywheel is required to store we can calculate the Inertia of the flywheel using I=2(KE/ω2), which equals 1.21kgm3.

From this we can calculate the minimum mass of the flywheel using m=I/k2, this equals 60.68kg.

For low RPM flywheels steel is usually the chosen material, Peter Davis (2020) suggests using Steel (AICI 4340) with density 7800kg/m3.

6.0 Appendices

- Fastrac Tractor 4220, Nick Needham (2016) JCB Fastrac Spec https://jcbcea.com.au/wp-content/uploads/2016/02/specsheet_fastrac4220.pdf Accessed on: 2/01/2023

- Tractor Drag Coefficient Researched Value Sang Joon Lee (2019) https://www.sciencedirect.com/science/article/abs/pii/S0167610519302259 Accessed on: 11/01/2023

- Tractor Rolling Resistance Coefficient Researched Value http://hpwizard.com/tire-friction-coefficient.html Accessed on: 11/01/2023

- Racecar engineering, Sam Collins (2009) Flywheel hybrid systems (KERS) https://www.racecar-engineering.com/articles/f1/flywheel-hybrid-systems-kers/#:~:text=Very%20simply%20the%20system%20comprises,down%20then%20it%20releases%20energy. Accessed on: 12/01/2023

- Clemson University Vehicular Electronics Laboratory, John Batalha (2009) Kinetic Energy Recovery Systems https://cecas.clemson.edu/cvel/auto/AuE835_Projects_2009/batalha_project.html#:~:text=There%20are%20three%20basic%20types,is%20stored%20within%20the%20vehicle. Accessed on: 14/01/2023

- Peter Davis (2020) Energy conversion systems for a sustainable world https://illinois.digication.com/flywheel-energy-storage/materials Accessed on: 15/01/2023