A Simple Smart Home

Household is:
1 x parent
1 x teenager
1 x 5 year old

Wifi router is located centrally in the living room.

Home is controlled by Amazon Alexa devices, several of which have built in Zigbee. Chosen as they are user friendly, have built in mesh networking hub, and are reasonably good speakers.

Proximity sensors – on parent and teens smart phone. When phone detected within X-far of home routines can run to warm/cool house, make coffee etc.

All lights are on a smart switch, except:
Bathrooms, garage and utility room have a light and motion detector – if it’s dark and movement is detected, the light activates. It switches off after motion has not been detected for a set amount of time.

2 x fans are controlled by a temperature sensor during set hours, but can be manually over ridden.

Smart thermostat controls heating system.

Teens bedroom has a smart power bar for media and games devices, allowing it to be monitored and switched on/off remotely.

There are 2 x smart vacuums that activate during set times, when house is usually empty.

Younger child’s room has a multi-purpose sensor used as a contact sensor – if door is opened during night, parent is alerted.

Garage – car has a proximity sensor in the form of a fob. As it approaches home the garage door opens automatically.

There are security cameras inside the garage, monitoring the door to the house, and on the rear deck to monitor the garden.

Porch – externally there is a motion and light sensor, to light up as people approach. And a smart doorbell with built in camera. 
Inside the porch - the door to the house has a contact sensor to alert when door is open, and a smart lock that opens when an authorized proximity sensor is in the porch.

Rear Deck – doors have contact  and vibration sensors. 
There are fairy lights decorating area, operated by a smart plug.

All windows have contact and vibration sensors  - be notified if they are opened or someone strikes the glass.

Parent and teens rooms have smart blinds – can be programmed to open at set time, or when light sensor on deck or porch detects it is bright enough.

This design is:

- Scalable. It incorporates a hub from a major provider that works with a huge number of devices.

- Resilience. With most smart home systems doing the processing in the cloud, the loss of the internet means the loss of the processor. There are resilient hubs available, but at a cost. In this case the decision is to use a standard hub. It is a small smart home set up and the only notable risk should the internet fail is with the smart door locks.

- Fault Tolerance. This could be also be phrased as redundancy – the ability to cover a devices role should it fail. This design has multiple smart assistants, several of which are also a hub, so while a sensor might fail there will still be a hub able to run the home. The router is a single point of failure.

- Accessibility. This design is simple but the technology is going to be life changing for people with disabilities. Not just the smart kitchen devices but potentially smart beds, voice activated hoists, wheelchairs that dock themselves to charge. The smart assistants such as Alexa and Google allow you to run a house just with your voice or an app.

Security essentials:

- Do not use the default name or password of your router. Give it a name that does not identify it - ‘Michelle at number 12’ is a no-no.

- Use a strong complex password that is kept in the family.

- Do not broadcast the SSID unless you have to (ie. when discovering new devices).

- Set up a guest network if router can support it, and allow guests access to just the internet.

- Use the highest encryption your router offers, such as WPA2.

- Change the default passwords of devices.

- Keep devices patched and updated.

- User 2FA – two factor authentication.

- Do not manage your home remotely via public wi-fi – they are not secure or private.