CLOSED SYSTEMS

Exercise 1:

P1 = P2 = 900 kPa

V1 = 200 L = 0.2 m3

T2 = 70C

Q = 47.5 MJ

- State 1: (Table A-12)

P1=900 kPasaturated∴u1f=100.83 kJ/kgv1f=0.000858 m3kg

Hence,

V1=m∙v1f      ∴     m=V1v1f=0.20.000858 =233.1 kg

- State 2: (Table A-13)

P2=900 kPaT2=70°C∴ u2=280.72 kJ/kg

Therefore,

∆U=Q-W

∴       U2-U1=Q-W

∴       W =Q-U2-U1

∴       W =Q-m∙u2-u1

∴       W =47.5×103-233.1×280.72-100.83

∴       W =5567.6 kJ

Exercise 2:

AIR

V = 456

= 120 m3

T1 = 7C

T2 = 23C

∆U=Q-We         where     Q=0

∴       ∆U=-We

∴       U2-U1=-We             where      u2-u1=cv∙T2-T1

∴       m∙cv∙T2-T1 =-We

According to Ideal Gas Equation P∙v=R∙T,

P1∙v=R∙T1

∴       P1∙Vm=R∙T1

∴       m=P1∙VR∙T1=100×1200.287×280

∴       m=149.33 kg

Therefore,

m∙cv∙T2-T1 =-We

∴       We=m∙cv∙T1-T2

∴       We=149.33×0.718×7-23

∴       We=-1715.48 kJ

Hence,

We=Wet=-1715.4815×60=-1.91 kW

Exercise 3:

WFan=-150 W

P1 = 100 kPa

ROOM

V = 4  6  6 = 144 m3

T1 = 15C

∆U=Q-WFan         where     Q=0

∴       ∆U=-WFan

∴      m∙ u2-u1=-WFan             where      u2-u1=cv∙T2-T1

∴       m∙cv∙T2-T1 =-WFan      where      WFan=WFan∙t

∴       m∙cv∙T2-T1 =-WFan∙t

According to Ideal Gas Equation P∙v=R∙T,

P1∙v=R∙T1

∴       P1∙Vm=R∙T1

∴       m=P1∙VR∙T1=100×1440.287×288

∴       m=174.22 kg

And,

WFan∙t=-150×10×3600

∴       WFan∙t=-5400 kJ

Therefore,

m∙cv∙T2-T1 =-WFan∙t

∴       T2-T1 =-WFan∙tm∙cv

∴       T2=-WFan∙tm∙cv+T1

∴       T2=5400174.22×0.718+288

∴       T2=331.17 K

∴       T2=58.17°C

Discussion:

Note that a fan actually causes the internal temperature of a confined space to rise. In fact, a 100W fan supplies a room with as much energy as 100W resistance heater.

Exercise 4:

wb=P.v2-v1

q>0

Air

500 kPa

27C

wsw=-50 kj/kg

∆u=q-wb+wsw

∴       q=∆u+wb+wsw                   where    ∆u=cv∙∆T

∴       q =cv∙T2-T1+P.v2-v1-50         where    T2=T1

∴       q =P.v2-v1-50

According to Ideal Gas Equation P∙v=R∙T,

v1=R∙T1P1=0.287×300500

∴       v1=0.1722 m3kg

And,

v2=3∙v1

∴       v2=3×0.1722=0.5166 m3kg

Therefore,

q =P.v2-v1-50

∴       q =500.0.5166-0.1722-50

∴       q =122.2 kJ/kg

STEADY FLOW OPEN SYSTEMS

Exercise 5:

We take the air dryer as the system, which is a control volume since mass crosses the boundary. Moreover this is steady-flow process since there is no change in time.

- There is only one inlet and one exit, therefore the mass balance for this steady-flow system is:

- Mass balance:

inm=outm

∴     m1=m2=m

The energy balance for this steady-flow system can be expressed in the rate form as:

- Energy balance:

Q-W=outm.h+12.c2+g.z-inm.h+12.c2+g.z

∴   -W=outm.h-inm.h               since Q≅ke≅pe≅0

∴   -W=mh2-mh1

∴   -W=mh2-h1

∴   -W=m.cpT2-T1        since h2-h1=cpT2-T1 for ideal gas

∴   m=-WcpT2-T1

According to Table A-2 the constant pressure specific heat of air at room temperature is

cp = 1.005 kJ/kg.K

Therefore the mass flow rate is:

m=--1.2 kJ/s1.005×47-22=0.04776 kg/s

By definition the volume flow rate (at the inlet) is:

V1=m.v1

The specific heat v1 can be calculated using the ideal-gas equation P . v = R . T

P1.v1=R.T1

∴    v1=R.T1P1     where the gas constant R=0.287kJkg  according to Table A-1

∴    v1=0.287.22+273100=0.8467 m3kg

Therefore:

V1=0.04776×0.8467=0.0404 m3s

- The velocity of the air at the exit can be detrmined using the continuity of mass flow equation m = A.cv

m=A2.c2v2

∴    c2=m.v2A2=0.04776×v260×10-4

The specific heat v2 can be calculated using the ideal-gas equation P . v = R . T

P2.v2=R.T2

∴    v2=R.T2P2     where the gas constant R=0.287kJkg  according to Table A-1

∴    v2=0.287.47+273100=0.9184 m3kg

Therefore:

c2=m.v2A2=0.04776×0.918460×10-4=7.31 ms

Exercise 6:

Win

Compressor

1 Mpa

330°C

120 kpa

20°C

10 l/s

- The mass and energy balances for this steady-flow system can be expressed in the rate form as follows:

- Mass balance:

inm=outm

∴     min=mout=m

- Energy balance:

Q-W=outm.h+12.c2+g.z-inm.h+12.c2+g.z

∴    -W=outm.h-inm.h               since  Q≅0, ke≅pe≅0

∴    -W=mhout-hin

∴    w=hin-hout

∴    w=cpTin-Tout        since h2-h1=cpT2-T1  (ideal gas)

∴    w=1.00520-330       since cp=1.005 kJ/kg.K  (Table A-2)

∴    w=-311.55 kJ/kg

- Power is:

Power=w.m

According to ideal gas equation:

P.v=R.T

∴    vin=R.TinPin

∴    vin=0.287.20+273120

∴    vin=0.7 m3/kg

Calculation of mass flow rate:

m=Vinvin=10l/s0.7

∴    m=10×10-30.7

∴    m=14.27×10-3 kg/s

Therefore the power input is:

Power=-311.55×14.27×10-3

∴    Power=-4.45 W

Exercise 7:

- Mass Balance:

inm=outm

∴      m1+m2=m3

- Energy Balance:

Q-W=outm.h+12.c2+g.z-inm.h+12.c2+g.z

∴     0=m3.h3-m1.h1+m2.h2        since Q=W=∆ke=∆pe=0

∴     m1.h1+m2.h2=m3.h3

- Combining the Mass and Energy Balances and solving for m2  gives :

m1.h1+m2.h2=m1+m2.h3

∴     m2.h2-m2.h3=m1.h3-m1.h1

∴     m2.h2-h3=m1.h3-h1

∴     m2=m1.h3-h1h2-h3

- According to Table A-4:

h1=335.02 kJ/kgh2=83.915 kJ/kgh3=175.894 kJ/kg

- Therefore:

m2=0.5.175.894-335.0283.915-175.894

∴     m2=0.865 kg/s

Exercise 8:

- Mass Balance:

inm=outm

∴      m1+m2=m3

∴      3∙m2=m3                   since m1=2∙m2

- Energy Balance:

Q-W=outm.h+12.c2+g.z-inm.h+12.c2+g.z

∴     0=m3.h3-m1.h1+m2.h2        since Q=W=∆ke=∆pe=0

∴     m1.h1+m2.h2=m3.h3

- Combining the Mass and Energy Balances and solving for h3  gives :

2m2.h1+m2.h2=3m2.h3

∴     h3=2m2.h1+m2.h23m2

∴     h3=m22.h1+h23m2

∴     h3=2.h1+h23

- According to Table A-11:

h1=hf@12°C=68.18 kJ/kg

- According to Table A-13:

h2=h@1MPa and 60°C=293.38 kJ/kg

- Therefore:

h3=2×68.18+293.383

∴     h3=143.246 kJ/kg

- According to Table A-12, at 1MPa:

hf=107.32 kJ/kghg=270.99 kJ/kghfg=163.67 kJ/kgT3=Tsat@1MPa=39.37°C

First we can conclude that the exit stream is a saturated mixture since  hf<h3<hg.

Therefore,

h3=hf+x3∙hfg

∴    x3=h3-hfhfg

∴    x3=143.246-107.32163.67=0.220