| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 4 | LEFEL 4 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| Jan 2021 | JaN 2021 |

HNC / HND in Electrical and Electronic ENGINEERING

MODULE CODE: GOAC4004

Micros Peripherals and Interfaces

TIME:  2 HOURS

answer All Five   questions

Equipment or materials required

Instruction set for PIC16f84

Calculators may be used.

Adnoddau neu ddefnyddiau sydd eu hangen -

First Question (10 marks)

- In the context of a PIC processor explain the purpose and meaning of the following terms:-

- ALU

- Program Counter

- TRISA, (TRISB)

(3 marks)

- Explain why some ASM commands take longer to execute than others

(2 marks)

- A PIC processors are known as RISC devices, explain what this means and what the advantages / disadvantages of this  are.

(3 marks)

- Programs for processors can be written using ‘high’ level languages such as C, C++ etc. Or they can be programmed using Assembly Language. For a time critical process which would you use and explain why?

(2 marks)

Second Question (10 marks)

- What is the main purpose for the stack in a processors, give an

example of its use

- marks)

- Subroutines are used in programming explain what they are and give three reasons why they are used.

- marks)

- Explain in detail how external hardware interrupts are handled

by a PIC16F84. Describe all actions necessary to ensure correct

program operation following the interrupt.

(4 marks)

Third Question  (10 marks)

a) Explain the key limitations for connecting loads to the output of a PIC16F84

(1 mark)

b) With the aid of a sketch describe a suitable solution that would allow a

PIC to control the ON OFF state of a 230 V a.c. mains operated loads.

(2 marks)

- What issues often arise when connecting mechanical switches to the input

of a microprocessor? Describe the normal methods of ensuing safe reliable

interfacing. (both from an electrical and functional perspective)

(3 marks)

- A processor with a 12 bit analogue input has a maximum input voltage

range 0 to +10V. What is the binary code for an input voltage 6.5V

(4 marks)

Fourth Question  (10 marks)

You are provided with a microprocessor with a 13 bit external address bus along

with memory devices having only a 1024 byte capacity.

- Show how you would connect the memory to the microprocessor such that the entire memory addressing capability of the processor is utilised (include a truth table for any decoder required).

(5 marks)

- Show how memory location 4699 (decimal) would be accessed.

- marks)

- Explain the key differences between Von Neumann and Harvard computer architectures.

(2 marks)

Fifth Question  (10 marks)

Microcontrollers are often required to transfer data between themselves and peripheral   devices.

- Describe the principle of Direct Memory Access (DMA) compare the three main modes of transfer.

- marks)

- Clearly explain the difference between synchronous and asynchronous serial interfaces, and give an example of each type.

(1 mark)

- Explain the differences between SPI and I2C interfaces.

(3 marks)

- How does a I2C indicate the start and end of a message transfer

(2 marks)

END

Total 50 Marks