Year  1 Physics module: Example Sheet 1:

Assume g=9.81 ms-2

- Calculate the horizontal and vertical components of a force of 50N which is acting at 40 o  to horizontal.

- Find Horizontal  and Vertical components of a

  - Force of 30N acting at 30 o to  the horizontal

  - Velocity of 50 ms-1 at 60 o  to horizontal

  - Velocity of 60 ms-1 at 60 o  to  horizontal

- A particle of weight 200N rests on a plane inclined at 50o to  horizontal. What are components of weight parallel and perpendicular to plane.

- A body of weight 100N rests on a plane  inclined at 30 o to  horizontal. Calculate the components of the weight parallel and perpendicular to  the plane.

- What is the force required to  maintain point P in static equilibrium?

P

55o

30o

5N

10N

40N

Find the resultant of the following system of forces

60N

70o

30o

50N

  - A mass of 3kg rests on a plane inclined at 40 o  to  horizontal. Calculate the reaction force R acting on mass by plane.

  - What is friction force required to  keep mass stationary on this inclined plane.

- A body of mass 7kg rests on the floor of a lift. Calculate the force, R, exerted on the body by the floor when the lift

  - Has upward acceleration of 2ms-2

  - Has downward acceleration of 3ms-2

  - Is moving down with constant velocity .

- Two bloc, A of mass m and B of mass 3m, are side by side and in contact with each other. They are pushed along a smooth floor under the action of  a constant force F applied to  A. Find:

  - The acceleration of the blocks

  - The force exerted on B by A.

- A body of mass 1 tonne( = 103 kg) is sliding at an acceleration of 0.025ms-2 down an plane that is at an incline of angle α to  the horizontal, where sin α =5/13. Find:

  - The reaction force of the plan on the body

  - The friction force

  - The coefficient of friction