Owain Plant (1910280)

University of Wales Trinity Saint David

Chemistry Assignment

Table of Contents

1.	Tables of Figures, Tables and Equations	2

2.	Introduction	3

3.	Method	4

4.	Screening Smoke Grenade	5

4.1.	Ignition Train	5

4.2.	Primary explosive	6

4.2.1.	Material	6

4.2.2.	Material Manufacture	7

4.2.3.	Reactions	7

4.3.	Delay Composition	8

4.3.1.	Material	8

4.3.2.	Material Manufacture	9

4.3.3.	Reactions	9

4.4.	Bursting Charge	9

4.4.1.	Material Manufacture	9

4.4.2.	Material Manufacture	10

4.4.3.	Reactions	10

4.5.	Main Filling	11

4.5.1.	Material	12

4.5.2.	Material Manufacture	12

4.5.3.	Reactions	12

5.	Conclusion/ Discussion	13

6.	References	14

# Tables of Figures, Tables and Equations

Figure 1- Grenade Hand Smoke Screening Red Phosphorus L84A1 [3]	5

Table 1-Table of K.W Rules [2]	4

Table 2- Primer chemical composition	7

Table 3- Table of the heat of formation for chemicals in the primary explosive	8

Table 4- Delay composition chemical composition	8

Table 5- Bursting charge chemical composition	10

Table 6- Nitro Cellulose Fibre chemical composition	10

Table 7- Table of the heat of formation for chemicals in the Nitro Cellulose Fibre	11

Table 8- Main filling chemical composition	12

Table 9- Polyethene separator discs	12

Equation 1- Oxygen Percentage Formula [2]	4

Equation 2- Calculation for the heat of formation [4]	4

Equation 3- Oxygen Percentage Formula for (C6H2N4O5) +(H4N4) +(C5H8N4O12) +(Al) +(C18H21N11O38) +(BaN2O6)	7

Equation 4- Oxygen Percentage Formula for C18H21N11O38	11

# Introduction

This assignment will break down and discuss the function of an explosive device that has a simple top-level function. There are many of these types of devices that are used in a wide array of different industries such as construction, demolition, entertainment, and mining. The field that has been chosen for this assignment is the defence industry. The device that has been chosen is a military hand grenade that produces smoke for screening purposes. [1]

The smoke grenade should be designed in such a way that it is effective in producing a dense smoke screen that is visible to the naked eye. Each stage of the ignition train will need to have its own mechanism for releasing energy to the next stage to get to the final desired output. The chosen chemicals for each stage must be stable enough so that they cannot ignite by shock, vibration, or other external factors that they are likely to encounter. It is vital that a sufficient time delay is put into the device for the protection of the user, to ensure that the bursting charge does not initiate until it is a sufficient distance away from the user. There must be a safety device built into the device to stop accidental ignition. It is vital that all potential safety issues relating to the user of the device are kept as low as reasonably practical.

# Method

To best do this assignment it has been split into sections, it begins with the method that is going to be undertaken to gain the relevant inform required to produce a successful OME (Ordnance munitions and Explosives) device. This will be done by doing research into existing designs and previous design of devices with the same function. There will then be research into literature for OME chemicals that are new and upcoming as well as chemicals that are reliable and commonly used for this type of device.

Once the relevant research and the method is completed the assignment will then describe the ignition train from initiation until the smoke screen is produced. The assignment will then look at the materials that have been chosen for the device followed by how these will be manufactured. It will then calculate the reactions that happen at each stage in the ignition train. This will be done by firstly creating balanced equations for the chemical reactions, to do this the oxygen percentage will need to be calculated firstly. This can be done for an explosive of molecular formula CaHbNcOd with a molecular weight M by using the formula below:

Ω=[ d – (2a) – (b/2) ] x 1600M

Equation 1- Oxygen Percentage Formula [2]

Once this has been calculated the rules below must be applied to balance the equation:

| K-W Rules (For explosive molecules with an oxygen balance greater than -40%) | Modified K-W Rules (For explosive molecules with an oxygen balance less than -40%) |
| --- | --- |
| 1. Convert as many available C atoms to CO as O atoms allow | 1. Convert H atoms to H2O |
| 2. If there is any oxygen left use it to oxidise H atoms to water | 2. If any oxygen left convert C atoms to CO |
| 3. If there is then any oxygen left uses it to convert CO to CO2 | 3. If any oxygen left convert CO to CO2 |
| 4. Convert all the nitrogen atoms to nitrogen gas molecules N2 | 4. Convert N atoms to N2 gas molecules |

Table 1-Table of K.W Rules [2]

At this stage the heat of formation and its exothermicity can then be calculated. [3] The heat of formation gets calculated using the formula below:

ΔHreaction=∑ΔHof(products)-∑ΔHof(Reactants)

Equation 2- Calculation for the heat of formation [4]

Once this information is gathered the exothermicity can be determined. It will conclude by explaining the reason for the selection of final design and describe the requirements that will need to be in place to ensure quality and safety standards are met.

# Screening Smoke Grenade

## Ignition Train

As discussed above each type of explosive device has its own ignition train for producing its unique desired outcome. The ignition train for the smoke grenade begins with the grenade being held in the hand with the fly off lever in the palm of the hand and fingers wrapped around the grenade holding firmly. Once the user is prepped and ready to throw the grenade the safety pin is removed from the striker mechanism by twisting and pulling it. When the grenade is thrown and leaves the users hand the fly-off lever gets ejected by the force applied to it from the helical spring acting through it. At this point the striker will be released and will actuate against the percussion cap where the primary explosive is held. The primary explosive inside the percussion cap will initiate creating a blast that will travel towards the delay compassion and ignite it. The delay composition is there as a safety measure to protect the user by ensuring that there is adequate time from the time that the user has thrown the device until the bursting charge ignites.

Once the delay composition has completed its burning it will go on to ignite the bursting charge. The bursting charge has a Nitro Cellulous Fibre that runs down the centre of it which transmits the flash from the delay composition to the bursting charge. The bursting charge has two functions firstly to ignite the main filling and to also rupture the body of grenade to allow the smoke to be released. To ensure that an instant and dense cloud of smoke is produced the burning and scattering of the main filling must happen in an even pattern. Once the ignition train described above has been completed a dense screen of smoke will be produced that will be affective in the visible spectrum. [5] Below is a labelled sectioned drawing of the grenade hand smoke screening Red Phosphorus L84A1 in Figure 1.

Figure 1- Grenade Hand Smoke Screening Red Phosphorus L84A1 [3]

## Primary explosive

The first material that was required to be chosen in the ignition train was the primary explosive, which will be inside the smoke grenades percussion cap. There are many different types of chemicals that could have been chosen such as Lead Azide, Mercury Fulminate or Pentaerythritol Tetranitrate however Diazodinitrophenol was chosen instead. [6]

The reason that DDNP (Diazodinitrophenol) was chosen is because of its low levels of toxicity due to the Lead and antimony being replaced with alternative chemicals. The chosen primary explosive is more powerful than the alternative mercury fulminate but is less sensitive than it and does not decompose in acid.  Its disadvantages are that it cannot be dead pressed into shape and is found to lacks consistency with effects being shown when used in centrefire ammunition. [7] [8]

### Material

The information on the chosen primary explosive composition is shown in the table 1 below:

| Chemicals |
| --- |
|  | Percentage [9] | Molecular Formula | Molar Weight | Density | Chemical Drawing |
| DDNP (Diazodinitrophenol) | 30% | C6H2N4O5 [10] | 210.1 g/mol [10] | 1.63 g/cm³ [11] | [10] |
| Tetrazene | 10% | H4N4 [12] | 60.06 g/mol [12] | 1.7 g/cm3 [13] | [12] |
| PETN (Pentaerythritol Tetranitrate) | 5% | C5H8N4O12 [14] | 316.14 g/mol [14] | 1.77 g/cm3 [15] | [14] |
| Aluminium | 5% | Al [16] | 26.98 g/mol [16] | 2.7 g/cm³ [16] | N/A |
| Nitrocellulose (Double-Based) | 10% | C18H21N11O38 [17] | 999.4 g/mol [17] | 1.682 g/ml [18] | [17] |
| Barium Nitrate | 40% | BaN2O6 [19] | 261.34 g/mol [19] | 3.24 g/cm³ [20] | [19] |
| Binder (phenacetin) | 20% | C10H13NO2 [21] | 179.22 g/mol [21] | 1.0±0.1 g/cm3 [22] | [21] |

Table 2- Primer chemical composition

### Material Manufacture

The chosen primary explosive described above will be manufactured by mixing the ingredients shown in table 1 to the shown percentages. The mixtures will be added to the mix using a granular laser measuring system to ensure a precise mix and to avoid rapid pouring. The DDNP will be immersed in water due to it being desensitized by it until its required to be used. The mixture will be pressed to shape containing 0.034 g of the primary explosive mix and will be held together with a phenacetin binder.  Handling of all these chemicals requires a high level of care due to their highly sensitive nature. They must be kept in a temperature-controlled environment and avoid any form of friction, impact or static. The process must be kept to a high level of cleanliness and any residues disposed of appropriately.  [23] [24]

### Reactions

The first reaction in this process as discussed above is the primary explosive, the equations for the reaction, heat released and its exothermicity is shown below:

Ω=[ 61 – (2*29) – (35/2) ] x 16001874.02=-12.380 3.D.P

Equation 3- Oxygen Percentage Formula for (C6H2N4O5) +(H4N4) +(C5H8N4O12) +(Al) +(C18H21N11O38) +(BaN2O6)

Heat of Formation

Can’t be calculated

2(C6H2N4O5) +2(H4N4) +2(C5H8N4O12) +2(Al) +2(C18H21N11O38) +2(BaN2O6)

(Al2O3) +2(BaO)+ 35(H2O) +34(CO) +24(CO2) +35(N2)

The Formula (C6H2N4O5) +(H4N4) +(C5H8N4O12) +(Al) +(C18H21N11O38) +(BaN2O6) will be multiplied by 2 for ease of balancing.

|  | X1 | X2 |
| --- | --- | --- |
| Carbon | 29 | 58 |
| Hydrogen | 35 | 70 |
| Nitrogen | 35 | 70 |
| Oxygen | 61 | 122 |
| Aluminium | 1 | 2 |
| Barium | 1 | 2 |

The heats of formation for some of the chemical compounds could not be found so the known ones are shown in table 3, because of this the heat of reaction and the exothermicity cannot be determined. If they were found the equation shown in the method under Equation 2 would be used. This would be done by taking the sum of the chemical products heat of formation minus the sum of the chemical reactions heat of formation. This result would then be used to see if the reaction is endothermic giving a positive answer or exothermic giving a negative answer. Due to the nature of this reaction its likely to be exothermic. [4]

| Chemical Products | Heat of Formation/ Bond energy (kJ/mol) | Chemical Reactions | Heat of Formation/ Bond energy (kJ/mol) |
| --- | --- | --- | --- |
| C6H2N4O5 | Unknown | Al2O3 | -1675.7 [25] |
| H4N4 | 115.5 [25] | BaO | -548.0 [25] |
| C5H8N4O12 | -538.5 ± 0.8 [26] | H2O | -241.8 [25] |
| Al | 0 | CO | -110.5 [25] |
| C18H21N11O38 | Unknown | CO2 | -393.5 [25] |
| BaN2O6 | 988.0 [25] | N2 | 0 |
| Total: | Can’t be calculated | Total: | -2,969.5 |

Table 3- Table of the heat of formation for chemicals in the primary explosive

## Delay Composition

The next material that was required to be chosen in the ignition train is the delay composition. The selection of this next chemical was vitally important in ensuring that a sufficient time delay is put into the device for the protection of the user. The delay composition that was chosen for the smoke grenade is a Zirconium-Nickel Alloy, Barium Chromate and Potassium Perchlorate mixture.

The reason that the Barium chromate and the Potassium Perchloride was chosen in the delay composition is because it can be used to modify the burn rate this is because it is an oxidising agent. The downside to the Barium chromate is that its toxic and if its inhaled as dust or fumes it is a carcinogen. The Zirconium Nickel alloy mixed 70 to 30 is used due to it being highly flammable and will act as the fuel. The downside to this chemical is that the dust is explosive and hazardous so would need to be kept away from flames, sparks, electrostatic and heat during manufacturing to avoid any accidental ignition. [27]

### Material

The information for the chosen chemical composition is shown in table 2 below:

| Chemicals |
| --- |
|  | Percentage [28] | Molecular Formula | Molar Weight | Density | Chemical Drawing |
| Barium Chromate | 60% | BaCrO4 [29] | 253.32 g/mol [29] | 4.5 g/cm³ [30] | [29] |
| Zirconium-Nickel Alloy 70/30 | 26% | NiZr [31] | 147.84 g/mol [31] | Unknown | Unknown |
| Potassium Perchlorate | 14% | ClKO4 [32] | 138.55 g/mol [32] | 2.52 g/cm³ [32] | [32] |

Table 4- Delay composition chemical composition

### Material Manufacture

The Delay Unit will contain 1.2 g of Delay Composition in three separate pellets each pellet being 0.4 g each. The Barium Chromate, Zirconium-Nickel Alloy and the Potassium Perchlorate delay composition is made by using the agglomerate mixing method. The chemicals will be suspended in a fluid during mixing making a wet mix.  The mixing will be done mechanically by mills and high-speed blenders, this will produce a homogeneous mixture of the chemicals described above. [33]

### Reactions

The second stage in the ignition train is the delay composition, the equations for the reaction, heat released and its exothermicity could not be calculated due to lack of required information. If this was to be calculated the equation showed below would need to balance so that the chosen input chemicals and its reactance are the same. Once this was completed the rules shown above in the method would need to be used to calculate its heat of formation. This result would then be used to see if the reaction is exothermic if the rection

is negative or if its endothermic giving a positive answer. [4] The chemicals that would react

are shown below:

| Barium | 1 |
| --- | --- |
| Chromium | 1 |
| Oxygen | 8 |
| Nickel | 1 |
| Zirconium | 1 |
| Potassium | 1 |
| Chloride | 1 |

(BaCrO4) +(NiZr) +(ClKO4)

Ba +Cr +O8 +Ni +Zr +Cl +K

Heat of Formation

Can’t be calculated

## Bursting Charge

The next material that was required to be chosen in the ignition train is the bursting charge composition. The bursting charge will have a Nitro Cellulose fibre put along its length through the centre to aid its ignition. The chosen composition for the bursting charge is a Barium Nitrate and Magnesium powder mix.

The reason these chemicals were chosen is because the Barium Nitrate is not combustible but will accelerate the burning by acting as an oxidiser to the Magnesium. This chemical composition is stable and requires the initiation of delay composition before it will react. The downside is that the Barium Nitrate is soluble in water and becomes toxic. [34]  The reaction produced by these chemicals are highly exothermic although more heat could have been produced by reacting the Magnesium with Sodium Oxide that level of energy isn’t required. [35]

### Material Manufacture

The information for the chosen chemical composition of the bursting charge is shown in table 3 on the overleaf:

| Chemicals |
| --- |
|  | Percentage [36] | Molecular Formula | Molar Weight | Density | Chemical Drawing |
| Barium Nitrate | 70% | BaN2O6 [19] | 261.34 g/mol [19] | 3.24 g/cm³ [20] | [19] |
| Magnesium powder | 30% | Mg [37] | 24.305 g/mol [37] | 1.74 g/cm³ [38] | N/A |

Table 5- Bursting charge chemical composition

The information on the Nitro cellulose Fibre for inside the bursting charge is shown in table 4 below:

| Chemicals |
| --- |
|  | Percentage [36] | Molecular Formula | Molar Weight | Density | Chemical Drawing |
| Nitro cellulose (Fibre) | 100% | C18H21N11O38 [17] | 999.4 g/mol [17] | 1.682 g/ml [18] | [17] |

Table 6- Nitro Cellulose Fibre chemical composition

### Material Manufacture

The Nitrocellulose fibre will be produced by the process of electrophilic substitution where the Oxygen and Hydrogen will be replaced by the nitro group on the cellulose structure leaving water as a by-product. This process starts with acids and cellulous being added to a mix where it will nitrate. Once the nitration process is completed it will need to go through a pre-stabilisation process before it gets pressure boiled and then post stabilising. To then finish creating the nitro cellulous it must be mixed with alcohol before getting sent to the next stage. At this stage the Nitrocellulose will be a wet watery like paste and will require additives and stabilisers to be added. To finish it will need to be rolled, pressed, cut, dried, blended and sorted into different lots before it gets fitted. [39]

The fibre will be in the centre of the Bursting Charge with Barium Nitrate and Magnesium Powder mixed 70 to 30 around it, all encased in an aluminium tube. This barium nitrate for the bursting charge is made by causing a reaction between barium chloride and sodium nitrate. This product will then need to be recrystallized and filtered to gain the Barium Nitrate for the mixture. [40] The Magnesium powder for the mixture is made by firstly going through the cutting milling process to get the piece small enough to then be finished in the Vortex milling process. [41]

### Reactions

The next stage in the ignition train is the bursting charge the equations for the reaction is shown in equation 4 on the overleaf:

Nitro cellulose (Fibre)

Ω=[ 38– (2*18) – (21/2) ] x 1600999.4=-13.608 3.D.P

Equation 4- Oxygen Percentage Formula for C18H21N11O38

The Formula C18H21N11O38 will be multiplied by 2 for ease of balancing.

Heat of Formation

Can’t be calculated

2(C18H21N11O38)

21(H2O) +17(CO) +19(C02) +22(N)

|  | X1 | X2 |
| --- | --- | --- |
| Carbon | 18 | 36 |
| Hydrogen | 21 | 42 |
| Nitrogen | 11 | 22 |
| Oxygen | 38 | 76 |

The heats of formation for the chosen chemical could not be found but the reactants chemicals could, these are shown in the table below:

| Chemical Products | Heat of Formation/ Bond energy (kJ/mol) | Chemical Reactions | Heat of Formation/ Bond energy (kJ/mol) |
| --- | --- | --- | --- |
| C18H21N11O38 | Unknown | H2O | -241.8 [23] |
|  |  | CO | -110.5 [23] |
|  |  | CO2 | -393.5 [23] |
|  |  | N2 | 0 |
|  |  | Total: | -745.8 |

Table 7- Table of the heat of formation for chemicals in the Nitro Cellulose Fibre

This means as stated previously that the heat released by the reaction and its exothermicity cannot be determined.

Bursting charge

Heat of Formation

ΔH = -2,702

(BaN2O6) + 6(Mg)

(Ba) + (N2) +6(MgO)

For the ease of balancing the equation Mg 6 will be used

| Barium | 1 |
| --- | --- |
| Nitrogen | 2 |
| Oxygen | 6 |
| Magnesium | 6 |

The equation and heat of formation above was gained from the book ‘fundamentals of Pyrotechnics’ by ‘A.A. Shidlovskys’ [35]. This means that the reaction is exothermic and will release heat.

## Main Filling

The next chemicals in the ignition train that were chosen are a red phosphorus and binder mix for the main filling. The reason that this was chosen is because it is a very effective material for forming a dense cloud of smoke. Red phosphorous is chemically reactive and is friction sensitive so the handling of it should be treated as a hazardous material and precautions taken to avoid any contact and inhalation of the dust. The red Phosphorus when it is burning reacts with moisture in the air that causes scattering. There are many cases of people becoming ill from contact with one of the possible products of red phosphorus phosphine, so it is vital that precautions are taken with the substance. This substance is poisonous, can corrode metals, can cause cracking, and can spontaneously combust. The other hazards associated with the smoke produced include disorientation, suffocation, induce asthmatic attack and burns.  [42]

### Material

The information on the chemical composition of the main filling is shown in table 5 below:

| Chemicals |
| --- |
|  | Percentage [36] | Molecular Formula | Molar Weight | Density | Chemical Drawing |
| Red Phosphorus | 80% | P4 [43] | 30.97 g/mol [43] | 2.34 g/cm³ [44] | N/A |
| Binder | 20% | C10H13NO2 [21] | 179.22 g/mol [21] | 1.0±0.1 g/cm3 [22] | [21] |

Table 8- Main filling chemical composition

The information for the chosen chemical composition of the Polyethene separator discs that go between the layers of main filling is shown in table 6 below:

| Chemicals |
| --- |
|  | Percentage [34] | Molecular Formula | Molar Weight | Density | Chemical Drawing |
| Polyethylene separator discs | 100% | (C2H2) n [45] | 103–107 g mol−1 [45] | 0.97 g/cm3 [46] | [45] |

Table 9- Polyethene separator discs

### Material Manufacture

The red phosphorous for the main filling is made by sealing white phosphorous in an iron vessel and heating it to around 400°C for a few hours. It’s important that the temperature for this process is controlled to control the reaction precisely because the conditions can affect the degree of crystallinity. To ensure that there is no white phosphorous remaining this product will then need to be split up mechanically and heated with a solution of caustic soda. To prepare the red phosphorous for mixing it will need to be graded and ground by sieving it. This product is then mixed with the binder composition to a ratio of 80 to 20 and pressed into hexagonal Discs. The disks are then stacked inside the smoke grenades body with the polyethene separator disks between each layer until it is filled to 205 g. The bursting charge will sit down the centre of the disks as could be seen above in figure 1.  [47]

### Reactions

The final stage in the ignition train is the main charge the equations for the reaction, heat released and it’s exothermicity is shown on the overleaf.

Main Charge Red Phosphorous

The Red phosphorous will react with the oxygen in the air and will burn the equation for this is below:

Heat of Formation

ΔH = -3010 kJ

4P + 5O2

P4O10

| Phosphorous | 4 |
| --- | --- |

The equation and heat of formation above was gained from ‘David Holley, Power Point presentation, Slide 96, The Chemistry of Pyrotechnics’ [47] The gained value for the heat of formation shown above shows that the red phosphorous reacts exothermically.

# Conclusion/ Discussion

The materials that were chosen for the device were paramount in the desired outcome being produced successfully and reliably. The grenade was designed in such a way that it would meet the requirements previously stated above and produce an effective dense smoke screen that is in the visible spectrum. The chemicals that are chosen for each stage in the ignition train are stable enough so that they cannot be ignited by shock, vibration, or any other external factors that they are likely to encounter. There are two safety devices built into the system to protect the user, the first is the delay composition. The other is the safety pin needing to be twisted and pulled before ignition to stop accidental ignition for example if it got accidently caught on the users webbing.

The chemicals that are used need to be manufactured by using remote operation wherever feasible due to its sensitivity. It is also important that if an event is to occur that it cannot propagate through any of the processes. An example of this is one primer to another or from the plate to the hopper. It is also safety critical that standards of cleanliness are kept to the highest level to prevent sensitisation.

The Grenade will require artificial aging tests to be carried out to see how the chosen chemicals are affected. It is vital that after the aging process that the ignition train will still work and produce the desired outcome effectively. The grenade would also require ISS (In Service Surveillance) to be carried out to check the serviceability of the systems that are being stored throughout its lifecycle.

I believe that this assignment was successful, however if it was to be done again one of the things that would have been done different is choosing different chemicals. The reason for this is due to the lack of information that was available on the chosen chemicals and the calculations for each stage could not be complete because of this. The other thing that would be changed is to look for an alternative chemical that could be used to produce a dense smoke screen instead of the Red Phosphorous due to its associated hazards.

# References

| [1] | E. S. S. Sydney Meyers, “Industrial explosives - a brief history of their development,” Journal of Hazardous Materials, vol. 23, no. 2, pp. 183-201, 1990. |
| --- | --- |
| [2] | H. David, High Explosives Chemistry, Swansea, 2020, pp. 28-33. |
| [3] | D. Holley, ACDA4011 Chemistry Assessment Specification, Swansea, 2020. |
| [4] | Chemistry Libretexts, “ Standard Enthalpy of Formation,” Chemistry Libretexts, 05 June 2019. [Online]. Available: https://chem.libretexts.org/Courses/Valley_City_State_University/Chem_121/Chapter_7%3A_Thermochemistry/7.4%3A_Standard_Enthalpy_of_Formation#:~:text=This%20equation%20essentially%20states%20that,of%20formation%20of%20the%20reactants.&text=and%20the%20stan. [Accessed 20 November 2020]. |
| [5] | The Defence Council, Pamphlet-13: Grenades Part-12:Grenade Hand Smoke Screening, vol. 3, Bristol: The Defence Council, 2002. |
| [6] | N. G. Johnson, “Explosive Chemical product,” Britannica, 2019. |
| [7] | e. a. Mucong Deng, “A green metal-free fused-ring initiating substance,” NCBI, 22 March 2019. [Online]. Available: https://www.ncbi.nlm.nih.gov/pmc/articles/PMC6430815/. [Accessed 19 November 2020]. |
| [8] | K. Oyler, Green Primary Explosives, Rockaway: US Army Armament Research, Development and Engineering Center, 2014. |
| [9] | R. A. C. Inc, “Lead-free primer mix”. Madison, NC Patent PCT/US96/16021, 26 September 1996. |
| [10] | National Library of Medicine, “Pub Chem Diazodinitrophenol,” National Center for Biotechnology Information, 07 November 2020. [Online]. Available: https://pubchem.ncbi.nlm.nih.gov/. [Accessed 15 November 2020]. |
| [11] | Chemcal Book, “Diazodinitrophenol Properties,” Chemical Book, [Online]. Available: https://www.chemicalbook.com/ChemicalProductProperty_EN_CB71106843.htm. [Accessed 15 November 2020]. |
| [12] | ChemSpider, “Tetrazene,” ChemSpider, [Online]. Available: http://www.chemspider.com/Chemical-Structure.4576001.html. [Accessed 2020 November 15]. |
| [13] | National Library of Medicine, “PubChem Tetrazene,” National Centre for Biotechnology Information, 07 November 2020. [Online]. Available: https://pubchem.ncbi.nlm.nih.gov/compound/5463295 . [Accessed 15 November 2020]. |
| [14] | Nationl Centre for Biotechnology Information, “Pentaerythritol tetranitrate,” Nationl library of Medicine, 07 November 2020. [Online]. Available: https://pubchem.ncbi.nlm.nih.gov/compound/6518. [Accessed 15 November 2020]. |
| [15] | ChemSpider, “Pentaerithrityl tetranitrate,” Chem Spider, [Online]. Available: http://www.chemspider.com/Chemical-Structure.6271.html?rid=e608098c-65b1-4923-8139-0dea459dd6bc. [Accessed 2020 November 15]. |
| [16] | PubChem, “Aluminum,” PubChem, 07 November 2020. [Online]. Available: https://pubchem.ncbi.nlm.nih.gov/compound/5359268. [Accessed 2020 November 2020]. |
| [17] | PubChem, “Nitrocellulose,” PubChem, 07 November 2020. [Online]. Available: https://pubchem.ncbi.nlm.nih.gov/compound/44263835. [Accessed 15 November 2020]. |
| [18] | D. M. FRENC, “The Density of Cellulose Nitrate,” Naval Surface Weapons Center, 04 February 1977. [Online]. Available: https://onlinelibrary.wiley.com/doi/pdf/10.1002/app.1978.070220123. [Accessed 15 November 2020]. |
| [19] | PubChem, “Barium nitrate,” PubChem, 07 November 2020. [Online]. Available: https://pubchem.ncbi.nlm.nih.gov/compound/24798#section=Structures. [Accessed 15 November 2020]. |
| [20] | Merck, “Barium nitrate,” Merck, [Online]. Available: https://www.sigmaaldrich.com/catalog/product/aldrich/202754?lang=en&region=GB. [Accessed 15 November 2020]. |
| [21] | PubChem, “2-Propenoic acid, polymer with 1,3-butadiene and 2-propenenitrile,” PubChem, 07 November 2020. [Online]. Available: https://pubchem.ncbi.nlm.nih.gov/compound/168343. [Accessed 16 November 2020]. |
| [22] | ChemSpider, “phenacetin,” ChemSpider, [Online]. Available: http://www.chemspider.com/Chemical-Structure.4590.html . [Accessed 16 November 2020]. |
| [23] | ResearchGate, “Synthesis and characterization of spherical 2-diazo-4,6-dinitrophenol (DDNP),” ResearchGate, 23 August 2020. [Online]. Available: https://www.researchgate.net/publication/41189260_Synthesis_and_characterization_of_spherical_2-diazo-46-dinitrophenol_DDNP. [Accessed 19 November 2020]. |
| [24] | Cornell University, “Performance testing of lead free primers: blast waves, velocity variations, and environmental testing,” Cornell University, 23 October 2014. [Online]. Available: https://arxiv.org/abs/1410.6390. [Accessed 19 November 2020]. |
| [25] | Unknown, “STANDARD THERMODYNAMIC PROPERTIES OF CHEMICAL SUBSTANCES,” Unknown, [Online]. Available: https://www.update.uu.se/~jolkkonen/pdf/CRC_TD.pdf. [Accessed 20 November 2020]. |
| [26] | NIST, “Pentaerythritol Tetranitrate,” NIST, 2018. [Online]. Available: https://webbook.nist.gov/cgi/cbook.cgi?ID=C78115&Mask=2. [Accessed 20 Novemeber 2020]. |
| [27] | W. W. E. All, “Review of Gasless Pyrotechnic Time Delays,” Propellants, Explosives, Pyrotechnics, vol. 44, no. 1, 30 May 2018. |
| [28] | Pyro Data, “Delay composition,” Pyro Data, [Online]. Available: https://pyrodata.com/compositions:/Delay-composition-2 . [Accessed 16 November 2020]. |
| [29] | PubChem, “Barium chromate,” PubChem, 07 November 2020. [Online]. Available: https://pubchem.ncbi.nlm.nih.gov/compound/Barium-chromate . [Accessed 16 November 2020]. |
| [30] | Chemical Book, “Barium chromate,” Chemical Book, [Online]. Available: https://www.chemicalbook.com/ChemicalProductProperty_EN_CB4674975.htm . [Accessed 16 November 2020]. |
| [31] | American Elements, “Zirconium Nickel Alloy,” American Elements, [Online]. Available: https://www.americanelements.com/zirconium-nickel-alloy-11146-73-9. [Accessed 16 November 2020]. |
| [32] | PubChem, “Potassium perchlorate,” PubChem, 07 November 2020. [Online]. Available: https://pubchem.ncbi.nlm.nih.gov/compound/Potassium-perchlorate . [Accessed 16 November 2020]. |
| [33] | R. H. Comyn, PYROTECHNIC RESEARCH AT DOFL, Virginia: ARMED SERVICES TECHNICAL INFORMATION AGENCY, 1962. |
| [34] | RSC, The thermite reaction- 43, 2009. |
| [35] | A. Shidlovsky, fundamentals of Pyrotechnics, 3 ed., Dover, 1965. |
| [36] | The Defence Council, Grenade Hand Smoke Screening, The Defence Council, 2002, p. Part 12 Sect 4 Page 3. |
| [37] | National Library of Medicine, “Magnesium,” National centre for biotechnology information, 07 November 2020. [Online]. Available: https://pubchem.ncbi.nlm.nih.gov/compound/5462224. [Accessed 15 November 2020]. |
| [38] | Lenntech, “Chemical properties of magnesium,” Lenntech, [Online]. Available: https://www.lenntech.com/periodic/elements/mg.htm. [Accessed 15 November 2020]. |
| [39] | Britannica, “Nitrocellulose,” Britannica, [Online]. Available: https://www.britannica.com/science/nitrocellulose. [Accessed 18 November 2020]. |
| [40] | Science madness, “Barium nitrate,” Science madness, 29 June 2019. [Online]. Available: http://www.sciencemadness.org/smwiki/index.php/Barium_nitrate. [Accessed 20 November 2020]. |
| [41] | e. A. Li Gang, “Experiment-based fire and explosion risk analysis for powdered magnesium production methods,” Journal of Loss Prevention in the Process Industries, vol. 21, no. 4, pp. 461-465, 2008. |
| [42] | D. Holley, The Chemistry of Pyrotechnics, Swansea: UWTSD, 20020, pp. 102-104. |
| [43] | PubChem, “Phosphorus,” PubChem, 07 November 2020. [Online]. Available: https://pubchem.ncbi.nlm.nih.gov/compound/5462309. [Accessed 15 November 2020]. |
| [44] | Merck, “Phosphorus, red,” Merck, [Online]. Available: https://www.sigmaaldrich.com/catalog/product/aldrich/343242?lang=en&region=GB&gclid=Cj0KCQiAwMP9BRCzARIsAPWTJ_EnQxuWusclQUdyCtQpjwECsKjmi2bD2SrmMiES8MlV2Knr7IkNZ3kaApc9EALw_wcB. [Accessed 15 November 2020]. |
| [45] | Omnexus, “Polyethylene (PE),” Omnexus, [Online]. Available: https://omnexus.specialchem.com/selection-guide/polyethylene-plastic#:~:text=Polyethylene%20is%20a%20lightweight%2C%20durable,plastic%20parts%2C%20laminates%2C%20etc.. [Accessed 16 November 2020]. |
| [46] | Science Direct, “Polyethylene,” Science Direct, [Online]. Available: https://www.sciencedirect.com/topics/medicine-and-dentistry/polyethylene. [Accessed 16 November 2020]. |
| [47] | D. Holley, The Chemistry of Pyrotechnics, Swansea: UWTSD, 2020, p. 96. |