- A transmission line has a characteristics impedance of 50 Ω  and a load impedance of ZL=40+j60Ω.  Calculate the SWR, Voltage reflection coefficient and input impedance for 0.18𝜆 long transmission line. Use smith chart to calculate.

Answer:

- VSWR

Normalised load impedance ZL=ZLZ0=40+j6050

ZL=0.8+j1.2

VSWR =3.59 (From smith chart)

- Voltage reflection Coefficient

Γ=0.56 (From smith chart)

- Input impedance of the line, given that the line is 0.18𝜆 long.

Zin=0.82-j1.22 (From smith chart)

Zin=ZinZ0

Zin=ZinZ0=(0.82-j1.22)50

Zin=(41-j61)Ω

Fig: 1

- A transmission line is terminates at a load impedance of  ZL=25+j50Ω.  Determine the following if the characteristics if the characteristic impedance of 50Ω. Use smith chart to calculate the following

- VSWR

- Voltage Reflection Coefficient

- Return loss

- Input impedance of the line, given the line is 3.3𝜆 long

- Input admittance of the line.

Answer:

- VSWR

Normalised load impedance ZL=ZLZ0=25+j5050=0.5+j1

ZL=0.5+j1

VSWR =4.3 (From smith chart)

- Voltage reflection Coefficient

Γ=0.62 (From smith chart)

- Return loss

Return Loss = 4.05db (From smith chart)

- Input impedance of the line, given that the line is 3.3𝜆 long.

Zin=0.28-j0.4 (From smith chart)

Zin=ZinZ0

Zin=ZinZ0=(0.28-j0.4 )50

Zin=(14-j20)Ω

- Input admittance of the line.

Yin=1.15+j1.66  (From smith chart)

Yin=YinG0

Yin=YinG0

Yin=YinZ0=1.15+j1.66   50

Yin=0.023+j0.0332

Fig: 2

- A transmission line with a length of 0.1λ has a characteristic impedance of 75Ω. The line is terminated at a load impedance of ZL=35-j75Ω. Use the smith chart to calculate the following

- VSWR

- Voltage Reflection Coefficient

- Return loss

- Input impedance of the line,			.

- Input admittance of the line.

Answer:

- VSWR

Normalised load impedance ZL=ZLZ0=35-j7575

ZL=0.47-j1

VSWR =4.4 (From smith chart)

- Voltage reflection Coefficient

Γ=0.64 (From smith chart)

- Return loss

Return Loss = 4db (From smith chart)

- Input impedance of the line, given that the line is 0.1𝜆 long.

Zin=0.23-j0.2 (From smith chart)

Zin=ZinZ0

Zin=ZinZ0=(0.23-j0.2)75

Zin=(17.25-j15)Ω

- Input admittance of the line.

Yin=2.3+j2.1  (From smith chart)

Yin=YinG0

Yin=YinG0

Yin=YinZ0=2.3+j2.1    75

Yin=0.031+j0.028

Fig: 3

- A RF transmission line with a length of 2.5cm  and a frequency of 3GHz, has a characteristic impedance of 50Ω. The line is terminated at a load impedance of ZL=50+j40Ω. Use the smith chart to calculate the following

- VSWR

- Voltage Reflection Coefficient

- Return loss

- Input impedance of the line,			.

- Input admittance of the line.

Answer:

- VSWR

Normalised load impedance ZL=ZLZ0=50+j4050

ZL=1+j0.8

VSWR =2.2 (From smith chart)

- Voltage reflection Coefficient

Γ=0.38 (From smith chart)

- Return loss

Return Loss = 8.5db (From smith chart)

- Input impedance of the line

λ=cf=3×1083×109=0.1m=10cm

Length of the transmission line is = 0.25 λ

Zin=0.6-j0.5 (From smith chart)

Zin=ZinZ0

Zin=ZinZ0=(0.6-j0.5)50

Zin=(30-j25)Ω

- Input admittance of the line.

Yin=1+j0.8  (From smith chart)

Yin=YinG0

Yin=YinG0

Yin=YinZ0=1+j0.8      50

Yin=0.02+j0.016

Fig: 4

- A transmission line with a length of 0.07λ has a characteristic impedance of 50Ω, frequency of 21MHz  and input impedance of R=40Ω and C=180PF. The line is terminated at an unknown load impedance of ZL. Use the smith chart to calculate the following

- VSWR

- Voltage Reflection Coefficient

- Return loss

- Load impedance		.

Answer:

- VSWR

XL=12πfc=12π×21×106×180×10-12=63Ω

R=40Ω

Zi=R-jXC=40-j63

Normalised input impedance Zi=ZiZ0=40-j6350

Zi=0.8-j1.26

VSWR =3.7 (From smith chart)

- Voltage reflection Coefficient

Γ=0.57 (From smith chart)

- Return loss

Return Loss = 4.8db (From smith chart)

- Load impedance, given that the line is 0.07𝜆 long.

ZL=3.1-j1.4   (From smith chart)

ZL=ZLZ0

ZL=ZLZ0=(3.1-j1.4 )50

ZL=(155-j70)Ω

Fig: 5