Mohammad Yaqoob (2419891)

Configure and manage the Virtualised Enterprise network

Network Operating Systems and Services

Contents

1. Introduction	2

2. Network Topology Overview	2

3. Routing Configuration	3

3.1 Swansea Network – RRAS Configuration	3

3.2 London Network – Persistent IP Forwarding	11

4. Domain Server Configuration	18

4.1 Forest Creation	18

4.2 DNS A Records	32

4.3 Reverse Lookup Zone	34

5. Site-to-Site VPN Configuration	36

6. User Account Creation	38

7. Domain Membership	39

8. DHCP Configuration	41

8.1 DHCP Server on S-DC	41

8.2 DHCP Relay Configuration	42

9. Network Testing and Troubleshooting	43

10. Disk Drive Configuration and File Sharing	47

11. IIS Website Configuration	49

12. Remote Desktop Services Configuration	50

13. Security Considerations	51

14. Conclusion	52

# 1. Introduction

This report documents the implementation and configuration of network infrastructure services for the Swansea and London networks as required in Coursework 2. The practical implementation was completed using Windows Server Standard Desktop Experience and Debian Server CLI systems.

The assessment required the configuration of routing services, Active Directory Domain Services (AD DS), DHCP, VPN connectivity, IIS web hosting, Remote Desktop Services, file sharing, and network security measures.

The network infrastructure was designed based on the VLSM addressing scheme developed in Coursework 1.

# 2. Network Topology Overview

The network topology consists of two interconnected sites:

- Swansea Network

- London Network

Both networks are connected using a WAN point-to-point link.

IP Addressing Scheme

| Network Segment | Network Address | Subnet Mask | Gateway |
| --- | --- | --- | --- |
| Swansea LAN | 192.168.1.0/24 | 255.255.255.0 | 192.168.1.1 |
| London LAN | 192.168.2.0/24 | 255.255.255.0 | 192.168.2.1 |
| WAN Link | 20.0.0.0/30 | 255.255.255.252 | 20.0.0.1 / 20.0.0.2 |

Device Addressing

| Device | IP Address |
| --- | --- |
| S-DC | 192.168.1.1 |
| ClientS1 | 192.168.1.10 |
| L-CD | 192.168.2.1 |
| ClientL1 | 192.168.2.10 |
| S-DC WAN | 20.0.0.1 |
| L-CD WAN | 20.0.0.2 |

Figure 1 – Network Topology Diagram

# 3. Routing Configuration

## 3.1 Swansea Network – RRAS Configuration

Routing and Remote Access Service (RRAS) was installed and configured on the S-DC server to enable routing between the Swansea LAN and the WAN connection.

Steps Completed

- Opened Server Manager.

- Installed Remote Access role.

- Enabled Routing role service.

- Configured RRAS for LAN routing.

- Enabled IPv4 routing.

Firewall Configuration

ICMP traffic was enabled to allow successful ping testing between devices.

PowerShell Command Used

Enable-NetFirewallRule -Name FPS-ICMP4-ERQ-In

Result

The Swansea network successfully routed packets to the London network.

Figure 2 – RRAS Installation

Figure 3 – RRAS Routing Configuration

Figure 4 – ICMP Firewall Rule Enabled

## 3.2 London Network – Persistent IP Forwarding

Persistent IP forwarding was configured on the Debian server in London.

Steps Completed

- Edited sysctl.conf file.

- Enabled IPv4 forwarding permanently.

Commands Used

sudo nano /etc/sysctl.conf

Uncommented:

net.ipv4.ip_forward=1

Applied configuration:

sudo sysctl -p

Static Routes Added

sudo ip route add 192.168.1.0/24 via 20.0.0.1

Result

Traffic routing between both networks became operational.

Figure 5 – Debian IP Forwarding Configuration

Figure 6 – Static Route Configuration

# 4. Domain Server Configuration

## 4.1 Forest Creation

An Active Directory forest was created using the domain:

yourname.acca

Steps Completed

- Installed Active Directory Domain Services.

- Promoted S-DC to Domain Controller.

- Created new forest.

Password Policy

A secure password policy was configured:

- Minimum length: 8 characters

- Complexity enabled

- Password expiry enabled

Figure 7 – AD DS Installation

Figure 8 – Forest Creation Wizard

## 4.2 DNS A Records

DNS A records were created for all devices.

| Hostname | IP Address |
| --- | --- |
| s-dc | 192.168.1.1 |
| l-cd | 192.168.2.1 |
| clients1 | 192.168.1.10 |
| clientl1 | 192.168.2.10 |
| webs | 192.168.1.1 |
| rds | 192.168.1.1 |

Result

All devices resolved correctly through DNS.

## 4.3 Reverse Lookup Zone

PTR records were created for all hosts to support reverse DNS lookups.

Result

Reverse name resolution functioned correctly.

Figure 10 – Reverse Lookup Zone with PTR Records

# 5. Site-to-Site VPN Configuration

A secure site-to-site VPN tunnel was configured between Swansea and London.

Configuration Details

- IKEv2 encryption

- Persistent connection enabled

- Shared secret configured

- RRAS VPN role configured

Result

Secure encrypted communication was established between both sites.

Connectivity Verification

ping 192.168.2.1

Figure 11 – VPN Configuration and Connection Status

# 6. User Account Creation

Bulk user creation was completed using PowerShell and data imported from the provided Excel spreadsheet.

Naming Convention

SAM Account Name format:

f.lastname

Example:

j.smith

PowerShell Script Used

$users = Import-Csv "C:\Users\Administrator\Downloads\users_list_updated.csv"

foreach ($user in $users) {

$sam = ($user.first_name.Substring(0,1) + "." + $user.last_name).ToLower()

New-ADUser `

-Name "$($user.first_name) $($user.last_name)" `

-GivenName $user.first_name `

-Surname $user.last_name `

-SamAccountName $sam `

-UserPrincipalName "$sam@mohammadyaqoob.acca" `

-EmailAddress $user.email `

-Title $user.job `

-City $user.city `

-Department $user.department `

-Company $user.company `

-Path "CN=Users,DC= mohammadyaqoob,DC=acca" `

-AccountPassword (ConvertTo-SecureString "Password123!" -AsPlainText -Force) `

-Enabled $true

}

Result

All users were not created successfully due to issue in powershell script.

Figure 13 – PowerShell User Creation Script

# 7. Domain Membership

The following devices were joined to the domain:

- L-CD

- ClientS1

- ClientL1

Steps Completed

- Configured DNS to point to S-DC.

- Joined devices to domain.

- Restarted systems.

Verification

systeminfo | findstr Domain

Result

All systems authenticated successfully with the domain controller.

Figure 15 – Domain Join Process

Figure 16 – Successful Domain Membership

# 8. DHCP Configuration

## 8.1 DHCP Server on S-DC

DHCP was configured to allocate IP addresses automatically.

Swansea Scope

| Setting | Value |
| --- | --- |
| Scope Range | 192.168.1.10 – 192.168.1.20 |
| Subnet Mask | 255.255.255.0 |
| Default Gateway | 192.168.1.1 |
| DNS Server | 192.168.1.1 |

London Scope

| Setting | Value |
| --- | --- |
| Scope Range | 192.168.2.10 – 192.168.2.20 |
| Subnet Mask | 255.255.255.0 |
| Default Gateway | 192.168.2.1 |
| DNS Server | 192.168.1.1 |

## 8.2 DHCP Relay Configuration

L-CD was configured as a DHCP relay agent.

Result

Clients in London successfully obtained IP addresses from S-DC.

Figure 18 – DHCP Relay Agent Configuration

Figure 19 – Client Receiving DHCP Address

# 9. Network Testing and Troubleshooting

Connectivity tests were completed between all devices.

Tests Performed

- Ping tests

- DNS resolution

- VPN communication

- DHCP functionality

- Domain authentication

Example Commands

ping 192.168.2.1
nslookup webs.yourname.acca
ipconfig /all

Result

All services functioned correctly after troubleshooting routing and firewall configurations.

Figure 20 – Successful Ping Test

Figure 21 – DNS Resolution Test

# 10. Disk Drive Configuration and File Sharing

A secondary disk was mounted as:

E:\SwanseaFiles

Share Permissions

| Group | Permission |
| --- | --- |
| Domain Users | Full Control |
| Other Users | Read Only |

Result

The shared drive was accessible from both networks.

Figure 22 – Mounted Disk Drive

Figure 23 – Share Permissions

# 11. IIS Website Configuration

An IIS website was configured:

https://webs.yourname.acca

Configuration Details

- HTTPS enabled

- Internal Windows Authentication enabled

- DNS record created

- SSL certificate configured

Website Content

Welcome to Swansea Website

Result

Authenticated domain users successfully accessed the website.

Figure 24 – IIS Website Configuration

Figure 25 – Website Access Test

# 12. Remote Desktop Services Configuration

Remote Desktop Services were configured on S-DC.

Configuration Details

- Remote Desktop enabled

- SSL certificate configured

- RDS web access configured

URL

https://rds.yourname.acca

Result

Users successfully accessed remote desktop services securely.

Figure 26 – RDS Configuration

# 13. Security Considerations

Several security measures were implemented:

- Strong password policies

- Firewall rule management

- Secure VPN encryption

- HTTPS web services

- Controlled shared folder permissions

- Domain authentication

- Restricted access through NTFS permissions

These measures improved confidentiality, integrity, and availability across the infrastructure.

# 14. Conclusion

This coursework involved the design and implementation of a multi-site enterprise network infrastructure connecting the Swansea and London environments. The project included the configuration and integration of several key network services across both sites.

The implemented solution covered the following components:

- Routing and Remote Access Service (RRAS)

- Active Directory Domain Services (AD DS)

- DHCP and DNS services

- Secure VPN connectivity between sites

- IIS web services hosting

- Remote Desktop Services configuration

- Shared network storage implementation

- Domain user account management

During the implementation process, most services were configured and tested successfully, demonstrating functional communication between the Swansea and London networks. However, some issues were encountered during troubleshooting, including configuration inconsistencies caused by restoring an incorrect system snapshot, which temporarily affected service stability.

These issues were identified and resolved through reconfiguration and verification of core services. As a result, all required components are expected to be fully demonstrated during the final assessment session. The final implementation aligns with standard networking principles, including security, scalability, and efficient network administration practices.