Figure 1-1 – University of Wales Trinity Saint David Logo (UWTSD, 2017)

Assignment title: Powertrain Report

Student name: Bradley Thomas

Student number: 1901066

Course module: Powertrain Technology

Module code: M4x02196

Lecture: Abi Summerfield

Contents

Introduction	3

Experimental procedure	3

Results and analysis	3

Transmission efficiency	3

Torque at the wheels	3

Tractive effort	4

Final drive ratio	4

Max tractive effort for each gear	4

Speed ratio	5

Theoretical gear ratios	5

Drag power against road speeds	5

Conclusion	6

Bibliography	7

# Introduction

This report will look at different results gathered during live testing of a BMW S1000 RR bike. Throughout this report, the test procedures will be explained, and the data gathered will be studied and compared with manufacture data, looking at differences within the data and discussing why there are different trends within the data.

# Experimental procedure

The first part of this report looks at the experimental procedure, the bike that was tested as stated in the introduction is the BMW S1000 RR. The squeak test is the following test procedure that was Implemented, similar to a sweep test it looks at constant traction and constant speed by simulating it. This is the steps taken to gather the results from live testing. The test data starts from 1000 RPM however it was determined that there was a fault with the data between 1000 and 2000 RPM because there was a fault with data collection during acceleration. The bike finished on 14000 rpm just under its max of 14200 rpm. The gear which was used to give the test ratio was 5th gear and this gear was used throughout the test. The data which is gathered from the test will allow for results to be interpreted because it would give best 1:1 ratio on the bike.

(BMW motorrad, 2020)

# Results and analysis

The results gathered from the test will now be discussed in order of the report brief.

## Transmission efficiency

The transmission efficiency is how efficiently the transmission is working on the vehicle, during the test the TE was 0.89% efficient which means it was close to 90% efficiency. Reason why this result was produced could be due to frictional loss between components of the vehicle such as the gears or clutch plates, however this is only a small possibility due to the age of the bike that was tested.

## Torque at the wheels

The torque at the wheels is how much torque is being produced when the vehicle is in motion. Looking at our data, the graph (figure 2.1) produced below shows the rate of torque increase through the RPM ranges. There are some interesting points to note about this graph, the torque line dips slightly between 7500 and 8000 RPM, this could be due to the engine overcoming mass and tractive effort. Another point to note is that when the speed is at 11000 rpm, the torque is potentially at its highest output of 1000Nm.

Figure 2.1 Wheel torque graph.

## Tractive effort

Tractive effort is force that is applied to the engine from the transmission at the road wheels to overcome any tractive resistance on the vehicle. Below is a graph (figure 2.2) which shows the data gathered from the practical test carried out. The torque produced through each of the gears, there tractive effort is only relevant as the vehicles speed increase, however when the vehicle reaches its optimum torque output in this case 3000-3250Nm. This graph shows the torque vs the tractive effort and this data shows that as the torque increase the more tractive effort there is needed when there is tractive resistance on the road wheels. When the tractive resistance is cover come by the tractive effort, it will decrease and so will the torque as there is less resistance for the engine to overcome, which in turn allows for a speed to increase.

(Zammit, 1987)

Figure 2.2 Tractive effort graph.

## Final drive ratio

The next result gathered from the data was the final drive ratio, this is the ratio which the bikes rear sprocket, using gear box ratio multiplied the axle ratio. The result gathered by using the data from live testing produced a set of data which placed the final drive ratio as 2.68:1 which is slightly different to the manufacture data of 2.65:1, this could be down to friction loss or wear in transmission components. Also required was the average ratio for the speed range, this produced an average of 2.69:1.

(Bosch Automotive hand book 10th Edition, 2018)

## Max tractive effort for each gear

Max tractive effort is the next result gathered, this was done for each gear and is the maximum tractive effort that each gear can produce. The table below (figure 2.3) shows the max TE for each of the six gears and it tell us how much force is needed at each gear to over-come the tractive resistance. As you can see form the data the higher the gear ratio the less tractive effort is needed to overcome any resistance. Looking at 5th gear our 1:1, the data shows there is little resistance to over come at that point.

| Ratios |  | Max TE |
| --- | --- | --- |
| Primary | 1.652 | N |
| 1st | 2.65 | 3777.74726 |
| 2nd | 2.09 | 2979.430858 |
| 3rd | 1.73 | 2466.227457 |
| 4th | 1.5 | 2138.347506 |
| 5th | 1.36 | 1938.768405 |
| 6th | 1.26 | 1796.211905 |

Figure 2.3 Max tractive effort for each gear set.

## Speed ratio

The gear ratio (also known as speed ratio) is the ratio of angular velocity of a driver gear to the driven gear or input and output. Speed ratio is then calculated by taking the max torque engine speed and dividing that by the max power engine speed. The result produced from the data was 0.800986, this result is the speed which the gears are traveling.

## Theoretical gear ratios

Theoretical gear ratios are taking the speed ratio (gear ratio) and multiplying it by the original ratio an increasing the multiplication for each gear change. The results produced below (figure 2.4) show the ratio for each gear one to six. The reason why the results of theoretical and actual differ is due to there being more angular velocity, therefor the gears are travelling faster and lowing the ratio, for example looking at gear 5, the result is more closer to a 1:1 ratio that the standard ratio.

(MCS, 2020)

| Gear | Gear ratio | Theoretical gear ratios |
| --- | --- | --- |
| 1 | 2.65 | 0 |
| 2 | 2.09 | 2.122611827 |
| 3 | 1.73 | 1.700181498 |
| 4 | 1.5 | 1.361820889 |
| 5 | 1.36 | 1.090798915 |
| 6 | 1.26 | 0.873714218 |

Figure 2.4 Gear ratios vs Theoretical gear ratios.

## Drag power against road speeds

The bikes drag power is the next result gathered from the data. The graph below (figure 2.5) shows the Pdrag against the velocity of the vehicle, there is a distinct black line on the graph, this is marking the point which maximum Engine road speed is achieved. The max engine road speed produced was 135.28Kw. To achieve this, result the max road speed power was taken and multiplied by the transmission efficiency to produce that result. Pdrag is the power drag acting against the bike and how much force there is will have an effect on the max road speed, this is due to the engine needed to work harder to increase the out put speed in order to over come those forces. Another effect that this can have on the bike is the fuel efficiency can be affected as the engine as stated is working extra hard to over come the forces acting on the bike.

(Drivetribe, 2020)

Figure 2.5 Pdrag Vs Velocity note the black line to symbolise the Max engine power.

# Conclusion

In conclusion to this report, the live testing session with the dyno produced interesting results and it was a good way to see the process of how the data is gathered through those means. The results produced from the live data testing have shown that for a two-year-old bike, it is preforming as expected and the data is accurate. Looking at the graphs the output from the bike, show that the standard engine is performing at the best it can for the different RPM ranges and that the transmission system of the bike is able to push through those tractive resistances and allow the rider to have a good performance ride. The data also shows that the different factors can have clear effects on performance for example, looking at the Pdrag data as forces acting are evermore increasing, the more velocity is increasing and how much more the bike needs to work to overcome it. I feel that the group sessions with the dyno are a good way to learn about testing vehicles and are a good way to show how data is interpreted. In motorsport applications dynos would be used to see performance trends and learn how the vehicle is driving but also it can be used to look at altering the vehicle for a different application of motorsport.

# Bibliography

(2020, 04 10). Retrieved from BMW motorrad: https://www.bmw-motorrad.co.uk/en/models/sport/s1000rr-2019/technicaldata.html

(2020, april 11). Retrieved from Drivetribe: https://drivetribe.com/p/how-is-a-cars-drag-coefficient-RUb1vQtoTlyeBW9yR7fmRw?iid=VkfoVOfDSmS0DtpAJNYOMA

(2020, 04 11). Retrieved from MCS: https://www.motorcyclespecs.co.za/Techinicale/Gear_Ratios.html

Bosch Automotive hand book 10th Edition. (2018). Stuttgart: GMBH.

N/A. (2020). Air - Density, Specific Weight and Thermal Expansion Coefficient at Varying Temperature and Constant Pressures. Retrieved from Engineeringtoolbox.com: https://www.engineeringtoolbox.com/air-density-specific-weight-d_600.html

Zammit, S. (1987). Motor Vehicle Enginnering Science . london: longman Group .