Take off and BoQ

Prepared for:

D. Hughes

Written by:

S. Williams

Date completed:

15th May 2023

Table of Contents

Page

Number

1.0 Executive Summary                                                                                             3

2.0     Findings                                                                                                               3

2.1     Cost of Gross Internal Floor area for ground floor extension                                                       3

2.2     Cost of Gross Internal Floor area for existing ground floor                                                           3

2.3     Cost of Gross Internal Floor area for existing first floor                                                              3

2.4     Centreline calculations for extension.                                                                                               4

2.5      Measuring the existing roof tiles.                                                                                                      5

2.6      BoQ information and extra drawing details.                                                                                   7

3.0     Conclusion                                                                                                          7

4.0     References                                                                                                          7

1.0 Executive summary

This report focuses on doing a take-off for elements of the proposed extension and replacing roof tiles on the existing building. It BoQ is then produced showing an estimate of the external envelope and tiles.

2.0 Findings

2.1 Cost of Gross Internal Floor area for ground floor extension

| Ground Floor | Length (M) | Width (M) | Area (M2) | Estimated cost per (M2) £ |
| --- | --- | --- | --- | --- |
| Section A | 1.75 | 1.75 | 3.063 | 300.17 |
| Section B | 7.43 | 1.48 | 10.996 | 1077.61 |
| Section C | 2.95 | 2.78 | 8.201 | 803.70 |

Total Gross Internal Floor area for extension = 22.26 M2

Total estimated cost for extension = £2182.00

2.2 Cost of Gross Internal Floor area for existing ground floor

| Ground Floor | Length (M) | Width (M) | Area (M2) | Estimated cost per (M2) £ |
| --- | --- | --- | --- | --- |
| Section A | 5.45 | 2.35 | 12.808 | 1255.19 |
| Section B | 1.75 | 1.6 | 2.8 | 274.40 |

Total Gross Internal Floor area for existing ground floor = 15.608 M2

Total estimated cost for existing ground floor = £1530.00

2.3 Cost of Gross Internal Floor area for existing first floor

| First Floor | Length (M) | Width (M) | Area (M2) | Estimated cost per (M2) £ |
| --- | --- | --- | --- | --- |
| Section A | 5.45 | 2.35 | 12.808 | 960.60 |
| Section B | 1.75 | 1.6 | 2.8 | 210.00 |

Total Gross Internal Floor Area for existing first floor = 15.608 M2

Total estimated cost for existing first floor = £1171.00

2.3.1 Notes for client

- The ground floor for the existing build and proposed extension consists of 150mm thick concrete slab with 1 layer of A195 fabric reinforcement that has a cost of £98.00 per M2.

- The first floor for the existing build consists of a joisted floor, plasterboard ceiling, skim, emulsion, t&g chipboard, sheet vinyl flooring and painted softwood skirtings that has a cost of £75.00 per M2 (AECOM, 2020, p. 185, 189).

2.4 Centreline calculations for extension (see sheet for original calculations)

Internal living space girth (m)

1.475 A

5.725 B

2 / 1.700 C/D

3.400

5.950 E

2.950 F

2.775 G

4.500 H

Total internal girth = 26.775m

2.4.1 Centreline calculations

Inner blockwork

4/2/0.5/0.1 = 0.4

4/2/0.5/0.1 = 0.4

T = 0.8m

Insulation

4/2/0.5/0.0375 = 0.15

4/2/0.5/0.0375 = 0.15

T = 0.3m

Cavity

4/2/0.5/0.015 = 0.06

4/2/0.5/0.015 = 0.06

T = 0.12m

Outer blockwork

4/2/0.5/0.1 = 0.4

4/2/0.5/0.1 = 0.4

T = 0.8m

26.775 + 0.8 + 0.3 + 0.12 + 0.8

Total girth (internal to external) = 28.795m

2.5 Measuring the existing roof tiles.

Roof measurements

Width of roof (including overhang) = 6.6m

Height from eaves to apex = 1.15m

Length of roof = 7.75m

Roof area (m2)

2.8 + 0.5 = 3.3m

(run) + (overhang)

= (3.3 x 1.15) x2 = 7.59m2

Roof pitch

= 1.15 ÷3.3 = 0.348 (3 d.p.)

tan-1 (0.348) = 19.19˚

Rafter length

C2 = 3.32 + 1.152

= 10.89 + 1.3225

= 12.2125

= √12.2125

C = 3.49m

2.5.1 Notes for client

- A Welsh slate finish has been selected for the existing roof.

- Sloping per m2 = £211.00.

- Estimated price = £1602.00 (AECOM, 2020, p. 190)

2.6 BoQ information and extra drawing details

See excel spreadsheet for BoQ.

Proposed rear elevation.

Door = 2.05 x 0.9m

Windows = 2/ 1.3 x 0.8

Proposed side elevation (to the right)

Door = 2.05 x 0.9m

Window = 1.3 x 0.8m

Proposed front elevation.

Window = 1.1 x 0.6m

Girth = 28.795m x 2.3m height to soffit

= 66.2285 – 7.47

= 58.7585 + 0.5775

Total wall area = 59.34m2

Total area of windows and doors for extension = 7.47m2

- Mineral fibre quilt was selected to increase the overall thermal performance of the extension.

3.0 Conclusion

The total estimated cost which includes the gross internal floor area for the extension, the external envelope of the extension and the amount of tiles for the existing build equals £8206.00.

4.0 References

AECOM (2020) SPON’S ARCHITECTS AND BUILDERS PRICE BOOK. 145th edn. Taylor & Francis Group.