| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 5 | LEFEL 5 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| may 2024 | mai 2024 |

MODULE CODE: ACDA5006

Transducers and Signal Conditioning

TIME:  2 HOURS

answer all questions

Equipment or materials required – Calculator, Graph Paper

Adnoddau neu ddefnyddiau sydd eu hangen - Dim

50 MARKS AVAILABLE IN TOTAL.

QUESTIONS HAVE UNEQUAL WEIGHTING.

PLEASE PLAN YOUR TIME CAREFULLY.

ANSWER ALL QUESTIONS

- (20 Marks available)

Define the following terms:

  - Transducer

[2 Marks]

  - Accuracy

[2 Marks]

  - Precision

[2 Marks]

  - Repeatability

[2 Marks]

  - Reproducibility

[2 Marks]

  - Range

[2 Marks]

  - Nonlinearity

[2 Marks]

  - Resolution

[2 Marks]

  - Sensitivity

[2 Marks]

  - Hysteresis

[2 Marks]

- (4 Marks available)

The following table shows the output measurements of a voltmeter under two sets of conditions: used in an environment kept at 20°C, which is the temperature that it was calibrated at; and used in an environment at a temperature of 50°C

| Voltage readings at calibration temperature of 20°C (assumed correct) | Voltage readings at temperature of 50°C |
| --- | --- |
| 10.2 | 10.91 |
| 15.3 | 16.265 |
| 20.5 | 21.725 |
| 24.9 | 26.345 |

- Determine the zero drift when it is used in the 50°C environment, assuming that the measurement values when it was used in the 20°C environment are correct.

[2 Marks]

- Calculate the zero drift coefficient

[2 Marks]

- (8 Marks available)

For the transducer characteristics given in the table below, where X is the value of the measurand, and Y is the transducer output in volts.

| X | 0 | 10 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y | -2.71 | -0.69 | 0.85 | 1.79 | 3.69 | 5.04 | 5.97 | 8.32 | 9.5 | 11.33 | 12.41 |

- Plot the characteristics and draw a line of best fit.

[4 Marks]

- Determine the maximum non-linearity of the transducer and express it as a percentage of full-scale deflection.

[2 Marks]

- Calculate the sensitivity of the transducer.

[2 Marks]

- (8 Marks available)

Fig Q4 shows a simple signal conditioning circuit.

  - Describe the function of the following circuit elements:

    - Network R4, D1 & D2

    - U1A

    - U1B, R1, R2, R3

    - RV1

[4 Marks]

  - Given the Zener diode of 5.1V, an input voltage of 2.7V, and a voltage of 1.8V present on the wiper of RV1; calculate the output voltage.

[4 Marks]

Fig Q4

- (10 Marks available)

Compare and contrast the requirements and specifications for a temperature sensor for the following applications:

  - Air temperature measurement for a weather station

  - Coolant temperature of a car engine.

[10 Marks]

Diwedd/End