| Drawing Name/No. |  |  |  | CAM assign Task 1 |  |  |  |  | Tooling and Setup Data Tool 2 – End mill – Dia 12mm – Length 40mm Tool 5 – Twist drill – Dia 4mm - Length 55mm Tool 6 – Twist drill – Dia 5mm – Length 60mm Assume all tools are made from Cemented Carbide |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Description: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Program No. |  |  |  | 1 |  |  |  |  |  |  |  |  |  |
| Material: |  |  |  | Mild Steel |  |  |  |  |  |  |  |  |  |
| Program by: |  |  |  | Ben Leach |  |  |  |  |  |  |  |  |  |
| Date: |  |  |  | 03/06/2020 |  |  |  |  |  |  |  |  |  |
| N | G | M | X |  | Y | Z |  | R |  | F | Remarks |
| 01 | G80 |  |  |  |  |  |  |  |  |  | Cancel any previous can cycles |
| 02 | G20 | M08 |  |  |  |  |  |  |  |  | Turn coolant on and metric input |
| 03 |  | M03 |  |  |  |  |  |  |  |  | Spindle rotation CW |
| 04 |  | M06 | T02 |  |  |  |  |  |  | S4510 | Select Tool 2 Set speed for tool 2 at 4510mm/min |
| 05 | G41 |  | D02 |  |  |  |  |  |  |  | Set Cutter radius compensation for tool 2 |
| 06 | G43 |  | H02 |  |  |  |  |  |  |  | Set tool length compensation for tool 2 |
| 07 | G54 |  |  |  |  |  |  |  |  |  | Work coordinate system 1 |
| 08 | G90 |  |  |  |  |  |  |  |  |  | Absolute axis movement |
| 09 | G00 |  | X0 |  | Y0 | Z 15 |  |  |  |  | Move tool into initial position |
| 10 | G91 |  |  |  |  |  |  |  |  |  | Turn on Incremental axis |
| 11 | G01 |  | X0 |  | Y0 | Z-4 |  |  |  | F3247 | Plunge down with speed rate of 3247mm/min |
| 12 | G01 |  | X0 |  | Y24 |  |  |  |  |  | Move up linear |
| 13 |  |  | X18 |  | Y18 |  |  |  |  |  | Linear move to produce 45Degree edge |
| 14 |  |  | X61 |  |  |  |  |  |  |  | Linear move to right. |
| 15 | G03 |  | X6 |  | Y6 |  |  | R6 |  |  | Change to ACW movement to produce R6 corner |
| 16 | G01 |  |  |  | Y-26 |  |  |  |  |  | Linear movement down |
| 17 | G02 |  | X-10 |  | Y-10 |  |  | R10 |  |  | CW interpolation to produce R10 corner |
| 18 | G01 |  | X-75 |  |  |  |  |  |  |  | Linear movement to left back to start. |
| 19 | G01 |  | X0 |  | Y0 | Z-4 |  |  |  |  | Plunge down another 4mm to bottom of part |
| 20 | G01 |  | X0 |  | Y24 |  |  |  |  |  | Move up linear |
| 21 |  |  | X18 |  | Y18 |  |  |  |  |  | Linear move to produce 45Degree edge |
| 22 |  |  | X61 |  |  |  |  |  |  |  | Linear move to right. |
| 23 | G03 |  | X6 |  | Y6 |  |  | R6 |  |  | Change to ACW movement to produce R6 corner |
| 24 | G01 |  |  |  | Y-26 |  |  |  |  |  | Linear movement down |
| 25 | G02 |  | X-10 |  | Y-10 |  |  | R10 |  |  | CW interpolation to produce R10 corner |
| 26 | G01 |  | X-75 |  |  |  |  |  |  |  | Linear movement to left back to start. |
| 27 | G90 |  |  |  |  |  |  |  |  |  | Change to absolute movement |
| 28 | G00 |  | X0 |  | Y0 | Z15 |  |  |  |  | Rapid movement 15mm above part |
| 29 |  |  | X48.5 |  | Y23 |  |  |  |  |  | Move to Spaced holes section of drawing. |
| 30 |  | M06 |  |  |  |  |  |  |  | T06 | Set tool unit 6 |
| 31 | G43 | G40 |  |  |  |  |  | S |  | H06 | Cancel cutter radius and set tool length compensation for tool 6 |
| 32 | G83 | G98 |  |  |  | Z-8 |  | R1 |  | L00 | Set pecking cycle with zero reps |
| 33 | G70 | I8 | J0 |  | L4 |  |  |  |  |  | Will carry out in circular array of 8mm radius at 0-degree angle with 4 equally spaced holes |
| 34 | G00 |  | X0 |  | Y0 | Z15 |  |  |  |  | Rapid movement 15mm above part |
| 35 |  | M06 | T05 |  |  |  |  |  |  |  | Change to tool 5 |
| 36 | G43 | H05 | S451 |  |  |  |  |  |  |  | Set tool length compensation and feed rate for tool 5 |
| 37 |  | X20 | Y10 |  |  |  |  |  |  |  | Move to first whole postion |
| 38 | G83 | G98 | Z-8 |  | R1 | Q4 |  |  |  |  | Peck drill cycle, depth 8mm, starting at 1mm from surface with 4mm increments. |
| 39 |  | X20 | Y20 |  |  |  |  |  |  |  |  |
| 40 | G83 | G98 | Z-8 |  | R1 | Q4 |  |  |  |  | Peck drill cycle, depth 8mm, starting at 1mm from surface with 4mm increments. |
| 41 |  | X20 | Y30 |  |  |  |  |  |  |  |  |
| 42 | G83 | G98 | Z-8 |  | R1 | Q4 |  |  |  |  | Peck drill cycle, depth 8mm, starting at 1mm from surface with 4mm increments. |
| 43 | G80 | M09 | M08 |  |  |  |  |  |  |  | Cancel can cycle, turn off coolant, stop spindle |
| 44 |  |  |  |  |  |  |  |  |  |  |  |
| 45 |  |  |  |  |  |  |  |  |  |  |  |
| 46 |  |  |  |  |  |  |  |  |  |  |  |
| 47 |  |  |  |  |  |  |  |  |  |  |  |
| 48 |  |  |  |  |  |  |  |  |  |  |  |
| 49 |  |  |  |  |  |  |  |  |  |  |  |
| 50 |  |  |  |  |  |  |  |  |  |  |  |
| 51 |  |  |  |  |  |  |  |  |  |  |  |
| 52 |  |  |  |  |  |  |  |  |  |  |  |
| Continuation of program no: |  |  |  |  |  |  |  |  |  |  |  |  |
| N | G | M | X |  | Y | Z |  | R |  | F | Remarks |

Spindle Speed = 1000 x 1703.141 x 12 = 4510rev/min

Tool 2 Feed rate (mm/rev) = 0.18 x 2 = 0.72mm/rev

Feed rate (mm/min) = 0.72 x 4510 = 3247mm/min

Tool 5 feed rate = 0.1 x 4510 = 451mm/min

Tool 6 feed rate = 0.11 x 4510 = 496mm/min