Site Set Up Method Statement

Introduction.

The project involves the build of a single, two story detached domestic building with a loft room. It will be built on a Greenfield site with access directly on to a public highway via a short track of 3.5m width and the work is to start on 6th May 2019.

This method statement will focus on the major tasks related to the construction of the build and take into account any constraints. A copy of the schedule or works has been sent to Building Regulations so they can plan their site inspections. The scheduled inspections have been annotated on the programme of works.

Site Set up

Security –

The site will be secured using closeboard fencing that has pedestrian and vehicle access. This has been chosen over other forms of fencing as it gives the impression that the site is occupied. If a timber or Heras fencing were to be used, it would appear that the site was under construction and might encourage vandalism or theft.

It is suggested that there is a gateman onsite 24hrs a day to prevent unauthorised access and record any site personnel.

It is suggested that 3 gatemen are employed on a rotational basis of 8hrs a shift.

Signage is to be marked outside the site for locations of access and any hazardous activities on site.

Parking/Reception Access:

The car park is to be segregated from the main work area. The main reception is to be clearly marked and a safe route is to be identified for pedestrian access.

Site Layout:

There will be an appointed first aider and fire marshal. The muster points will be marked out and appropriate signage erected around the site. There will also be first aid kits and fire stations clearly marked.

Spill kits at designated points and signage to mark their location

Traffic Management:

Traffic and pedestrians are segregated by fencing and there will be a manned guard at the entrance to stop unauthorised access.

A competent Banksman will be on site to monitor plant movements.

Due to the close location of site to a public highway, vehicles will need to be washed before exiting the site and travelling along public highways.

Housekeeping/Storage:

Separate skips will be delivered, and waste will be sorted according to their use.

Any plasterboard cut offs are to be stored separately and kept dry and contaminate free for recycling. Ideally there should be no cut offs left but in case of errors the above note should be taken.

A waste transfer licence has been applied for and any waste transfer notes will be kept and filed as per CDM regulations.

All chemicals will be stored in locked bonded cages as per CDM regs 2015.

All areas will be kept clean and tidy so as not to cause a hazard.

Inductions

All personnel authorised to enter the site will be given a site inspection before any work is to start (CDM Regs)

All personnel are to sign in and out of site any time they enter/leave the site.

Hot works permit/ confined spaces permit and permit to dig are to be read, understood and signed before any work is commenced by those competent to do so.

Excavation Method Statement

Supervisor is to make sure all staff have had a site induction and are trained and competent to start the work and read and sign the permit to dig (if correct).

Carry out safety checks on the plant and tools at the start of each shift and that all operators hold a valid certificate and are competent in operating the machinery.

Ensure that all workers wear the correct personal protective equipment.

Contractors are to be aware of the risks of weal’s disease and to wash hands before consuming any food or drink or smoking.

Let other contractors working nearby know that you are about to start the work.

Liaise with the principal contractor and other contractors to ensure safe operation

Cordon off work area.

Ensure the area is clear of any obstructions

Ensure that safe access and egress is maintained and that a safety plan is in place, and those working informed during the site induction.

Locate any drawings/site plans of existing ground

Determine location of underground services

Mark out area to be excavated with marker paint

Carry out RD8000/CAT scans of ground to check and locate live services

Arrange for existing live services to be shutdown where possible

Ensure trained and competent banksman is available to watch excavation work

Dig out ground as per contract drawings to required size and depth

Where live services have been located machines must not dig within 0.5 meters (Reference)

Hand dig around live services where necessary using spade and shovel

All excavations must be left safe at the end of each day (Reference)

Methods of Working

Excavations of Greenfield site trench

The topsoil will be lifted using a 360 Excavator to a depth of 150-300mm and set aside adjacent to each proposed area of work for reuse as landscaping.

The trench will be dug to a depth of 1m for a concrete strip. The remaining soil (not topsoil) will be set aside for either fill, or again recycling.

The excavation is to be dug evenly throughout and measured before fill.

The excavation will need to be shored and batted ready for the concrete with a ladder for easy egress.

The wooden shutters will be cut by circular saw and treated posts will be hammered in at corners and equal centres along the side. Once in place galvanised nail will be used to keep the wooden edgings in place. All small tool users are to hold a small tools card and are competent in using the equipment.

The building inspector will need to inspect the excavation before any concrete is poured.

Once the inspection has been approved, the concrete can be poured into the excavation. . The final level compaction will be compacted using the plate compactor.

Timber edging 150mm x 50mm will be laid on to the hard-core and levelled using either a rubber hammer or wooden mallet. The hard-core type 1 stone levelled up to the edgings and compacted using a small compactor.

Building up to Roof Weathering

Once concrete has dried, and has been inspected, start install block and beam

The beams will be delivered and placed next to the excavation to ensure there is minimal manual handling of materials.

Install the beams and blocks according to manufacturer’s guidelines.

Use lifting equipment to install the beams

Once the beams have been laid, insert the blocks

Once in place, a level of screed is to be laid.

DPM is to be laid

Start build of ground floor walls up to 1st floor using scaffolding

Timber 1st floor to be constructed

Walls/ Partitions, 1st Floor

2nd Floor

Walls/ Partitions, 2nd Floor

Wall Sheathing

Check Level, Square & Layout as work progresses

Ceiling Joists

Rafters & Trusses

Space for a loft window

Roof Sheathing and Felt

Framing Inspection

Install remaining Doors/Window using scaffolding

Clear site for client.

Who is Responsible?

According to Annex A of the CDM Regulations 2015, the following people have these responsibilities:

Principal Contractor

The Principal contractor’s main duties are to plan, manage, and monitor and co-ordinate health and safety in the construction phase of a project.

They are responsible for the following:

•	Liaising with the client and principal designer

•	Preparing the construction phase plan

•	Organising co-operation between contractors and coordinating their work

They are also to ensure that:

•	Suitable site inductions are provided

•	Reasonable steps are taken to prevent unauthorised access

•	Workers are consulted and engaged in securing their health and safety

•	Welfare facilities are provided.

Contractors

Those who are an individual or a company who do the construction work:

•	Plan, manage and monitor work under their controls that it is carried out without risks to health and safety

Workers

Those who work under a contractor in a construction site:

•	Be consulted about matters which affect their health and safety

•	Take care of their own health and safety and of others who may be affected by their actions

•	Report anything they see which is likely to endanger either their own or others health and safety

•	Co-operate with their employer, fellow co-workers, contractors and other duty holders.

Domestic Client

People who have constructed work on their own home.

•	Their duties have been transferred to the principal contractor on a project which involves more than one contractor.

RISK

| Likelihood | Severity |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Trivial | Minor Injury | Over 3 Day Injury | Major Injury | Incapacity or Death |
| Highly Unlikely | 1 | 2 | 3 | 4 | 5 |
| Unlikely | 2 | 4 | 6 | 8 | 10 |
| Possible | 3 | 6 | 9 | 12 | 15 |
| Probable | 4 | 8 | 12 | 16 | 20 |
| Certain | 5 | 10 | 15 | 20 | 25 |

| Falls from height | Major injury or death | Use Guardrail, toe guards |
| --- | --- | --- |
| Excavations | Possible injury or death | Keep plant away from edge, Place a stop guard on edge. |
| Public Highway | Possible car accident | Ensure highway code is followed |
| Theft of Materials and Plant | Possible | Ensure materials are locked away when not in use. Fit plant with trackers and immobilisers. |
| Contact with underground services | Possible injury or death | Ensure an RD8000 scanner is used or cat scan for buried services |
| Inhalation of hazardous chemical |  | Wear correct PPE and wet cut |

References:

CDM Regulations 2015

Site safety handbook 4th Edition 1997 by MWL Digital, Pontypool

HSE (2006) Health and Safety in construction, 3rd Edition, HSE Books, London.

Working at height regulations 2005

Control of Asbestos regulations 2012

Safety Representatives and Safety committees Regulations 1977

Health & Safety (Consultation with Employees) Regulations 1996

COSHH - regulations 2002

Factories Act 1961