FORMULEA BOOKLET

- Stress and Strain, Modulus of Elasticity and Rigidity:

N/m2  Pa

direct stress  =PA	Direct Strain ε=δ LL

unitless

shear stress τ =PA	Shear Strain γ=∆ lL

N/m2  Pa

Modulus of elasticity E=σε                  Modulus of rigidity G=τγ

- Equilibrium Condition of Forces and Moments

Moment M = F  d

N.m

Condition for static equilibrium:

sum of the leftward component of forces=sum of the rightward  component of forcessum of the upwardcomponent of forces=sum of the downward component of forces

- Dynamics

- Linear motion:

Newton’s Second Law for linear systems:	F=m .a

Linear motion of bodies moving with constant acceleration:

(1)

v=v0+a.t

(2)

v2=v02+2.a.s

(3)

s=v0.t+12.a.t2

(4)

s=12.v+v0.t

Where:	v0 = initial velocity at t = 0                  v = velocity at time t

a = constant acceleration                  s = displacement

- Rotational Motion:

Relationships between linear and angular speed:

angular speed

rad/s

linear speed

m/s

v=r.ω

Relationships between angular speed and frequency of rotation:

frequency

rev/s

angular speed

rad/s

ω=2.π.n

Relationships between linear and angular acceleration:

linear acceleration

m/s2

angular acceleration

rad/s2

a=r.α

Rotational motion of bodies moving with constant angular acceleration:

ω=ω0+α.t                   1

ω2=ω02+2.α.θ          2

θ=ω0.t+12.α.t 2        3

θ=12.ω+ω0.t           4

Where:	ω0=initial angular velocity at t=0             ω=angular velocity at time t

α=constant angular acceleration                  θ=angle turned through in time t

Newton’s Second Law for rotating systems:	M=I .α

k is the radius of gyration in m

Where Second Moment of Inertia I is:

Moment of Inertia in kg.m2

I=m.k2

- Work, Energy and Power

- Work:

- The work done by a constant force is given by

F

direction of motion

J

W=F.s

m

N

direction of motion

F

F cos 𝛳

Suppose that a constant force F acts on a body so as to move it in a direction other than its own

The component of F in the direction of motion is F cos 𝛳, in which case the work done W is given by

W=F.s.cosθ

- Work done by a torque T

W=T.θ

- Energy:

- Kinetic energy for a body undergoing translational motion

KE=12.m.v2

J

- Kinetic energy for a body undergoing rotational motion

Rotational KE=12.I.ω2

- Total kinetic energy of a rolling wheel

total kinetic energy=12 m v2+12 I ω2

- Gravitational Potential Energy

PE = m.g.h

- Work-Energy Principle

work done byexternal forces=change inKE+change inPE

- Power:

- First expression

If a machine is working at a steady rate, the power of the machine is

power=work donetime taken

Watt (W)

When the rate of working is not steady, the above equation give the average power.

- Instantaneous Power

power=force ×speed=F.v

- Power developed by a torque T applied to rotate an axle through an angle 𝛳 in time t

power developed by a torque=T.θt=T.ω

- Efficiency 𝜂

η=work outputwork input   in a given time

η=power outputpower input

η=enery outputenergy input   in a given time

Example:

Efficiency η of a car=power carpower engine

- Fluid Flow

kg/m3

Density ρ:

ρ=mV

The density of water is usually taken to be 1000 kg/m3 although the value:

- decreases slowly as temperature increases,

- increases very slowly as pressure increases.

Definition of Newton

N≡kg∙ms2

Specific Weight γ:

γ=ρ∙g

m/s2

kg/m3

N/m3

- Specific Gravity or Relative Density:

Relative Density=Density of substanceDensity of water at 4°C

unitless

N/m2 or bar

Pressure:

P=FA

1 bar	≡	105 N/m2

- Compressibility:

m2 / N

compressibility=change in volumeoriginal volume×pressure change

- Pressure energy of unit mass or specific pressure energy:

N / m2 or bar

specific pressure energy=Pρ

kg / m3

J / kg

- Total specific energy E:

E=Pρ+v22+g.h

J / kg

h is the height of the fluid above some datum line.

- Bernoulli’s equation:

P1ρ∙g+v122∙g+h1=P2ρ∙g+v222∙g+h2+hf

where hf  is the frictional resistance “head”

Each quantity in the latter form of the Bernoulli’s equation is measured in terms of “head of fluid”, i.e. height of liquid above a given datum. The unit for each quantity is the meter.

Therefore,

(pressure head + kinetic head + potential head) all for the section 1

=

(pressure head + kinetic head + potential head + frictional resistance head) all for the section 2

- Continuity equation:

mass flow rate m kgs=volume flow rate×density ρ

volume flow rate Q m3s=area×velocity

Q=   A1∙v1  =   A2∙v2

Extra Notes:

- 1 cubic meter is equal to 1,000 litres:

1 m3≡1000 l

- For water:

1 kg≡1 l       (for water)

- The energy lost in friction, i.e. the loss of power, is given by:

m

loss of power=mass flow rate×g×hf

J/s or Watt

kg / s

m / s2

- Heat Transfer

- Conduction

Fourier’s law of heat conduction (1D, steady-state)

Q=-k.A.dTdx

Where the derivative term dTdx is called the temperature gradient; k is called the thermal conductivity which is a material property; A is the surface area of the material considered; Q is the steady heat flow rate through the material, i.e. the amount of heat transferred, i.e. heat leakage, i.e. heat loss.

- Plane wall with fixed surface temperatures / Conduction through composite wall.

Q=-U.A.T2-T1

T as for units C or Kelvin. ∆x as for units m. A as for units m2. k as for units W/(m.C) or W/(m.K). Q as for units W. U is either the heat transfer coefficient U=kx or the overall heat transfer coefficient 1U=1Ua+1Ub+1Uc+⋯

- Radial system with fixed surface temperatures.

For a single-layer cylindrical wall of a homogeneous material with constant thermal conductivity and uniform inner (1) and outer (2) surface temperatures:

Q=2πkLT1-T2lnr2r1

For a two layer cylinder the heat transfer rate is:

Q=2πLT1-T31kalnr2r1+1kblnr3r2

- Convection

Without regard to the classification of convection, the heat flow between a solid and a fluid is governed by Newton's law of cooling, defined by:

Q=h∙A∙Ts-Tf

Where Q is the rate of heat transfer by convection, in W; h is the convective heat transfer coefficient, in W/m2.K; A is the surface area of the solid in contact with the fluid, in m2; Ts is the temperature at the surface of the solid, in K; Tf is the temperature of the fluid stream, in K.

- Radiation

- The Stefan-Boltzmann Law

Q=ε∙σ∙A∙T4

Where Q is the energy radiated by the surface, in kW; A is the surface area of the body, in m2; ε is the emissivity of the surface (varies between 0 and 1) at the temperature T; σ is the Stefan-Boltzmann constant, 56.7×10-12 kW/m2K4; T is the temperature of the body, in K.

In practice, bodies do not radiate into perfect heat sinks (bodies that can absorb finite amounts of heat without undergoing any change in temperature) but also receive radiation from the surroundings. Thus if the body is at a temperature T2 and the surroundings at a lower temperature T1, the nett loss is given by:

Q=56.7∙ε∙A∙T210004-T110004

Difficulty is experienced when the body is outside, since it is hard to determine the sky radiation temperature. On a cold night, the sky radiation temperature will be of the order of 230 K. During the day, the sky temperature is often taken to be about 5C below ambient. The re-radiation from the atmosphere is often known as diffuse radiation and expressed as a percentage of the direct solar radiation.

Emissivity varies with temperature and also throughout the spectrum. For simplified analysis, the emissivity is often taken as an approximate mean value, as shown in following Table.

| Material | Emissivity ε |
| --- | --- |
| Polished aluminium Oxidised (corroded) aluminium Anodised (anti-corrosion) aluminium Polished steel Oxidised steel Asbestos Building brick Water and Ice | 0.095 0.2 0.95 0.07 0.8 0.95 0.45 0.96 |

A surface with an emissivity of 1 defines a black body

The Inverse Square Law

Intensity at Surface of Sphere

S4πr2=I

Source Strength

The energy twice as far from the source is spread over four times the area, hence one-fourth the intensity.

Sphere Area

4πr2

C = K – 273.15

K = C + 273.15