Rebar spacing

Longitudinal bars: the maximum distance of the following:

- Maximum nominal diameter

- Maximum aggregate size plus 5 mm

- 20 mm

Minimum vertical distance between groups: the maximum distance of the following:

- Maximum nominal diameter

- 20 mm

Maximum distance between longitudinal bars without any link: 150 mm

Jail construction: Maximum distance between bars: 250 mm

EXERCISE

Calculate the following rebar distribution (nominal cover 35mm):

1.- A column of 400x400mm with 4Ø20 at each side. Links Ø8

2.- A column of 600x400mm with 3Ø25+4Ø16 at the long side, and 2Ø25+2Ø16 at the small side.

3.- A beam of 400x500mm with 5Ø20 at the bottom and 3Ø12 at the top.

4.- A beam of 400x500mm with 10Ø20 at the bottom and 5Ø16 at the top.