A Study of the Driving Behaviours Monitoring

by

Qirui Wang

Supervisor: Gaynor Thomas

Project submitted as part of the requirements for the award of

MSC-SEAI Software Engineering and Artificial Intelligence

September 2022

DECLARATION

I, Qirui Wang declare that I am the sole author of this Project; that all references cited have been consulted; that I have conducted all work of which this is a record, and that the finished work lies within the prescribed word limits.

This work has not previously been accepted as part of any other degree submission.

Signed :

Date  : 6/9/2022

FORM OF CONSENT

I ___Qirui Wang__ hereby consent that my Project, submitted in candidature for the MSC-SEAI Software Engineering and Artificial Intelligence, if successful, may be made available for inter-library loan or photocopying (subject to the law of copyright), and that the title and abstract may be made available to outside organisations.

Signed __________________________

Date _____6/9/2022_______________________

# Abstract

The increasing number of motor vehicles and drivers makes the road driving environment more complex, which increases the probability of traffic accidents to a certain extent. A large number of analysis studies have shown that driver human factors are an important cause of road traffic accidents, and the main dangerous driving behaviour is distracted driving. Therefore, effective monitoring and real-time early warning for such dangerous driving behaviours have become more and more important. There is a problem with the current monitoring methods for driver distraction and fatigue driving behaviour, that is, it fails to take into account the accuracy and real-time requirements of driving behaviours monitoring at the same time. In view of the above problems, based on the deep learning target detection and behaviours recognition method, this paper realizes the accurate and rapid identification of the driver's distracted driving behaviours in the surveillance video. The method in this paper is of great significance to reduce the probability of traffic accidents and reduce the hidden dangers of driving safety. The main work and results of this paper are as follows:

Based on the characteristics of driver's distracted driving behaviours, a distracted driving behaviours detection algorithm based on improved SSD is proposed. Aiming at the problem of missed detection caused by serious occlusion of mobile phones and cups in driving images, this paper introduces a receptive field module in the low-level feature map of the SSD target detection model to enhance the feature extraction capability of the low-level feature map, thereby improving the model's detection of small objects Effect. In order to make SSD make full use of the semantic features of high-level feature maps and the detailed features of low-level feature maps, this paper first introduces the feature pyramid structure into the SSD model, and then adopts the adaptive spatial feature fusion method. Achieve optimal fusion of feature maps of different layers . Aiming at the problem that the default frame ratio of the SSD model is not applicable, this paper uses the k -means clustering algorithm to cluster the aspect ratio of the labeled images to obtain a set of default frame ratios that meet the current detection task. The experimental results show that the proposed algorithm has good performance in detection accuracy and speed.

Key words : distracted driving; deep learning; behaviour recognition

Content

Abstract	I

Content	III

Figure Content	V

Table content	VII

1. Introduction	1

1.1 Research background	1

1.2 Purpose and significance	4

2. Research status and existing problems	7

2.1 Research status	7

2.2 The research content of this paper	13

2.3 Chapter Organizational Structure	14

3 Distracted Driving Behaviour Detection	17

3.1 Distracted driving behaviour characteristics and detection process	18

3.2 Analysis of deep learning target detection algorithm	20

4. Research on Distracted Driving Behaviour Detection Algorithm Based on Improved SSD	24

4.1 Principle of SSD target detection algorithm	24

4.2 Low-level feature map feature extraction optimization	27

4.3 Feature fusion based on adaptive space	32

4.4 Optimized design of default frame ratio	41

5. Experimental results and analysis of distracted driving behaviour detection	47

5.1 Distracted driving behaviour discrimination method	47

5.2 Experimental dataset production	48

5.3 Model training strategy	51

5.4 Experiment and result analysis	53

5.4.1 Experimental design	53

5.4.2 Analysis of experimental results	54

6. Summary and Outlook	64

7. Reflection	66

References	67

Figure Content

Figure 3-1 Research ideas in this chapter	18

Figure 3-2 Driver distracted driving behaviour	19

Figure 3-3 Distracted driving behaviour detection process	20

Figure 3-4 R -CNN target detection process	22

Figure 4-1 SSD Model Structure	24

Figure 4-2 Cell phone/water cup occlusion	27

Figure 4-3 Schematic diagram of the receptive field	29

Figure 4-4 Schematic diagram of atrous convolution	30

Figure 4-5 Receptive Field Module	31

Figure 4-6 Visualization of SSD convolution feature map	32

Figure 4-7 Adaptive Spatial Feature Fusion	34

Figure 4-8 Size comparison of driver detection frame	37

Figure 4-9 Improved SSD model based on FPN	38

Figure 4-10 Conv4_ 3_p feature fusion process	39

Figure 4-11 SSD adaptive spatial feature fusion	40

Figure 4-12 RFAS-SSD distracted driving behaviour detection model	40

Figure 4-13 Distracted driving behaviour detection target box size	41

Figure 4-14 Object Labeling and Labeling Data	42

Figure 4-15 Schematic diagram of clustering results	44

Figure 5-1 cross-over	47

Figure 5-2 Partial sample image	49

Figure 5-3 Schematic diagram of image annotation	50

Figure 5-4 Training Loss-Epoch change curve	52

Figure 5-5 SSD test results	55

Figure 5-6 RFAS-SSD test results	55

Figure 5-7 FSSD detection sample diagram	57

Figure 5-8 M 2Det detection sample diagram	57

Figure 5-9 RFAS-SSD detection results	57

Figure 5-10 R FAS-SSD original default box detection example	59

Figure 5-11 R FAS-SSD default box detection sample image after optimization	59

Figure 5-12 Comprehensive comparison of target detection results	61

Figure 5-13 misidentified sample	62

Figure 5-14 Identify the sample correctly	63

Table content

Table 4-1 Cluster Center Results	44

Table 4-2 Default box aspect ratio for different layer feature maps	45

Table 5-1 Distracted driving behaviour discrimination method	48

Table 5-2 Distracted Driving Behaviour Detection Dataset Details	51

Table 5-3 Model training hyperparameter settings	51

Table 5-4 Comparison of experimental results between SSD and RFAS-SSD algorithm	56

Table 5-5 Comparison of experimental results of FSSD , R -SSD and RFAS-SSD algorithm	58

Table 5-6 Comparison of experimental results of RFAS-SSD algorithm before and after default frame optimization	60

Table 5-7 Distracted driving behaviour classification experiment comparison	61

1. Introduction

1.1 Research background

With the rapid socio-economic development of countries and the gradual improvement of people's living standards, more and more families own private cars, and the number of automobiles is constantly growing. According to the relevant data from China's National Bureau of Statistics[1], by the end of 2019, the number of motor vehicles in China had reached 348 million, of which the number of cars was 260 million, accounting for about 75% of motor vehicle ownership, an increase of 21.22 million, or about 8.83%, compared with the number of cars at the end of 2018. 2020 has been affected by the new crown epidemic, and the growth rate of cars has been reduced However, the total number is still in a state of continuous growth and, after the epidemic, more and more households are putting car purchase plans on the agenda to avoid using public transport on their way to travel. As a result, the number of motor vehicles in China will continue to grow, but this will be accompanied by an increase in traffic accidents.

According to statistics, traffic accidents are now the leading cause of death among China's teenage population, and with many drivers becoming progressively more reliant on making or receiving phone calls or sending messages, distracted behaviour has become the leading factor in causing traffic accidents. According to statistics from the China Statistical Yearbook-2015[2], in 2014, there were 196,812 traffic accidents causing major casualties in China, resulting in 58,523 deaths, 211,882 injuries, and direct property losses of 1.075 billion yuan. Among them, there were 74,746 traffic accidents caused by distracted behaviour, accounting for about 37.98% of the total, resulting in a total of 21,570 deaths, 76,984 injuries and direct economic losses of up to 458 million yuan. According to a study conducted by the Road Traffic Safety Research Centre of the Chinese Ministry of Public Security, a driver's distracted behaviour while driving can cause a serious lag in his or her reaction. For example, the reaction time of a driver in a normal state in response to an unexpected situation is 0.3s─1s, while the reaction time in a distracted state can reach about three times normal, leading to a significant reduction in the driver's ability to cope with risk, and the distance of following the car while driving gradually increases, making the road less efficient for vehicle traffic, and also reducing the driver's ability to control the vehicle. For example, when a driver makes or receives a phone call, his or her lateral control of the vehicle is reduced, including increased lateral acceleration, more lane departures or lane changes, and more frequent changes in the vehicle's distance from the lane line [3]. Experimental results from related studies also confirm this idea, with drivers making phone calls while driving causing a 20% decrease in their attention span, and even a 37% decrease when answering important calls [4]. Similarly, messaging while driving can increase the likelihood of a traffic accident by 50%, and the series of actions involved in messaging can lead to distractions that can affect normal driving behaviour [5]. In addition, actions such as reaching for objects in the car, turning the head to talk to passengers or staring at the in-car information system for long periods of time while driving are also harmful distracting driving behaviours that can lead to serious traffic accidents[6]. In response to bad driving behaviour, China's new traffic law stipulates that drivers must not engage in distracted driving behaviours such as making or receiving phone calls, sending messages, eating or drinking and watching mobile phone videos while driving, otherwise they will be punished according to the law. Therefore, the detection and early warning of distracted driving behaviour not only prevents violations of traffic laws, but more importantly, reminds drivers to avoid distracted driving, thereby reducing the number of traffic accidents.

Since the 1970s, many companies and researchers have researched distracted driving behaviour from various perspectives and have come up with their own detection methods, of which there are three main types: physiological data-based, vehicle control data-based and machine vision data-based. Firstly, physiological data includes EEG and ECG, and the detection method uses the brain activity reflected in the EEG signal and the heart rate inferred from the ECG signal as a feature vector to detect the driver's state [7]. Secondly, vehicle control data includes information such as steering wheel movement and pedal position. The detection principle is to take advantage of the feature that drivers usually act differently from their normal driving behaviour when driving distracted, and to determine distracted behaviour by collecting information such as steering wheel movement and application of pedals [8]. Finally, machine vision data includes image information such as the driver's face, eyes and body movements. The detection principle is to analyse the data from the captured cab images to derive whether the driver is engaged in distracted driving behaviour [9].

Of the three detection methods mentioned above, the physiological data-based detection method requires the driver to wear some physiological sensors while driving, thus causing sensory discomfort and thus interfering with the driver's normal driving behaviour [10]; while the vehicle control data-based method detects the driver's control behaviour of the vehicle only after it has been issued, and cannot give the driver a timely Vision data can detect the driver's state in a non-contact way, which does not interfere with the driver's normal driving and can also make timely and accurate judgments on the driver's state. Therefore, distracted driving behaviour detection methods based on machine vision data have become one of the most reliable detection and perception methods in practical applications, and this paper will also start research from machine vision-based detection methods.

1.2 Purpose and significance

There are a number of distracted driving detection methods that have emerged in the transport sector, and they are implemented in a variety of complex ways, including video surveillance by transport regulators to monitor distracted driving behaviour; there are also operating companies that install monitoring equipment on their vehicles and remind drivers to drive safely by means of remote measurement; and there are also companies that design detection systems for private cars that use vehicle collection information plus cloud-based processing. However, all of the above methods have certain drawbacks, including the inability to give drivers timely alerts and the high operating costs of the system, making it unsuitable for widespread use. However, with the development of technologies such as image processing, computer hardware and software, deep learning and high-performance embedded systems, it has become possible to monitor driver distraction behaviour in a low-cost, efficient and fully intelligent manner, making more and more private cars willing to install distraction detection systems to improve driving safety.

In recent years, the explosion of data volume and the rapid development of computer hardware have made deep learning an important research method in various fields, especially in image processing, where it has wider application [11]. As an emerging image processing technique, deep learning is not only different from traditional image processing methods, but also differs greatly from machine learning. The original research was inspired by the way the human brain processes information, by simulating the operation mode of the human brain to classify or process different things, and in learning the features of different things, instead of relying on the researcher's experience to extract features, the feature extraction is done automatically through different levels of networks, which not only simplifies the learning workflow, but also makes the extracted image features richer. As a result, the design of a more practical distracted driving behaviour detection system to aid safe driving is currently a hot topic of research. Therefore, this study adopts a machine vision-based approach, incorporating a variety of deep learning algorithms to extract deep potential features from multi-dimensional inputs, and uses a large amount of experimental data to train distracted driving behaviour models in order to obtain a high detection accuracy. At the same time, after the algorithm is ported to the hardware system, the system resources are used to optimise and accelerate the algorithm to further improve the real-time performance of the algorithm.

Based on the above design ideas, the aim of this paper is to investigate a timely and accurate distracted driving behaviour detection system that can be used in real driving scenarios, so as to improve the driver's alertness on the road and reduce the probability of traffic accidents, which is important for the safety of drivers and passengers.

2. Research status and existing problems

2.1 Research status

Since the 1970s, T.B. Sheridan [12] has decomposed the driving process into three levels of control tasks: strategic, tactical and operational, followed by J.A. Michon [13] T.A. Ranney [14] and others who have conducted research on the continuous control process of distracted driving behaviour. J.D. Lee [15] and others have summarised distracted driving behaviour as a process in which the continuous control process of driving behaviour is disrupted and different levels of distracted behaviour are produced, including talking on the phone, smoking, eating and drinking, fetching and dropping objects and drifting.

Professor Cheng Bo of Tsinghua University, whose team has been involved in driving-related research since 2000, has led the design of a driver status and behaviour monitoring technology and device, which analyses the driver's image information to determine whether the driver is in a distracted or dangerous driving state, which mainly includes the driver's hands off the steering wheel at the same time and other dangerous driving behaviour. These include the driver's hands off the steering wheel at the same time and other dangerous driving behaviours, such as eating, playing with a mobile phone or answering a phone call while driving, etc. If the driver is in such a distracted state, the system will issue an alert to remind the driver to drive safely [25].

In 2007, M. Kutila [16] et al. used traffic rule-based and SupportVectorMachine (SVM) classification methods to detect driver visual and cognitive workload by fusing stereo vision and lane tracking data, and tested the results with data collected on trucks and buses, which showed that the success rate of detecting visual distraction was over 80% and the average success rate of detecting cognitive distraction was 68-86%. M. Miyaji [17] et al. used a stereo camera system to conduct experiments on a driving simulator in which they enhanced the talk or arithmetic load of the testers and extracted their features by tracking the driver's eye and head movements, and to further improve the detection accuracy, the ECG presented heart rate data with pupil diameter data. To further improve detection accuracy, heart rate data from ECG and pupil diameter data were fused together as classification features; SVM and AdaBoost algorithms were then used for classification and recognition respectively. Finally, after evaluating the driver's cognitive load distraction performance, it was found that AdaBoost achieved an average accuracy of 92.6% on the driving simulator, while SVM achieved an average accuracy of 91.8%, so AdaBoost was able to achieve a higher accuracy. liNanxiang [18] et al. conducted experiments on the designed UT-Drive platform, which mainly includes two on-board cameras and embedded microphone arrays, one of which is used to acquire the image information inside the cab and the other is used to acquire the road environment information outside the vehicle; meanwhile, the driving information of the car is recorded by analysing the CAN-BUS bus signal of the car; then six algorithms, LDC, KNN, SVM1, SVM2, QDC and RUSB, are used to analyse the collected The information was then analyzed using six algorithms: LDC, KNN, SVM1, SVM2, QDC and RUSB, so as to establish a detection model for driver distracted behaviour, and the final detection accuracy of this platform was around 80%. t.H. Dinh [19]et al. separated human targets by depth-valued histograms, and then detected distracted driving behaviour by analyzing the difference between the initial frame and the current frame. Ma Xin [26]from Northern Polytechnic University conducted a study using visual distraction and cognitive distraction as detection targets and tested the experiments on a specially designed driving behaviour simulation experimental platform. The main indicators used in the experiments were the characteristics of the driver's eyes, which were collected through the distracted driving simulation platform and analysed by combining information on the driver's state and pupil diameter, including variance, root mean square and recurrence plots. Through experimental research, we know that in order to achieve fast and accurate driver status recognition, the data should be analysed in different ways according to the actual detection needs.

Most of the above methods are based on traditional algorithms or machine learning algorithms to detect distracted driving behaviour, and although they can achieve certain detection results, there is still a certain gap between them and the actual application requirements. With the development of deep learning, more researchers have started to use deep learning for distracted behaviour detection. 2016 StateFarm held a competition on Kaggle with the goal of detecting distracted driving behaviour on a provided dataset of dashboard camera images, which consisted mainly of images of drivers driving distracted and driving normally, with the final convolutional neural network ( Yu Zhiding [20]et al. used a cascaded convolutional neural network to identify driver facial features and detect distracted driving behaviour by analysing a series of changes in their eyes, mouth, etc. F. Vicente [21]et al. D. Tran [22]et al. used a simulated driving environment in the laboratory to detect ten behaviours such as normal driving, talking on the phone, texting, drinking, talking and operating the radio. , GoogleNet and ResNet, yielding test accuracies of 86%, 88%, 89% and 92%, respectively, at 14Hz, 12Hz, 11Hz and 8Hz on a JetsonTX1 embedded computer board, yielding a better overall performance of GoogleNet in the laboratory simulation environment. m. Leekha [23]et al. used foreground extraction to exclude background interference and a simple convolutional neural network architecture to detect driver human posture for distracted driving detection.

In addition, foreign car manufacturers have developed their own distracted driving behaviour detection systems [24]. For example, Mercedes-Benz has developed a driver attention assistance system, which mainly obtains the state of the vehicle by recording and analysing information about the vehicle and road environment, including the steering wheel rotation angle, vehicle speed, signals from the accelerator and brake pedal, lateral acceleration, changes in the distance between the vehicle and the lane line, the position of surrounding vehicles and the duration of continuous driving, and finally obtains the driver attention state, which is When it is below the safety threshold, a warning will be issued to remind the driver to drive safely or take a proper rest.

Wu Hao [27]of Southeast University proposed a driver behaviour classification method based on Long-term Recurrent Convolutional Networks (LRCN), which is based on the detection of the driver's upper body and head posture. The design idea is to first pre-process the driver image, then combine the convolutional neural network with the Gated Recurrent Unit (GRU) and add some optimization strategies to build a complete set of classification models for the driver's upper body posture and head posture, and combine the changes of the two body postures to distinguish different driving behaviours.

Chen Jun [28]et al. used a two-stage cascaded convolutional neural network to detect distracted driving behaviour, where the first stage was a pre-screening network for distracted behaviour, a lightweight image classification network, and the second stage was a convolutional neural network for accurate detection, mainly using a VGG network to extract features from images and accurately identify driving behaviour, which was finally tested on a publicly available dataset from State Farm, USA tested and the detection accuracy obtained was 93.3%. Ming Li [29]from the University of Electronic Science and Technology proposed a dynamic recognition algorithm that fuses CNN and Recurrent Neural Network (RNN) for distracted behaviours in the daze state, where the CNN network uses the ResNet-50V2 network, which is an improvement on ResNet-50, and fuses the RNN network by This algorithm is good at detecting normal and dazed behaviour, but not distracted driver behaviour.

Volvo has developed a driver safety warning system, which consists of an image acquisition module, a vehicle status monitoring module, a vehicle trajectory monitoring module and a control processing module, which analyses the driver's head position and deflection angle, the direction and duration of eye gaze, the relative position of the vehicle to the lane line, the steering wheel rotation data and other information to obtain the driver's The driver's driving status data is then compared with the driver's normal driving status data recorded in the control centre to determine whether the driver is in a distracted driving state at this time, and finally to provide early warning based on the detection results. On the commercial side, some manufacturers have gradually started to equip their distracted driving behaviour detection and warning systems on certain models, such as the Haval H9, BYD G6, Geely Link, Hongqi H7 and Changan UNI-T [30]. The technology used by BYD, Hongqi and Changan mainly uses a system consisting of cameras, infrared sensors and controllers to capture the driver's facial expressions, eye and head movements, and combine them with the vehicle's position in relation to the lane to determine whether the driver is in a distracted or fatigued driving state; the driver warning system used by Geely works on essentially the same principle as Volvo; while Haval uses an indirect monitoring method, mainly using steering wheel angle, lateral acceleration and pedal position to monitor and classify the driver's status into nine levels from mild to severe, and corresponding to different alarm methods.

In summary, from searching and analysing relevant research today, we can see that many scholars have conducted a large amount of research on the definition, classification, task rating, detection methods and warning methods of distracted driving behaviour, which not only lays the foundation for in-depth research on distracted behaviour, but also expands the research ideas in this thesis. For example, the detection accuracy of traditional algorithms is low in complex environments; most of the existing deep learning-based methods do not take into account the processing speed of the hardware system, resulting in poor real-time performance during actual detection; and the driver's driving habits and common distracted behaviours are not taken into account, resulting in inaccurate driver This paper uses the following methods to address these issues Therefore, to address these issues, this paper uses deep learning techniques to achieve fast and accurate monitoring of driver distracted driving behaviour by weighing the implementation of driving behaviour recognition and recognition accuracy.

2.2 The research content of this paper

The purpose of this paper is to reduce the probability of road traffic accidents and reduce safety hazards in the driving environment. Combined with deep learning target detection and behaviour recognition methods, effective monitoring and real-time early warning of driver distraction and fatigue driving behaviour are realized. The main research contents of this paper are as follows:

Based on the research of distracted driving behaviour detection method based on improved SSD, for the task of distracted driving behaviour detection, the current mainstream target detection algorithm is firstly analyzed, and the algorithm is improved according to the characteristics of driver's distracted driving behaviour. Aiming at the problem of missed detection caused by the serious occlusion of small targets such as mobile phones and cups when the driver is making a phone call or drinking a drink, the research uses the receptive field module to enhance the feature extraction ability of the low-level feature map of the target detection model and improve the model's detection of small targets. Effect. Aiming at the problem that the target detection model cannot fully utilize the semantic features of high-level feature maps and the detailed features of low-level feature maps, an adaptive spatial feature fusion method is studied to achieve the optimal fusion of feature maps of different layers and improve the detection performance of the model. Aiming at the problem that the default frame aspect ratio is not applicable, the k-means clustering algorithm is studied to cluster and optimize the default frame ratio of the target detection model.

2.3 Chapter Organizational Structure

This paper mainly focuses on the task of monitoring driver distraction and fatigue driving behaviour based on deep learning. It is divided into 5 chapters, and the chapter organization structure is shown in table 2-1.

Table 2-1 Chapter Organization Chart

| Charter1 | Research background and significance |
| --- | --- |
| Charter2 | Research status and existing problems. |
| Charter3 | Distracted driving behaviour detection; Analysis of deep learning target detection algorithm. |
| Charter4 | Research on distracted driving dehavior detection algorithm based on improved SSD - Principle of SSD target detection algorithm - Low-level feature map feature extraction optimization - Optimized design of default frame ratio - Optimized design of default frame ratio |
| Charter5 | Experimental results and analysis of distracted driving behaviour detection - Distracted driving behaviour discrimination method - Experimental dataset production - Model training strategy - Experiment and result analysis |
| Charter6 | Summary |
| Charter7 | Reflection |

Chapter 1: Introduction. Explains the development trend of road traffic accidents and the causes of accidents in recent years, and explains the significance of research on distracted and fatigued driving behaviour monitoring.

Chapter 2: Introduces the current status of research on driver distraction and fatigue monitoring at home and abroad, and analyses the problems and difficulties of the current distraction and fatigue monitoring methods. In response to these problems, the main research content of this paper is proposed. Finally the chapter organisation of this paper is expressed.

Chapter 3: From the analysis of the source of driver behaviour, distracted driving behaviour can be divided into three categories: firstly, distracted driving behaviour that satisfies the driver's need for physical comfort; secondly, distracted driving behaviour that satisfies the driver's need for pleasure; and thirdly, distracted driving behaviour triggered by the surrounding environment. Among the above distracted driving behaviours, this paper selects several of the more common distracted driving behaviours as objects of study, namely talking on the phone, drinking and sending messages, and then selects the SSD target detection algorithm as the basic algorithm for distracted driving behaviour detection.

Chapter 4: Introduces the SSD principle, design concept, and optimises the SSD target detection algorithm using the perceptual field module, adaptive feature fusion method, and default frame aspect ratio optimisation algorithm, respectively, in conjunction with the characteristics of distracted driving behaviour. The implementation of improved SSD-based distracted driving behaviour detection is presented.

Chapter 5: Introduces the distracted driving behaviour discrimination method, experimental dataset and network training strategy in this chapter. The detection results of the improved SSD-based distracted driving behaviour detection algorithm in this chapter are analysed through experimental comparisons.

Chapter 6: summarises the research work in this paper.

Chapter 7: Analyses the strengths and weaknesses of this paper's research and provides an outlook on distracted and fatigued driving behaviour monitoring methods.

3 Distracted Driving Behaviour Detection

Distracted driving is a phenomenon in which attention is directed to activities unrelated to normal driving while driving , resulting in a decrease in driving ability. The National Highway Traffic Safety Administration classifies distracted driving into four categories: visual distraction, auditory distraction, behavioural distraction, and cognitive distraction. In addition to cognitive distraction, the other three types of distracted driving can be manifested by the driver’s external behaviour, such as texting while driving (visual distraction), talking on the phone while driving (auditory distraction), and drinking while driving (behavioural distraction). distraction) etc. This chapter mainly studies the three types of distracted driving, firstly analyzes the characteristics of distracted driving behaviour, and proposes a method for detecting distracted driving behaviour. Then combined with the problems in the detection process, the corresponding optimization strategy is proposed. Finally, the feasibility and effectiveness of the distracted driving behaviour detection method are verified by experimental comparison. The specific research ideas of this chapter are shown in Figure 3-1.

Figure 3-1 Research ideas in this chapter

3.1 Distracted driving behaviour characteristics and detection process

With the development of modern information technology, smart phones, car navigation and other technological products are used more and more frequently in daily life, which makes the behaviour of distracted driving more and more serious, and more and more road traffic accidents are caused by this. From the analysis of the source of driver behaviour, distracted driving behaviour can be divided into three categories: one is the distracted driving behaviour that meets the driver's physical comfort needs, such as smoking, drinking, eating, etc.; Distracted driving behaviours, such as chatting, making phone calls, sending messages, etc.; third, distracted driving behaviours caused by the surrounding environment, such as taking care of children, paying attention to emergencies outside the car, etc. [31] . Among the above distracted driving behaviours, this paper selects several more common distracted driving behaviours as research objects, as shown in Figure 3-2, which are making phone calls, drinking drinks and sending messages.

( a ) Left-handed call    (b) Right-hand call

( c ) Right-hand drink    (d) Left-hand texting   (e) Right-hand texting

Figure 3-2 Driver distracted driving behaviour

In Figure 3-2, it can be seen from Figures (a) and ( b ) that when the driver makes a call, the position of the mobile phone overlaps with the position of the driver's hand and head to a certain extent; from Figure ( c ) , it can be seen that the driver drinks When drinking, the position of the water cup coincides with the position of the driver's hand in a certain proportion, and may coincide with the position of the driver's head; it can be seen from Figures ( d ) and ( e ) that when the driver sends a message, the mobile phone position only matches the position of the driver 's head. The hand positions coincide. According to the characteristics of the above distracted driving behaviour, this paper adopts the method of target detection, taking the driver's head, the driver's left/right hand, mobile phone and cup as the detection targets, and by analyzing the relative positions of different detection targets to determine the driver's Distracted driving behaviour category. The distracted driving behaviour detection process is shown in Figure 3-3.

Figure 3-3 Distracted driving behaviour detection process

3.2 Analysis of deep learning target detection algorithm

Object detection is one of the basic tasks in the field of computer vision, with a research history of nearly 20 years. In recent years, with the rapid development of deep learning technology, deep learning models have gradually replaced traditional machine vision methods and become the mainstream algorithms in the field of object detection. At present, target detection algorithms based on deep learning mainly include two-stage detection algorithms based on candidate regions and single-stage detection algorithms based on regression.

The detection algorithm based on candidate regions divides the detection problem into two stages, first generating candidate regions, and then performing category prediction and boundary adjustment on the candidate regions. The representative of such algorithms is the R-CNN [32] series of algorithms, such as R-CNN, SPP - Net , Fast R-CNN, Faster R-CNN , etc. Taking R-CNN as an example, the detection process is shown in Figure 3-4. First, the image is input, and then the selective search algorithm is used to generate about 2000 candidate boxes from the image, and then each candidate box is normalized to a unified Size, input to convolutional neural network, and finally use support vector machine and linear regression to achieve object detection. Although the proposal of R -CNN has greatly improved the accuracy of target detection, it also has serious problems such as slow detection speed, large memory footprint, and information loss. In response to these problems, researchers have successively proposed a variety of improved target detection algorithms based on R-CNN. SPP-Net cancels the image normalization process. In order to adapt to feature maps of different sizes, spatial pyramid pooling is introduced, and the dimension unification of feature candidate boxes is completed in the case of extracting features from the original image once . Fast R-CNN draws on the idea of SPP-Net , only performs convolution processing on the original image, and maps with the candidate frame when outputting features. At the same time, it adopts a multi-task loss function to achieve weight sharing during classification and regression, and has a significant improvement in detection speed and accuracy compared to R-CNN. Based on Fast R-CNN , Faster R-CNN uses RPN (Region Proposal Networks ) instead of the original selective search method to generate candidate frames by itself, which greatly reduces the number of candidate frames, and the quality of candidate frames is also substantially improved. . Since then, such as Mask Two-stage detection algorithms such as R- CNN and Cascade R-CNN are proposed for further optimization of speed or accuracy on the basis of existing models. In general, the target detection algorithm based on candidate regions has a good performance in detection accuracy, but this type of algorithm takes a lot of time to process the candidate frame, and it is difficult to meet the real-time requirements.

Figure 3-4 R -CNN target detection process

The regression-based detection algorithm does not require pre-extraction of candidate regions, and can directly regress the category probability and frame position of the target object from the image, and the final detection result can be obtained after a single detection. Such methods include the YOLO [33] family of algorithms and the SSD [34] family of algorithms. YOLO is the pioneering work of the single-stage detection method. It divides the image into cells, and then predicts whether each cell contains the target object, the category to which the target object belongs, and the position coordinates of the target object, and finally filters through non- maximum suppression. out the prediction frame of the target object. The detection speed of this algorithm is fast, but its measurement accuracy is relatively low, and the detection effect of small objects and nearby objects is not good. Like YOLO, SSD is a regression-based detection algorithm. The difference is that SSD uses multi-scale feature maps to detect targets of different sizes, which improves the detection accuracy of small target objects to a certain extent. In addition, SSD adopts the anchor idea to sample candidate regions in feature maps of different scales, which improves the recall rate of detection and the detection effect of small targets. YOLO On the basis of YOLO, v2 has carried out various improvements and optimizations in combination with the characteristics of SSD, so that the detection accuracy has also been significantly improved while ensuring a faster detection speed. YOLO v 3 at YOLO On the basis of v2, the residual module and feature pyramid structure are introduced to realize multi-scale prediction and improve the detection effect of small objects. For the optimization of SSD algorithms, researchers have successively proposed many algorithms such as R-SSD , FSSD, STDN and M 2Det . The emergence of these algorithms makes the regression -based single-stage detection not only has obvious advantages in detection speed, but also the detection accuracy can gradually be comparable to the detection algorithm based on candidate regions.

To sum up, the missed detection rate of the two-stage detection algorithm based on candidate regions is relatively low, but its detection speed is very slow, and it is difficult to meet the real-time detection requirements. The detection speed of the regression-based single-stage detection algorithm is very fast, and with the development in recent years, the detection accuracy of this type of algorithm has also been greatly improved. In the regression -based single-stage detection algorithm, SSD combines the advantages of YOLO and Faster R-CNN , and uses multi-scale feature maps for detection, which has good performance in detection accuracy and speed. In the task of driver distracted driving behaviour detection in this paper, it is necessary to complete the detection of small objects of different scales such as cups, mobile phones and driver's hands. Combined with practical application scenarios, this chapter adopts an improved SSD-based target detection algorithm to realize the detection of drivers. Detection of cardiac driving behaviour.

4. Research on Distracted Driving Behaviour Detection Algorithm Based on Improved SSD

4.1 Principle of SSD target detection algorithm

SSD [34] is a regression-based single-stage detection algorithm proposed by Liu et al. in 2016. Its network model consists of two parts, namely a feature extraction network and a multi-scale feature detection network. The model adds a convolutional layer on the basis of the feature extraction network to obtain feature maps of different scales, and then uses these feature maps to predict the target category and the offset of the target bounding box, and finally generates the final image through NMS (Non - Maximum Suppression). Test results. Figure 4-1 shows the SSD network structure.

Figure 4-1 SSD Model Structure

As can be seen from Figure 4-1, SSD uses VGG16 as the backbone network for feature extraction, and converts the fully connected layers fc 6 and fc7 of VGG16 into convolutional layers conv 6 and convolutional layers conv7. At the same time, the pooling layer pool 5 is composed of The original stride=2 becomes stride=1 . The conv 4_3 in VGG16 is the first feature map in the multi-scale feature detection network, followed by conv 7, conv8_ 2, conv9_ 2, conv10 _2 and conv11 _2 , a total of 6 feature maps, and the size of the feature maps are ( 38,38 ), ( 19,19 ), ( 10,10 ), ( 5,5 ), ( 3,3 ) , ( 1,1).

The design concept of SSD includes the following three points:

(1) Multi-scale feature map to achieve target detection

Multi-scale feature maps are feature maps of different sizes. In general, the low-level feature maps of convolutional neural networks are relatively large, and later convolution or pooling operations with a step size of 2 will be gradually used to reduce the size of the feature maps. The advantage of using multi-scale feature maps to achieve target detection is that the large feature maps of the low-level network can be used to detect relatively small objects, while the small feature maps of the high-level network are responsible for detecting large objects, so as to improve the detection effect of the network model.

(2) Convolutional Neural Network Direct Detection

SSD uses convolution to detect feature maps of different scales. When the feature map scale is , the convolution kernel can be used to obtain the detection result.

(3) Default frame size and aspect ratio settings

SSD sets default boxes with different scales or aspect ratios for each unit in the feature map, and the predicted bounding boxes are based on these default boxes. Usually, multiple default boxes are set for each unit, and the number of default boxes for different feature maps is different. The default frame size calculation is shown in formula (4-1).

(4-1)

in, Represents the default frame size of the feature map of the k - th layer , and represents the minimum and maximum values of the default frame size, which are 0.2 and 0.9 , that is, the default frame size corresponds to 0.2 and 0.9 of the original image, respectively . m represents the number of feature maps, m = 5. The default box size of the conv 4_3 layer feature map is set separately, and its value is 0.1.

SSD is generally set to , and the calculation of the width and height of the default box is shown in equations (4-2) and (4-3).

(4-2)

(4-3)

Among them, represents the default box width of the k -th layer feature map, and the scale is . Represents the default box height of the k -th layer feature map with a scale of . By default, each feature map will have a default box of scale . In addition, a scale is set to And the default box, so that each feature map is set with two square default boxes with an aspect ratio of 1 but different sizes. Therefore, each feature map has a total of 6 default boxes , but when implemented, only four default boxes are set for the conv 4_3, conv 10_2 and conv 11_2 layers , which is due to the large or small size of the feature maps of these three layers, Setting too many default boxes will bring a certain amount of computational waste, so the default boxes with aspect ratios of 3 and 1/3 are removed.

4.2 Low-level feature map feature extraction optimization

It can be seen from Section 4.1 that SSD uses low-level feature maps to detect small target objects, and high-level feature maps to detect large objects. Since the low-level feature maps have fewer convolutional layers, the feature extraction effect will be better than that of high-level features. inferior. At the same time, in the distracted driving behaviour detection task of this paper, when the driver makes a phone call or drinks a drink, the hand will cover more than half of the mobile phone or cup , as shown in Figure 4-2, which makes the mobile phone and the driver's image in the image. The extractable features of the cup are reduced. Therefore, the SSD network model will have serious missed detections when detecting small target objects such as mobile phones and water cups.

( a ) call                               ( b ) drink

Figure 4-2 Cell phone/water cup occlusion

In order to improve the feature extraction effect of the network model and reduce the occurrence of missed detection, the method in most cases is to use deep networks such as ResNet 101 and Inception as the backbone network of the detection model to extract image features. The deep network model has a powerful feature representation ability, which can greatly improve the detection effect. However, with the deepening of the network layer, the high computational cost brought by it cannot be ignored, which will seriously affect the real-time detection. In order to ensure the detection speed of the network model and improve the detection effect of the model for objects such as mobile phones and cups, this paper introduces the receptive field module and adds it to the convolutional layer conv 4_3 on the premise of keeping the structure of the SSD network model unchanged. And after conv 7 , to optimize the feature extraction ability of low-level feature maps of SSD network model.

The receptive field module simulates the relationship between the size of the receptive field and the eccentricity in the human visual system, strengthening the deep features learned from the convolutional neural network model, making the detection model faster and more accurate. In the human visual cortex, the receptive field of neuronal populations increases with increasing eccentricity in the retina [35] . This conclusion also proves that the area closer to the center has a higher proportion and role in recognizing objects. The receptive field module utilizes multi-branch convolution layers with different convolution kernels corresponding to receptive fields of different sizes, and adopts atrous convolution to control their eccentricity. constitute. Multiple convolution kernels resemble neuron population receptive fields of different sizes, while atrous convolution assigns individual eccentricities to each branch to simulate the ratio between the size and eccentricity of neuron population receptive fields by combining all The branch merges the convolutional transformations performed, resulting in the final spatial array of receptive fields. As shown in Figure 4-3, the receptive field module consists of two parts: the atrous convolution layer and the multi-branch convolution layer. Dilated convolutional layers can improve the receptive field of feature maps, while multi-branched convolutional layers can enhance the discriminability and robustness of features.

Figure 4-3 Schematic diagram of the receptive field

(1) Atrous convolutional layer

Atrous convolution [36] is widely used in tasks such as semantic segmentation and object detection. It can expand the receptive field without increasing the extra computational load, so that the output of each convolution contains a large range of information. Compared with standard convolution, atrous convolution adds a new parameter, dilation factor. rate). The standard convolution can be regarded as an atrous convolution with a dilation factor of 1. A dilated convolution with a dilation factor of n means that n-1 holes are injected between each element of the convolution kernel. For standard convolution, the receptive field calculation formula is shown in formula (4-4).

(4-4)

where is the receptive field of each point in the kth layer, is the size of the convolution kernel of the kth layer, and is the stride of the ith layer of convolution. The difference between the calculation method of the receptive field of the atrous convolution and the standard convolution is that the expansion factor in the atrous convolution can be understood as expanding the size of the convolution kernel. Insert 0 between each element of , then the size is , and the effect size of the equivalent convolution of the convolution kernel with the expansion factor d is . In the standard convolution receptive field calculation formula, the substitution is the receptive field of the hole convolution, as shown in formula (4-1).

(4-1)

Figure 4-4 is a schematic diagram of the operation of atrous convolution.

Figure 4-4 Schematic diagram of atrous convolution

, Figure ( a ) shows that the convolution kernel is a hollow convolution with an expansion factor of 1, and its receptive field is is 2, the receptive field is expanded to 2 ; Figure (c) is located in the next layer of Figure (b), the convolution kernel is , the expansion factor becomes 4, and the receptive field is expanded to . In contrast, using ordinary convolution with stride 1, the receptive field after three layers is only .

(2 ) Multi-branch convolutional layer

The multi-branch convolutional layers borrow ideas from Inception V4 [37] . Each branch contains a convolutional layer for reducing the number of feature channels and a convolutional layer for feature extraction . The convolution kernel sizes of the feature extraction convolution layers of different branches are respectively , and , in order to reduce the parameters and build a deeper nonlinear mapping, the convolution layers can be replaced by two stacked convolution layers. Similarly, convolutional layers can also be replaced by and convolutional layers. Finally, combine ResNet and Inception- ResNet The tributary module (shortcut) in the V2 structure constitutes the entire structure, as shown in Figure 4-5 .

Figure 4-5 Receptive Field Module

As can be seen from Figure 4-5, each branch of the receptive field module consists of two parts, a convolutional layer with a specific kernel size and a dilated convolutional layer with a corresponding expansion factor. The convolution kernel size and dilation factor have similar proportional functions to the receptive field size and eccentricity of neuronal populations in the visual cortex. In this section, the receptive field module is introduced after the low-level feature maps conv 4_3 and conv 7 of the SSD model to improve the feature extraction ability of the low-level feature maps for heavily occluded small target objects.

4.3 Feature fusion based on adaptive space

The inherent problem of convolutional neural network structure is that the high-level network has a strong ability to represent semantic information, and a weak ability to represent geometric detail information. The low-level network has a strong ability to represent geometric detail information, but a weak ability to represent semantic information. Figure 4-6 is the visual feature map of the SSD network model Conv 1_2, Conv 2_2 , Conv 3_3 and Conv 4_3 layers. It can be seen that the shallow features are more inclined to the detection of image edges, the detected content is comprehensive, and there will also be key information is extracted. As the number of layers deepens, the feature map becomes more and more abstract, and more semantic information of the image is extracted.

original image    Conv1_2       Conv2_2       Conv3_3      Conv4_3

Figure 4-6 Visualization of SSD convolution feature map

Although the SSD network model uses feature maps of different levels for prediction, it treats them as the same level, so local detailed features (target positioning) and global semantic features (target recognition) cannot be fully utilized. Therefore, this section combines the low-level detailed features of the detection model with the high-level semantic features, adopts an adaptive spatial feature fusion method, and makes full use of the detailed features and semantic features of multi-scale feature maps to achieve target location and recognition.

At present, in order to make full use of the semantic information of high-level features and the fine-grained information of low-level features, most target detection algorithms use Feature Pyramid (FPN) to achieve feature fusion, such as YOLOv 3, FSSD, etc. Concatenation ) or element-wise direct connection or addition, which cannot make full use of features of different scales. Moreover, another disadvantage of FPN is its inconsistency at different scales. When using feature pyramids to detect objects, large objects are usually associated with high-level feature maps, and small objects are associated with low-level feature maps. When an object is at a certain level When the feature map of is considered a positive sample, the corresponding regions in the feature maps of other levels are considered as background. Therefore, if the image contains both large and small objects, the conflicts between features at different levels often occupy the main part of the feature pyramid. This inconsistency interferes with gradient computation during training and reduces the effectiveness of the feature pyramid. The Adaptive Spatial Feature Fusion (ASFF) [38] method can solve the above problems well, which enables the network to directly learn how to spatially filter features from other layers in order to retain only useful information for combination. For a feature map of a certain layer, the feature maps of other layers are first adjusted to the same resolution and fused, and then trained to find the optimal fusion. At each spatial location, features from different levels are adaptively fused, some features may be filtered out due to carrying contradictions at that location, and some features may be retained due to more discriminative cues. ASFF has the following advantages: (1) Since differentiation is used to search for the best fusion method, it can be easily learned in backpropagation; (2) It is independent of the backbone model and is suitable for a single-stage detection model with a feature pyramid structure; (3) The implementation is simple, and the increased calculation amount is very small. The ASFF structure is shown in Figure 4-7.

Figure 4-7 Adaptive Spatial Feature Fusion

The green box in Figure 4-7 describes how ASFF performs feature fusion. Taking level3 as an example, , , are the features of level1, level2 and level3 respectively ( meaning that the leveli feature map is adjusted to the same size of the level3 feature map), , , are from Weight parameters for different layers. The new fusion features can be obtained by multiplying the features of different layers with the weight parameters of the corresponding layers, and then summing them up. The calculation formula of feature fusion is shown in formula (4-2).

(4-2)

Among them , it represents the feature fusion result of the first layer, which represents the features of different layers, and , , and , which represent the spatial weights of the feature maps of different layers. These weights are adaptively learned through the neural network and can be shared among all channels. In the above formula, the summation method is used to fuse the features, and it is necessary to ensure that the feature maps of the level1, level2 and level3 layers have the same size and the same number of channels when adding . Therefore, it is necessary to upsample or downsample the feature maps of different layers and adjust the number of channels. For layers that need upsampling, for example, to obtain ASFF3, level1 and level2 need to be adjusted to the same size as level3. The number of channels can be adjusted by convolution first, and then the size can be increased by interpolation ; for layers that need downsampling For example, to get ASFF1, for level2 to level1 , you only need to use convolution with stride 2. If it is level3 to level1, you need to add max pooling with stride 2 before convolution with stride 2 layers to achieve consistent size.

For the weight parameter, the formula is specified , and the formula is shown in formula (4-3).

(4-3)

In the above formula, it is defined by the softmax function and is the control parameter of the softmax function. These control parameters are calculated by convolution of the adjusted feature maps of level~level3 , so they can be learned by standard backpropagation. they.

Usually, after the single-stage detection model is introduced into the feature pyramid, it follows the chain rule, and the gradient calculation formula is:

(4-4)

Since the transformation between different scale feature layers is up-sampling or down-sampling, it is usually a fixed value. To simplify the expression, set its value to 1. Moreover , the derivatives of the two activation operations with respect to the output feature will also be fixed values, so their values are also simplified to 1 . The formula (4-4) can be written as:

(4-5)

Assuming that there is a target and a positive sample on the feature map of the level 1 layer , the corresponding position of other layers may just be a negative sample, so that both positive samples and negative samples are included in the backpropagation process, which will affect the gradient result. cause interference and reduce training efficiency. In the ASFF method, the gradient expression of backpropagation is:

(4-6)

It can be controlled by adjusting the weight parameters. When the above situation occurs, let , the gradient of the negative sample will not interfere with the result.

In order to improve the detection accuracy and make full use of the semantic features of high-level feature maps and the detailed features of low-level feature maps, this paper adopts the adaptive spatial feature fusion method on the basis of SSD to enhance the feature fusion effect. First, the feature pyramid is introduced into the SSD feature map. Before that, the size of the detection target required for the current detection task is analyzed. As shown in Figure 4-8, in the pixel input picture, The head sizes are , , , and the largest of the remaining targets is only , these targets are small targets relative to the entire picture.

Figure 4-8 Size comparison of driver detection frame

10_2 and Conv 11_2 in the original SSD model are too small, the images are too abstract, and there is relatively little information that can be fused. And these two layers of feature maps are used to detect large targets, which are not very useful in the detection task of this paper. Therefore, in this paper, the last two layers of the SSD network are removed, and only the first 9 layers of SSD are used for target detection, and feature pyramids are introduced into them. , the improved detection model is shown in Figure 4-9 .

Figure 4-9 Improved SSD model based on FPN

Figure 4-9 , there are 4 prediction feature maps, namely Conv4_3_p , Conv7_p , Conv 8_2_p and Conv 9_2_p. Taking the first layer feature map as an example, Conv 4_3_fus in the figure is the first layer feature map Conv4_ 3 of the original SSD and the next layer Conv5_ 3 are upsampled by bilinear interpolation, and the features are extracted by convolution and normalized. Then Conv 4_3_fus is dimensionally reduced to the same size as the original feature map Conv4_3 to obtain the predicted feature map Conv4_3_p through convolution, and the feature fusion process is shown in Figure 4-10 . The generation of feature maps of the latter two layers is similar to that of Conv4_3_p , and the final predicted feature map is the same size as the original SSD feature map.

Figure 4-10 Conv4_ 3_p feature fusion process

In Figure 4-10, the scale of Conv4_3_p is 512, which is obtained by reducing the dimension of Conv 4_3_fus through the convolution kernel . The scale of Conv 4_3_fus is 1024 and the number of channels is 1024, which is composed of bilinear interpolation cascades of feature maps of scale of original Conv4_3 and high-level Conv5_3 . The fusion method of Conv7 _p and Conv 8_2_p layer scale is the same as that of Conv4 _3_p , while Conv9 _2_p is directly obtained by convolution and convolution of the feature map Conv9 _2 in the original SSD .

After the feature pyramid is introduced into the improved SSD, adaptive spatial feature fusion needs to be completed. The original ASFF only fuses feature maps of 3 scales, and the number of feature map scales here is 4. Therefore, it is necessary to make the ASFF feature fusion formula. The corresponding modification, taking ASFF-4 as an example, the formula after modification is:

(4-7)

where X is the feature of different layers, They are the feature weights of Conv 9_2_p, Conv 8_2_p, Conv7 _p , and Conv4 _3_p , respectively. The features of different layers are multiplied by the weights of the corresponding layers, and the results obtained by each layer are added to obtain the final fusion feature. The fusion feature map is shown in Figure 4-11.

Figure 4-11 SSD adaptive spatial feature fusion

In Section 4.2, a receptive field module is added after the convolutional layers conv 4_3 and conv 7 to optimize the feature extraction capability of the low-level feature maps of the SSD detection model. Combining it with this section, the structure of the final SSD detection model based on receptive field module and adaptive feature fusion ( RFAS-SSD ) is shown in Figure 4-12. The RFB structure is shown in Figure 4-5 in Section 4.2, which is bilinear upsampling feature fusion, and ASFF represents adaptive spatial feature fusion.

Figure 4-12 RFAS-SSD distracted driving behaviour detection model

4.4 Optimized design of default frame ratio

In the original SSD detection model, the aspect ratio, size, and quantity of the default frame are manually set in advance, in order to better detect 20 types of objects with different shapes. The aspect ratio of the default frame is generally set to , The default box total is 8732 . However, in the distracted driving behaviour detection task of this paper, the required detection targets are the driver's head, driver's hand, mobile phone and cup. Figure 4-13 shows two randomly selected driver distracted driving images. It can be seen from the figure that the ratio of the default box of the original SSD detection model is not well suited for the distracted driving behaviour detection task in this paper.

Figure 4-13 Distracted driving behaviour detection target box size

In order to obtain a set of default frame aspect ratios suitable for the distracted driving behaviour detection model, this paper first collects the a priori frame aspect ratios in the labeled original data as sample data, and then uses the k -means clustering algorithm to calculate k By adjusting the value of k, the final default frame ratio data set after clustering is selected , and this data set is used as the default frame ratio of the SSD detection model to optimize the detection effect of the SSD model. Figure 4-14 shows some annotated images and corresponding data.

( a ) Partially annotated image

( b ) Screenshot of annotated image data

Figure 4-14 Object Labeling and Labeling Data

k -means clustering algorithm is an unsupervised learning algorithm, which divides n points in the sample into k clusters , each cluster has a cluster center, which is determined by the value contained in the cluster The mean is calculated. Each point in the sample belongs to the cluster corresponding to its nearest cluster center. For the original sample data , k - means clustering is to divide the n data into k sets ( ) , so that the sum of squares of errors between the data in the set and the cluster center is the smallest, and the corresponding formula is as follows.

(4-8)

where is the cluster center, which is the mean of all points in the cluster. represents the Euclidean distance between x and y . Algorithm 4-1 is the specific implementation steps of the k -means clustering algorithm.

Algorithm 4-1 k -means clustering algorithm Input: n data points // target aspect ratio The number of clusters k Output: Clustering results

1 BEGIN 2 Get the number of data n, the data dimension d 3 Randomly generate k initial cluster center points 4 While not converge do 5 For p = 1,2,… ,n do// traverse n input data points (aspect ratio) 6 Calculate which type of cluster each point belongs to: // t is the t-th operation 7 8 End For 9 For i = 1,2,… ,k do // traverse k center points 10 Find all data points that belong to this cluster center 11 Update the center point coordinates of the clusters, and modify the cluster center coordinates to the center coordinates of these data points: 12 End For 13 End While 1 4 End

Using the above k -means clustering algorithm, collect the aspect ratio of the labeling target frame, and set the number of cluster centers k=4, 5, 6, 7, 8, 9 for clustering calculation, and the obtained clustering results are shown in Figure 2 -19 shown. (a)~ ( f ) in Figure 4-15 correspond to the clustering results with k values of 4, 5, 6, 7, 8, and 9, respectively.

(a) k =4 							   ( b ) k =5

( c ) k =6 							   ( b ) k =7

( e ) k =8 								(f) k =9

Figure 4-15 Schematic diagram of clustering results

In Figure 4-15, different colors and shapes are used to represent different clusters, and the cluster centers are identified by red diamonds. When the k value is 4, 5, 6, 7, 8, and 9, the corresponding cluster centers are shown in Table 4-1 shown.

Table 4-1 Cluster Center Results

| k -means cluster centers | Clustering results |
| --- | --- |
| 4 | 0.48, 0.70, 0.84, 1.07 |
| 5 | 0.42, 0.59, 0.74, 0.86, 1.11 |
| 6 | 0.42, 0.59, 0.73, 0.82, 0.93, 1.18 |
| 7 | 0.38, 0.52, 0.63, 0.73, 0.81, 0.91, 1.17 |
| 8 | 0.38, 0.52, 0.64, 0.74, 0.79, 0.85, 0.95, 1.20 |
| 9 | 0.37, 0.51, 0.61, 0.71, 0.78, 0.85, 0.94, 1.11, 1.44 |

According to the clustering results in Table 4-1, combined with the actual distracted driving behaviour detection scene, this paper selects the clustering results with 8 k -means cluster centers as the benchmark ratio of the default box, and uses rounding to choose the results. , the final selected default frame aspect ratio is . Considering the different sizes of the objects detected by the feature maps of different layers, the low-level feature maps detect small objects, and the high-level feature maps detect large objects. The large target in distracted driving behaviour detection is the driver's head, whose length-width ratio has a small variation range. The small targets are mobile phones, water cups and the driver's hand, and the ratio of length to width varies widely. Therefore, this paper adjusts the number of default frame aspect ratios of feature maps of different layers according to the actual situation, and the results are shown in Table 4-2.

Table 4-2 Default box aspect ratio for different layer feature maps

| Feature map | Aspect ratio |
| --- | --- |
| Conv 4_3 |  |
| Conv7 |  |
| Conv8_2 |  |
| Conv9_2 |  |

distracted driving behaviour detection model, the scale of the feature map Conv 4_3 is 38 38, corresponding to 8 groups of default box ratios. The scale of feature map Conv 7 is 19 19, corresponding to 8 groups of default box scales . The scale of the feature map Conv 8_2 is 1 0 10, corresponding to 5 groups of default box scales . The scale of the feature map Conv 9_2 is 5 5, corresponding to 5 groups of default box scales . There are n n center points in the feature map of size n n , each center point generates k default boxes, and the number of default boxes generated by each center point of the four-layer feature map is 8, 8, 5, and 5, respectively . A total of 15065 default boxes are generated.

5. Experimental results and analysis of distracted driving behaviour detection

5.1 Distracted driving behaviour discrimination method

In this paper, the problem of discriminating driver's distracted driving behaviour is transformed into a target detection problem. First, the RFAS-SSD algorithm is used to detect and output the position and category of key targets (driver's head, hand, mobile phone and water cup) in the distracted driving image. Then the intersection ratio (IOU) between target detection frames of different categories is calculated according to the target detection results, and finally the distraction driving behaviour classification is realized by using these intersection ratios as the basis for discriminating the driver's distracted driving behaviour.

(1) Intersection over Union (IOU)

The intersection and union ratio refers to the ratio of the intersection and union of two regions. As shown in Figure 5-1, the intersection of rectangle A and rectangle B is the yellow border area in the figure, and the union is the red border area. The size of the intersection ratio is the area of the yellow border area divided by the area of the red border area in the figure .

Figure 5-1 cross-over

(2) Classification of distracted driving behaviours

The above intersection ratio can well represent the positional coincidence relationship between the two target detection frames. If the IOU is greater than 0, there is an overlap between the target detection frames, and vice versa. According to this characteristic, the classification method of distracted driving behaviour is shown in Table 5-1. The detection box of the target category is represented in the table .

Table 5-1 Distracted driving behaviour discrimination method

| Distracted Driving Category | Judgment basis |
| --- | --- |
| Call up | Premise : mobile phone , hand and head are detected ; Conditions : and |
| send Message | Premise : mobile phone , hand and head are detected ; Condition : and |
| drink | Premise : detect the water cup, hand , head; Condition : or |
| normal driving | 1. Mobile phones and water cups are not detected; 2. Detect the water cup, hand , head , |

5.2 Experimental dataset production

The distracted driving behaviour detection task in this paper adopts the public Kaggle distracted driving dataset [39] as the standard dataset. The data set is divided into a training set and a test set. The training set contains 10 types of driving behaviours, namely normal driving, sending messages with the left hand, sending messages with the right hand, making a phone call with the left hand, making a phone call with the right hand, operating the car radio, drinking a drink, turning around, Make up and talk to others, the number of pictures in each category is between 2000 and 2500, a total of 22424 pictures. The test dataset contains a total of 79,726 test images of the above 10 categories of driving behaviours.

Since the Kaggle distracted driving data set cannot be directly used in the distracted driving behaviour detection task in this paper, in order to meet the current task requirements, this paper selects 6 types of driving behaviour data in its training set to recreate the data set, as shown in Figure 5-2. The 6 types of driving behaviours are normal driving, calling with the right hand, sending a message with the right hand, making a phone call with the left hand, sending a message with the left hand and drinking a drink. There are 14,070 pictures in total. Some low-quality data are filtered out of these pictures, and 12,800 pictures are selected. Pictures as raw data numbers.

Figure 5-2 Partial sample image

After obtaining the original data, this paper follows the following steps to create a distracted driving behaviour detection dataset:

(1) Data renaming

Create a JPEGImages folder, place all categories of pictures under this folder, and rename each picture in the folder in the format "0xxxxx.jpg", where " x xxxx " means all from 00000 to 1 2799 Numbering.

(2) Data labeling

Open the labelImg annotation tool and change the classification category in labelImg to the target categories "head", "hand", "phone" and "bottle" that need to be detected for the current task. The annotation tool labelImg reads the image to be annotated from the JPEGImages folder, and stores the “ .xml ” file generated after the annotation in the newly created “ Annotations ” folder. The schematic diagram of image annotation is shown in Figure 5-3.

Figure 5-3 Schematic diagram of image annotation

(3) Divide training set, validation set and test set

After the image data annotation is completed, the data set is divided into training set, validation set and test set according to the ratio of 6:2:2. After the division, the number of samples in the training set is 7680, and the sample data in the validation set and test set are 2560 each. Table 5-2 is a detailed data list of the six driving behaviour categories.

Table 5-2 Distracted Driving Behaviour Detection Dataset Details

| driving behaviour category | Training set | validation set | test set | total |
| --- | --- | --- | --- | --- |
| normal driving | 1306 | 436 | 436 | 2178 |
| Send a message (right hand) | 1294 | 431 | 431 | 2156 |
| call (right hand) | 1262 | 421 | 421 | 2104 |
| Send a message (left hand) | 1281 | 427 | 427 | 2135 |
| call (left hand) | 1270 | 423 | 423 | 2116 |
| drink | 1267 | 422 | 422 | 2111 |
| total | 7680 | 2560 | 2560 | 12800 |

5.3 Model training strategy

The distracted driving behaviour detection model is trained and validated on the dataset produced in Section 4.2. The parameter initialization method and training optimization algorithm adopted by the model are shown in Sections 4.2.1 and 4.2.2 . The settings of hyperparameters during training are shown in Table 5-3.

Table 5-3 Model training hyperparameter settings

| Hyperparameters | parameter value |
| --- | --- |
| The number of iterations (epoch) | 300 |
| batch size ( batch _size ) | 64 |
| Momentum factor (momentum) | 0.9 |
| learning rate ( learning_rate ) | 0.001 |
| Learning rate decay rate ( lr_decay_rate ) | 0.1 |
| learning rate decay steps ( l r_ decay_ steps ) | 150, 200 |
| Weight decay factor ( weight _decay ) | 0.0005 |

The deep learning framework used in this section is Pytorch , and the experimental environment is configured as follows: memory capacity 32G ; CPU model INTEL I7 8700K; GUP model GTX1080 Ti ( quantity 2 ) . The number of iterations during training is set to 300 rounds . Considering factors such as the convergence speed of the network and the memory capacity of the experimental environment, the training batch size is set to 64, that is, the number of samples for one batch training is 64 . The initial learning rate was set to 0.001, and the learning rate was decayed to 0.0001 at 150 epochs and 0.00001 at 200 epochs . The Loss-Epoch change curve during the training process is shown in Figure 5-4.

Figure 5-4 Training Loss-Epoch change curve

As can be seen from the above figure, during the training process, the model loss value gradually decreases. The loss decreases rapidly in the first 100 epochs, the learning rate decays at the 150th epoch and the 200th epoch, and the loss decreases slowly. At the 250th epoch, the model starts to converge, and the loss value is 0.186.

5.4 Experiment and result analysis

In order to verify the detection effect of the distracted driving behaviour detection model RFAS-SSD based on the improved SSD , this paper uses the average precision (AP) and the frame rate per second (FPS) as the evaluation criteria for target detection, and designs four sets of comparative experiments. The results of each group of experiments were analyzed and summarized.

5.4.1 Experimental design

Experiment 1 : Compare the original SSD detection model and the improved RFAS-SSD detection model on the distracted driving behaviour detection dataset produced in Section 5.2, and analyze and compare the average accuracy and frame rate per second before and after model optimization. In the training process, the original default frame generation strategy of SSD is adopted. There are six different SSDs, and the default frame sizes of the layer feature map are 4, 6, 6, 6, 4, and 4 respectively . For details , see Section 4.1 . The default frame sizes of the four different layer feature maps of the improved RFAS-SSD are 4 , 6, 6, and 6, respectively .

Experiment 2: The RFAS-SSD model based on adaptive spatial feature fusion in this paper, the FSSD [40]model (2017) based on multi-feature fusion, and the M2Det [41] model (2019) based on multi-level feature pyramid (MLFPN) are respectively Comparative experiments are carried out on the distracted driving behaviour detection dataset. By comparing the average accuracy and frame rate per second of the three algorithms, the detection effect of different feature fusion methods is verified. The above algorithms all use the S SD original default box generation strategy.

Experiment 3: Use the original default frame generation strategy of SSD and the default frame generation strategy in Section 4.4 to conduct experiments on the RFAS-SSD algorithm, verify and analyze the average accuracy and frames per second of the RFAS-SSD algorithm when using different default box generation strategies Rate. When using the default box generation strategy in Section 4.4, the default box scales of the four different layer feature maps of the RFAS-SSD algorithm are 8, 8, 5, and 5, respectively.

Experiment 4: Select 200 pictures of each of the four driving behaviours of normal driving, calling while driving, sending a message, and drinking in the experimental test set, set driving behaviour category labels for all 800 pictures, and use the distracted driving behaviour in Section 5.1 The discrimination method is based on the four algorithms of SSD, FSSD, M2Det and RFAS-SSD ( default frame optimization) to conduct classification experiments, and analyze and compare the false recognition rates of the four algorithms.

5.4.2 Analysis of experimental results

Experiment 1 : Figures 5-5 and 5-6 are sample images of SSD and RFAS-SSD detection models for the driver to make calls, drink drinks and send messages while driving.

Figure 5-5 SSD test results

Figure 5-6 RFAS-SSD test results

Analysis of Figures 5-5 and 5-6 above , in Figure 5-5, the SSD model misses the mobile phone in the image of the driver calling, and the beverage bottle in the image where the driver sends a message. The main reason is that the mobile phone and the beverage bottle are about five Four-thirds of the parts are occluded by the driver's head and hands, resulting in the small visible size of these targets in the picture, so missed detection occurs. In Figure 5-6, the RFAS-SSD model in this paper accurately detects the driver's head, the driver's left and right hands, mobile phones and beverage bottles without any missed detection, mainly because RFAS-SSD detects the features of the low-level feature map of the model. The extraction is optimized, and the features of different layers of high and low are fully integrated, which greatly improves the detection effect of small target objects. Overall, the detection effect of the RFAS-SSD model in this paper is generally better than the original SSD model. Table 5-4 shows the detailed experimental results of the two models on the distracted driving behaviour detection dataset.

Table 5-4 Comparison of experimental results between SSD and RFAS-SSD algorithm

| algorithm | mAP | head _ | h and | phone | b ottle | F PS |
| --- | --- | --- | --- | --- | --- | --- |
| S SD | 0.838 _ | 0.978 | 0.7 72 | 0.8 19 | 0.784 | 43 |
| R FAS-SSD | 0.875 _ | 0.972 | 0.8 31 | 0.8 7 | 0.8 25 | 35 |

RFAS -SSD algorithm is increased by 3.7 percentage points compared with the SSD , which is mainly reflected in the detection of small objects such as hands, mobile phones and water cups. For the detection of the head, both algorithms are used. Good results have been achieved. Comparing the frame rate per second of the two algorithms, RFAS-SSD drops 8FPS compared to SSD . In general, the RFAS-SSD algorithm sacrifices the target detection speed and improves the average target detection accuracy. The main reason is that RFAS-SSD introduces a receptive field module and performs adaptive feature fusion on feature maps of different layers , which improves the detection effect of small targets, but the model structure is also more complex.

Experiment 2: Figures 5-7, 5-8, and 5-9 are sample images of FSSD, M2Det , and RFAS-SSD detection models for the driver to call, drink, and send messages while driving.

Figure 5-7 FSSD detection sample diagram

Figure 5-8 M 2Det detection sample diagram

Figure 5-9 RFAS-SSD detection results

Comparing the detection results of Figures 5-7 , 5-8, and 5-9, M2Det and the RFAS-SSD model in this paper have good detection results , all targets are detected, and no missed detection occurs. The FSSD model misses the phone in the driver's phone calls and texting images. The reasons for the missed detection are analyzed. First, when the driver holds the mobile phone , the palm of the driver almost holds most of the position of the mobile phone, which leads to the reduction of extractable features of the target. Second, the feature fusion method of the FSSD model and the feature pyramid structure used are simpler than M 2Det and RFAS-SSD, and its detection effect for small targets is inferior to M 2Det and RFAS-SSD networks. To further verify the detection effects of the three models, Table 5-5 shows the detailed experimental results on the distracted driving behaviour detection dataset.

Table 5-5 Comparison of experimental results of FSSD , R -SSD and RFAS-SSD algorithm

| algorithm | mAP | head _ | h and | phone | b ottle | F PS |
| --- | --- | --- | --- | --- | --- | --- |
| FS SD | 0.852 _ | 0.981 _ _ | 0.779 _ | 0.83 _ | 0.817 _ | 3 7 |
| M 2Det | 0.879 _ | 0.985 | 0.827 _ | 0.861 _ | 0.842 _ | 14 |
| R FAS-SSD | 0.875 _ | 0.972 | 0.8 31 | 0.8 7 | 0.8 25 | 35 |

From the data in the table, it can be seen that among the three algorithms, R FAS-SSD has the best detection effect on the mobile phone and the driver's hand, and M 2Det has the best detection effect on the water cup. All three have the same effect on the driver's head detection. better performance. Compared with mAP , the algorithm R FAS-SSD in this paper is 0.4 percentage points lower than that of M 2Det , and 2.3 percentage points higher than that of FS SD , which has a certain improvement in the detection of hands, mobile phones and water cups. Compared with the frame rate per second, the detection speed of M 2Det is the slowest, and the FPS is only 1 4 , which is difficult to meet the real-time requirements. Both R FAS-SSD and FSSD have reached FPS of more than 35, which has certain advantages in detection speed. Analyzing the above results, the feature fusion effect of R FAS-SSD and M 2Det is better than that of FS SD , which is mainly reflected in the detection of small target objects. Since M2Det adopts a multi-level feature pyramid structure MLFPN, MLFPN consists of multiple modules, and its structure is much more complicated than that of R FAS-SSD and FS SD , which affects its target detection speed to a certain extent. In general, the R FAS-SSD algorithm in this paper can take into account the speed and accuracy of target detection at the same time , and has a good detection effect on the basis of ensuring the detection speed.

Experiment 3: This experiment mainly compares the detection effect of R FAS-SSD before and after optimization of the default frame ratio , as shown in Figures 5-10 and 5-11 , which are sample images of R FAS-SSD model detection before and after the default frame optimization.

Figure 5-10 R FAS-SSD original default box detection example

Figure 5-11 R FAS-SSD default box detection sample image after optimization

As can be seen from the above figure, all key targets in Figure 5-11 can be detected after the default frame is optimized, and no missed detection occurs. When the original SSD default frame is used, the mobile phone is not detected in the image of the driver's message in Figure 5-10. Part of the reason is that the way the driver holds the mobile phone makes the mobile phone almost vertical in the image, and its size ratio is larger. In line with the optimized default box ratio , in order to further analyze the impact of the default box generation strategy in this paper on the model performance, Table 5-6 shows the comparison of the experimental results of the two detection methods on the distracted driving behaviour detection dataset.

Table 5-6 Comparison of experimental results of RFAS-SSD algorithm before and after default frame optimization

| algorithm | mAP | head _ | h and | phone | b ottle | F PS |
| --- | --- | --- | --- | --- | --- | --- |
| R FAS-SSD ( default box ) | 0.875 _ | 0.972 | 0.8 31 | 0.8 7 | 0.8 25 | 35 |
| R FAS-SSD ( Clustering Optimization ) | 0.882 _ | 0.974 | 0.84 _ | 0.876 _ | 0.837 _ | 3 1 |

It can be seen from the data in the table that the default frame optimized by the k -means clustering algorithm can improve the detection effect of R FAS-SSD on mobile phones and water cups, and its mAP increases by 0.7 percentage points compared with the original default frame. , which indicates that for different object detection tasks, selecting a default box that matches the task can improve the accuracy of object detection. In this paper, since the number of optimized default boxes is increased a little more than the original default box, which will have a certain impact on the detection speed of the model, so the FPS of R FAS-SSD after the default box optimization is reduced by 4FPS.

SSD, FSSD, M 2Det and the R FAS-SSD model optimized by the default frame ratio of this paper are comprehensively compared to verify the improvement of the detection performance of the proposed algorithm for small objects in driver images. The comparison of the test results of the above four models is shown in Figure 5-12.

Figure 5-12 Comprehensive comparison of target detection results

In Figure 5-12, the algorithm in this paper has a certain improvement in the detection of targets such as the driver's hand, mobile phone and water cup, among which the mobile phone has the most obvious improvement. The detection of the driver's head is not much different from other algorithms, and both have good detection results.

Based on the four algorithms of SSD, FSSD, M 2Det and R FAS-SSD ( default frame optimization ) , and according to the method of discriminating distracted driving behaviour in Section 5.1, a classification experiment was performed on 800 sample data. Table 5-7 lists the number of misrecognitions and misrecognition rates of categories.

Table 5-7 Distracted driving behaviour classification experiment comparison

| algorithm | Call up | send Message | drink | normal driving | total number of misidentifications | false recognition rate |
| --- | --- | --- | --- | --- | --- | --- |
| SSD | 19 | 15 | twenty two | 0 | 56 | 7.00% |
| F SSD | 14 | 7 | 17 | 0 | 3 8 | 4.75% |
| M 2Det | 10 | 2 | 13 | 0 | 25 | 3.13% |
| R FAS-SSD | 8 | 4 | 9 | 0 | twenty one | 2.63% |

It can be seen from the data in the table that the original SSD has the highest false recognition rate, the R FAS-SSD in this paper has the lowest false recognition rate, and the recognition results of M 2Det are not much different from those of the model in this paper. The reasons for the misidentification of R FAS-SSD in this paper are analyzed, as shown in Figure 5-13. Figures (a), (b), and (c) are three misidentified samples.

( a )                   (b)                      (c)

Figure 5-13 misidentified sample

The driving behaviour categories in (a), (b), and (c) above are calling, texting, and drinking, respectively. In Figures (a) and (b), the driver's mobile phone is completely blocked by the driver's head and hands, respectively, resulting in no mobile phone detected during target detection. In Figure (c), the driver's water cup is transparent. It was also not detected under the light conditions in . The missed detection of key targets led to the misidentification of the three driving behaviours as normal driving in the above figure. Figure 5-14 shows the correct identification samples of three types of driving behaviours randomly selected.

( a )                   (b)                     (c)

Figure 5-14 Identify the sample correctly

According to 5.1 Distracted Driving Behaviour Discrimination Method, in Figure 5-14(a), and , its distracted driving behaviour classification result is making a phone call; in Figure 5-14(b), and , its distracted driving behaviour classification The result is to send a message; in Figure 5-14(c), , its distracted driving behaviour classification results drink a drink.

6. Summary and Outlook

Driver distraction and fatigue driving behaviour monitoring and early warning is of great significance to reduce the probability of road traffic accidents and reduce driver safety hazards. However, the current distraction and fatigue driving monitoring methods, including driving behaviour monitoring based on driving data, driving behaviour based on physiological signals, and driving behaviour monitoring based on image processing, all have shortcomings in practical applications. Therefore, starting from the characteristics of driver's distracted and dangerous driving behaviour, based on the driver's video data in real scenes, combined with the target detection and behaviour recognition method based on deep learning, this paper studies a driver classification method based on an improved SSD network model. Cardiac driving behaviour monitoring method.

The main research work of this paper is as follows:

The target detection algorithm is studied, and a distracted driving behaviour detection algorithm based on improved SSD is proposed.

The characteristics of the driver's distracted driving behaviour are analyzed, compared with the current classic target detection algorithms, and the SSD algorithm is used as the basic algorithm for distracted driving behaviour detection. Aiming at the problem of missed detection caused by the serious occlusion of small objects such as mobile phones and cups when the driver makes a phone call or drinks a drink in the distracted driving behaviour detection task, a receptive field module is added after the SSD network models conv4_3 and conv7 to optimize The low-level feature map feature extraction capability of the SSD model improves the detection effect of the model on small targets. Aiming at the problem that the SSD model cannot make full use of the semantic features of the high-level feature maps and the detailed features of the underlying feature maps, a feature pyramid structure is introduced into the SSD model, and an adaptive spatial feature fusion method is used to obtain feature fusion weights through back-propagation learning. Optimal fusion of feature maps from different layers . Aiming at the problem that the default box ratio of the original SSD model is not applicable, the k-means clustering algorithm is used to cluster the aspect ratio of the labeled data set, so as to obtain the default box ratio data set that is more suitable for the current task. Finally, through four sets of comparative experiments, the feasibility and effectiveness of the improved SSD algorithm model in the task of distracted driving behaviour detection are verified.

7. Reflection

In this paper, a method for monitoring driver's distracted and fatigue-hazardous driving behaviour based on deep learning target detection and behaviour recognition is proposed. The experimental results show that the method can better identify the driver's dangerous driving behaviour state, and has a certain significance for the monitoring and early warning of the driver's dangerous driving behaviour. Affected by factors such as time and data set, there are also some contents that need to be improved in this paper:

( 1) In the distracted driving behaviour detection task in this paper, the distracted driving behaviour is only determined by the detected key targets and the positions between the key targets. When the key targets in the driving image are completely occluded, the driving behaviour classification results will be generated. The driver's driving posture can be considered as an important factor in the follow-up. In the fatigue driving state recognition task in this paper, the driver's fatigue driving state is judged only by yawning behaviour, which is biased, and the driver's eye state can be considered as an important factor in the follow-up.

(2) This paper only considers three kinds of dangerous driving behaviours , such as calling, texting, and drinking while driving. But the dangerous driving behaviours in the real scene are far more than these, and also include eating, smoking, makeup and other behaviours while driving. In the future, it can be considered to create a distracted driving behaviour monitoring dataset that contains more driving behaviour categories and a larger amount of data, and increases the monitoring and early warning categories of the model.

References

- Statistical Bulletin of the People's Republic of China on National Economic and Social Development in 2019 [OL]. URL: [2020-02-28]. http://www.stats.gov.cn/tjsj/zxfb/202002/t20200228_1728913.html .

- National Bureau of Statistics of the People's Republic of China. China Statistical Yearbook-2015[M]. Beijing: China Statistics Press, 2015: 813-814.

- Ma Yong, Shi Yongquan, Fu Rui, et al. Real vehicle test of the effect of driver distraction time on lane departure [J]. Journal of Jilin University (Engineering Science), 2015, 45(4): 1095-1101.

- Haque M., Washington S. The impact of mobile phone distraction on the braking behaviour of young drivers: A hazard-based duration model[J]. Transportation Research Part C Emerging Technologies, 2015, 50(1):13- 27.

- Alonso F., Esteban C., A. Useche S. A., et al. Smoking while driving: frequency, motives, perceived risk and punishment[J]. World Journal of Preventive Medicine,2017, 5(1): 1-9.

- Ma Yong, Fu Rui, Wang Chang, et al. Analysis of driver's gaze behaviour characteristics during visual distraction [J]. Chinese Journal of Safety Science, 2013, 23(5): 10-14.

- Almahasneh H. S., Kamel N., Malik A. S., et al. EEG based driver cognitive distraction assessment[C]. 2014 5th International Conference on Intelligent and Advanced Systems (ICIAS). Kuala Lumpur: IEEE, 2014: 1-4.

- Nirmali B., Wickramasinghe S., Munasinghe T., et al. Vehicular data acquisition and analytics system for real-time driver behaviour monitoring and anomaly detection[C]// 2017 IEEE International Conference on Industrial and Information Systems (ICIIS ). Peradeniya: IEEE, 2018: 1-6.

- Yao Ying. Research on Distracted Driving Behaviour Discrimination Method Based on Visual Features [D]. Beijing University of Technology, 2017.

- Li Li. Research on driver fatigue and distraction state recognition based on CNNs and LSTM [D]. Hunan University, 2018.

- Zhang Jianming, Zhan Zhicai, Cheng Keyang, et al. Research and Development of Deep Learning [J]. Journal of Jiangsu University (Natural Science Edition), 2015, 36(2): 191-200.

- Sheridan T. B. Big brother as driver: new demands and problems for the man at the wheel[J]. Human Factors, 1970, 12(1): 95-101.

- Michon J. A. Explanatory pitfalls and rule-based driver models[J]. Accident Analysis & Prevention, 1989, 21(4): 341-353.

- Ranney T. A. Models of driving behaviour: a review of their evolution[J]. Accident Analysis & Prevention, 1994, 26(6): 733-750.

- Lee J. D. Preface to the special section on driver distraction[J]. Human Factors, 2004, 46(4): 583-586.

- Kutila M., Jokela M., Markkula G., et al. Driver distraction detection with a camera vision system[C]// 2007 IEEE International Conference on Image Processing(ICIP). San Antonio: IEEE, 2007: 201 -204.

- Miyaji M., Kawanaka H., Oguri K. Driver's cognitive distraction detection using physiological features by the adaboost[C]// 2009 IEEE International Conference on Intelligent Transportation Systems(ITSC). St. Louis: IEEE, 2009: 1 -6.

- Li Nanxiang., Busso C. Predicting perceived visual and cognitive distractions of drivers with multimodal features[J]. IEEE Transactions on Intelligent Transportation Systems, 2015, 16(1): 51-65.

- Dinh T. H., Pham M. T., Phung M. D., et al. Image segmentation based on histogram of depth and an application in driver distraction detection[C]// 2014 International Conference on Control Automation Robotics & Vision(ICARCV). Singapore: IEEE , 2014: 969-974.

- Yu Zhiding, Zhang Cha. Image based static facial expression recognition with multiple deep network learning[C]// 2015 International Conference on Multimodal Interaction (ICMI). New York: ACM, 2015: 435-442.

- Vicente F., Huang Zehua, Xiong Xuehan, et al. Driver gaze tracking and eyes off the road detection system[J]. IEEE Transactions on Intelligent Transportation Systems, 2015, 16(4): 2014-2027.

- Tran D., Do H. M., Sheng Weihua, et al. Real-time detection of distracted driving based on deep learning[J]. IET Intelligent Transport Systems, 2018, 12(10): 1210-1219.

- Leekha M., Goswami M., Shah R. R., et al. Are you paying attention? Detecting distracted driving in real-time[C]// 2019 IEEE International Conference on Multimedia Big Data (BigMM). Singapore: IEEE, 2019: 171-180.

- Jurgen, Goroncy. Driving assistance systems improve vehicle safety performance [J]. Automobile Manufacturing, 2014(9): 70.

- Liao Yuan. Research on driver distraction monitoring based on multi-source information fusion [D]. Tsinghua University, 2015.

- Ma Xin. Research on driving distraction in virtual reality driving simulation environment [D]. North China University of Technology, 2017.

- Wu Hao. Research on driving behaviour recognition method based on deep learning [D]. Southeast University, 2018.

- Chen Jun, Zhang Li, Zhou Bo, et al. Driver distracted driving behaviour detection based on cascaded convolutional neural network [J]. Science Technology and Engineering, 2020, 20(14): 5702-5708.

- Li Ming. Research and implementation of an artificial intelligence algorithm for distracted driving monitoring [D]. University of Electronic Science and Technology of China, 2020.

- Yu Lijiao, Wu Zhenxin, Wang Wenbin, et al. A review of driver fatigue state monitoring system [J]. Automotive Digest, 2019(3): 24-30.

- Zhao Yujuan. Analysis of Distracted Driving Behaviour and Management Countermeasures [N]. People's Public Security Daily Traffic Safety Weekly, 2016-09-27(003).

- Girshick R, Donahue J, Darrell T, et al. Rich feature hierarchies for accurate object detection and semantic segmentation[C]. Proceedings of the IEEE conference on computer vision and pattern recognition. 2014: 580-587.

- Redmon J, Divvala S, Girshick R, et al. You only look once: Unified, real-time object detection[C]. Proceedings of the IEEE conference on computer vision and pattern recognition. 2016: 779-788.

- Liu W, Anguelov D, Erhan D, et al. Ssd: Single shot multibox detector[C]. European conference on computer vision. Springer, Cham, 2016: 21-37.

- Wandell B A, Winawer J. Computational neuroimaging and population receptive fields[J]. Trends in cognitive sciences, 2015, 19(6): 349-357.

- Yu F, Koltun V. Multi-scale context aggregation by dilated convolutions[J]. arXiv preprint arXiv:1511.07122, 2015.

- Szegedy C, Ioffe S, Vanhoucke V, et al. Inception-v4, inception-resnet and the impact of residual connections on learning[C]. Thirty-first AAAI conference on artificial intelligence. 2017.

- Liu S, Huang D, Wang Y. Learning Spatial Fusion for Single-Shot Object Detection[J]. arXiv preprint arXiv:1911.09516, 2019.

- Competition K. State Farm Distracted Driver Detection[J]. 2017.

- Cao G, Xie X, Yang W, et al. Feature-fused SSD: Fast detection for small objects[C]. Ninth International Conference on Graphic and Image Processing (ICGIP 2017). International Society for Optics and Photonics, 2018, 10615: 106151E.

- Zhao Q, Sheng T, Wang Y, et al. M2det: A single-shot object detector based on multi-level feature pyramid network[C]. Proceedings of the AAAI Conference on Artificial Intelligence. 2019, 33: 9259-9266.