UNSTEADY FLOW OPEN SYSTEMS

EXTRA EXERCISES

Exercise 1:

A 0.12 m3 rigid tank initially contains refrigerant-134a at 1 MPa and 100 percent quality. The tank is connected by a valve to a supply line that carries saturated liquid refrigerant-134a at 36C. Now the valve is opened, and the refrigerant is allowed to enter the tank. The valve is closed when it is observed that the tank contains saturated liquid at 1.2 MPa. Determine:

- The mass of the refrigerant that has entered the tank.

- The amount of heat transfer.

Sat. liquid

36C

Assumptions:

- Kinetic and potential energy changes are negligible.

- There are no work interactions involved.

- Table A-11 and A-12 will be required.

Solution:

- According to first law of thermodynamics for steady-flow open systems,

- Mass Balance:

mi-me=m2-m1CV         where me=0

∴ mi=m2-m1

P1=1MPaV1=0.12 m3sat. vap.∴v1=0.020313m3kg    (Table A-12)

Therefore,

m1=V1v1=0.120.020313=5.91 kg

P2=1.2MPaV2=0.12 m3sat. liq.∴v2=0.0008934m3kg    (Table A-12)

Therefore,

m2=V2v2=0.120.0008934=134.32 kg

Thus,

mi=134.32-5.91=128.41 kg

- According to first law of thermodynamics for steady-flow open systems,

- Energy Balance:

Q-W=outmh-inmh+m2u2-m1u1CV

∴   Q=mihi+m2u2-m1u1

T1=36℃sat. liq.∴hi=102.33 kJkg   (Table A-11)

P1=1MPaV1=0.12 m3sat. vap.∴u1=250.68kJkg    (Table A-12)

P2=1.2MPaV2=0.12 m3sat. liq.∴u2=116.70kJkg    (Table A-12)

Thus,

Q=-128.41×102.33+134.32×116.70-5.91×250.68

∴   Q=1053.77 kJ

Exercise 2:

An insulated piston-cylinder device initially contains 0.2 m3 of air at 200 kPa and 22C and internal energy 211.81 kJ/kg. At this state, a linear spring touches the piston but exerts no force on it. The cylinder is connected by a valve to a line that supplies air at 800 kPa and 22C. The valve is opened, and air from the high-pressure line is allowed to enter the cylinder. The valve is turned off when the pressure inside the cylinder reaches 600 kPa. If the enclosed volume inside the cylinder doubles during this process, determine,

- The mass of air that entered the cylinder.

- The final temperature of the air inside the cylinder.

INLET

Pi = 800 kPa

Ti = 22C

AIR

V1 = 0.2 m3

P1 = 200 kPa

T1 = 22C

u1=211.81 kJ/kg

Assumptions:

- Air is considered as an ideal gas.

- The gas constant of air and the properties of air can be found in Table A-2.

- Kinetic and potential energy changes are negligible.

- There is no heat transfer.

Wb=P1+P22V2-V1

The spring is a linear spring, therefore boundary work for this process can be determined from,

Solution:

According to first law of thermodynamics for steady-flow open systems,

- Mass Balance:

mi-me=m2-m1CV         where me=0

∴ mi=m2-m1

- Energy Balance:

Q-Wb=outmh-inmh+m2u2-m1u1CV

∴  -Wb=-mihi+m2u2-m1u1

T1=22℃=295KP1=200 kPaV1=0.2 m3∴P1V1m1=RT1 ∴  m1=P1V1RT1=200×0.20.287×295=0.472 kg

T2P2=600 kPaV2=0.4 m3∴m2=P2V2RT2=600×0.40.287×T2

Thus,

Wb=mihi-m2u2+m1u1

∴   Wb=m2-m1×cpTi-m2×cvT2-T1+u1+m1u1

∴   P1+P22V2-V1=m2-m1×cpTi-m2×cvT2-cvT1+u1+m1u1

∴   200+60020.4-0.2=m2-m1×cpTi-m2×cvT2-cvT1+u1+m1u1

∴   80=m2-0.472×1.005×295-m2×0.718T2-0.718×295+211.81+0.472×211.81

∴   80=600×0.40.287×T2-0.472×296.475-600×0.40.287×T2×0.718×T2+0.472×211.81

∴   80+0.472×296.475-0.472×211.81=600×0.40.287×T2×296.475-600×0.40.287×0.718

∴   119.96188+600.4181185=711540.287×T2

∴   0.287×T2=71154119.96188+600.4181185=98.773

∴   T2=99.7730.287=344.16 K

Thus,

m2=600×0.40.287×T2=2.43 kg

And,

mi=m2-m1=1.96 kg