#include <main.h> // Include main header file for the microcontroller

#define LCD_ENABLE_PIN PIN_D2 // Define pin for LCD enable

#define LCD_RS_PIN PIN_D0 // Define pin for LCD register select

#define LCD_RW_PIN PIN_D1 // Define pin for LCD read/write

#define LCD_DATA4 PIN_D4 // Define pin for LCD data line 4

#define LCD_DATA5 PIN_D5 // Define pin for LCD data line 5

#define LCD_DATA6 PIN_D6 // Define pin for LCD data line 6

#define LCD_DATA7 PIN_D7 // Define pin for LCD data line 7

#include <lcd.c> // Include LCD control library

// Function prototypes

void analogs(void);

void printanalogs(void);

void motor1(void);

void printmotor1(void);

void digLED(void);

void clearall(void);

void terminal(void);

void button_on(void);

void button_off(void);

void knight_rider(void);

void kitt_mode(void);

// Global variables

unsigned int pot; // Variable to store potentiometer value

int last_button_state = 0; // Variable to track the last button state

int kitt_mode_displayed = 0; // Flag to indicate if KITT mode is displayed

unsigned int sequence[] = {0x01, 0x02, 0x04, 0x08, 0x10, 0x20, 0x40, 0x80, 0x40, 0x20, 0x10, 0x08, 0x04, 0x02, 0x01}; // Sequence for Knight Rider effect

// Function to set the state of the Knight Rider LEDs

void knightrider_state(unsigned int state);

void main() {

// Set up ADC for potentiometer input

setup_adc_ports(sAN0);

setup_adc(ADC_CLOCK_INTERNAL | ADC_TAD_MUL_0);

lcd_init(); // Initialize the LCD

while(TRUE) { // Infinite loop

// Check if button connected to PIN_A2 is pressed

if(input(PIN_A2)) {

motor1(); // Control motor based on potentiometer

analogs(); // Read potentiometer value

printanalogs(); // Display potentiometer value on LCD

printmotor1(); // Display motor state on LCD

output_b(0); // Clear output B

}

// Check if button connected to PIN_A3 is pressed

else if(input(PIN_A3)) {

analogs(); // Read potentiometer value

printanalogs(); // Display potentiometer value on LCD

digLED(); // Control LEDs based on potentiometer digital output

terminal(); // Print potentiometer value to terminal

}

// Check if button connected to PIN_A4 is pressed

else if(input(PIN_A4)) {

output_b(0); // Clear output B

int current_button_state = input(PIN_A1); // Read current button state

// Check for button state change

if (current_button_state != last_button_state) {

if (current_button_state) {

button_on(); // Call function to handle button ON state

}

else {

button_off(); // Call function to handle button OFF state

}

last_button_state = current_button_state; // Update last button state

}

}

// Check if button connected to PIN_A5 is pressed

else if(input(PIN_A5)) {

knight_rider(); // Activate Knight Rider LED effect

kitt_mode(); // Activate KITT mode

}

}

}

// Function to read the potentiometer value

void analogs(void) {

set_adc_channel(0); // Set ADC channel to 0

delay_us(30); // Delay for ADC stabilization

pot = read_adc(); // Read ADC value and store in pot

}

// Function to print the potentiometer value on the LCD

void printanalogs(void) {

lcd_gotoxy(2, 1); // Set cursor position on LCD

printf(lcd_putc, "Potentiometer = %3u", pot); // Print potentiometer value

}

// Function to control motor based on potentiometer value

void motor1(void) {

if (pot < 75) { // If potentiometer value is low

output_high(PIN_C1); // Set motor to rotate anticlockwise

output_low(PIN_C0);

} else if (pot > 175) { // If potentiometer value is high

output_high(PIN_C0); // Set motor to rotate clockwise

output_low(PIN_C1);

} else {

output_low(PIN_C0); // Stop motor

output_low(PIN_C1);

}

}

// Function to print the motor state on the LCD

void printmotor1(void) {

if (pot < 75) { // If potentiometer value is low

lcd_gotoxy(5, 2); // Set cursor position on LCD

printf(lcd_putc, "Anticlockwise"); // Display motor direction

} else if (pot > 175) { // If potentiometer value is high

lcd_gotoxy(5, 2); // Set cursor position on LCD

printf(lcd_putc, "  Clockwise  "); // Display motor direction

} else {

lcd_gotoxy(5, 2); // Set cursor position on LCD

printf(lcd_putc, "   Stopped   "); // Display motor stopped state

}

}

// Function to control digital LEDs based on potentiometer value

void digLED(void) {

output_b(pot & 0xFF); // Output the lower 8 bits of the potentiometer value to PORTB

}

// Function to clear all outputs and the LCD display

void clearall(void) {

output_low(PIN_C0); // Turn off motor control pin C0

output_low(PIN_C1); // Turn off motor control pin C1

printf(lcd_putc, "\f"); // Clear the LCD display

output_b(0); // Clear PORTB outputs

}

// Function to print the potentiometer value to the terminal

void terminal(void) {

printf("\f"); // Clear terminal display

printf("Potentiometer = %u \r\n", pot); // Print potentiometer value

delay_ms(100); // Delay for readability

}

// Function to handle button ON state

void button_on(void) {

output_high(PIN_C0); // Activate output C0

output_low(PIN_C1); // Deactivate output C1

lcd_gotoxy(26, 1); // Set cursor position on LCD

printf(lcd_putc,"Button On "); // Display button ON state

printf("\f"); // Clear terminal display

printf("Button = On \r\n"); // Print button state to terminal

}

// Function to handle button OFF state

void button_off(void) {

output_low(PIN_C0); // Deactivate output C0x

output_low(PIN_C1); // Deactivate output C1

lcd_gotoxy(26, 1); // Set cursor position on LCD

printf(lcd_putc,"Button Off"); // Display button OFF state

printf("\f"); // Clear terminal display

printf("Button = Off \r\n"); // Print button state to terminal

}

// Function to create the Knight Rider LED effect

void knight_rider(void) {

for (int q = 0; q < sizeof(sequence) / sizeof(sequence[0]); q++) {

knightrider_state(sequence[q]); // Set LED state based on sequence

delay_ms(200); // Delay

}

}

// Function to set the state of the Knight Rider LEDs

void knightrider_state(unsigned int state) {

// Turn off all LEDs initially

output_low(PIN_B0);

output_low(PIN_B1);

output_low(PIN_B2);

output_low(PIN_B3);

output_low(PIN_B4);

output_low(PIN_B5);

output_low(PIN_B6);

output_low(PIN_B7);

// Set the appropriate LED based on the current state

if (state & 0x01) output_high(PIN_B0);

if (state & 0x02) output_high(PIN_B1);

if (state & 0x04) output_high(PIN_B2);

if (state & 0x08) output_high(PIN_B3);

if (state & 0x10) output_high(PIN_B4);

if (state & 0x20) output_high(PIN_B5);

if (state & 0x40) output_high(PIN_B6);

if (state & 0x80) output_high(PIN_B7);

}

// Function to activate KITT mode and display a message

void kitt_mode(void) {

if (!kitt_mode_displayed) { // Check if KITT mode has not been displayed yet

lcd_gotoxy(26, 3); // Set cursor position on LCD

printf(lcd_putc, "KITT Mode On"); // Display KITT mode message

printf("KITT Mode On \r\n"); // Print KITT mode activation message to terminal

kitt_mode_displayed = 1; // Set flag to indicate KITT mode has been displayed

}

}

Comments

2.. 20   3.. 30    4.. 40   5.. 10

***************************************

2.. Your code has a number of your functions and all work except you don't put the value of RV1  on the LEDS or on the virtual terminal in section 1. Your statements indicate what most lines do. 65%

3.. Your program flows nicely 75%

4.. Your program achieves the desired results. 75%

5.. Your program is nicely laid out and well indented but could have been remarked better. 70%