University of Wales Trinity Saint David

Wales Institute of Science & Art

Motorcycle Geometry Analysis and Dynamic Handling Investigation

BEng Motorcycle Engineering|ACHA4003

John Firth

2004927

December/2020

# Contents Page

1	Contents Page	1

2	Introduction	2

3	Methodology	3

4	Separate Results Section	4

5	Results & Analysis / Discussion Conclusions	6

6	References	8

# Introduction

The purpose of this report is to compare the power of two high performance modern motorcycles with two older motorcycles. The two high end motorcycles used in this report are the Honda CRF450R and the Ducati 1199 Panigale and they are being compared against the KTM 450 Rally Replica, and the Suzuki GSX r1000. The objective is to suggest ways in which the two older motorcycles could be altered to improve their handling characteristics and performance.

The first objective is to acquire accurately measured geometry data and centre of gravities from the Honda CRF450R and the Ducati 1199 Panigale. This data is then presented on schematic diagrams. The second objective is to calculate the maximum possible speeds of the bikes and, considering forces, determine if the motorcycles have enough power to reach these top speeds. The third objective is to use the data to explain, in detail, the known handling characteristics of the Honda CRF450R and the Ducati 1199 and differences in their attitudes under dynamic situations. The fourth objective is to then acquire geometry data for the KTM 450 Rally Replica, and the Suzuki GSX r1000, use this geometry information to explain how the design of the motorcycle has been tailored to its application, and determine how the geometry of each motorcycle relate to its handling characteristics and performance. The fifth objective is to use all the information gathered in the report to recommend any alterations, which could be made to improve the handling characteristics of any of the bikes that have been physically measured.

# Methodology

To acquire accurately measured geometry data and centres of gravities from the Honda CRF450R and the Ducati 1199 Panigale, the motorcycles were weighed by balancing them upon scales underneath their wheels, being careful not to corrupt the data with any false readings from balancing the bikes. The scale underneath the rear wheels of the bikes were then placed upon a stool and the bikes were re-balanced upon the stools. Once accurate measurements were taken of the motorcycles weight distribution, and the stool height was measured, the radius of the wheels and the wheelbase were measured. Using the data acquired, the motorcycles centre of gravities could be derived, and the data was presented on a schematic diagram. Then from the motorcycles workshop manuals the gear ratios for each motorcycle is determined, the maximum possible speed of the bikes can then be derived. Using data acquired throughout the report it is then possible to explain the known handing characteristics of the two motorcycles and their different attitudes under dynamic situations. Geometry data was then acquired for the KTM 450 Rally Replica, and the Suzuki GSX r1000. The geometry of each motorcycle was then analysed and related to its handling characteristics and performance.

# Separate Results Section

Motorcycle Specifications:

| Ducati 1199 Panigale |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Level | Level R | Lifted | Lifted R |
| Front Wheel | 86.Kg | 143.5Kg | 80.Kg | 124.5Kg |
| Rear Wheel | 97.5Kg | 137.Kg | 104.5Kg | 136.5Kg |
| Wheelbase | 1.44m |  | Weight of Bike | 183.5Kg |
| Fr Rad | .318m |  | Rider Weight | 97.Kg |
| Rw Rad | .295m |  | Total Weight | 280.5Kg |
| Rav | .307m |  |  |  |
| Stool Height | .344m |  | Stool Weight | N/A |
| Lifted Angle | 13.821° |  |  |  |
| Xcog | .765m |  | Xcog R | .703m |
| Ycog | .53m |  | Ycog R | .317m |
| Honda CRF 450R |  |  |  |  |
|  | Level | Level R | Lifted | Lifted R |
| Front Wheel | 47.1Kg | 79.Kg | 50.3Kg | 92.5Kg |
| Rear Wheel | 48.9Kg | 114.Kg | 45.35Kg | 97.Kg |
| Wheelbase | 1.49m |  | Weight of Bike | 96.Kg |
| Fr Rad | .318m |  | Rider Weight | 97.Kg |
| Rw Rad | .334m |  | Total Weight | 193.Kg |
| Rav | .326m |  |  |  |
| Stool Height | .344m |  | Stool Weight | N/A |
| Lifted Angle | 13.348° |  |  |  |
| Xcog | .759m |  | Xcog R | .88m |
| Ycog | .558m |  | Ycog R | .879m |

Motorcycle Gear Ratios:

Ducati 1199 Panigale

| Primary | 30 | 53 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Final | 15 | 39 |  |  |  |  |  |
| Diam | Wheel 1 |  |  | 0.295 | m |  |  |
| RPM | 10000 | rev/min |  |  |  |  |  |
| Ratios |  |  |  |  | Speeds | Revs | Falls |
| 1st |  | 15 | 37 | 2.466667 | 49.07776 | 10000 |  |
|  |  |  |  |  |  | 7601.351 | 2398.649 |
| 2nd |  | 16 | 30 | 1.875 | 64.56452 | 10000 |  |
|  |  |  |  |  |  | 8000 | 2000 |
| 3rd |  | 18 | 27 | 1.5 | 80.70565 | 10000 |  |
|  |  |  |  |  |  | 8333.333 | 1666.667 |
| 4th |  | 20 | 25 | 1.25 | 96.84677 | 10000 |  |
|  |  |  |  |  |  | 8727.273 | 1272.727 |
| 5th |  | 22 | 24 | 1.090909 | 110.9703 | 10000 |  |

Honda CRF450R

| Primary | 23 | 63 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Final | 13 | 48 |  |  |  |  |  |
| Diam | Wheel 1 |  |  | 0.334 | m |  |  |
| RPM | 10000 | rev/min |  |  |  |  |  |
| Ratios |  |  |  |  | Speeds | Revs | Falls |
| 1st |  | 15 | 27 | 1.8 | 34.58315 | 10000 |  |
|  |  |  |  |  |  | 8169.935 | 1830.065 |
| 2nd |  | 17 | 25 | 1.470588 | 42.32977 | 10000 |  |
|  |  |  |  |  |  | 8400 | 1600 |
| 3rd |  | 17 | 21 | 1.235294 | 50.39258 | 10000 |  |
|  |  |  |  |  |  | 8500 | 1500 |
| 4th |  | 20 | 21 | 1.05 | 59.28539 | 10000 |  |
|  |  |  |  |  |  | 8658.009 | 1341.991 |
| 5th |  | 22 | 20 | 0.909091 | 68.47463 | 10000 |  |

# Results & Analysis / Discussion Conclusions

Honda CRF450R schematic diagram:

The Honda CFR450R is a lightweight off-road bike developed for competition racing, it has signature off road motorcycle attributes such as a high centre of gravity, low trail (109.4mm) and rake angle, and a large rear wheel. The higher centre of gravity allows for better handling at lower speeds, it also allows for larger rear suspension. The low rake and trail angle allow for quick steering, however at the cost of stability. The large rear wheel allows for the motorcycle to clear larger obstacles.

Ducati 1199 Panigale schematic diagram:

The Ducati 1199 Panigale is a sport motorcycle, it has a high rake angle and trail (127mm), low centre of gravity, a small rear wheel, and a small wheelbase. The high rake and trail angle increase the motorcycles stability, especially on straights, however it makes the steering heavier forcing the rider to lean deeper into corners. The low centre of gravity allows for the motorcycle to corner at greater speeds, at the cost of stability at low speeds. The motorcycle has a small wheelbase to allow for a lighter, more compact build, it also allows for faster cornering.

The KTM 450 Rally Replica has a high rake angle and a trail of approximately 139mm, a tall seat hight of 960mm, and a wheelbase of 1482mm. The motorcycle was designed as a replica based on the KTMs Dakar Rally race teams rally bike, its tall seat hight and high centre of gravity allow for nimble handling at low speeds. The large trail provides straight-line stability tailored for rally racing.

The Suzuki GSX R1000 has a wheelbase of 1420mm, a trail of approximately 135mm and a low centre of gravity. The bike is based on the Suzuki Ecstar MotoGP bike the Suzuki GSX-RR, developed for Grand Prix racing. The motorcycle has a small wheelbase to allow for faster corners however its large trail makes it difficult to change direction at higher speeds.

The Honda CRF450R has a lower trail than the KTM 450 Rally Replica which is better for short dirt track races however long-distance rallying would most likely benefit a larger trail and rake angle. The Honda has incredibly low gear ratios, allowing for extremely quick acceleration but lower top speeds, it will most likely get closer to those speeds than the KTM however, as the Honda is light, slim and the KTMs gear ratios are extremely high, especially towards the higher gears. Both have a high seat and ground clearance which is ideal for off-road motorcycles.

The Ducati 1199 Panigale has a larger wheelbase than the Suzuki GSX R1000, which will benefit its stability however it will not be as nimble around corners. The Ducati has a shorter trail and rake angle which allows for sharper turning circles, however its handling will be significantly worse, this might be worth the trade-off for the road however not for the track.

# References

En.wikipedia.org “Bicycle and motorcycle geometry” https://en.wikipedia.org/wiki/Bicycle_and_motorcycle_geometry

En.wikipedia.org “Honda CRF450R” https://en.wikipedia.org/wiki/Honda_CRF450R

En.wikipedia.org “Ducati 1199” https://en.wikipedia.org/wiki/Ducati_1199

1199-Workshop-Manual “2013 Ducati Superbike Panigale S Tricolore 1199 Workshop Manual” http://www.ducati-manuals.com no. 140312

Honda Motor Co., Ltd. “2009-2010 HONDA SERVICE MANUAL CRF450R”

2020 KTM Sportmotorcycle GmbH “KTM 450 RALLY REPLICA” https://www.ktm.com/en-gb/models/travel/ktm-450-rally-replica2021.html

En.wikipedia.org “Suzuki GSX-R1000” https://en.wikipedia.org/wiki/Suzuki_GSX-R1000

Suzuki “GSX-R1000” https://bikes.suzuki.co.uk/bikes/sport/gsx-r1000/