# Lab - Calculate IPv4 Subnets (Answers)

| Given: |  |
| --- | --- |
| Host IP Address: | 192.168.200.139 |
| Original Subnet Mask | 255.255.255.0 |
| New Subnet Mask: | 255.255.255.224 |

| Find: |  |
| --- | --- |
| Number of Subnet Bits | 3 |
| Number of Subnets Created | 8 |
| Number of Host Bits per Subnet | 5 |
| Number of Hosts per Subnet | 30 |
| Network Address of this Subnet | 192.168.200.128 |
| IPv4 Address of First Host on this Subnet | 192.168.200.129 |
| IPv4 Address of Last Host on this Subnet | 192.168.200.158 |
| IPv4 Broadcast Address on this Subnet | 192.168.200.159 |

| Given: |  |
| --- | --- |
| Host IP Address: | 10.101.99.228 |
| Original Subnet Mask | 255.0.0.0 |
| New Subnet Mask: | 255.255.128.0 |

| Find: |  |
| --- | --- |
| Number of Subnet Bits | 9 |
| Number of Subnets Created | 512 |
| Number of Host Bits per Subnet | 15 |
| Number of Hosts per Subnet | 32,766 |
| Network Address of this Subnet | 10.101.0.0 |
| IPv4 Address of First Host on this Subnet | 10.101.0.1 |
| IPv4 Address of Last Host on this Subnet | 10.101.127.254 |
| IPv4 Broadcast Address on this Subnet | 10.101.127.255 |

| Given: |  |
| --- | --- |
| Host IP Address: | 172.22.32.12 |
| Original Subnet Mask | 255.255.0.0 |
| New Subnet Mask: | 255.255.224.0 |

| Find: |  |
| --- | --- |
| Number of Subnet Bits | 3 |
| Number of Subnets Created | 8 |
| Number of Host Bits per Subnet | 13 |
| Number of Hosts per Subnet | 8,190 |
| Network Address of this Subnet | 172.22.32.0 |
| IPv4 Address of First Host on this Subnet | 172.22.32.1 |
| IPv4 Address of Last Host on this Subnet | 172.22.63.254 |
| IPv4 Broadcast Address on this Subnet | 172.22.63.255 |

| Given: |  |
| --- | --- |
| Host IP Address: | 192.168.1.245 |
| Original Subnet Mask | 255.255.255.0 |
| New Subnet Mask: | 255.255.255.252 |

| Find: |  |
| --- | --- |
| Number of Subnet Bits | 6 |
| Number of Subnets Created | 64 |
| Number of Host Bits per Subnet | 2 |
| Number of Hosts per Subnet | 2 |
| Network Address of this Subnet | 192.168.1.244 |
| IPv4 Address of First Host on this Subnet | 192.168.1.245 |
| IPv4 Address of Last Host on this Subnet | 192.168.1.246 |
| IPv4 Broadcast Address on this Subnet | 192.168.1.247 |

| Given: |  |
| --- | --- |
| Host IP Address: | 128.107.0.55 |
| Original Subnet Mask | 255.255.0.0 |
| New Subnet Mask: | 255.255.255.0 |

| Find: |  |
| --- | --- |
| Number of Subnet Bits | 8 |
| Number of Subnets Created | 256 |
| Number of Host Bits per Subnet | 8 |
| Number of Hosts per Subnet | 254 |
| Network Address of this Subnet | 128.107.0.0 |
| IPv4 Address of First Host on this Subnet | 128.107.0.1 |
| IPv4 Address of Last Host on this Subnet | 128.107.0.254 |
| IPv4 Broadcast Address on this Subnet | 128.107.0.255 |

| Given: |  |
| --- | --- |
| Host IP Address: | 192.135.250.180 |
| Original Subnet Mask | 255.255.255.0 |
| New Subnet Mask: | 255.255.255.248 |

| Find: |  |
| --- | --- |
| Number of Subnet Bits | 5 |
| Number of Subnets Created | 32 |
| Number of Host Bits per Subnet | 3 |
| Number of Hosts per Subnet | 6 |
| Network Address of this Subnet | 192.135.250.176 |
| IPv4 Address of First Host on this Subnet | 192.135.250.177 |
| IPv4 Address of Last Host on this Subnet | 192.135.250.182 |
| IPv4 Broadcast Address on this Subnet | 192.135.250.183 |

# Reflection Question

Why is the subnet mask so important when analyzing an IPv4 address?

Type your answers here.

The subnet mask determines everything about the address: the network, number of host bits, number of hosts and the broadcast address. Merely looking at an IPv4 address tells you nothing. You need the subnet mask to fill in all the important pieces of information.

End of Document

End of document