25th January 2018

An Analysis of the Resonance and Power Factors of a Series RLC Circuit

P151664 Derek Lockhart

M5X00372 Dr Steve Mosey

Electrical Principles Theory

| Contents | Page |
| --- | --- |
| Abstract | 2 |
| Introduction | 2 |
| Method Task 1 Resonance | 2 |
| Method Task 2 Phasor Diagrams | 7 |
| Method Task 3 Max Power Transfer | 8 |
| Conclusion | 10 |
| References | 12 |

1.	Abstract

An analysis of a series circuit containing resistance, inductance and capacitance (RLC) including current, voltage, power, resonance, power factor and power factor corrections.

2.	Introduction

This paper will analyse the performance of a series RLC circuit. It shall identify the current within the circuit and the voltage drops across components. Further it shall analyse the active, reactive and apparent power factor against frequency and discuss the effect on power factor across the frequency range.  The paper will then investigate the effect on component voltages and circuit current through the point of resonance before investigating maximum power transfer.

3. 	Method

A series RLC circuit is one where resistive, inductive and capacitive components are place in series with an AC supply voltage, as defined by Prommee et al [1] the series RLC circuit is a bandpass filter. The circuit to be used is this paper is shown in Figure 1.

Figure 1 Series RLC Circuit

The circuit component values used in this paper were derived from the student’s number (P151664), this gives component values of:	L = 115 mH, C = 16μF, R = 64Ω.

From these figures a resonant frequency of 117.33Hz was calculated using the equation

12πLC

3.1	This Series RLC was subjected to a range of frequencies from 1 to 10,000 Hz, and the resulting circuit current plotted in Figure 2.

Figure 2 Circuit current over frequency range.

3.2	The voltage drops across the circuit components were measured across the frequency range, the results were plotted and are shown in Figures 3, 4 & 5.

Figure 3 Resistor voltage over frequency range.

Figure 4 Inductor voltage over frequency range.

Figure 5 Capacitor voltage over frequency range.

3.3 	Power in reactive circuits differs from dc circuits in that reactance must also be considered, this reactance varies depending on frequency. This gives three power quantities that can be calculated for any given frequency. Tooley [2]explained that True Power (P) is that which dissipated as heat in the resistive components and is measured in Watts (W). Akagi et al [3] explained Reactive Power (Q) is that which is stored in capacitors and inductors at any given time and is measures I Volt-Amperes Reactive (VAR). Hickman [4] further explains that mathematically, Apparent Power (S) is the combination of the real True Power (P) and the imaginary Reactive Power and is measured in Volt-Amperes (VA).

Figure 6 True power over frequency range.

Figure 7 Reactive power over frequency range.

Figure 8 Apparent power over frequency range.

3.4	Horowitz et al [5]explains that Power Factor is a mathematical expression of the quantity of reactive power in a circuit. The power factor tends towards one at the point of resonance in a Series RLC circuit.

Figure 9 Power factor over frequency range.

3.5	Energy storage in a RLC circuit at resonance are stored in either the Capacitor, the Inductor or between them. The point of resonance at the point where stored energy is passed from the Inductor to the Capacitor and vice versa, algebraically the net energy would be zero, therefore at this point the circuit is purely resistive[6].

4.	Phasor diagrams are  a graphical representation of the magnitude and phase of voltages and current within a circuit for a given frequency[7]. In this case we shall look at the phase relationships in the circuit at, below and above resonance.

4.1	Phasor diagrams of RLC circuit at resonance +25Hz

Figure 11 Resultant phasor diagram at resonance +25Hz.

Figure 10 Phasor diagram at resonance +25Hz.

Figure 13 Resultant phasor diagram at resonance.

Figure 12 Phasor diagram at resonance.

4.2	Phasor diagrams of RLC at resonance

Figure 15 Resultant phasor diagram at

resonance -25Hz.

Figure 14 Phasor diagram at resonance -25Hz.

4.3	Phasor diagrams of RLC circuit at resonance -25Hz

5.	Maximum power transfer is achieved when the load impedance matches the source impedance. If however, a circuit is found to be either inductive or capacitive, this will affect the power that is apparent at the supply, correction of this requires the correction of the phase difference induced. This would be done by adding the opposite effect. Mathematically this is done using the complex conjugate of the cause of the initial phase error.

Figure 16 Series RL Circuit

A series RL circuit is shown in Figure 16. For given component values of 10Ω and 100μH the circuit will have a cut off frequency given by the equation

Fco=R2πL

This would give a cut-off frequency of 15915Hz. At this frequency the circuit would have an impedance of 10+j9.99Ω. This gives a circuit impedance of 14.13Ω ∠ 45⁰.

To counteract the phase shift, the complex conjugate of 10+9.99Ω must be applied at the load. Therefore 10-j9.99Ω. At the cut-off frequency of 15915Hz this can be calculated by transposing

XC=12πfC

This gives an approximate capacitor value of 1μF. When placed in series with the load this will correct the phase shift. And ensure maximum power transfer is completed.

Figure 17 RL Circuit Impedance

Figure 17 shows that Impedance of the circuit caused by the circuit inductance. Figure 18 shows the same phasor diagram with the Capacitive correction.

Figure 18 RL Circuit with Rc load

Since the resultant is parallel to the reference it can therefore be seen that the power correction used to correct the phases to max power transfer is the same used to bring a circuit to resonance and consequently bring the power factor to one.

6. 	Conclusions

6.1	It can be seen that the circuit current peaks at the same point of resonance previously calculated. The circuit current peak matches perfectly with the resistor voltage drop, this can also be seen in the Capacitor and Inductor voltages, which show a characteristic steady state voltage equal to the supply voltage on either the capacitor or inductor before peaking at resonance and decaying as the equivalent to the supply voltage drops over the other reactive component.

The Active, Reactive and Apparent power graphs show the same resonance point, the only remarkable point being the inversion of the reactive power moves from the capacitive at the low frequencies to the inductive at the higher frequencies.

The power factor follows a similar trend. The nearest sample to the resonance point was plotted at 100Hz which produced a power factor of 0.92.

The energy stored in the reactive components at resonance transfer between the capacitor and inductor and cancel each other out.

6.2	The phasor diagrams plotted at resonance and 25Hz either side show the reactive components will produce reactive power either above resonance (Inductor) or below resonance (Capacitor) and consequently force a value of apparent power that is greater than true power.

6.3	As previously stated the same component values used for maximum power transfer also bring the circuit to resonance and bring the power factor toward one.

7.0	References

[1]	P. Prommee, P. Thongdit, and K. Angkeaw, “Log-domain high-order low-pass and band-pass filters,” AEU - Int. J. Electron. Commun., vol. 79, pp. 234–242, Sep. 2017.

[2]	M. H. Tooley, Electronic circuits : fundamentals and applications. Elsevier, 2006.

[3]	H. Akagi, E. H. Watanabe, and M. Aredes, “Electric Power Definitions: Background,” in Instantaneous Power Theory and Applications to Power Conditioning, Hoboken, NJ, USA: John Wiley & Sons, Inc., 2017, pp. 17–36.

[4]	I. Hickman, Analog electronics. Newnes, 1999.

[5]	P. Horowitz, The art of electronics. .

[6]	Ohio State Faculty, “R-L-C Circuits and Resonant Circuits,” Ohio State.

[7]	D. I. Crecraft and D. (David) Gorham, Electronics. Open University, 2003.