| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 5 | LEFEL 5 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| January 2023 | IONAWR 2023 |

MODULE CODE: ACDA5006

Transducers and Signal Conditioning

TIME: 2 HOURS

answer all questions

Equipment or materials required – Calculator, Graph paper

Adnoddau neu ddefnyddiau sydd eu hangen – Cyfrifiannell, Papur Graff

ANSWER ALL QUESTIONS

- Define the following terms:

  - Repeatability

[3 Marks]

  - Reproducibility

[3 Marks]

  - Accuracy

[3 Marks]

  - Precision

[3 Marks]

  - Sensitivity

[3 Marks]

- For the circuit shown in Fig Q2,

  - Explain the purpose of the networks R8, R9 & D1 – D4

[3 Marks]

  - Explain the purpose of U1 – U3

[1 Marks]

  - Explain the purpose of R7 and how this is achieved

[4 Marks]

  - State if the filter is a high pass or low pass filter along with its order.

[3 Marks]

  - Calculate the cut off frequency of the filter

[4 Marks]

Fig Q2

- A digital voltmeter has a resistance of 500kΩ and is connected in parallel with the 2MΩ resistor in figure Q3.

  - What would the true voltage across the 1MΩ resistor be if the voltmeter was not connected?

[4 Marks]

  - Assuming the voltmeter is calibrated correctly, what is the reading shown by the voltmeter?

[5 Marks]

  - State one method of making the voltmeter more accurate.

[5 Marks]

2MΩ

Figure Q3

- A load cell is calibrated in an environment at a temperature of 21 °C and has the following deflection / load characteristics:

| Load (kg) | 0 | 25 | 50 | 75 | 100 |
| --- | --- | --- | --- | --- | --- |
| Deflection (mm) | 0.0 | 1.0 | 2.0 | 3.0 | 4.0 |

When used in an environment at 35 °C, its characteristic changes to the following:

| Load (kg) | 0 | 25 | 50 | 75 | 100 |
| --- | --- | --- | --- | --- | --- |
| Deflection (mm) | 0.2 | 1.3 | 2.4 | 3.5 | 4.6 |

- Plot the load cell characteristics for both temperatures

[6 Marks]

- Determine the sensitivity at 21 °C and 35 °C.

[5 Marks]

- Calculate the total zero drift and sensitivity drift at 35 °C in units of (µm/°C) and (µm/kg)/°C

[5 Marks]

- Explain what methods could be used to reduce noise when transmitting transducer signals using wires in a noisy environment.  Give an example circuit and explain how it works.

[10 Marks]

- Explain the need for and the process of calibration.

[10 Marks]

- Compare and contrast the requirements and specifications for a temperature sensor for the following applications:

  - Air temperature measurement for a weather station

  - Coolant temperature of a car engine.

[10 Marks]

Diwedd/End

Student Number ___________________________

Question Number __________________________