| LOCATION: Swansea |  |
| --- | --- |
| LEVEL 4 |  |
| EXAMINATION PERIOD |  |
| January 2019 |  |

MODULE CODE: M4X01163

MATERIALS AND INTRODUCTION TO MANUFACTURING

TIME:  2 HOURS (MOCK EXAM)

answer all questions FROM SECTION A

ANSWER THREE QUESTIONS SECTION B

Equipment or materials required – Calculator

ANSWER ALL QUESTIONS

SECTION A

Multiple choice

1. 	A material will undergo elastic deformation:

A	After the limit of proportionality

B 	Slightly before the ultimate tensile strength

C 	Between yield point 1 and yield point 2

D 	At the breaking point

2.	The young’s modulus can be calculated by finding:

A 	The gradient of a stress – strain graph

B	The area under a stress – strain graph

C	The gradient of a force - extension graph

D	The area under a force - extension graph

3.	The co-ordination number of a cubic structure refers to:

A	The number of atoms within a unit cell

B	The length of a side of a unit cell

C	The number of nearest neighbours of an atom within a crystalline structure

D	The 3D stacking array of a crystal system

4.	A material with a low Young’s modulus is likely to have:

A	Cross-linking type bonds

B	High ductility

C	A high degree of stiffness

D 	A polymer base

5.	Certain types of ceramics are:

A 	Linear molecular structures

B	Cross-linked molecular structures

C	Silicates

D	Repeated units of hydrocarbon molecules

6.	The area under a force-extension graph provides information on:

A	Young’s modules

B	Spring constant for springs in a series arrangement

C	Spring constant for springs in a parallel arrangement

D	Stored elastic energy

7.	A transgraular crack is the propagation of a crack:

A 	Along grain boundaries

B	Through the centre of grains

C	At dislocations in a vertical plane

D	At dislocations in a horizontal plane

SECTION B

1.	Figure 1 shows a tower crane that has two identical steel cables. The length of each steel cable is 55m from the jib to the hook.

Figure 2

Figure 2

45m

45m

Figure 1

Figure 1

- Each cable has a mass of 7.2kg per meter. Calculate the weight of a 55m length of one cable.										[3]

- The cables would break if the crane attempted to lift a load of 2.5×106N or more. Calculate the breaking stress of one cable.						[3]

Cross-sectional area of each cable = 5.8×10-4m2

- When the crane supports a load each cable experiences a stress of 550MPa. Each cable obeys Hooke’s Law. Ignore the weight of the cables.

Young modulus of steel = 2.1×1011Pa

- Calculate the weight of the load.							[3]

- If the unstretched length of each cable is 55m. Calculate the extension of each cable when supporting the load.							[3]

- Calculate the combined stiffness constant, k, for the two cables. 		[3]

- Calculate the total energy stored in both stretched cables.			[3]

- When the load is removed it is assumed that the cable returns to its original length. State what assumptions are made here.						[2]

- The material used in figure 1 has a body centred cubic structure. Given that

- 4R=3a, show that the atomic packing fraction is 0.63.				[5]

2.	a)	A material with an FCC crystal structure, an atomic radius of 0.1473nm, and an atomic weight of 87.43 g/mol. Calculate its theoretical density. 			[5]

b)	Calculate the radius of an atom, given that it has an FCC structure, a density of 13.2gcm-3 and an atomic weight of 170.9g/mol.					[5]

c)	Some hypothetical metal has a simple cubic structure. If its atomic weight is 74.5g/mol and the atomic radius is 0.145nm, calculate its density.			[5]

Diwedd/End