# REDACTED

# Major Project:

Disposal of Legacy Warheads Post-Environmental Testing & Long-Term Storage

Student:

Creighton Rees - 1910564

Lecturer:

Jenni Steen

Course:

Ordinance Munitions and Explosives:                                          ACDA60200: Major Project

Abstract

‘During a site wide inspection at a large munitions factory two warheads that had been in long term deep storage were found in extremely poor condition. The warheads were found inside an un-conditioned storage unit with little information available on their history, date of manufacture, functional design, and energetic materials contained within.

Due to the lack of information and history on the warheads, combined with the fact both are development trial items there is no transport classification in place to allow the shipment of the munitions to an established Explosive Ordinance Disposal (EOD) site. The site where the warheads were found currently has no capability or explosive license in place to allow for the disposal of highly hazardous large munitions

There are various approaches that can be taken to dispose of high hazard munitions, such as full-order detonation, low-order detonation, incineration, and remote breakdown of the munition [1]. The mitigations in place during the disposal activity must be suitable to ensure full containment of a full-order detonation and its associated hazards including, blast, fragmentation, thermal flux and Jet propagation.

A detailed Risk Assessment for the disposal activity has been produced and shown in Annex A1, with the risk scoring matrix shown in Annex A2.

Prior to any mobilisation activities commencing, formal approval of a temporary explosive disposal licence for the Mitigated Disposal Area (MDA) from the Heath & Safety Executive (HSE) was required. In additon to the HSE, the local government and coast guard were informed of the planned disposal date.

The MDA consisted of large shipping containers double stacked as inner walls, around a central 10M X 10M Disposal Area. All outer walls were doubled buttressed with earth mounding up to the mid-point of the upper container. The central disposal area was lined and filled with an infill of multiple 1 tonne sandbags up to 800 tonnes in total.

Both high-order detonations took place as planned with no residual un-exploded material remaining on completion, this was confirmed by EOD personnel after following a 24-hour stand-down period, with re-entry authorised after EOD inspection.

When all scap metal waste was collected, it was placed into an appropriate stillage and transported to the Original Equipment Manufacturers (OEM) facility proving oven to Certified Free From Explosive(CFFE). All sand waste was treated as hazardouse waste and was disposed of through the appropirate disposal routes. The shipping containers were dissassembled and transported from the OEM site by EOD contractor for future use.

For the purpose of this report, precise design details of the warheads, specific locations of the munitions manufacturer and names of relevant stakeholders have been omitted for security reasons. Some theoretical values and locations have been used within this report for illustration purposes, although some information can be made available on request if required.’

Contents

1	Introduction	5

2	Legistlation & Regulation For Ordinance Disposal	6

3	Disposal Munition Design	10

4	Assessment of Risk	16

5	Disposal Process	24

6	Discussion	31

7	Conclusion	32

References	33

Annex A – Risk Assessment	37

Annex A1 –  General Risk Assessment	34

Annex A2 – Risk Scoring Matrix	41

Annex B – Disposal Project Estimated Cost	45

Annex C – Disposal Project Estimated Timeline	46

Table of Tables

Table 1 – Article 1 – CBF Warhead Component Description [10]	11

Table 2 – ExRAB Hazard Properties [13]	12

Table 3 – ExRAB RE Factors [16]	13

Table 4 – Article 2 – SC Warhead Component Description [9]	14

Table 5 – ExHB Hazard Properties [13] [17]	15

Table 6 – ExHB RE Factors [16]	15

Table 7 – Blast Parameters of Article 1 (CBF) Warhead at 50m [21]	18

Table 8 – Blast Parameters of Article 2 (SC) Warhead at 50m [21]	18

Table of Figures

Figure 1 – STANAG 4518 – Demilitarization and Disposal Process [6]	9

Figure 2 – Article 1 – Configured Blast Fragmentation (CBF) Warhead [10]	11

Figure 3 – Article 2 – Shaped Charge (SC) Warhead [9]	14

Figure 4 – MDA – Rectangular Destruction Area with Four Containing Faces & Open Roof [7]	17

Figure 5 – Effects of Blast Pressure on Structures and the Human Body [22]	19

Figure 6 – Site Overview	25

Figure 7 – Overview of Disposal Route to MDA	25

Figure 8 – Overley of Hazard Zones on MDA	26

Figure 9 – Disposal Process Flow	27

Figure 10 – Side View MDA with SC Warhead Present	29

Figure 11 – Plan View of Complete MDA with SC Warhead	30

Figure 12 – Side View of MDA with CBF Warhead	31

Table of Equations

Equation 1 – TNTe by RE Factor & Mass Fraction [14]	13

Equation 2 – TNTe of ExRAB Explosive	13

Equation 3 – TNTe of ExHB Explosive	16

Equation 4 - Hopkinson-Cranze scaling law [19]	17

Equation 5 – Article 1 CBF Warhead Scaled Distance at 50m	18

Equation 6 – Article 2 SC Warhead Scaled Distance at 50m	18

Equation 7 – Idealised Hydrodynamic Penetration [16]	19

Equation 8 – National Defence Research Committee (NDRC) Modified Formula [23]	20

Equation 9 – Gurney Equation Fragment Velocity [24]	20

Equation 10 – CBF Warhead Fragment Velocity	21

Equation 11 – SC Warhead Fragment Velocity	21

Equation 12 – Kinetic Energy [16]	21

Equation 13 – Fragment Mass	22

Equation 14 – Fragment Kinetic Energy	22

Equation 15 – Typical Synthetik© Blast FEM Simulation Output [26]	Error! Bookmark not defined.