Series Hybrid Vehicle CW1

ACEA7007: Hybrid Powertrain and Control

Student: 2109787 (Jaimishkumar V. Navadiya )

Lecturer: Mr. Mark Sandford

Content

- Abstract ………………………………………………………………………….……….3

- Introduction…………………………………………………………………….………….3

- Dual purpose course track…………………...…………………………………….…………4

- Calculations…………………………...………………………….……………………….4

  - Assumptions………………………………………………………………….………...4

    - Gear ratio……………………………………………………………………….4

    - Masses…………………...………………………………………….…………5

      - Test mass……………………………………………………………………5

      - Wheel assembly………………………………………………………………5

      - Reduced mass…………………………………..………………….………...5

    - Frontal area………………………………………..………….………………...6

    - Coefficient if Aerodynamic drag………………………………………….…………6

  - Forces acting on the course…………………………………………………….………….6

    - Aerodynamic resistance ……………………………...……………….…………...7

    - Rolling resistance……………………………………...……………...……...…...7

    - Gradient resistance…………………………………………………….……….....7

    - Inertia resistance………………………………….……...…………………….....8

  - Traction Force………………………………………………….….……...…………… 8

  - Engine Torque…………………………….……...…………………………………… 8

  - Brake Specific Fuel Consumption (BSFC) ………….……………...………………………..9

  - Fuel Consumption……………………………………………..…….…………………..9

- Results…….……………………………………………………….……………………10

  - Dyno chart ……………………….…………………………………………………...10

  - Velocity ……………………...…………………………….………………………...10

  - Acceleration & Deceleration……………………..…….….……...…...…………………11

  - Resistance Force…………….………………………………………….……………...11

  - Net Resistance Force…………………………………………………………………....12

  - Power………………………………….……………...……………………………...12

  - Work done……………….……...............................................................…………….12

  - Fuel Flow Rate……………….….............................................................…………….13

  - Total Fuel Consumption…………...............................................................…………….13

  - Energy Dissipation…….............................................................................…………13

- Results…………………….…………………………………………………………….14

- Conclusion……………………………………………………………………………….15

- References………………………………..…………...…………………………………16

- Appendix……………………………………………...…………………………………17

- Abstract

In this simulated study, the duty cycle of the off-road buggy (renegade dx 10C) over a dual-purpose racecourse is generated to analyze the fuel consumption and vehicle performance. The course is a combination of 3 different surfaces, designed to utilize optimum performance of torque-oriented power trains that consists of a total climb of 20 meters. To analyze the dynamic behavior of the buggy, the document will explore the resistance forces acting on buggy and fuel consumption for one lap.

2. Introduction

The overall purpose of a buggy is to efficiently accelerate in different course irregularities such as bumps, surface conditions, and transfer torque for a hassle-free enduring experience without compromising on the agility of the vehicle. The geometry with extremely low center of gravity helps the vehicle to perform in tricky weather conditions. The primary objective of this test is to obtain the fuel consumed under maximum performance, using appropriate calculations regarding the dynamic processes experienced throughout a lap, to output a simulated lap time for Renegade Dx-10C.Secondly, obtaining maximum fuel consumption for a full tank of gasoline gives the maximum fuel efficiency of the buggy. Mapping the track layout is the first step into creating the model and categories the circuit into a series of straights and corners, with which the dynamic processes can be split into steady state cornering, acceleration, and braking.

3. Dual purpose course track

I consider waterfront visitors car parking as course track. I assume that the layout in this experiment for Renegade dx 10c consisted of four different surfaces of tracks. The whole track is 1971m long.

The course track layout considered by me for experiment is shown in below figure (1).

Figure 1: Course Track Layout

4. Calculations

4.1 Assumption

4.1.1 Gear Ratio

Renegade Dx-10c consists of a CVT transmission assembly where an infinite number of gear ratio possible. But for simulation we assume that convention gear transmission is used in this vehicle which has 3 speed gearboxes. The gear ratio we used in this experiment is shown in the following table 1.

| Gear | Input | Output | Gear Ratio | Overall Ratio | Top Speed(m/s) | Top Speed (km/h) | Top Speed (mph) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1st | 11 | 29 | 2.636363636 | 22.72922078 | 7.602753669 | 27.36991321 | 17.00690379 |
| 2nd | 15 | 29 | 1.933333333 | 16.66809524 | 10.36739137 | 37.32260892 | 23.19123244 |
| 3rd | 21 | 31 | 1.476190476 | 12.72687075 | 13.57793837 | 48.88057813 | 30.37303346 |
| Final Drive | 15 | 51 | 3.4 |  |  |  |  |
| Primary Drive | 28 | 71 | 2.535714286 |  |  |  |  |

Table 1: Gear Ratio

4.1.2 Masses

4.1.2.1 Test Mass

Kerb weight of renegade was mentioned in the specifications data. The buggy was assumed to be tested with a fuel of 10 liters, pouring 1.5 liters of oil and a passenger for prime dynamics at challenging sectors during corporate event. The values of lubricant and fuel in kg are considered by multiplying the available quantity in liters with the density of respective liquid.

| Driver + Passenger Weight (kg) | 144 |
| --- | --- |
| Fuel + Oil Weight (kg) | 16.11 |
| Kerb Weight (kg) | 345 |
| Total Weight (kg) | 521.71 |

Table 2: Test Mass

4.1.2.2 Wheel Assembly

The wheel and tire masses are assumed by converting the relevant tire specifications 24*8*12 to 175/60R12. The more mass an object has the more energy it takes to accelerate. To accelerate a rolling object such as wheel, we must both accelerate its mass plus overcome its rotational inertia.

| Tires Mass (kg) | 10.2 |
| --- | --- |
| Wheels Mass (kg) | 6.4 |
| Total Assembly Mass (kg) | 16.6 |

Table 3: Wheel and Tire Mass

(http://the-welters.com/racing/rotational.html)

4.1.2.3 Reduced Mass

Reduced Mass=Total test mass+4*Wheel InertiaWheel Radius2 +Engine Inertia* Gear RatioWheel Radius2

(https://en.wikipedia.org/wiki/Reduced_mass)                                                                          …………… (1)

Equation 1 depicts that reduced mass is directly proportional to the gear ratio. So the different gear ratio has different reduced mass. A reduced mass with respect to all 3-gear ratio is given in table 4.

| Equivalent Mass in Gear 1 | 575.2066326 |
| --- | --- |
| Equivalent Mass in Gear 2 | 568.9254141 |
| Equivalent Mass in Gear 3 | 565.8780235 |

Table 4: Reduced Mass

4.1.3 Frontal Area

Renegade Dx-10c buggy has drag effects on 2 tires, wide outside the body panel as it is open to longitudinal acceleration. From specifications, the frontal area of the tire can be calculated by considering it as a rectangular area of cross section from front view. Driver and passenger should experience the drag. The frontal area can be calculated as the sum of Frontal area of driver, passenger and panel, tires.

Frontal area = 1.151 m2

4.1.4 Coefficient of Aerodynamic Drag

As per given data, we can measure frontal effective area of vehicle. In this experiment, frontal area is measured as 1.151 m2. The drag coefficient for renegade had to be assumed as 0.7 which is an average of values of drag coefficient of Open top convertible vehicle and flat plate (face on flow). The assumption is made from https://www.engineersedge.com/fluid_flow/aerodynamic_drag_13632.htm .

4.2 Forces acting on the course

There are four resistive forces acting on moving vehicles. There are Aerodynamic force, Rolling force or Friction force, Gradient force, and Inertia force.

Figure 2: Forces acting upon moving vehicle

4.2.1 Aerodynamic Resistive Force (Fa)

Fa= 12 ρAfCd(Vcar±Vwind)2   ………………….. (2)

Fa= Air Resistance (N)​

ρ= Air Density (kg/m3)​= 1.167 kg/m3

Af = Car frontal area (m2)​= 1.151 m2

Cd = Coefficient of Aerodynamic Resistance (drag coefficient)​= 0.7

Vcar = Car velocity (m/s)​

Vwind = Wind velocity (m/s)​

(+) with head wind, the wind velocity opposite the car velocity direction​

(-) with tail wind, the wind velocity in the same direction of car velocity

https://slidetodoc.com/12-tractive-effort-and-tractive-resistance-nazarin-b/

4.2.2 Rolling Resistance (Fr)

Fr = μR W;   ​…………… (3)

Fr = μR  mg​      ……………… (4)

​

Fr= Rolling resistance (N)​

μR = Coefficient of rolling resistance

W= Car weight (N)​

M= Car mass (kg)​

g= acceleration due to gravity = 9.8 m/s2

|  | µR |
| --- | --- |
| Rolled gravel | 0.200 |
| Concrete Asphalt | 0.013 |
| Standing water | 0.155 |
| Field | 0.100 |

​​Table 5: Rolling coefficient

4.2.3 Gradient Resistance (Fg)

Fg = ±W Sinθ      …………… (5)

Fg = Gradient resistance (N)​

W = Car weight (N) = mg (kg m/s2)

θ= Gradient angle < 2.3° for highways​

5.7°: 6.9° for local road​

11.5° max grad​

(+) with car going up to hill (ascending) {Resistance effort}​

(-) with car going down to hill (descending) {Tractive effort}​

| Sinθ1 (slop 1 uphill) | 0.13864 |
| --- | --- |
| Sinθ2 (slop 1 downhill) | 0.20372 |
| Sinθ3 (slop 2 uphill) | 0.11803 |
| Sinθ4 (slop 2 downhill) | 0.16229 |

Table 6: Gradient values

4.2.4 Inertia Resistance (Fi)

IR = ± ma = ± (mcar + meq) a​    ………………. (6)

​

m= Car mass + Equivalent mass of rotating parts (kg)​

a= Acceleration (m/s2)​

From 0 to 100 km/h in 6 seconds a= 4.63 m/s2​

10seconds   a= 1.543 m/s2

(+) with car in acceleration (Tractive resistance)​

(-) with car in deceleration (Tractive effort) ​

​​

4.3 Traction Force

Traction force can be calculated by equation 7.

Traction Force =Traction Coefficient ×  Resistance Forces  ………. (7)

| Traction Coefficient (Asphalt) | 0.85 |
| --- | --- |
| Traction Coefficient (Gravel) | 0.7 |
| Traction Coefficient (Field) | 0.6 |
| Traction Coefficient (Water splash) | 0.3 |

Table 7: Traction coefficients

4.4 Engine Torque

If traction force is known, then we can calculate required torque at wheel by equation 8.

Torque at wheel = Traction force × Rear wheel effective radius                   …....……. (8)

From wheel torque, we can calculate engine torque by equation 9. Gear ratios are calculated in table 1.

Tw= Te×ig×if×η   …...……. (9)

4.5 Brake Specific Fuel Consumption (BSFC)

Table (7) is considered as BSFC data. I use mean method to find BSFC for changing of every 500 rpm.

Table 8: BSFC

4.6 Fuel Consumption

The fuel usage per metre and across the whole lap was calculated using equation (14). From the simulated data buggy consumes around 2.65 litres of fuel in one complete lap, considering performance with torque utilisation in competitive terrains. The fuel usage is strictly dependant on the driver where a professional sportsperson is considered to drive the lap. For calculations density of petrol is taken as 0.755 kg/cm3.

If the specific consumption in g/kW·h is known, we can calculate fuel consumption in liter per hour by equation (14).

Q= BSFC × Pe ρ  …………………………. (7)

Q = maximum theoretical fuel consumption in grams per 1 hour of engine operation at maximum power [in liter/hour],

Pe = Engine power [in hp],

ρ = Fuel density [in kg/m3]

5. Results

Data to analyze the performance of renegade dx-10c in distinct parts of the simulated track conditions are follows.

5.1 Dyno chart

Graph (1) Dyno chart

Power and torque are related to each other in every increment of rpm to that of gear ratios. According to the higher gear ratios used for the simulation of renegade dx-10c the torque is more than power until 6500 rpm where both power and torque curves intersect as obtained in Graph (1).

5.2 Velocity (mph)

Graph (2) Velocity and Gear

Graph (2) demonstrates the velocity pattern followed by vehicle in particular gear selection by buggy. As per graph, it is seen that maximum velocity of vehicle does not exceed more than 30 mph. Gear selection is also shown in this graph which is also satisfied for velocity. Maximum velocity for gear 1 selection does not exceed more than 17 mph. For gear 2, maximum velocity can be achieved is 23mph, while for gear 3 it is 30mph.

5.3 Acceleration (m/s2) & Deceleration (m/s2)

Graph (3) Acceleration & Deceleration

Graph (3) depicts acceleration and deceleration of buggy during course.  It is seen that maximum acceleration is 2.54 m/s2 is due to an initial inertia resistance force.

5.4 Resistance Forces (N)

Graph (4) Resistance Forces

Graph (4) shows all 4 resistive forces throughout travel. Inertia resistance depends on acceleration and deceleration of vehicle, so it changes throughout the course. Where buggy accelerate, force will be positive and resist to accelerate the vehicle. Where the vehicle decelerates, the resistance force will be negative. Rolling resistance only depends upon friction coefficient, and so in distinct terrain has constant rolling resistance force and it is always opposite the motion of vehicle. Gradient resistance force only changes when gradient changes. Gradient resistance will be either positive or negative. When a vehicle is acceding uphill, it resists motion and so it will be positive. While a vehicle is descending downhill, it becomes supportive force and helps to increase motion of a vehicle. It will not resist motion of vehicle so it will be negative. Aero drag force directly proportional to the square of velocity. When velocity increases aero drag will increase. It is only zero when a vehicle is not moving. Here, we do not consider wind velocity in this simulation. That means we only consider vehicle speed.

5.5 Net Resistance Force (N)

Graph (5) Net resistance force

Graph (5) illustrates total resistive forces acting on buggy.

5.6 Power (kW)

Graph (6): Power

Graph (6) shows power in kilowatts that we exert from engine throughout the course. The power is highest while a buggy is moving at maximum velocity and while ascending the uphill. Throughout the course, a vehicle used average 2.328972913 kW/s. During travelling uphill and maximum speed, if any additional power is given to a vehicle, it is beneficial to decrease fuel consumption and engine load.

5.7 Work done

Graph (7): Work done

Vehicles need to work for continuous motion throughout the track. As shown in graph (7), areas under positive curse, it depicts that during this time engine need to produce power or can say that propeller need power to move a vehicle. But areas under the negative curve, we can say that during this time the vehicle does not need to produce power, or we do not need to throttle to the engine. It is time when net resistive forces will be negative. Because work done can be calculated by Force multiply by distance.

5.8 Fuel Consumption (g), Total Fuel Burned (L)

Graph (8) Fuel consumption

Graph (8) gives information about fuel mass rate per second as well as total fuel burn during completion of one lap. Fuel mass flow rate can be calculated by equation 14. Brake Specific Fuel Consumption (BSFC) is taken from table 7.  It is seen that during max speed and uphill, a vehicle burns maximum fuel due to work done by engine. For complete one lap, a vehicle consumed 89.76-gram fuel which equals to approximately 0.12 liters. That means a vehicle can travel 16.42-km per liter. Suppose fuel tank capacity is 29 liters, then a buggy can travel approximately 216 complete laps.

5.9 Energy Dissipation (kWh)

Graph (9): Energy Dissipation

Graph (9) shows total traction energy, drag energy and braking energy during one lap. Total traction energy used by a vehicle is approx. 0.175 kWh while braking energy is 0.039 kWh which is 23% of traction energy.

6. Result

| Lap Distance (km) | 1.97 |
| --- | --- |
| Fuel consumed (l) | 0.119995787 |
| Fuel consumed (g) | 89.7568485 |
| Liters per km | 16.41724308 |
| Maximum speed (mph) | 30 |
| Maximum speed (km/h) | 48.28023997 |
| Average speed (km/h) | 25.62571052 |
| Maximum lap possible with full tank | 216.67 |
| Total traction energy (kWh) | 0.175221095 |
| Total energy consumed by Drag (kWh) | 0.016510405 |
| Total energy consumed by Braking (kWh) | 0.039608906 |
| Braking energy percentage (%) | 23% |
| Time taken for one lap | 4:36 |

Table 9: Lap result

A vehicle takes 4 minutes 36 seconds to complete one lap. Total lap distance is 1.97 kilometers. To complete one lap, a vehicle consumes 0.119 liters of fuel or 89.76 grams. A vehicle can gain a maximum of 30 mile per hour speed. If we consider the fuel tank to be full, then a buggy can travel 216 complete 216 laps.

| Maximum Engine Speed (RPM) | 6500 |
| --- | --- |
| Maximum Engine Torque (Nm) | 22 |
| Maximum Power Develop by Engine (W) | 14967.33333 |
| Maximum Power Developed by Engine (HP) | 20.06344951 |
| Effective Frontal Area (m2) | 1.151 |
| Front Wheel Radius (mm) | 304.8 |
| Rear Wheel Radius (mm) | 254 |
| Front tyre Radius (24*8-12) (m) | 0.307692308 |
| Rear tyre Radius (23*10.5-12) (m) | 0.294871795 |
| Driver + Passenger Weight (kg) | 144 |
| Fuel + Oil Weight (kg) | 16.11 |
| Kerb Weight (kg) | 345 |
| Total Weight (kg) | 521.71 |
| Drag Coefficient | 0.7 |
| Wind Speed (m/s) | 2.23 |
| Air Density (kg/m3) | 1.167 |
| Rolling Coefficient (μR) Asphalt | 0.013 |
| Rolling Coefficient (μR) Gravel | 0.02 |
| Rolling Coefficient (μR) Standing water | 0.155 |
| Rolling Coefficient (μR) Field | 0.1 |
| Traction Coefficient ( Asphalt) | 0.85 |
| Traction Coefficient ( Gravel) | 0.7 |
| Traction Coefficient ( Field) | 0.6 |
| Traction Coefficient ( Water splash) | 0.3 |
| Wheelbase (mm) | 1940 |
| Height of COG from Ground (mm) | 488 |
| Distance of COG from Front Wheel Centre (mm) | 1070 |
| Distance of COG from Rear Wheel Centre (mm) | 870 |
| Angle of Buggy Lifted (Degree) | 18 |
| Angle of Buggy Lifted (Radians) | 0.314159 |
| tan(18°) | 0.32491 |
| Load on Rear Wheel (kg) | 207 |
| Load on Rear Wheel while Lifted (kg) | 193 |
| YCOG | 769.737977 |
| XCOG | 521.696752 |
| Tires Mass (kg) | 10.2 |
| Wheels Mass (kg) | 6.4 |
| Total Assembly Mass (kg) | 16.6 |
| Radius of Gyration Front Wheel (Kfront) | 0.27432 |
| Radius of Gyration Rear Wheel at 6500rpm 1st Gear | 0.2286 |
| Inertia of Front Wheel | 1.249174276 |
| Inertia of Rear Wheel | 0.867482136 |
| Inertia of Engine | 0.17 |
| Equivalent Mass in Gear 1 | 575.2066326 |
| Equivalent Mass in Gear 2 | 568.9254141 |
| Equivalent Mass in Gear 3 | 565.8780235 |

Table 10: Specifications and Data

7. Conclusion

This report has shown the Renegade dx 10c characteristics on a dual-purpose terrain course across a lap. Considering the design of renegade to driver characteristics the test was conducted by a professional sportsperson to obtain maximum fuel consumption for the given engine specifications. Looking for the wide perspective of Renegade users there are long distance tourer and adventure seekers who want to serve the purpose where terrain and weather challenges need to be considered. The test was conducted in dry conditions. The wet conditions would be difficult to obtain the maximum efficiency where the real performance should have been tested to ensure the reliability of Renegade in outdoor conditions.

Rolling resistive and gradient resistive forces are acting on Renegade dx 10c influence on the buggy performance. Introducing adventure tires which can accumulate a vast volume of terrain surface between the grooves can improve the rolling characteristics in different terrains. Due to the absence of front wheel drive, buggy finds it hard to negotiate with the gradient and drive conditions, introducing an electric motor drive for the front wheel drive can improve the efficiency of the renegade on real world terrain conditions.

The quantity of fuel used by the renegade is a considerable amount at maximum performance considering the brake specific fuel consumption generated. The CVT transmission of buggy can save a considerable amount of fuel loss due to gear shifting. Meanwhile introduction of all wheel drive can have supplementary improvement in fuel efficiency.

8. References

- http://the-welters.com/racing/rotational.html &

https://view.officeapps.live.com/op/view.aspx?src=http%3A%2F%2Fthe-                welters.com%2Fracing%2Frotational.xls&wdOrigin=BROWSELINK , March 17, 2002

- Aleksander UBYSZ

http://transportproblems.polsl.pl/pl/archiwum/2010/zeszyt1/2010t5z1_04.pdf (2010)

- https://en.wikipedia.org/wiki/Reduced_mass

- https://www.engineeringtoolbox.com/rolling-friction-resistance-d_1303.html

- https://buildingclub.info/calculator/g-kwh-to-l-h-online-from-gram-kwh-to-liters-per-hour/

- https://slidetodoc.com/12-tractive-effort-and-tractive-resistance-nazarin-b/

- http://www.thecartech.com/subjects/auto_eng/car_performance_formulas.htm

- https://www.engineeringtoolbox.com/car-acceleration-d_1309.html

9. Appendix

Table (1) Gear Ratio

Table (2) Test Mass

Table (3) Wheel and tire mass

Table (4) Reduced Mass

Table (5) Rolling Coefficient

Table (6) Gradient values

Table (7) Traction coefficients

Table (8) BSFC

Table (9) Lap Results

Table (10) Specifications and Data

Graph (1) Dyno chart

Graph (2) Velocity and Gear

Graph (3) Acceleration and Deceleration

Graph (4) Resistive forces

Graph (5) Net resistance force

Graph (6) Power

Graph (7) Work done

Graph (8) Fuel consumption

Graph (9) Energy dissipation

Equation (1) Reduced Mass

Equation (2) Aerodynamic Resistive Force

Equation (3) Rolling Resistance

Equation (4) Rolling Resistance

Equation (5) Gradient resistance

Equation (6) Inertia Resistance

Equation (7) Traction force

Equation (8) Torque at wheel

Equation (9) Engine Torque

Equation (10) Fuel mass flow rate

Figure (1) Course Track Layout

Figure (2) Forces acting upon moving vehicle