Lab - Accessing Network Devices with SSH

Topology

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/1 | 192.168.1.1 | 255.255.255.0 | N/A |
| S1 | VLAN 1 | 192.168.1.11 | 255.255.255.0 | 192.168.1.1 |
| PC-A | NIC | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 |

Objectives

Part 1: Configure Basic Device Settings

Part 2: Configure Basic Security Measures on the Router and Switch

Part 3: Configure the Router for SSH Access

Part 4: Configure the Switch for SSH Access

Part 5: SSH from the CLI on the Switch

Background / Scenario

In the past, Telnet was the most common network protocol used to remotely configure network devices. Telnet does not encrypt the information between the client and server. This allows a network sniffer to intercept passwords and configuration information.

Secure Shell (SSH) is a network protocol that establishes a secure terminal emulation connection to a router or other networking device. SSH encrypts all information that passes over the network link and provides authentication of the remote computer. SSH is rapidly replacing Telnet as the remote login tool of choice for network professionals. SSH is most often used to log in to a remote device and execute commands; however, it can also transfer files using the associated Secure FTP (SFTP) or Secure Copy (SCP) protocols.

The network devices that are communicating must be configured to support SSH in order for SSH to function. In this lab, you will enable the SSH server on a router and then connect to that router using a PC with an SSH client installed. On a local network, the connection is normally made using Ethernet and IP.

Note: The routers used with CCNA hands-on labs are Cisco 1941 Integrated Services Routers (ISRs) with Cisco IOS Release 15.2(4)M3 (universalk9 image). The switches used are Cisco Catalyst 2960s with Cisco IOS Release 15.0(2) (lanbasek9 image). Other routers, switches, and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and the output produced might vary from what is shown in the labs. Refer to the Router Interface Summary Table at the end of this lab for the correct interface identifiers.

Note: Make sure that the routers and switches have been erased and have no startup configurations. If you are unsure, contact your instructor.

Required Resources

1 Router (Cisco 1941 with Cisco IOS Release 15.2(4)M3 universal image or comparable)

1 Switch (Cisco 2960 with Cisco IOS Release 15.0(2) lanbasek9 image or comparable)

1 PC (Windows 7 or 8 with terminal emulation program, such as Tera Term, and Wireshark installed)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet cables as shown in the topology

Configure Basic Device Settings

In Part 1, you will set up the network topology and configure basic settings, such as the interface IP addresses, device access, and passwords on the router.

Cable the network as shown in the topology.

  - Initialize and reload the router and switch.

  - Configure the router.

    - Console into the router and enable privileged EXEC mode.

    - Enter configuration mode.

    - Disable DNS lookup to prevent the router from attempting to translate incorrectly entered commands as though they were host names.

    - Assign class as the privileged EXEC encrypted password.

    - Assign cisco as the console password and enable login.

    - Assign cisco as the VTY password and enable login.

    - Encrypt the plaintext passwords.

    - Create a banner that will warn anyone accessing the device that unauthorized access is prohibited.

    - Configure and activate the G0/1 interface on the router using the information contained in the Addressing Table.

    - Save the running configuration to the startup configuration file.

Configure PC-A.

Configure PC-A with an IP address and subnet mask.

Configure a default gateway for PC-A.

Verify network connectivity.

Ping R1 from PC-A. If the ping fails, troubleshoot the connection.

- Configure Basic Security Measures on the Router

  - Encrypt the clear text passwords.

R1(config)# service password-encryption

  - Strengthen passwords.

An administrator should ensure that passwords meet the standard guidelines for strong passwords. These guidelines could include combining letters, numbers and special characters in the password and setting a minimum length.

Note: Best practice guidelines require the use of strong passwords, such as those shown here, in a production environment. However, the other labs in this course use the cisco and class passwords for ease in performing the labs.

    - Change the privileged EXEC encrypted password to meet guidelines.

R1(config)# enable secret Enablep@55

    - Require that a minimum of 10 characters be used for all passwords.

R1(config)# security passwords min-length 10

  - Secure the console and VTY lines.

    - You can set the router to log out of a connection that has been idle for a specified time. If a network administrator was logged into a networking device and was suddenly called away, this command automatically logs the user out after the specified time. The following commands cause the line to log out after five minutes of inactivity.

R1(config)# line console 0

R1(config-line)# exec-timeout 5 0

R1(config-line)# line vty 0 4

R1(config-line)# exec-timeout 5 0

R1(config-line)# exit

R1(config)#

    - The following command impedes brute force login attempts. The router blocks login attempts for 30 seconds if someone fails two attempts within 120 seconds. This timer is set especially low for the purpose of this lab.

R1(config)# login block-for 30 attempts 2 within 120

What does the 2 within 120 mean in the above command?

The above means that if you enter the incorrect login twice with 120 seconds that you will be blocked for a set period of time.

____________________________________________________________________________________

What does the block-for 30 mean in the above command?

Block-for-30 means that if a user meets a set criteria, block that user from logging in for 30 seconds.

____________________________________________________________________________________

  - Verify that all unused ports are disabled.

Router ports are disabled by default, but it is always prudent to verify that all unused ports are in an administratively down state. This can be quickly checked by issuing the show ip interface brief command. Any unused ports that are not in an administratively down state should be disabled using the shutdown command in interface configuration mode.

R1# show ip interface brief

Interface                  IP-Address      OK? Method Status                Protocol

Embedded-Service-Engine0/0 unassigned      YES NVRAM  administratively down down

GigabitEthernet0/0         unassigned      YES NVRAM  administratively down down

GigabitEthernet0/1         192.168.1.1     YES manual up                    up

Serial0/0/0                unassigned      YES NVRAM  administratively down down

Serial0/0/1                unassigned      YES NVRAM  administratively down down

Configure the Router for SSH Access

Using Telnet to connect to a network device is a security risk because all information is transmitted in a clear text format. SSH encrypts the session data and provides device authentication, which is why SSH is recommended for remote connections. In Part 2, you will configure the router to accept SSH connections over the VTY lines.

Configure device authentication.

The device name and domain are used as part of the crypto key when it is generated. Therefore, these names must be entered prior to issuing the crypto key command.

Configure the domain for the device.

R1(config)# ip domain-name ccna-lab.com

Configure the encryption key method.

R1(config)# crypto key generate rsa modulus 1024

The name for the keys will be: R1.ccna-lab.com

% The key modulus size is 1024 bits

% Generating 1024 bit RSA keys, keys will be non-exportable...

[OK] (elapsed time was 1 seconds)

R1(config)#

*Jan 28 21:09:29.867: %SSH-5-ENABLED: SSH 1.99 has been enabled

Configure a local database username.

R1(config)# username admin privilege 15 secret adminpass

Note: A privilege level of 15 gives the user administrator rights.

Enable SSH on the VTY lines.

Enable Telnet and SSH on the inbound VTY lines using the transport input command.

R1(config)# line vty 0 4

R1(config-line)# transport input ssh

Change the login method to use the local database for user verification.

R1(config-line)# login local

R1(config-line)# end

R1#

Save the running configuration to the startup configuration file.

R1# copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

R1#

Establish an SSH connection to the router.

Start Tera Term from PC-A.

Use Tera Term to telnet to R1.

Does R1 accept the Telnet connection? Explain.

No it did not accept the telnet connection. The transport input is set to SSH and not telnet.

Use Tera Term, establish an SSH session to R1. Use the username admin and password Admin1p@55. You should be able to establish an SSH session with R1.

Does R1 accept the SSH connection?

YES

Intentionally mistype the user and password information to see if login access is blocked after two attempts.

What happened after you failed to login the second time?

- It blocks logins for 30 seconds

From your console session on the router, issue the show login command to view the login status. In the example below, the show login command was issued within the 30 second login blocking period and shows that the router is in Quiet-Mode. The router will not accept any login attempts for 14 more seconds.

R1# show login

A default login delay of 1 second is applied.

No Quiet-Mode access list has been configured.

Router enabled to watch for login Attacks.

If more than 2 login failures occur in 120 seconds or less,

logins will be disabled for 30 seconds.

Router presently in Quiet-Mode.

Will remain in Quiet-Mode for 14 seconds.

Denying logins from all sources.

R1#

After the 30 seconds has expired, SSH to R1 again and login using the admin username and Admin1p@55 for the password.

After you successfully logged in, what was displayed?

If I use the password ‘Admin1p@55’ it allows me to login. It shows the MOTD Banner that was set earlier and It then displays the prompt R1# meaning I am at privilege EXEC mode.

Enter privileged EXEC mode and use Enablep@55 for the password.

If you mistype this password, are you disconnected from your SSH session after two failed attempts within 120 seconds? Explain.

When using SSH you are already at privilege EXEC mode. If you type exit it closes the SSH session.

____________________________________________________________________________________

____________________________________________________________________________________

Issue the show running-config command at the privileged EXEC prompt to view the security settings you have applied.

- Due to inconsistency in the questions I have copied the running-config to display what commands I had to enter to get the above to work.

Building configuration...

Current configuration : 1110 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

security passwords min-length 10

!

hostname R1

!

login block-for 30 attempts 2 within 120

!

!

enable secret 5 $1$mERr$5wXi8fVAu3dNc0gNCjvIQ1

!

!

!

!

!

!

ip cef

no ipv6 cef

!

!

!

username admin privilege 15 secret 5 $1$mERr$uMzC7If3IbCZVifeGQ3rg/

!

!

license udi pid CISCO1941/K9 sn FTX1524H439-

!

!

!

!

!

!

!

!

!

no ip domain-lookup

ip domain-name ccna-lab.com

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0

no ip address

duplex auto

speed auto

shutdown

!

interface GigabitEthernet0/1

ip address 192.168.1.1 255.255.255.0

duplex auto

speed auto

!

interface Vlan1

no ip address

shutdown

!

ip classless

!

ip flow-export version 9

!

!

ip access-list extended sl_def_acl

deny tcp any any eq telnet

deny tcp any any eq www

deny tcp any any eq 22

permit tcp any any eq 22

!

banner login ^C Unauthorised Access is Prohibited ^^C

!

!

!

!

!

line con 0

exec-timeout 5 0

!

line aux 0

!

line vty 0 4

exec-timeout 5 0

login local

transport input ssh

!

!

!

end

Configure the Switch for SSH Access

In Part 3, you will configure the switch in the topology to accept SSH connections. After the switch has been configured, establish an SSH session using Tera Term.

Configure the basic settings on the switch.

Console into the switch and enable privileged EXEC mode.

Enter configuration mode.

Disable DNS lookup to prevent the router from attempting to translate incorrectly entered commands as though they were host names.

Assign class as the privileged EXEC encrypted password.

Assign cisco as the console password and enable login.

Assign cisco as the VTY password and enable login.

Encrypt the plain text passwords.

Create a banner that will warn anyone accessing the device that unauthorized access is prohibited.

Configure and activate the VLAN 1 interface on the switch according to the Addressing Table.

Save the running configuration to the startup configuration file.

Configure the switch for SSH connectivity.

Use the same commands that you used to configure SSH on the router in Part 2 to configure SSH for the switch.

Configure the device name as listed in the Addressing Table.

Configure the domain for the device.

S1(config)# ip domain-name ccna-lab.com

Configure the encryption key method.

S1(config)# crypto key generate rsa general-keys modulus 1024

Configure a local database username.

S1(config)# username admin privilege 15 secret Admin1p@55

Enable Telnet and SSH on the VTY lines.

S1(config)# line vty 0 15

S1(config-line)# transport input telnet ssh

Change the login method to use the local database for user verification.

S1(config-line)# login local

S1(config-line)# end

Establish an SSH connection to the switch.

Start Tera Term from PC-A, and then SSH to the SVI interface on S1.

Are you able to establish an SSH session with the switch?

Yes I get a password prompt and am able to login. MOTD banner is displayed and I am at the privilege EXEC prompt.

_______________________________________________________________________________________

SSH From the CLI on the Switch

The SSH client is built into the Cisco IOS and can be run from the CLI. In Part 4, you will SSH to the router from the CLI on the switch.

View the parameters available for the Cisco IOS SSH client.

Use the question mark (?) to display the parameter options available with the ssh command.

S1# ssh ?

-c    Select encryption algorithm

-l    Log in using this user name

-m    Select HMAC algorithm

-o    Specify options

-p    Connect to this port

-v    Specify SSH Protocol Version

-vrf  Specify vrf name

WORD  IP address or hostname of a remote system

SSH to R1 from S1.

You must use the –l admin option when you SSH to R1. This allows you to log in as user admin. When prompted, enter adminpass for the password.

S1# ssh -l admin 192.168.1.1

Password:

***********************************************

Warning: Unauthorized Access is Prohibited!

***********************************************

R1#

You can return to S1 without closing the SSH session to R1 by pressing Ctrl+Shift+6. Release the Ctrl+Shift+6 keys and press x. The switch privileged EXEC prompt displays.

R1#

S1#

To return to the SSH session on R1, press Enter on a blank CLI line. You may need to press Enter a second time to see the router CLI prompt.

S1#

[Resuming connection 1 to 192.168.1.1 ... ]

R1#

To end the SSH session on R1, type exit at the router prompt.

R1# exit

[Connection to 192.168.1.1 closed by foreign host]

S1#

What versions of SSH are supported from the CLI?

Using the command SSH -v ? it showed Version 1 and 2 protocols were available through the CLI.

How would you provide multiple users, each with their own username, access to a network device?

Creating additional usernames with appropriate privilege levels and a password at the configure terminal prompt. i.e.

Username Bob privilege 15 password LETMEINBOB!

Reflection (max 800 words):

Write a research report comparing various Encryption standards, e.g AES, SHA, DES, RSA.

Report:

Title: Comparing Encryption Methods

AES, DES, SHA, RSA.

Contents

Part 1:	Configure Basic Device Settings	2

Part 2:	Configure Basic Security Measures on the Router	3

Part 3:	Configure the Router for SSH Access	5

Part 4:	Configure the Switch for SSH Access	9

Part 5:	SSH From the CLI on the Switch	10

Introduction	12

AES	12

DES	12

SHA	13

RSA	13

Summary	14

# Introduction

This report gives a brief history, description and finally a summary of AES, DES, SHA and RSA. A summary is given describing some usages of each method and why more than 1 method may be the best approach.

# AES

AES stands for Advanced Encryption Standard. It is an encryption method with a block size of 128bits and key sizes of 128 bits, 192 bits or 256 bits.

According to Rouse (Rouse, Advanced Encryption Standard, 2017) NIST started development of the encryption method in 1997 to replace DES due to vulnerabilities to brute-force attacks. Joan Daemen and Vincent Rijmen are cryptographers from Belgium who developed the algorithm which was adopted as a standard in 2000. AES is a symmetric key algorithm which means the same key is required for both encryption and decryption, this key needs to be sent to the recipient of the data to decrypt it.

# DES

DES stands for Data Encryption Standard.

DES was created by IBM for the US government and was considered uncrackable until the late 1990’s where computing power was powerful enough to brute force DES. The first demonstration of DES being successfully cracked was in 1997. [2]

DES uses a 64 bit key however 8 bits are used for parity checks therefore DES only supports 56 bit key. A brute force attack to find the key will use 2^56 iterations which works out at a maximum of 72,057,594,037,927,936 attempts to crack the algorithm.

A variant of DES called 3DES or triple DES has since also been retired in favor of AES.

# SHA

SHA stands for Secure Hash Algorithm. This algorithm doesn’t encrypt or decrypt data, it only creates a hash of it. The hash is used to validate the integrity of the data.

There are a number of SHA versions, however only SHA-2 is currently widely used. SHA-2 can be identified by other names such as SHA-256, SH-512 etc. These are all SHA-2 algorithms however the number identifies the bit length.

SHA-1 was reevaluated due to its weakness. SHA-2 was published in 2001 but not adopted (completely replacing SHA-1) until 2016 and is predominately used in SSL/TLS Certificates.

SHA3 is a result from a competition run by NIST in which NIST chose the winner in 2013. Unlike the 2 previous versions of SHA which use MD (Merkle-Damgard) SHA-3 uses a Sponge function. For more details comparing the version and function are covered by Penard [3]. As of yet SHA-3 hasn’t been widely adopted due to compatibility with software and hardware.

# RSA

RSA stands for Rivest-Shamir-Adleman.

RSA is an asymmetric algorithm it uses a publicly known key and a private key to encrypt and decrypt the data. The public key is shared so that the data can be encrypted but only those who have the private key can decrypt the data. This type of algorithm is computationally intensive and is much slower than AES. Generally, this is used to encrypt small amounts of data.

# Summary

All the encryption methods above have their place in our digital lives except for DES which has been superseded by AES. AES and RSA can be used together, for example RSA can be used to encrypt the exchange of keys while AES is used to encrypt the data. This is mainly due to the extra computational power RSA requires which wouldn’t be suitable for large amounts of data. This negates the issue of having to distribute the AES key which is used for encrypting and decrypting [4].

SHA is used for SSL/TLS and digital signature which is often seen when accessing HTTPS websites. Whereas the other 3 mentioned above are encryption methods, SHA is a hashing algorithm. A hashing algorithm is used to check the sender authenticity and data integrity when being sent from A to B, generally seen in Internet traffic accessing websites.

WORDS = 635

# References

| [1] | M. Rouse, "Advanced Encryption Standard," 2017. [Online]. Available: https://searchsecurity.techtarget.com/definition/Advanced-Encryption-Standard. [Accessed 03 December 2019]. |
| --- | --- |
| [2] | I. P. Team, "IT PRO," 30 September 2019. [Online]. Available: https://www.itpro.co.uk/security/29671/what-is-aes-encryption. [Accessed December 2019]. |
| [3] | T. v. W. Wouter Penard, "On the secure hash algorithm family," 2016. |
| [4] | R. Franklin, "SyncSort," 13 March 2019. [Online]. Available: https://blog.syncsort.com/2019/03/data-security/aes-vs-rsa-encryption-differences/. [Accessed 3 December 2019]. |