## Front cover

BSc (Hons.)/BEng (Hons.) Ordnance, Munitions and Explosives ACDA 6017 – Level 6 Major Project: A Novel Propellant Small Scale Hazard Test

Authored by: Mike Peck Student number: 1907404 Date: July 2023

## ontents

Contents

Front cover	1

ontents	2

1.	Introduction	3

2.    Background to the Proposed Project	4

3.	Project Design	5

4.	Resources and Planning	6

4.1.	   Gantt Chart	6

5.	References	7

6.	Appendix	8

## Introduction

Typically a suite of Small Scale Hazard Tests (SSHT) are carried out in accordance with the Energentic Materials Testing Assessment Policy Manual (EMTAP) to allow essential classification of explosives as catergories within the UK Manual testing and classification of dangerous goods [1]. These are required to give data for explosive substance data sheets to allow catergorisation for purposes of use, handling, storage and transport. This particular project is focussing on the use of the BAM friction test, which is used to establish the sensativity of a substance to friction. This particular project will focus not only on friction, but the friction applied to propellants. The aims of the project will be fully discussed later on, but the key aim is to create a test that will allow propellant sticks to be tested in their true form i.e. in stick form (essentially as a solid) rather than the powdered/granullar form in which they are currently tested.

## 2. Background to the Proposed Project

As mentioned in the introduction the BAM Friction test normally uses the apparatus as shown below in Figure 1. This consists of a metal table clamped to which is a ceramic tile (just above weight 6 below) the black handle which is attached to the collet holds a ceramic peg, the sample to be tested is placed under the ceramic peg on the tile (which has striations to create friction perpendicular to the direction of motion). When the button is pressed, it allows the motor to move the peg and weighted arm through 1 full revolution. The tile is then inspected to see whether there has been a positive result or ‘go’ or no result a ‘no go’ [2].

Figure 1: BAM Friction tester

The key aspect to this project is to test propellant stick samples as they appear in the finished product. It can then be established whether the current test of powdered/granular propellant is representative of the end user product and if it is not, provide a means of testing propellant sticks in final form and giving a source of additional information to perhaps be added as an additional SSHT test.

Current research suggests that there has not been a test of this sort carried out before and that people in the energetics industry may not hold the current suite of tests with much regard due to their lack of representation of the finished articles. Discussions with Subject Matter Experts (SMEs) in the field have further demonstrated this to be the case. It is therefore the aim of this project to design and commission a test procedure that uses the BAM Friction Tester, but adapted for use with propellant sticks in their final product form instead of powder.

## Project Design

This project will have several aims/objectives to it, they are as follows:

Aim 1: Investigate/procure 4 suitable different types of sample with 4 different geometries

Objective A: investigate and procure 4 types of propellant composition of suitable samples within the first week of the project.

Objective B: investigate and procure 4 types of different propellant stick geometry, with the same composition of suitable samples within the first week of the project.

Aim 2: Design and Manufacture of Test Apparatus

Objective A: Measure both the physical collet hole in the BAM Friction tester and measure all propellant sticks to allow CAD of collet adapters.

Objective B: Discuss with engineering and get collet adapters manufactured (Design 1 week, manufacture 4 weeks).

Aim 3: Procurement of test samples

Objective: Within the first 8 weeks of the project and as a result of information gathered for Aims 1 & 2 obtain cut sections of propellant sticks for test and granular form for conventional test.

Aim 4: Test samples

Objective: Within the first 10 weeks of the project use the samples procured and test equipment as manufactured to test both different types of propellant in stick form and granular form (1 form in the morning and other in the afternoon).

Aim 5: Collect, analyse and evaluate data

Objective A: From week 10 to week 12 data should be collected by way of Bruceston staircase method as is normally conducted

Objective B: 12 to 14 weeks from start of project all data sets will be compared for consistency and to highlight any data needing repetition

Objective C: Weeks 14 to 16 will comprise of data evaluation to compare to already available data, from burn tests and Small Scale Hazard Tests (SSHT)

To achieve all of these aims and objectives strong working connections will be forged between the engineering department, Small Scale Hazard Testing department, and propellant extraction teams. Tooling likely to be utilised will include use of CAD packages such as Solidworks and possible CNC programmed milling of collet adapters. SQEP personnel from the propellant team will carry out actual extraction of propellant sticks. Micrometres will be used to ascertain correct dimensions for CAD diagrams.

## Resources and Planning

To summarise he deliverables from this project will be:

- Accurate CAD diagrams or collet adapters to hold stick propellant samples while testing

- Manufacture of collet adapters

- Testing of both stick and ground propellant samples

- Analysis of data to show whether the typical ground BAM Friction test results are repsentative of those given by the final end user stick propellant

- Conclusions of test and possible applications for more representative tests

## Gantt Chart

Below is an indication of the ideal estimate for the process of the project to be carried out. I should be understood that at this time there are many unknowns, such as time required to obtain authorisation to carry out the project, lead time on materials, lead time on workshop capability access to build the required apparatus and many other unforeseen complications that will undoubtedly present themselves. To help mitigate against these unforeseen events the scale of the project is estimated as follows:

| Aim: | Objective: | Ideal duration (weeks): |
| --- | --- | --- |
| 1 | A | 1 |
|  | B | 1 |
| 2 | A | 1 |
|  | B | 4 |
| 3 | A | 1 |
| 4 | A | 2 |
| 5 | A | 4 |
|  | B | 2 |
|  | C | 2 |

Giving an ambitious total of 18 weeks or about 4.5 months, which then allows a 1.5 month contingency for the unforeseen events as mentioned above.

## References

| [1] | HSE, “Explosives Testing,” Health and Safety Executive, 01 07 2023. [Online]. Available: https://www.hsl.gov.uk/product-and-process-validation-testing/explosives-testing. [Accessed 01 02 2023]. |
| --- | --- |
| [2] | L. R. S. a. M. F. Foltz, “LLNL Small-Scale Friction Sensitity (BAM) Test,” Lawrence Livermore National Laboratory, Springfield VA, 1996. |

## Appendix

APPLICATION FOR ETHICAL APPROVAL

In order for research to result in benefit and minimise risk of harm, it must be conducted ethically.

The University follows the OECD Frascati manual definition of research activity: “creative work undertaken on a systematic basis in order to increase the stock of knowledge, including knowledge of man, culture and society, and the use of this stock of knowledge to devise new applications”. As such this covers activities undertaken by members of staff, postgraduate research students, and both taught postgraduate and undergraduate students working on dissertations/projects.

The individual undertaking the research activity is known as the “principal researcher”.

This form must be completed and approved prior to undertaking any research activity.

SECTION A: About You (Principal Researcher)

| 1 | Full Name: | Mike Peck |
| --- | --- | --- |
| 2 | Student Number: | 1907404 |
| 3 | Email address: | 1907404@student.uwtsd.ac.uk |
| 4 | Programme of Study: | OME Apprenticeship Programme |
| 5 | Director of Studies/Supervisor: | Gareth Collett |

SECTION B:  Internal and External Ethical Guidance Materials

|  | Please list the core ethical guidance documents that have been referred to during the completion of this form (including any discipline-specific codes of research ethics, location-specific dodes of research ethics, and also any specific ethical guidance relating to the proposed methodology). Please tick to confirm that your research proposal adheres to these codes and guidelines. You may add rows to this table if needed. |  |
| --- | --- | --- |
| 1 | UWTSD Research Ethics & Integrity Code of Practice | ☒ |
| 2 | UWTSD Research Data Management Policy | ☒ |
| 3 |  | ☐ |

SECTION C: Details of Research Activity

| 1 | Indicative title: | A Novel Propellant Small Scale Hazard Test |  |  |
| --- | --- | --- | --- | --- |
| 2 | Proposed start date: | 01/09/2023 | Proposed end date: | 01/01/2024 |
|  | Introduction to the Research (maximum 300 words in each section) Ensure that you write for a Non-Specialist Audience when outlining your response to the three points below: - Purpose of Research Activity - Proposed Research Question - Aims of Research Activity - Objectives of Research Activity Demonstrate, briefly, how Existing Research has informed the proposed activity and explain - What the research activity will add to the body of knowledge - How it addresses an area of importance. |  |  |  |
| 3 | Purpose of Research Activity To design a test apparatus to measure the sensitivity of propellant when introduced to friction, when the propellant is in its useable end state i.e. stick form as opposed to ground/granulated form. (this box should expand as you type) |  |  |  |
| 4 | Research Question Is the sensitivity of ground propellant stick composition comparable to that of propellant stick in its end user form i.e. is the BAM Friction test of ground propellant representative of the finished product? (this box should expand as you type) |  |  |  |
| 5 | Aims of Research Activity To engineer, build and obtain reproducible/comparable results for propellant samples from both typically ground/granular composition and non-typical solid propellant stick cut sections. (this box should expand as you type) |  |  |  |
| 6 | Objectives of Research Activity Aim 1: Investigate/procure 4 suitable different types of sample with 4 different geometries Objective A: investigate and procure 4 types of propellant composition of suitable samples within the first week of the project. Objective B: investigate and procure 4 types of different propellant stick geometry, with the same composition of suitable samples within the first week of the project. Aim 2: Design and Manufacture of Test Apparatus Objective A: Measure both the physical collet hole in the BAM Friction tester and measure all propellant sticks to allow CAD of collet adapters. Objective B: Discuss with engineering and get collet adapters manufactured (Design 1 week, manufacture 4 weeks). Aim 3: Procurement of test samples Objective: Within the first 8 weeks of the project and as a result of information gathered for Aims 1 & 2 obtain cut sections of propellant sticks for test and granular form for conventional test. Aim 4: Test samples Objective: Within the first 10 weeks of the project use the samples procured and test equipment as manufactured to test both different types of propellant in stick form and granular form (1 form in the morning and other in the afternoon). Aim 5: Collect, analyse and evaluate data Objective A: From week 10 to week 12 data should be collected by way of Bruceston staircase method as is normally conducted Objective B: 12 to 14 weeks from start of project all data sets will be compared for consistency and to highlight any data needing repetition Objective C: Weeks 14 to 16 will comprise of data evaluation to compare to already available data, from burn tests and Small Scale Hazard Tests (SSHT) (this box should expand as you type) |  |  |  |
|  | Proposed data collection methods (maximum 600 words) Provide a brief summary of all the methods that may be used in the research activity to collect data, making it clear what specific techniques may be used. If methods other than those listed in this section are deemed appropriate later, additional ethical approval for those methods will be needed. You do not need to justify the methods here, but should instead describe how you intend to collect the data necessary for you to complete your project. |  |  |  |
| 7 | Data collection to be carried out by use of High Speed Video and IR Thermal imaging cameras (this box should expand as you type) |  |  |  |

SECTION D: Scope of Research Activity

|  | Will the research activity include: | YES | NO |
| --- | --- | --- | --- |
| 1 | Use of a questionnaire or similar research instrument? | ☐ | ☒ |
| 2 | Use of interviews? | ☐ | ☒ |
| 3 | Use of focus groups? | ☐ | ☒ |
| 4 | Use of participant diaries? | ☐ | ☒ |
| 5 | Use of video or audio recording? | ☒ | ☐ |
| 6 | Use of computer-generated log files? | ☒ | ☐ |
| 7 | Participant observation with their knowledge? | ☐ | ☒ |
| 8 | Participant observation without their knowledge? | ☐ | ☒ |
| 9 | Access to personal or confidential information without the participants’ specific consent? | ☐ | ☒ |
| 10 | Administration of any questions, test stimuli, presentation that may be experienced as physically, mentally or emotionally harmful / offensive? | ☐ | ☒ |
| 11 | Performance of any acts which may cause embarrassment or affect self-esteem? | ☐ | ☒ |
| 12 | Investigation of participants involved in illegal activities? | ☐ | ☒ |
| 13 | Use of procedures that involve deception? | ☐ | ☒ |
| 14 | Administration of any substance, agent or placebo? | ☐ | ☒ |
| 15 | Working with live vertebrate animals? | ☐ | ☒ |
| 16 | Procedures that may have a negative impact on the environment? | ☐ | ☒ |
| 17 | Other primary data collection methods. Please indicate the type of data collection method(s) below. | ☐ | ☒ |
|  | Details of any other primary data collection method: (this box should expand as you type) |  |  |

If you have ticked NO to every question then the research activity is (ethically) low risk and you may skip section E and continue to section F.

If YES to any question, then no research activity should be undertaken until full ethical approval has been obtained.

SECTION E: Intended Participants

|  | Who are the intended participants: | YES | NO |
| --- | --- | --- | --- |
| 1 | Students or staff at the University? | ☐ | ☒ |
| 2 | Adults (over the age of 18 and competent to give consent)? | ☒ | ☐ |
| 3 | Vulnerable adults? | ☐ | ☒ |
| 4 | Children and Young People under the age of 18? (Consent from Parent, Carer or Guardian will be required) | ☐ | ☒ |
| 5 | Prisoners? | ☐ | ☒ |
| 6 | Young offenders? | ☐ | ☒ |
| 7 | Those who could be considered to have a particularly dependent relationship with the investigator or a gatekeeper? | ☐ | ☒ |
| 8 | People engaged in illegal activities? | ☐ | ☒ |
| 9 | Others. Please indicate the participants below, and specifically any group who may be unable to give consent. | ☐ | ☒ |
|  | Details of any other participant groups: Complete this only if your participants cannot give consent. This includes animals (this box should expand as you type) |  |  |

|  | Participant numbers and source Provide an estimate of the expected number of participants. How will you identify participants and how will they be recruited? |  |
| --- | --- | --- |
| 10 | How many participants are expected? | 2-3 for build, and data collection (this box should expand as you type) |
| 11 | Who will the participants be? | SQEP DSTL staff (this box should expand as you type) |
| 12 | How will you identify the participants? | Via team project meeting (stakeholder meeting) (this box should expand as you type) |

|  | Information for participants: | YES | NO | N/A |
| --- | --- | --- | --- | --- |
| 13 | Will you describe the main research procedures to participants in advance, so that they are informed about what to expect? | ☒ | ☐ | ☐ |
| 14 | Will you tell participants that their participation is voluntary? | ☒ | ☐ | ☐ |
| 15 | Will you obtain written consent for participation? | ☐ | ☐ | ☒ |
| 16 | Will you explain to participants that refusal to participate in the research will not affect their treatment or education (if relevant)? | ☐ | ☐ | ☒ |
| 17 | If the research is observational, will you ask participants for their consent to being observed? | ☐ | ☐ | ☒ |
| 18 | Will you tell participants that they may withdraw from the research at any time and for any reason? | ☐ | ☐ | ☒ |
| 19 | With questionnaires, will you give participants the option of omitting questions they do not want to answer? | ☐ | ☐ | ☒ |
| 20 | Will you tell participants that their data will be treated with full confidentiality and that, if published, it will not be identifiable as theirs? | ☐ | ☐ | ☒ |
| 21 | Will you debrief participants at the end of their participation, in a way appropriate to the type of research undertaken? | ☐ | ☐ | ☒ |
| 22 | If NO to any of above questions, please give an explanation |  |  |  |
|  | You should be able to tick YES for all of these questions. If not, then explain why not in this box. Largely not required. (this box should expand as you type) |  |  |  |

|  | Information for participants: | YES | NO | N/A |
| --- | --- | --- | --- | --- |
| 24 | Will participants be paid? | ☐ | ☐ | ☒ |
| 25 | Is specialist electrical or other equipment to be used with participants? | ☐ | ☐ | ☒ |
| 26 | Are there any financial or other interests to the investigator or University arising from this study? | ☐ | ☐ | ☒ |
| 27 | Will the research activity involve deliberately misleading participants in any way, or the partial or full concealment of the specific study aims? | ☐ | ☐ | ☒ |
| 28 | If YES to any question, please provide full details |  |  |  |
|  | You should be able to tick NO for most of these questions. For any cases that you have ticked YES then provide details in this box. If you are using cameras/voice recorders to record interviews then please state that in this box. (this box should expand as you type) |  |  |  |

SECTION F: Anticipated Risks

|  | # Outline any anticipated risks that may adversely affect any of the participants, the researchers and/or the University, and the steps that will be taken to address them. |  |
| --- | --- | --- |
| 1 | Risks to participants For example: sector-specific health & safety, emotional distress, financial disclosure, physical harm, transfer of personal data, sensitive organisational information. If you have identified in section D that there are no participants then enter N/A and go skip to question 3. |  |
|  | Risk to participants: There are always risks. Do not write N/A unless you have no participants. Risk of bright lights, sound and possible burns, due to the nature of the propellant samples. (this box should expand as you type) | How you will mitigate the risk to participants: Risk will be mitigated by adhering to the RA and Safe Scheme of Work. Which would include all team members being at a safe distance if remote operation is not possible and all necessary training and PPE will be given. (this box should expand as you type) |
| 2 | If research activity may include sensitive, embarrassing or upsetting topics (e.g. sexual activity, drug use) or issues likely to disclose information requiring further action (e.g. criminal activity), give details of the procedures to deal with these issues, including any support/advice (e.g. helpline numbers) to be offered to participants. Note that where applicable, consent procedures should make it clear that if something potentially or actually illegal is discovered in the course of a project, it may need to be disclosed to the proper authorities |  |
|  | N/A (this box should expand as you type) |  |
| 3 | Risks to the investigator For example: personal health & safety, physical harm, emotional distress, risk of accusation of harm/impropriety, conflict of interest |  |
|  | Risk to the investigator: There are always risks. Do not write NA. Risk of bright lights, sound and possible burns, due to the nature of the propellant samples. (this box should expand as you type) | How you will mitigate the risk to the investigator: Risk will be mitigated by adhering to the RA and Safe Scheme of Work. Which would include all team members being at a safe distance if remote operation is not possible and all necessary training and PPE will be given. (this box should expand as you type) |
| 4 | University/institutional risks For example: adverse publicity, financial loss, data protection |  |
|  | Risk to the University: There are always risks. Do not write NA. This will be fully conducted at DSTL sites, as such will not pose a risk to the University. (this box should expand as you type) | How you will mitigate the risk to the University: Not needed (this box should expand as you type) |
| 5 | Environmental risks For example: accidental spillage of pollutants, damage to local ecosystems |  |
|  | Risk to the environment: All tests will be carried out in a controlled, independently HVAC’d specialist facility Possibility of small spills of pollutants. (this box should expand as you type) | How you will mitigate the risk to environment: All left test related materials will be properly cleared and cleaned and removed in the proper manner upon completion of tasks. (this box should expand as you type) |

SECTION G: Feedback, Consent and Confidentiality

If you have identified in section D that there are no participants then enter skip this section and continue to section H.

| 1 | Feedback What de-briefing and feedback will be provided to participants, how will this be done and when? |
| --- | --- |
|  | You don’t need to email your participants with your final report. A good alternative is to set up an email address that they will be able to contact for further details or results. The team will be notified of outcomes at de-brief meeting and via report (this box should expand as you type) |
| 2 | Informed consent Describe the arrangements to inform potential participants, before providing consent, of what is involved in participating. Describe the arrangements for participants to provide full consent before data collection begins. If gaining consent in this way is inappropriate, explain how consent will be obtained and recorded in accordance with prevailing data protection legislation. |
|  | If you are using a paper questionnaire then you should have the participants sign an appropriate consent form. These forms will count as personal data and should be noted as such in section J. If you are using an online questionnaire, then you should have a screen before the questions start that acts as a consent form, informing participants that by clicking on the NEXT button they are providing consent. N/A (this box should expand as you type) |
| 3 | Confidentiality / Anonymity Set out how anonymity of participants and confidentiality will be ensured in any outputs. If anonymity is not being offered, explain why this is the case. |
|  | Do not collect names unless you really need them. Do not name participants or organisations in any research publications (including the thesis) without their explicit permission. N/A (this box should expand as you type) |

SECTION H: Data Protection and Storage

|  | Does the research activity involve personal data (as defined by the General Data Protection Regulation 2016 “GDPR” and the Data Protection Act 2018 “DPA”)? | YES | NO |
| --- | --- | --- | --- |
| 1 | “Personal data” means any information relating to an identified or identifiable natural person (‘data subject’). An identifiable natural person is one who can be identified, directly or indirectly, in particular by reference to an identifier such as a name, an identification number, location data, an online identifier or to one or more factors specific to the physical, physiological, genetic, mental, economic, cultural or social identity of that natural person. Any video or audio recordings of participants is considered to be personal data. | ☐ | ☐ |
|  | If YES, provide a description of the data and explain why this data needs to be collected: |  |  |
| 2 | This includes audio/video data of participants, but can also include IP addresses and usernames. Names, addresses and emails also count, as do consent forms. (this box should expand as you type) |  |  |
|  | Does it involve special category data (as defined by the GDPR)? | YES | NO |
| 3 | “Special category data” means sensitive personal data consisting of information as to the data subjects’ – (a) racial or ethnic origin, (b) political opinions, (c ) religious beliefs or other beliefs of a similar nature, (d) membership of a trade union (within the meaning of the Trade Union and Labour Relations (Consolidation) Act 1992), (e) physical or mental health or condition, (f) sexual life, (g) genetics, (h) biometric data (as used for ID purposes), | ☐ | ☐ |
|  | If YES, provide a description of the special category data and explain why this data needs to be collected: |  |  |
| 4 | What counts as ‘sensitive’ will differ between cultures. Any information on behaviour that is not in accordance with cultural norms would count as sensitive personal data. (this box should expand as you type) |  |  |

|  | Will data from the research activity (collected data, drafts of the thesis, or materials for publication) be stored in any of the following ways? | YES | NO |
| --- | --- | --- | --- |
| 5 | Manual files (i.e. in paper form)? | ☒ | ☐ |
| 6 | University computers? | ☐ | ☒ |
| 7 | Private company computers? | ☐ | ☒ |
| 8 | Home or other personal computers? | ☐ | ☒ |
| 9 | Laptop computers/ CDs/ Portable disk-drives/ memory sticks? | ☒ | ☐ |
| 10 | “Cloud” storage or websites? | ☐ | ☒ |
| 11 | Other – specify: | ☐ | ☒ |
| 12 | For all stored data, explain the measures in place to ensure the security of the data collected, data confidentiality, including details of backup procedures, password protection, encryption, anonymisation and pseudonymisation: |  |  |
|  | If possible, save your data on computers that are secure and regularly backed up. Many cloud services only provide GDPR-compliant storage for business customers. An example of suitable text is given below. All manual files will be held in a secure locker appropriate to material classification, the keys or codes to which will only be help by the project lead or members of the project team, as designated. Any USB pen drives, removable hard drives and PCs used to store or transfer data, will be encrypted and password protected. (this box should expand as you type) |  |  |

|  | Data Protection |  |  |
| --- | --- | --- | --- |
|  | Will the research activity involve any of the following activities: | YES | NO |
| 13 | Electronic transfer of data in any form? | ☐ | ☒ |
| 14 | Sharing of data with others at the University outside of the immediate research team? | ☐ | ☒ |
| 15 | Sharing of data with other organisations? | ☐ | ☒ |
| 16 | Export of data outside the UK or importing of data from outside the UK? | ☐ | ☒ |
| 17 | Use of personal addresses, postcodes, faxes, emails or telephone numbers? | ☐ | ☒ |
| 18 | Publication of data that might allow identification of individuals? | ☐ | ☒ |
| 19 | If YES to any question, please provide full details, explaining how this will be conducted in accordance with the GDPR and Data Protection Act (2018) (and/or any international equivalent): |  |  |
|  | This includes data such as drafts of your thesis as well as experimental or survey data. An example of suitable text is given below. All data will be encrypted and kept in password protected cloud storage on the University Office 365 system which will not be shared. Any USB sticks used to store or transfer data will be password protected. All data transfers will be encrypted and password protected. All participants will be given a unique identifier to ensure confidentiality and this list will be kept securely in the password protected folder. The data will be stored until the completion of the project and then deleted. In accordance with the DPA2018, participants will have the right to ask to see what data is held relating to them, and this data will be deleted immediately if the participant requests this, in which case the data will not be used in the project. (this box should expand as you type) |  |  |
| 20 | List all who will have access to the data generated by the research activity: |  |  |
|  | Normally the principal researcher, possibly also the supervisor and, if the project has an industrial partner, a representative of that partner. Possibly also external examiner or second marker? Those within DSTL that require it AND hold necessary security clearance (this box should expand as you type) |  |  |
| 21 | List who will have control of, and act as custodian(s) for, data generated by the research activity: |  |  |
|  | Usually the principal researcher. Myself/Mentor (SME – SSHT Lead) (this box should expand as you type) |  |  |
| 22 | Give details of data storage arrangements, including security measures in place to protect the data, where data will be stored, how long for, and in what form. |  |  |
|  | Any USB pen drives, removable hard drives and PCs used to store or transfer data, will be encrypted and password protected. These will be stored as long as required and in whatever format is preferred by the team (this box should expand as you type) |  |  |
| 22 | Confirm that you have read the UWTSD guidance on data management (see https://www.uwtsd.ac.uk/library/research-data-management/) | ☒ |  |
| 23 | Confirm that you are aware that you need to keep all data until after your research has completed or the end of your funding | ☒ |  |

SECTION I: Declaration

|  |  | The information which I have provided is correct and complete to the best of my knowledge. I have attempted to identify any risks and issues related to the research activity and acknowledge my obligations and the rights of the participants. In submitting this application I hereby confirm that I undertake to ensure that the above named research activity will meet the University’s Research Ethics and Integrity Code of Practice which is published on the website: https://www.uwtsd.ac.uk/research/research-ethics/ |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | Signature of applicant: |  | Date: 01/07/2023 |  |  |
| 2 |  | Director of Studies/Supervisor: |  |  | Date: |  |
| 3 |  | Signature: |  |  |  |  |

FOR INTERNAL USE ONLY:

|  | Ethical approval given |  |  |
| --- | --- | --- | --- |
| 1 | Signature of assessor: |  | Date: |
| 2 | Name: |  |  |
| 3 | Role: |  |  |