| Substance Name: Zinc Chloride |  |
| --- | --- |
| Product Name: Zinc Chloride |  |
| Chemical Name: Zinc Chloride |  |
| Chemical formula: ZnCl2 |  |
| Synonyms: Dichlorozinc |  |
| CAS No. 7646-85-7 |  |
| Physical and Chemical Information |  |
| Material Safety Data Sheet Reference No |  |
| Physical state: Solid (white powder) |  |
| Corrosive? Corrosive solid |  |
| Odour: Odorless |  |
| Boiling point: 290°C |  |
| Flash point: N/A |  |
| Auto ignition Temperature: N/A |  |
| Occupational Exposure Limit: 2 mg/m3 (STEL) |  |
| Chemical Hazards Involved |  |
| Applicable Risk Phases: C Corrosive N Dangerous for the environment Xn Harmful R34 – Causes burns R22 Harmful if swallowed R50/53 – Very toxic to aquatic organisms, may cause long-term adverse effects In the aquatic environment. |  |
| Hazardous Properties of Material: H302 – Harmful if swallowed H314 - Causes severe skin burns and eye damage H400 – Very toxic to aquatic life H410 - Very toxic to aquatic life with long lasting effects Skin Corr. – Skin corrosion |  |
| Substance Hazard Category: Acute tox. |  |
| Interactions hazards: P273 – Avoid release to the environment P280 – Wear protective gloves/protective clothing/eye protection/face protection P305+P351+P338 - IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. Immediately call a POISON CENTER or doctor/ physician. P501 - Dispose of contents/ container to an approved waste disposal plant. |  |
| Quantities involved: <200g |  |
| Physical Hazards |  |
| What physical hazards need to be considered elsewhere? NA |  |
| Likely Exposures |  |
| Description Of Work To Be Undertaken: Dissolution in alcoholic solvents for electropolishing |  |
| Exposure: Measure and mix powder within a fume cupboard with nitrile gloves. Low risk of exposure |  |
| Operating procedure reference: |  |
| Those at Risk |  |
| List those at risk: Authorised SPECIFIC lab users. Primarily PV and coatings groups. Visitors |  |
| Control Approach |  |
| Engineering measures: Regular fume cupboard maintenance. Airflow and filter monitoring |  |
| Personal Protective Equipment: Eye protection, nitrile rubber gloves, lab coat |  |
| Precautions To Be Taken |  |
| General Handling/Usage Precautions: Avoid contact with skin and eyes. Nitrile gloves will break through after 480mins of full exposure. Only open container in a fume cupboard as hydrogen chloride gas may be generated under certain conditions. |  |
| Storage: Keep container tightly closed. Keep container in a cool, well ventilated area. Do not store above 25°C |  |
| Special requirements for Disposal: Dispose in solid waste, or halogenated solvent waste if in solution |  |
| Monitoring |  |
| Is static monitoring required? N/A |  |
| Is personal exposure monitoring required? N/A |  |
| Is airborne monitoring required? N/A |  |
| Engineering checks on LEV required? N/A |  |
| Where are monitoring results archived? N/A |  |
| Health Surveillance |  |
| Is health surveillance required? N/A |  |
| Where are health surveillance results archived? N/A |  |
| Emergency Plans |  |
| Do special Emergency/Spillage/First Aid Procedures apply? N/A |  |
| Training |  |
| Who will require training? N/A |  |
| What is the required training standard? N/A |  |
| Assessment Review Date: |  |
| Name of COSHH Assessor: Joel Troughton |  |
| Status of Assessor: Research Engineer | Date: 28/02/2014 |