| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 4 | LEFEL 4 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| MAY 2017 | MAI 2017 |

MODULE CODE: M4X00386

ENGINEERING SCIENCE 2

MODEL ANSWERS

- QUESTION FOUR / Closed System: (15 marks)

An insulated rigid tank initially contains 0.7 kg of helium at 27C and 350 kPa. A paddle wheel with a power rating of 0.015 kW is operated within the tank for 30 min. Determine,

- The final temperature of the helium gas.

- The final pressure of the helium gas.

Tank filled with helium

WPW=-0.015 kW

Assumptions:

- Helium can be considered as an ideal gas.

- Constant specific heat values and gas constant can be found in Table A-2.

SOLUTION:

- The first law of thermodynamics for closed system is,

∆U=Q-W

The system is adiabatic therefore there is no heat transfer, that is Q = 0.

Thus,

∆U=-WPW

Helium is an ideal gas, thus,

∆u=cv∙T2-T1

∴    ∆U=m∙cv∙T2-T1

∴    ∆U=m∙cv∙T2-T1t

∴    ∆U=0.7×3.1156×T2-27+27330×60

(5 marks)

Thus,

0.7×3.1156×T2-27+27330×60=--0.015

∴    T2=0.015×18000.7×3.1156+27+273

∴    T2=272.18092+300=12.38+300=312.38 K

∴    T2=39.38℃

(5 marks)

- According to ideal gas equation,

P1.v=R.T1

∴ v=R.T1P1=2.0769×300350=1.7802 m3kg

And,

P2.v=R.T2     ∴   P2=R.T2v=2.0769×312.381.7802=364.44 kPa

∴ P2∙R.T1P1=R.T2

∴ P2=R.T2.P1R.T1=T2.P1T1

∴ P2=312.38×350300=364.44 kPa

(5 marks)

- QUESTION FIVE  / Open system, steady-flow process:  (15 marks)

A house has an electric heating system that consists of a 300 W fan and an electric resistance heating element placed in a duct. Air flows steadily through the duct at a rate of 0.6 kg/s and experiences a temperature rise of 7C. The rate of heat loss from the air in the duct is estimated to be 305 W. Determine the power rating of the electric resistance heating element.

T2 = T1 + 7C

T1

Q=-305 W

Welec

Wfan=-300 W

2

1

Assumptions:

- Air can be considered as an ideal gas.

- Constant specific heat values can be found in Table A-2.

- Kinetic and potential energy changes are negligible.

SOLUTION:

According to first law of thermodynamics for steady-flow open systems,

- Mass Balance:

inm=outm

∴m1=m2=m=0.6 kg/s

(3 marks)

- Energy Balance:

Q-W=outm.h+12.c2+g.z-inm.h+12.c2+g.z

∴   Q-Welec+Wfan=m2.h2-m1.h1

∴   Q-Welec+Wfan=m.h2-h1

(6 marks)

∴   Q-Welec+Wfan=m.cp∆T

∴   Welec+Wfan=Q-m.cp∆T

∴   Welec=Q-m.cp∆T-Wfan

∴   Welec=-0.305-0.6×1.005×7--0.300

∴   Welec=-4.226 kW

(6 marks)

- QUESTION SIX  / Open system, unsteady-flow process:  (20 marks)

An air conditioning system is to be filled from a rigid container that initially contains 5 kg of saturated liquid R-134a at 24C. The valve connecting this container to the air-conditioning system is now opened until the mass in the container is 0.25 k, at which time the valve is closed. During this time, only liquid R-134a flows from the container. Thus, at the final state the container encloses a saturated liquid - vapour mixture. Presuming that the process is isothermal (that is constant temperature) while the valve is open, determine,

- The final quality of the remaining liquid-vapour mixture of the R-134a in the container.

- The total heat transfer.

he=84.98 kJ/kg

Assumptions:

- Kinetic and potential energy changes are negligible.

- There are no work interactions involved.

- The system is isothermal therefore T2 = T1.

- Specific enthalpy of the exit is he=84.98 kJ/kg.

- You will need to use Table A-11.

- In a saturated liquid-vapour mixture, the mass fraction of vapour is called the quality and is expressed as:

x=mvapourmtotal

Quality may have values between 0 (saturated liquid) and 1 (saturated vapour). It has no meaning in the compressed liquid or superheated vapour regions. In the saturated mixture region, the average value of any intensive property y is determined from:

y=yf+x.yfg

where f stands for saturated liquid, g for saturated vapour and y stands for v, u or h.

- For specific volume, vfg=vg-vf

SOLUTION:

- According to Table 11,

- At initial state 1,

T1=24℃m1=5kgsat. liq.∴v1=0.0008261 m3kg

Therefore,

V1=v1.m1=0.0008261×5=4.1305×10-3 m3

(4 marks)

- At final state 2,

T2=T1=24℃m2=0.25kgV2=V1=4.1305×10-3 m3mixture∴vf=0.0008261 m3kgvg=0.031834 m3kg

Therefore,

v2=V2m2=04.1305×10-30.25=0.016522 m3kg

(3 marks)

And, as we have a saturated liquid-vapour mixture, then,

v2=vf+x.vg-vf

∴  x=v2-vfvf-vg=0.016522-0.00082610.031834-0.0008621=0.01569590.0310079=0.5061

(3 marks)

- According to first law of thermodynamics for steady-flow open systems,

- Mass Balance:

mi-me=m2-m1CV         where mi=0, m1=5 kg and m2=0.25 kg

∴ -me=0.25-5

∴ me=4.75 kg

(2 marks)

- Energy Balance:

Q-W=outmh-inmh+m2u2-m1u1CV

∴   Q=mehe+m2u2-m1u1

(2 marks)

T1=24℃sat. liq.∴u1=84.44 kJkg

(2 marks)

T2=24℃x=0.5061∴uf=84.44 kJkgug=243.10 kJkgufg=158.65 kJkg

∴u2=84.44+  0.5061×158.65=164.747 KJ/kg

(2 marks)

Thus,

Q=4.75×84.98+0.25×164.75-5×84.44

∴   Q=22.64 kJ

(2 marks)