MODULE CODE: ACCA5024

Advance Networking FINAL Exam Answer Sheet

Student Name: Ivan Naydenov

Student ID: 2025312

Date: 21.11.2022

- I- IPv4 Addressing Table

| Specification | Subnet A | Subnet B |
| --- | --- | --- |
| Subnet IP address | 192.168.202.0 | 192.168.202.128 |
| Subnet mask (slash notation) | /26 | /25 |
| Subnet mask (dotted decimal) | 255.255.255.192 | 255.255.255.128 |
| 1st Host IP address | 192.168.202.1 | 192.168.202.129 |
| Last Host IP address | 192.168.202.62 | 192.168.202.254 |
| Number of usable hosts per subnet | 62 | 126 |
| Broadcast address | 192.168.202.63 | 192.168.202.255 |

- II- IPv4 Address assignment

| Device | IP address | Subnet Mask | Gateway |
| --- | --- | --- | --- |
| PC-A | 192.168.202.15 /26 | 255.255.255.192 | 192.168.202.1 |
| R1-G0/0/0 | 192.168.202.129 /25 | 255.255.255.128 |  |
| R1-G0/0/1 | 192.168.202.1 /26 | 255.255.255.192 |  |
| S1 | 192.168.202.2 /26 | 255.255.255.192 | 192.168.202.1 |
| PC-B | 192.168.202.150 /25 | 255.255.255.128 | 192.168.202.129 |

- III- IPv6 Addressing Table

| Assigned to Interface | IPv6 Subnet Address | Prefix Length |
| --- | --- | --- |
| R1-G0/0/1 | 2001:db8:acad:0001:: | /64 |
| R1-G0/0/0 | 2001:db8:acad:0002:: | /64 |

- IV-IPv6 Address assignment

| Device | IPv6 address | Prefix Length | Gateway |
| --- | --- | --- | --- |
| R1-G0/0/0 | 2001:db8:acad:0002::1 | /64 | Fe80::1 |
| R1-G0/0/1 | 2001:db8:acad:0001::1 | /64 | Fe80::1 |
| S1 | 2001:db8:acad:0001::2 | /64 | Fe80::1 |

# Task 2: Device Configuration

- Hostname

Router>enable

Router#conf

Configuring from terminal, memory, or network [terminal]?

Enter configuration commands, one per line.  End with CNTL/Z.

Router(config)#hostname R1

R1(config)#

Switch>enable

Switch#conf

Configuring from terminal, memory, or network [terminal]?

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#hostname S1

S1(config)#

- Encrypted privileges EXEC password

S1(config)#enable secret ccnaenpass

R1(config)#enable secret ccnaenpass

- Secure the console access password

S1(config)#line console 0

S1(config-line)#password ccnaconpass

S1(config-line)#login

S1(config-line)#end

R1(config)#line console 0

R1(config-line)#password ccnaconpass

R1(config-line)#login

R1(config-line)#end

R1#

- Minimum length passwords

R1#conf

Configuring from terminal, memory, or network [terminal]?

Enter configuration commands, one per line.  End with CNTL/Z.

R1(config)#security passwords mi

R1(config)#security passwords min-length 10

R1(config)#

- Create an administrative user

R1(config-line)#

R1(config-line)#exit

R1(config)#

R1(config)#username admin password 0 admin1pass

R1(config)#

S1(config-line)#

S1(config-line)#exit

S1(config)#username admin password 0 admin1pass

S1(config)#

- Configure an MOTD banner

S1#conf

Configuring from terminal, memory, or network [terminal]?

Enter configuration commands, one per line.  End with CNTL/Z.

S1(config)#line console 0

S1(config-line)#banner motd #No Unauthorised Access!#

S1(config)#

R1#conf

Configuring from terminal, memory, or network [terminal]?

Enter configuration commands, one per line.  End with CNTL/Z.

R1(config)#line console 0

R1(config-line)#banner motd #No Unauthorised Access!#

R1(config)#

- Encrypt all passwords

R1(config)#service password-encryption

R1(config)#

S1(config)#service password-encryption

S1(config)#

- Disable IP domain lookup

R1(config)#

R1(config)#no ip domain-lookup

R1(config)#

S1(config)#

S1(config)#no ip domain-lookup

S1(config)#

- Enable synchronous logging

S1(config)#line console 0

S1(config-line)#loggin synchronous

S1(config-line)#

R1(config)#

R1(config)#line console 0

R1(config-line)#loggin synchronous

R1(config-line)#

- Configure router’s interfaces

R1(config)#interface g0/0

R1(config-if)#description Connection to Subnet B

R1(config-if)#ip address 192.168.202.129 255.255.255.128

R1(config-if)#no shutdown

R1(config-if)#

%LINK-5-CHANGED: Interface GigabitEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0, changed state to up

R1(config-if)#

R1(config-if)#exit

R1(config)#interface g0/1

R1(config-if)#description Connection to Subnet A

R1(config-if)#ip address 192.168.202.1 255.255.255.192

R1(config-if)#no shutdown

R1(config-if)#

%LINK-5-CHANGED: Interface GigabitEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/1, changed state to up

R1(config-if)#

R1(config-if)#

R1(config-if)#exit

R1(config)#

R1(config)#

R1(config)#interface g0/0

R1(config-if)#ipv6 address 2001:db8:acad:0002::1/64

R1(config-if)#ipv6 address fe80::1 link-local

R1(config-if)#exit

R1(config)#ipv6 unicast-routing

R1(config)#

R1(config-if)#no shutdown

R1(config)#

R1(config)#interface g0/1

R1(config-if)#ipv6 address 2001:db8:acad:0001::1/64

R1(config-if)#ipv6 address fe80::1 link-local

R1(config-if)#

R1(config-if)#no shutdown

- Configure switch’s management (SVI) interface

No Unauthorised Access!

User Access Verification

Password:

Password:

Password:

S1>

S1>

S1>

S1>enable

Password:

Password:

Password:

S1#

S1#

S1#

S1#conf

Configuring from terminal, memory, or network [terminal]?

Enter configuration commands, one per line.  End with CNTL/Z.

S1(config)#

S1(config)#sdm prefer dual-ipv4-and-ipv6 default

Changes to the running SDM preferences have been stored, but cannot take effect until the next reload.

Use 'show sdm prefer' to see what SDM preference is currently active.

S1(config)#

S1(config)#interface vlan 1

S1(config-if)#description Connection to Lan A

S1(config-if)#ip address 192.168.202.02 255.255.255.192

S1(config-if)#ip default-gateway 192.168.202.1

S1(config)#

Press RETURN to get started!

No Unauthorised Access!

User Access Verification

Password:

S1>enable

Password:

S1#

S1#

S1#conf

Configuring from terminal, memory, or network [terminal]?

Enter configuration commands, one per line.  End with CNTL/Z.

S1(config)#

S1(config)#inter

S1(config)#interface vlan 1

S1(config-if)#ipv6 address 2001:db8:acad:0001::2/64 eui 64

^

% Invalid input detected at '^' marker.

S1(config-if)#ipv6 address 2001:db8:acad:0001::2/64

S1(config-if)#ipv6 address fe80::1 link-local

S1(config-if)#no shutdown

S1(config-if)#

%LINK-5-CHANGED: Interface Vlan1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan1, changed state to up

S1(config-if)#

# Task 3: Verify Network connectivity

| From | To | Protocol | IP Address | Ping Results |
| --- | --- | --- | --- | --- |
| PC-A | R1 G0/0/0 | IPv4 | 192.168.202.129/25 |  |
| PC-A | R1 G0/0/0 | IPv6 | 2001:db8:acad:0002::1 |  |
| PC-A | R1 G0/0/1 | IPv4 | 192.168.202.1 /26 |  |
| PC-A | R1 G0/0/1 | IPv6 | 2001:db8:acad:0001::1 |  |
| PC-A | S1 VLAN 1 | IPv4 | 192.168.202.2 /26 |  |
| PC-A | S1 VLAN 1 | IPv6 | 2001:db8:acad:0001::2 /64 |  |
| PC-A | PC-B | IPv4 | 192.168.202.150 /25 |  |
| PC-A | PC-B | IPv6 | 2001:db8:acad:2:200:cff:fe80:ae2 |  |
| PC-B | R1 G0/0/0 | IPv4 | 192.168.202.129/25 |  |
| PC-B | R1 G0/0/0 | IPv6 | 2001:db8:acad:0002::1 /64 |  |
| PC-B | R1 G0/0/1 | IPv4 | 192.168.202.1 /26 |  |
| PC-B | R1 G0/0/1 | IPv6 | 2001:db8:acad:0001::1 /64 |  |
| PC-B | S1 VLAN1 | IPv4 | 192.168.202.2 /26 |  |
| PC-B | S1 VLAN1 | IPv6 | 2001:db8:acad:0001::2 /64 |  |