# 14  Day  Weather  and  Air quality  measurement  and logging system Report

# Name: Morgan jones

Student number: 1706863

Module : (CDA6002)

Lecturer: Gary Jones

Course: Electric and Electronic Engineering

Table of Contents

Name: Morgan jones	1

Introduction	6

Components list	7

Temperature	7

Temperature	7

Humidity	7

Wind speed	7

Wind direction	7

Air borne particle count	8

Sunlight brightness	8

UV levels	8

GPS position	8

VOC level	9

Casing	9

Battery	9

Memory Card Connector	9

Memory Card	10

Connector	10

Microcontroller	10

LED	10

Prototype costs	11

# Introduction

This report will contain a literature review of portable weather measurement systems with their features with a personal view of the positives and negatives of each. There will also be a list of components with a choice of three for each component in a comparison table where a column will be highlighted showing the chosen component and the reasoning behind the choice. There is also a full circuit design and a PCB design with prototype costs. The chosen choice will be highlighted in green.

# A brief literature review of portable weather measurement systems

The ATMOS 41 is an all-in-one weather station designed to work in harsh environments and has the ability to measure a minimum of 12 weather phenomenon’s such wind speed and direction however there is no GPS, and it is limited on what it can connect to due to the type of connector it uses.

The Bresser 5-in-1 is a weather station with 5 sensor for wind speed, wind direction, humidity, temperature, and precipitation this means that it is limited to the parameters of the sensors which limits what can be measured  is also requires wi-fi for some of the functions to works

The Davis Vantage Vue is multi sensored weather station that has sensors that measure seven separate weather conditions these are wind speed, wind direction, Rainfall, Temperature, Humidity, UV & Sunburn ( light intensity ). There is also a wireless screen to view the sensor information

The Youshiko Professional is a weather station that measures seven separate weather phenomenon’s these are outdoor temperature, humidity , Weather index for outdoor dew-point, wind chill, heat index, , UV & Sunburn ( light intensity ).

| weather measurement systems | ATMOS 41 | Bresser 5-in-1 Complete Colour WiFi Weather Station with Weather Alerts | Davis Vantage Vue (Highly Accurate & Robust. Optional Mobile Phone App/Web/PC Connectivity) | Youshiko Professional 7-in-1 Weather station |
| --- | --- | --- | --- | --- |
| product |  |  |  |  |
| Features | Measures 12 weather variables including: air temperature, relative humidity, vapor pressure, barometric pressure, wind speed, gust and direction, solar radiation, precipitation, lightning strike counter and distance Easy installation No moving parts All weather station data transmitted over a single wire Digital SDI-12 communication Connect to ZL6 Designed for continuous deployment in harsh climates No louvered radiation shield to attract bugs Accurate at low wind speeds because no moving parts Detects fine-scale wind speed variations with 0.01 m/s resolution Integrated weather station accelerometer detects if sensor is off level Integrated spring acts as a rain gauge filter to keep out large particles but still allow enough water flow Optional bird deterrent fits perfectly on the weather station funnel | Connects to the Internet via your home WiFi Measures: Outdoor temperature and humidity Wind direction and speed Rainfall with history Barometric pressure in relative or absolute readings ice/frost alert Indoor base station/display console with integrated stand 5-in-1 multi-sensor suite | Easy installation - rain, wind, temperature & humidity sensors in self-contained sensor suite Transmits data wirelessly to your console anywhere up to 1,000ft (300m) away Weather data updates every 2.5 seconds! Wind tunnel tested to 200mph (321kmph) Solar-powered outdoor sensor suite, with battery backup View, store and graph your real-time weather data online/on the smartphone app/on your PC, with optional Weather Link software and data logger. | outdoor temperature, humidity , Weather index for outdoor dew-point, wind chill, heat index, , UV & Sunburn ( light intensity ). |
| Specification | Solar radiation Range: 0 to 1750 W/m2 Precipitation Range: 0 to 400 mm/h RELATIVE HUMIDITY Range 0 to 100% RH (0.00-1.00) Air temperature Range: -50 to 60 °C Humidity sensor temperature Range: -40 to 50 °C Barometric pressure Range: 50 to 110 kPa Horizontal wind speed Range: 0 to 30 m/s Wind direction Range: 0° to 359° Tilt Range: -90° to +90° Data logger compatibility METER ZL6 and EM60 data loggers or any data acquisition systems capable of switched 3.6- to 15.0-VDC excitation and SDI-12 communication Dimensions Diameter: 10 cm (3.94 in) Height: 34 cm (13.39 in), includes rain gauge filter temperature range –50 °C to60 °C Connector types 3.5-mm stereo plug connector or stripped and tinned wires Supply voltage (VCC to GND) Minimum: 3.6 VDC continuous Maximum: 15.0 VDC continuous | Indoor temperature (-5°C – 50°C) and humidity with comfort indication Outdoor temperature (-40°C – 60°C / -40°F – 140°F) and humidity (1 – 99%) Wind direction and speed (0 – 112 mph, 50 m/s, 180km/h, 97 knots) Rainfall (0 – 19999mm) with history (hourly, daily, weekly, monthly, total rainfall) Weather forecast (sunny, partly cloudy, cloudy, rainy, stormy, snowy) Barometric pressure in relative or absolute readings (hPa, inHg, mmHg) (540 – 1100hPa) (relative range: 930 – 1050hPa) Weather index (‘feels like’ function, dew point, heat index, wind chill factor) Moon phase Selectable ice/frost alert Programmable alarms for weather extremes: indoor temperature/indoor humidity/outdoor temperature/outdoor humidity/rainfall/wind speed 3-mode display brightness: Hi / Lo / Off Dimensions 168 x 143 x 24mm (display console) Batteries Display console (base station): CR2032 backup battery (not included) Power Display console (base station): mains power, with optional battery backup Sensors The 5-in-1 outdoor sensor suite includes sensors for wind speed, wind direction, humidity, temperature, and precipitation Wireless transmission range to base station: 150 metres (line of sight) RF frequency: 868 MHz Dimensions Outdoor sensor suite: 343 x 393 x 136mm Batteries | Barometric Pressure, Dew Point, Humidity, Rainfall, Temperature, Wind Direction, Wind Speed Outdoor Unit The fully integrated, corrosion-proof sensor suite is built to take all the weather your location can give. The ISS combines a rain collector, temperature/humidity sensors, and anemometer/direction vane into a single unit. Sensors Rain Collector Self-emptying tipping spoon measures rain with 0.2mm resolution. Anemometer Records wind speeds as low as 2 mph and as high as 200 mph. Temperature and Humidity Provides outside temperature readings from -40°C to 65°C. Also measures relative humidity from 0 to 100%. Wind Direction Measures wind direction in compass rose points or degrees. Dimensions 329 x 146 x 340 mm Power Solar-powered | 8” True Black & Colourful Display for Time & Full Weather Information Receive Weather Data From 7-in-1 Weather Sensor (YC1136) Via 868 MHz RF Able To Synchronize UTC Time From Internet And With DST On/Off Function , Moon Phase And Sunrise / Sunset Display For Northern / Southern Hemisphere Buzzer Alarm Clock With Ice Pre-alert Outdoor Temperature (°c/°f) & Humidity Readings (YC1136) With Trend Gust Or Average Wind Speed (mph, m/s, km/h, knots) Wind Direction In 360° UV Index (0 – 16) With UV Level Indicator Light Intensity Reading Measure Rain rate, Barometric Pressure in Relative Or Absolute Readings (hPa, inHg, mmHg) Heat Index, Wind Chill, Dew Point Readings Index, High Wind-speed, High Daily Rainfall & Pressure Weather Data Calibration (Via the Set-up User Interface) , 3-mode Support Firmware Update 1 Pc Wireless outdoor 7-in-1 Professional Weather Sensor (YC1136) & 1 x Up To 7 x Wireless Wall Mount / Desktop Usage / Power Supply UK Adaptor Included Product Size: 215 X 172 X 29 Mm (W X H X D) / Back-up batteries 3 x AAA (not included) |
| Positives | All in one. Multiple sensors. Small size Robust | Multiple sensors Has GPS Wireless viewing screen | Multiple sensors Wireless viewing screen Protection against the elements and corrosion | Multiple sensors Wireless viewing screen |
| Negatives | No GPS For the ATMOS 41 weather station to meet digital logic levels specified by SDI-12, it must be excited at 3.9 VDC or greater Limited to what data loggers it can connect to Requires constant power | Limited to 5 sensors and their parameters Requires wi-fi to use GPS and for information to be sent to base station | Requires wi-fi for certain functions to work | Requires a socket power supply Requires wi-fi for certain functions to work |
| Reference | [1] | [2] | [3] | [4] |

# Components list

## Temperature

| Reference | [5] | [6] | [7] |
| --- | --- | --- | --- |

## Humidity

| Reference | [6] | [8] | [9] |
| --- | --- | --- | --- |

## Wind speed

| Reference | [9] | [10] | [11] |
| --- | --- | --- | --- |

## Wind direction

| Wind direction | WindSonic Solid State Wind Speed and Direction Sensor | R. M. Young 5103 Wind Monitor Sensor, wind speed and direction | Wind Sensor Garden Signal Output Aluminium Alloy Wind Direction Sensor Wind Vane Speed Measuring Instrument |
| --- | --- | --- | --- |
| Product |  |  |  |
| Features | 0-60m/s (116 knots) Wind Speed 0-359° Wind Direction NMEA Output Low Power Consumption Optional Heating System on WindSonic M version Maintenance Free. Free WindCom Software | a high-resolution rugged wind sensor. Simple , lightweight and corrosion resistant construction. The wind speed sensor is a four-blade helicoid propeller . The wind direction sensor is a rugged yet lightweight vane with a sufficiently low aspect ratio to assure good fidelity in fluctuating wind conditions. Vane position is sensed by a precision conductive plastic potentiometer located in a sealed chamber. | With RS485 interface. Aluminium Alloy material |
| Specification | Starting Threshold 0.01m/s Accuracy Wind Speed ±2%, Direction ±2% @ 12m/s Resolution Wind Speed 0.01m/s, Direction 1° Ultrasonic Output Rate 0.25, 0.5, 1, 2 or 4Hz Parameters Wind Speed & Direction or U & V Vectors Units m/s, knots, mph, kph, ft/min Power Supply - Anemometer Only 5-30vDC (Dependant on Output) 24vDC - Heating (Optional) 4.2A @ 24vDC Size 142 x 160mm Weight 0.9Kg WindSonic Output Options RS232, RS422, RS485 and NMEA plus 2 x Analogue Outputs: 0-5V, 0-20mA or 4-20mA Or SDI-12 | Min Wind Speed (mph)0 Max Wind Speed (mph)224 Wind Speed Accuracy±0.6 mph (0.3 m/s) or 1% of reading Battery Type Potentiometer excitation: 15 VDC maximum | Material: aluminum alloy For: Wind direction sensor / weather station Product Type: Six elements of meteorology Interface type: RS485 Measuring range: 0 ~ 360 ° Accuracy: ±3° Starting wind speed: ≤0.5m/s Power supply mode: DC Output form: current 4~20ma voltage 0~5V 0~10V RS485 Instrument line length: standard: 2.5 meters Load capacity: current type output impedance ≤250Ω Voltage type output impedance ≥1KΩ Working environment: temperature -40 ° C ~ 50 ° C humidity ≤ 100% RH Protection level: IP45 Cable grade: temperature grade: 80 ° C Rated voltage: 300V Product power consumption: 5.5mW Wind direction measurement range; 16 orientation Wind direction measurement accuracy: 1 position Response time: less than 5 seconds Baud rate: 9600 Communication port: RS485 Power supply: 12V-24V DC Power consumption: <1W Operating temperature: -30-80 ° C Operating humidity environment: 0-100% RH (15-95% RH) Size: Approx. 16 x 23cm/6.3 x 9.06in |
| Price | £684.00 - £1,398.00 | £1,510.50 | £34.09 |
| Reference | [7] | [11] | [12] |

## Air borne particle count

| Reference | [13] | [14] | [15] |
| --- | --- | --- | --- |

## Sunlight brightness

| Reference | [16] | [18] | [18] |
| --- | --- | --- | --- |

## UV levels

| Reference | [17] | [18] | [22] |
| --- | --- | --- | --- |

## GPS position

| Reference | [19] | [20] | [21] |
| --- | --- | --- | --- |

## Volatile Organic Compounds (VOC)  level

| Reference | [22] | [23] | [28] |
| --- | --- | --- | --- |

## Casing

| Reference | [24] |
| --- | --- |

## Battery

| Battery | Portable Power Technology Li Po Powerpack 450+ |
| --- | --- |
| Product |  |
| Features | Delivers AC and DC power Solar charging compatible Multi safety protection high temperature resistance Recharging in the Powerpack 450+ is safe, quick, and easy using the included AC adapter and 12V car adapter. The Powerpack 450+ can also be recharged from solar using the supplied cable and up to a 60 watt solar panel. |
| Specification | Battery Capacity : 3.7V @ 120Ah (444 Watt Hour) USB Output : 3 x 5V @ 3.1A total USB Type C : 1 x 10W, 1 x 40W Weight: 5kg integrated LCD control panel display battery and output status Solar charging compatible that lets you plug in up to 60W of 12V solar without external controller |
| Price | £499.00 |
| Reference | [25] |

## Memory Card Connector

| Reference | [26] |
| --- | --- |

Figure 1  Memory card connector

## Memory Card

| Memory card | SanDisk Ultra 512 GB microSDXC and adapter |
| --- | --- |
| Product |  |
| Features | Brand SanDisk Memory type Micro SDXC Colour Red/Grey Hardware Interface USB 3.0 Speed class Class 10 |
| Specification | Product Dimensions 0.1 x 1.5 x 1.09 cm; 4.54 Grams Memory Clock Speed 100 Megabytes Per Second Up to 512 GB capacity |
| Price | £72.99 |
| Reference | [27] |

## Connector

| Connector | RS PRO Crimp Connector, 4 Contacts |
| --- | --- |
| Product |  |
| Features | Contacts are tin plated, and other metal parts are brass or mazac, nickel plated. |
| Number of Contacts: | 4 |
| Mounting Type: | Cable Mount |
| Plug/Socket: | Plug |
| price | £4.01 |
| Reference | [28] |

## Microcontroller

| reference | [29] |
| --- | --- |

## LED

| Reference | [30] |
| --- | --- |

# Timings

GPS 1 signal when activated no other signals

Temperature 1/15 sec

Humidity 1/15 sec

Wind speed 1/15 sec

Wind direction 1/15 sec

Sunlight brightness 1/10min

UV levels 1/1 hr

Volatile Organic Compounds (VOC) Level 1/1 hr

Air borne particle count 1/1 hr

# Prototype costs

| Component | Price |
| --- | --- |
| Temperature | £78.48 |
| Humidity | £78.48 |
| Wind speed | £684.00 |
| Wind direction | £684.00 |
| Air borne particle count | £81.82 |
| Sunlight brightness | £1.38 |
| UV levels | £63.19 |
| GPS position | £18.58 |
| VOC level | £10.00 |
| Casing | £200.83 |
| Battery | £499.00 |
| Memory card connector | £ 2.04 |
| Memory card | £72.99 |
| Connector | £4.01 |
| Microcontroller | £2.44 |
| LED | £ 0.28 |
| Total cost | £1,719.04 |

## PCB costs

| PCB | £15.00 with no Gerber file |
| --- | --- |
| Delivery | £8.53 (12-16 Days) the quicker the delivery the more expensive it is |

# Block Diagram

# Easy-PC

## Schematic View

## PCB View

## 3D view

# Proteus

# list of suggested improvements

suggested improvement are the component should be combined in to one casing instead of multiple. There should be a wireless screen to view the stored data without using a usb or SD card. The weather station should be able to run of a battery and on solar power. Include a wider variety of sensors for example lighting or rain

# References

| [1] | “ATMOS 41 WEATHER STATION,” [Online]. Available: https://www.metergroup.com/environment/products/atmos-41-weather-station/?creative=442739627234&keyword=%2Bweather%20%2Bmonitoring%20%2Bequipment&matchtype=b&network=g&device=c&gclid=EAIaIQobChMItuqrmLDG7QIVCr_tCh0qgQDjEAAYASAAEgJN0vD_BwE. [Accessed Decmber 2020]. |
| --- | --- |
| [2] | “home weather station,” [Online]. Available: https://www.weathershop.co.uk/shop/weather-stations/home-weather-stations/5-in-1-bresser-colour-wifi-weather-station. [Accessed December 2020]. |
| [3] | “davis-vantage-vue-weather-station,” [Online]. Available: https://www.weathershop.co.uk/shop/davis-vantage-vue-weather-station?gclid=EAIaIQobChMI48zsm57I7QIVdIBQBh3RKAMMEAYYBiABEgJ64fD_BwE. [Accessed December 2020]. |
| [4] | “youshiko.co.uk,” [Online]. Available: https://youshiko.co.uk/products/weather-station-premium-quality-official-uk-version-wifi-internet-wunderground-weathercloud-professional-7-in-1-wireless-sensor-wind-speed-direction-rainfall-temperature-humidity-uv-sunburn?variant=31664847585374&currency=G. [Accessed December 2020]. |
| [5] | “Temp data sheet,” [Online]. Available: https://cdn.shopify.com/s/files/1/0174/1800/files/DS18B20.pdf?v=1596806772. [Accessed December 2020]. |
| [6] | “RS datasheet,” [Online]. Available: https://docs.rs-online.com/fcd5/0900766b8142cdce.pdf. [Accessed December 2020]. |
| [7] | [Online]. Available: https://www.automation24.co.uk/outdoor-temperature-sensor-titec-autf-mua-d?previewPriceListId=1. [Accessed December 2020]. |
| [8] | “digital-temperature--humidity-sensor-with-stainless-steel-probe,” [Online]. Available: https://www.robotshop.com/uk/digital-temperature--humidity-sensor-with-stainless-steel-probe.html?gclid=CjwKCAiA25v_BRBNEiwAZb4-ZRvHE7FWRRzOdvs3GL4SHkWsUjB2MM_GWLHs2n6wMJE-uNjWmMpzRBoC6goQAvD_BwE. [Accessed December 2020]. |
| [9] | “windsonic,” [Online]. Available: https://www.omniinstruments.co.uk/weather-stations-and-instruments/wind-speed-and-direction-sensors/windsonic-solid-state-wind-speed-and-direction-sensor.html. [Accessed December 2020]. |
| [10] | “WE550,” [Online]. Available: http://www.globalw.com/products/we550.html#Options. [Accessed December 2020]. |
| [11] | “R. M. Young 5103 Wind Monitor Sensor,” [Online]. Available: https://www.coleparmer.co.uk/i/r-m-young-5103-wind-monitor-sensor-wind-speed-and-direction/9953618?PubID=UY&persist=true&ip=no. [Accessed December 2020]. |
| [12] | “wind direction,” [Online]. Available: https://uk.banggood.com/Wind-Sensor-Garden-Signal-Output-Aluminum-Alloy-Wind-Direction-Sensor-Wind-Vane-Speed-Measuring-Instrument-p-1624988.html?utm_source=googleshopping&utm_medium=cpc_organic&gmcCountry=GB&utm_content=minha&utm_campaign=minha-gbg-en-pc. [Accessed December 2020]. |
| [13] | “RS datasheet,” [Online]. Available: https://docs.rs-online.com/ab19/A700000006569693.pdf. [Accessed December 2020]. |
| [14] | [Online]. Available: https://uk.banggood.com/RS485-PM2_5PM10-Sensor-Modbus-Particle-Detection-Sensor-Transmitter-Air-Quality-Detection-p-1626018.html?utm_source=googleshopping&utm_medium=cpc_organic&gmcCountry=GB&utm_content=minha&utm_campaign=minha-gbg-en-pc&currency=GBP&cur. [Accessed December 2020]. |
| [15] | “Honeywell,” [Online]. Available: https://www.digikey.co.uk/product-detail/en/honeywell-sensing-and-productivity-solutions/HPMA115S0-XXX/480-7035-ND/7202204?utm_adgroup=General&utm_source=google&utm_medium=cpc&utm_campaign=Smart%20Shopping_Product_Zombie%20SKUs&utm_term=&productid=7202204. [Accessed December 2020]. |
| [16] | [Online]. Available: https://www.robotshop.com/uk/mini-photocell-light-sensor.html?gclid=Cj0KCQiA_qD_BRDiARIsANjZ2LAzFHpQsmSubuTeSYoAdBSUzfk5sUpxh4uRgyiTQ6QtQGfKIRWfaAkaAv28EALw_wcB. [Accessed December 2020]. |
| [17] | “UV sensor,” [Online]. Available: https://uk.rs-online.com/web/p/photodiodes/1837143/. [Accessed December 2020]. |
| [18] | [Online]. Available: https://uk.rs-online.com/web/p/photodiodes/1769786/. [Accessed December 2020]. |
| [19] | “RS datasheet,” [Online]. Available: https://docs.rs-online.com/3767/A700000006719961.pdf. [Accessed December 2020]. |
| [20] | “RS data sheet,” [Online]. Available: https://docs.rs-online.com/c562/0900766b814beeb7.pdf. [Accessed December 2020]. |
| [21] | [Online]. Available: https://uk.rs-online.com/web/p/gps-chips-gps-modules/1709178/. [Accessed December 2020]. |
| [22] | “Gas sensing,” [Online]. Available: https://www.gas-sensing.com/sm-50.html#960=. [Accessed December 2020]. |
| [23] | “VOC sensor,” [Online]. Available: https://www.winsen-sensor.com/sensors/voc-sensor/mp901.html. [Accessed December 2020]. |
| [24] | “peliproducts.co.uk/1510-protector-case,” [Online]. Available: https://peliproducts.co.uk/1510-protector-case. [Accessed December 2020]. |
| [25] | “Power Pack,” [Online]. Available: https://www.machinemart.co.uk/p/portable-power-technology-powerpack-450/?da=1&TC=GS-010312138&gclid=CjwKCAiA25v_BRBNEiwAZb4-ZQywPSJ8I8i1y8jkjl6Euz8TPeT2Lar0cFt9wo2fkNCEdTB1uwiFjxoCP3wQAvD_BwE. [Accessed December 2020]. |
| [26] | “farnell datasheet,” [Online]. Available: http://www.farnell.com/datasheets/2701368.pdf. [Accessed December 2020]. |
| [27] | “SD card,” [Online]. Available: https://www.amazon.co.uk/SanDisk-microSDXC-Memory-Adapter-Performance/dp/B07SC7BZHR/ref=sr_1_4?dchild=1&keywords=micro+sd+card+512gb&qid=1606226502&sr=8-4. [Accessed December 2020]. |
| [28] | “RS connector,” [Online]. Available: https://uk.rs-online.com/web/p/industrial-circular-connectors/1115756/?cm_mmc=UK-PLA-DS3A-_-google-_-PLA_UK_EN_Connectors_Whoop-_-Industrial+Circular+Connectors_Whoop-_-1115756&matchtype=&aud-813230962291:pla-339061618701&gclid=EAIaIQobChMIn-Oa3cG67QIVDNP. [Accessed December 2020]. |
| [29] | “PIC microcontoller,” [Online]. Available: https://uk.rs-online.com/web/p/microcontrollers/1460209/. [Accessed November 2020]. |
| [30] | “Rs components,” [Online]. Available: https://uk.rs-online.com/web/p/leds/2285821/?cm_mmc=UK-PLA-DS3A-_-google-_-CSS_UK_EN_Displays_%26_Optoelectronics_Whoop-_-LEDs_Whoop-_-2285821&matchtype=&aud-821594433763:pla-307757506505&gclid=EAIaIQobChMI_pGI5qXG7QIVD7LtCh2HLA7mEAQYASABEgLPfPD_BwE&gclsr. [Accessed December 2020]. |
| [31] | “Microchip PIC18F25K42,” [Online]. Available: https://www.microchip.com/wwwproducts/en/PIC18F25K42#datasheet-toggle. [Accessed December 2020]. |