LEVEL/ LEfEL 4

SEMESTER 2

MODULE CODE/ Cod Y modIwl: PSDE 4004 - Electronics 1

aSSESSMENT TYPE/ mATH O ASESU: aSSIGNMENT

Assessment/Component/ AsESIAD/CYDRAN: 2

COMPONENT WEIGHTING/ PWYSAU’R CYDRAN: 35%

WORD COUNT/ NIFER Y GEIRIAU: 2000 WORDS/O EIRIAU

MODULE TUTOR / TIWTOR Y MODIWL: Jason Pointer

SUBMISSION DETAILS/ MANYLION CYFLWYNO:

- Make sure that Your Name , Your Student P Number, Your Module Title, Assignment Title and Your Module Lecturer’s Name are clearly shown on the front page of your assignment

- All assignments must be submitted electronically to Turnitin unless indicated by the module lecturer here: …………………………………………...

- If a paper submission is required then it will be indicated here: Yes

you will then be required to submit the assignment parts in a suitable report folder– not in poly-pockets - to the library.

- For paper submissions this form (ALL PAGES) MUST be printed out, signed, attached to the front of your assignment and submitted to the library separately from your electronic assignment submission.

- Assignments will not be accepted without this form as it is a requirement that you sign the disclosure regarding the referencing convention.

- DO NOT put this form into Turnitin or it will match many similarities with other students’ submissions.

###### RESULT

#### School of Applied Computing ASSIGNMENT SPECIFICATION

| Student name: |  | Student P number: |  |
| --- | --- | --- | --- |
| Programme: | HND/HNC Electrical & Electronic Engineering |  |  |
| Lecturer: | Jason Pointer | Internal Verifier: | Steve Roberts |
| Assignment Title: | Transistor & Operational Amplifier Circuits |  |  |
| Hand Out Date: | 02/02/18 | Submission deadline: | 16/03/18 |

| Referencing: | In the main body of your submission you must give credit to authors on whose research your work is based. Append to your submission a reference list that indicates the books, articles, etc. that you have read or quoted in order to complete this assignment (e.g. for books: surname of author and initials, year of publication, title of book, edition, publisher: place of publication). |  |  |
| --- | --- | --- | --- |
| Disclosure: | I declare that this assignment is all my own work and that I have acknowledged all materials used from the published or unpublished works of other people. All references have been duly cited. |  |  |
| Student’s Signature: | Hardcopy assignments will not be accepted without a signature here | Date: |  |

Late submission will result in a late penalty mark, as follows: Up to one week late, maximum mark of 40% for first attempts and 0% for resubmissions. (Academic Quality Handbook 2017/18 7.5 (5))

Learning Outcomes tested (from module syllabus)

1. Demonstrate an understanding of semi-conductor components and characteristics and their application in analogue and digital electronic circuits.

2. To design, simulate, build, test and evaluate analogue and digital electronic circuits using information gained from datasheets.

TASK DESCRIPTION

Part 1 - Transistor Amplifier Design and Performance Measurement

Objective

To construct a transistor amplifier configured in CE mode, to measure its performance, and to compare measured results against theoretical performance.

Experimental Procedure

(a)	Design a common emitter amplifier using a suitable transistor that has a hfe value between 	100 to 400 (value to be checked using datasheets and multimeter). The circuit will have a 	12V supply, the decoupling capacitor will be 100F and coupling capacitors will be 10F.

(b)	Build the circuit and produce a record of test results under DC and AC conditions.

(c)	Analyse the performance of the circuit.

Part 2 - Tuned Amplifier

The circuit below shows a tuned amplifier (class C).

9V

100nF

1mH

10F

56kΩ

10F

Vout

Vin

12kΩ

1kΩ

100F

0V

Task 1 - Calculations

Using the values shown in the circuit calculate the following:

(a)	The resonant frequency

(b)	The quality (Q) factor

(c)	The bandwidth of the amplifier.

Task 2 - Simulation

Using Multisim determine the following:

(a)	The resonant frequency

(b)	The gain at resonant.

(c)	The bandwidth of the amplifier, including the upper and lower frequencies.

Compare the results comment on any discrepancies.

Part 3 - Operational Amplifier Circuits

Objectives:

(1) 	To investigate the DC properties of a 741 operational amplifier by constructing and studying 	4 basic circuits.

(2) 	To investigate the AC properties of the 741 operational amplifier.

Simulation Requirements: 	Multisim.

Experimental Equipment: 	DVM, +15/-15 volt power supply, frequency generator, 							oscilloscope.

Task 1: Simulation using Multisim

For each of the four circuits shown on page 5, enter their details into Multisim.

(a) 	Connect a variable DC source to the input and use a multimeter and oscilloscope to measure 	the output voltages for a range of input voltages. Comment on your results.

(b) 	Connect a 1 Volt peak-to-peak sine wave to the input of the circuit shown in Fig. 1, and the 	Bode plotter to both the input and output. Print out the frequency response of the circuit, 	noting the frequency at which the gain is 1 (0 dB).

(c) 	Repeat (b) with the voltage increased to 5 Volts peak-to-peak. Print out your results.

Task 2: Experimental measurement

Construct each of the four circuits on breadboard:

(a) 	Connect the circuits with a DC power supply at the input and a DVM at the output.

(b) 	Construct tables showing the relationship between the input and output voltages for each 		circuit. Select a suitable range of input voltages to cover a full output voltage range.

(c) 	For Fig. 1 connect a sine wave to the input and an oscilloscope to the output. Apply a 1 volt peak-to-peak 1kHz signal and note the magnitude and phase of the output signal.

Complete the table shown below, ensuring that the input voltage stays at 1 volt peak-to-peak.

Plot the magnitude frequency response (in dB) of the amplifier on a log-linear graph.

| Frequency (kHz) | Output Voltage | Phase Relationship |
| --- | --- | --- |
| 1 |  |  |
| 3 |  |  |
| 8 |  |  |
| 10 |  |  |
| 13 |  |  |
| 16 |  |  |
| 20 |  |  |
| 30 |  |  |
| 50 |  |  |
| 80 |  |  |
| 200 |  |  |
| 300 |  |  |
| 500 |  |  |
| 800 |  |  |
| 1000 |  |  |

(d) 	Increase the amplitude of the input sine wave to 5 volts peak-to-peak. Vary the input signal 	frequency between 1 kHz and 100 kHz. Draw the output waveforms and explain their shape.

Task 3: Comparison of simulation and experimental results

Compare the results obtained from both the simulation and experimental work and comment on how they agree with theoretical predictions. Wherever possible, plot the simulation and experimental results on the same graphs so that true comparisons can be made.

Operational Amplifier Circuits

+15V

R1

+

-

-15V

R2

R2

10k

10k

Vout

V1

V2

1k

1k

R1

Vout = V1 + V2

+

-

+15V

-15V

R1

R2

1k

10k

Vout

Vin

Vout = V1(R1 + R2)/R1

+

-

+15V

-15V

R1

R2

1k

10k

Vout

V1

10k

1k

R1

R2

V2

Vout = (V1 - V2)R2/R1

+

-

+15V

-15V

R1

R2

1k

10k

Vout

Vin

Vout = -R2/R1

Fig. 1

Fig. 2

Fig. 3

Fig. 4

Task 4 - Negative Feedback

Many of these amplifier circuits used negative feedback. Explain what is negative feedback and the advantages of using it. Derive the formula for the closed loop gain for a negative feedback system.

GUIDANCE FOR Students IN THE COMPLETION OF TASKS

- Research-informed Literature: You must provide evidence of individual research of the concepts of transistor amplifier, operational amplifier circuits and the concepts of negative feedback.

(15%)

- Knowledge and Understanding of Subject: Must show an understanding of concepts and underlying principles associated with the circuits covered in the assignment.

(15%)

- Analysis: Provide analysis of the practical activities, discussing any variation in the results.

(20%)

- Practical Application and Deployment: Show the ability to perform practical work and be able to verify the operation of the circuits against calculated and theoretical results.

(40%)

- Skills for Professional Practice: Your work should be well organised and presented in a professional manner.

(10%)

Report Guidance

Your write up should be produced as an engineering report containing the following:

- Front Page

- Summary

- Table of Content

- Introduction & Theory

- Experimental Apparatus and Procedure

- Results and Discussion

- Conclusions - conclude what was discussed in the Results and Discussion section. Think of the conclusion as a short restatement of important points being presented in the report. The conclusion should also make links to the theory discussed earlier in the report.

- References - any direct quotes or images taken from a source material must be referenced using the Harvard System in the main body of the report but they should also be listed in a reference section at the end of the report.

- Bibliography - a list of the books or other sources of information that you consulted when writing the report.

marking criteria and Student FEEDBACK

This section details the assessment criteria. The extent to which these are demonstrated by you determines your mark. The marks available for each criterion are shown. Lecturers will use the space provided to comment on the achievement of the task(s), including those areas in which you have performed well and areas that would benefit from development/improvement.

| Common Assessment Criteria Applied | Marks available | Marks awarded |
| --- | --- | --- |
| 1. Research-informed Literature Extent of research and/or own reading, selection of credible sources, application of appropriate referencing conventions. | 15 |  |
| 2. Knowledge and Understanding of Subject Extent of knowledge and understanding of concepts and underlying principles associated with the discipline. | 15 |  |
| 3. Analysis Analysis, evaluation and synthesis; logic, argument and judgement; analytical reflection; organisation of ideas and evidence | 20 |  |
| 4. Practical Application and Deployment Deployment of methods, materials, tools and techniques; application of concepts; formulation of innovative and creative solutions to solve problems. | 40 |  |
| 5. Skills for Professional Practice Attributes in professional practice: individual and collaborative working; deployment of appropriate media; presentation and organisation. | 10 |  |

| Assignment Mark (Assessment marks are subject to ratification at the Exam Board. These comments and marks are to give feedback on module work and are for guidance only until they are confirmed. ) | Late Submission Penalties (tick if appropriate) |  | % |
| --- | --- | --- | --- |
|  | Up to one week late (capped at 40%) |  |  |
|  | First Attempt? |  |  |

### FdSc & HN COMMON ASSESSMENT AND MARKING CRITERIA

|  | OUTRIGHT FAIL | UNSATISFACTORY | SATISFACTORY | GOOD | VERY GOOD | EXCELLENT | EXCEPTIONAL |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Assessment Criteria | REFER (equivalent to 0-29% | REFER (equivalent to 30-39%* | PASS (equivalent to 40-49%) | PASS (equivalent to 50-59%) | MERIT (equivalent to 60-69% | DISTINCTION (equivalent to 70-79% | DISTINCTION (equivalent to 80-100% |
| 1. Research-informed Literature Extent of research and/or own reading, selection of credible sources, application of appropriate referencing conventions | Little or no evidence of reading. Views and findings unsupported and non-authoritative. Referencing conventions largely ignored. | Poor evidence of reading and/or of reliance on inappropriate sources, and/or indiscriminate use of sources. Referencing conventions used inconsistently. | References to a limited range of mostly relevant sources. Some omissions and minor errors. Referencing conventions evident though not always applied consistently. | Inclusion of a range of research-informed literature, including sources retrieved independently. Referencing conventions mostly consistently applied. | Inclusion of a wide range of research-informed literature, including sources retrieved independently. Selection of relevant and credible sources. Very good use of referencing conventions, consistently applied. | A comprehensive range of research informed literature embedded in the work. Excellent selection of relevant and credible sources. High-level referencing skills, consistently applied. | Outstanding knowledge of research-informed literature embedded in the work. Outstanding selection of relevant and credible sources. High-level referencing skills consistently and professionally applied. |
| 2. Knowledge and Understanding of Subject Extent of knowledge and understanding of concepts and underlying principles associated with the discipline. | Major gaps in knowledge and understanding of material at this level. Substantial inaccuracies. | Gaps in knowledge, with only superficial understanding. Some significant inaccuracies. | Evidence of basic knowledge and understanding of the relevant concepts and underlying principles. | Knowledge is accurate with a good understanding of the field of study. | Knowledge is extensive. Exhibits understanding of the breadth and depth of established views. | Excellent knowledge and understanding of the main concepts and key theories. Clear awareness of challenges to established views and the limitations of the knowledge base. | Highly detailed knowledge and understanding of the main theories/concepts, and a critical awareness of the ambiguities and limitations of knowledge. |
| 3. Analysis Analysis, evaluation and synthesis; logic, argument and judgement; analytical reflection; organisation of ideas and evidence | Unsubstantiated generalisations, made without use of any credible evidence. Lack of logic, leading to unsupportable/ missing conclusions. Lack of any attempt to analyse, synthesise or evaluate. | Some evidence of analytical intellectual skills, but for the most part descriptive. Ideas/findings sometimes illogical and contradictory. Generalised statements made with scant evidence. Conclusions lack relevance. | Evidence of some logical, analytical thinking and some attempts to synthesise, albeit with some weaknesses. Some evidence to support findings/ views, but evidence not consistently interpreted. Some relevant conclusions and recommendations, where relevant | Evidence of some logical, analytical thinking and synthesis. Can analyse new and/or abstract data and situations without guidance. An emerging awareness of different stances and ability to use evidence to support the argument. Valid conclusions and recommendations, where relevant | Sound, logical, analytical thinking; synthesis and evaluation. Ability to devise and sustain persuasive arguments, and to review the reliability, validity & significance of evidence. Ability to communicate ideas and evidence accurately and convincingly. Sound, convincing conclusions / recommendations. | Thoroughly logical work, supported by evaluated evidence. High quality analysis, developed independently or through effective collaboration. Ability to investigate contradictory information and identify reasons for contradictions. Strong, persuasive, conclusions, justifiable recommendations. | Exceptional work; judiciously selected and evaluated evidence. Very high quality analysis, developed independently or through effective collaboration. Ability to investigate contradictory information and identify reasons for contradictions. Highly persuasive conclusions |
| 4. Practical Application and Deployment Effective deployment of appropriate methods, materials, tools and techniques; extent of skill demonstrated in the application of concepts to a variety of processes and/or contexts; formulation of innovative and creative solutions to solve problems. | Limited or no use of methods, materials, tools and/or techniques. Little or no appreciation of the context of the application. | Rudimentary application of methods, materials, tools and/or techniques but without consideration and competence. Flawed appreciation of the context of the application. | An adequate awareness and mostly appropriate application of well-established methods, materials, tools and/or techniques. Basic appreciation of the context of the application. | A good and appropriate application of standard methods, materials, tools and/or techniques. Good appreciation of the context of the application, with some use of examples, where relevant. | A very good application of a range of methods, materials, tools and/or techniques. Very good consideration of the context of the application, with perceptive use of examples, where relevant. Evidence of some innovation and creativity. | An advanced application of a range of methods, materials, tools and/or techniques. The context of the application is well considered, with extensive use of relevant examples. Application and deployment extend beyond established conventions. Innovation and creativity evident throughout. | Outstanding levels of application and deployment skills. Assimilation and development of cutting edge processes and techniques. |
| 5. Skills for Professional Practice Demonstrates attributes expected in professional practice including: individual initiative and collaborative working; deployment of appropriate media to communicate (including written and oral); clarity and effectiveness in presentation and organisation. | Communication media is inappropriate or misapplied. Little or no evidence of autonomy in the completion of tasks. Work is poorly structured and/or largely incoherent. | Media is poorly designed and/or not suitable for the audience. Poor independent or collaborative initiative. Work lacks structure, organisation, and/or coherence | Can communicate in a suitable format but with some room for improvement. Can work as part of a team, but with limited involvement in group activities. Work lacks coherence in places and could be better structured. | Can communicate effectively in a suitable format, but may have minor errors. Can work effectively as part of a team, with clear contribution to group activities. Mostly coherent work and is in a suitable structure. | Can communicate well, confidently and consistently in a suitable format. Can work very well as part of a team, with very good contribution to group activities. Work is coherent and fluent and is well structured and organised. | Can communicate professionally and, confidently in a suitable format. Can work professionally within a team, showing leadership skills as appropriate, managing conflict and meeting obligations. Work is coherent, very fluent and is presented professionally. | Can communicate with an exceptionally high level of professionalism. Can work exceptionally well and professionally within a team, showing advanced leadership skills. Work is exceptionally coherent, very fluent and is presented professionally. |

Cyfadran Dylunio Cymhwysol a Pheirianneg

Faculty of Applied Design and Engineering

Taflen Cyflwyno Aseiniad

Assignment Hand in Sheet

| MANYLION Y MYFYRIWR STUDENT’S DETAILS |  |  |  |  |
| --- | --- | --- | --- | --- |
| ENW: NAME: |  | RHIF P Y MYFYRIWR: STUDENT P NUMBER: | P |  |
| CWRS: COURSE: | HND Electrical/Electronic Engineering |  | LEFEL Y CWRS: LEVEL OF COURSE: | 4 |
| MODIWL: MODULE: | PSDE 4004 - Electronic 1 |  |  |  |
| TEITL YR ASEINIAD: ASSIGNMENT TITLE: | Transistor & Operational Amplifier Circuits |  |  |  |
| DARLITHIWR/WYR: LECTURER/S: | Jason Pointer |  | DYDDIAD: DATE: |  |
| Staff Llyfrgell YN UNIG / Library Staff ONLY Derbyniwyd gan / Accepted by: _______ Dyddiad Cyflwyno: Date Submitted: ________________________ Amser / Time: _____ |  |  |  |  |

----------------------------------------------------------------------------------------------------------------------------------------------------------

| DERBYNNEB / RECEIPT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ENW / NAME: |  | RHIF P Y MYFYRIWR: STUDENT P NUMBER: |  | P |  |
| CWRS / COURSE: | HND Electrical/Electronic Engineering |  | LEFEL Y CWRS: LEVEL OF COURSE: |  | 4 |
| MODIWL / MODULE: | PSDE 4004 - Electronic 1 |  |  |  |  |
| TEITL YR ASEINIAD/ ASSIGNMENT TITLE: | Transistor & Operational Amplifier Circuits |  |  |  |  |
| DARLITHIWR/WYR: LECTURER/S: | Jason Pointer |  | DYDDIAD: DATE: |  |  |
| Staff Llyfrgell YN UNIG / Library Staff ONLY Derbyniwyd gan / Accepted by: _______ Dyddiad Cyflwyno: Date Submitted: ________________________ Amser / Time: _____ |  |  |  |  |  |