#include <main.h>	// header file containing all relevant definitions

#include <lcd.c>	// LCD driver file

// Function Prototypes

void switchPosition1();

void switchPosition2();

void switchPosition3(unsigned buttonState);

void switchPosition4();

// Global variables

unsigned STATUS = 0;

unsigned inputState;

unsigned adcVal;

void main() {

// Setting up adc

setup_adc_ports(sAN0);																	// initialise adc channel

setup_adc(ADC_CLOCK_INTERNAL|ADC_TAD_MUL_0);											// setup adc operation

// Initialising LCD

lcd_init();

// Infinite loop

while(TRUE) {

// Getting inputs

inputState = (input_a() >> 1) & 0x1F;												// Input of port A with masking off unused pins to avoid acknowledging their floating state

// Reading ADC

set_adc_channel(0);																	// selecting adc channel

delay_us(30);																		// wait 30us for conversion

adcVal = read_adc();																// reading adc into adcVal

// Clearing outputs

lcd_putc("\f");																		// Flushing lcd display - lcd_putc is used as extra computation of printf not needed

output_b(0);																		// Turning off LEDs

// Testing state of inputs

switch(inputState) {

case 2:

case 3:

switchPosition1();															// If swtich is in position 1 and regardless of state of button

break;

case 4:

case 5:

switchPosition2();															// If swtich is in position 2 and regardless of state of button

break;

case 8:

switchPosition3(OPEN);														// If swtich is in position 3 and button not pressed

break;

case 9:

switchPosition3(CLOSED);													// If swtich is in position 3 and button is pressed

break;

case 16:

case 17:

switchPosition4();															// If swtich is in position 4 and regardless of state of button

break;

default:

break;

}

delay_ms(10);																		// Delay for debouncing switch/button inputs

}

}

/////////////////

//	 functions  //

/////////////////

void switchPosition1() {

LCD_PRINT(10, 1, "%u", adcVal);															// Print adcVal to LCD line 1 centre

printf("%3u\r", adcVal);																// Send adcVal to terminal

output_b(adcVal);																		// Output binary value of adcVal to LEDs

if(adcVal < 75) {

output_low(PIN_C0);																	// Rotate motor counter-clockwise

output_high(PIN_C1);

LCD_PRINT(3,2, "Counter-Clockwise");												// Print motor direction on LCD in centre of line 2

}

else if(adcVal > 175){

output_low(PIN_C1);																	// Rotate motor clockwise

output_high(PIN_C0);

LCD_PRINT(7,2,"Clockwise");															// Print motor direction on LCD in centre of line 2

}

else {

output_c(0);																		// Stop motor spinning

}

}

void switchPosition2() {

output_c(0);																			// Stop motor spinning

LCD_PRINT(10,1, "%u", adcVal);															// Print adcVal on LCD in centre of line 1

printf("%3u\r", adcVal);																// Send adcVal to terminal

output_b(adcVal);																		// Output binary value of adcVal to LEDs

}

void switchPosition3(unsigned buttonState) {

if (buttonState) {																		// If button is indicated to be pressed

output_c(0);																		// Spin motor clockwise

LCD_PRINT(9,3, "Open");																// Print state of button on LCD in centre of line 3 continually to replace from LCD flush

if(!(bit_test(STATUS, LATCH_FLAG))) printf("Open\r");								// If latch flag has not been set to indicate button has already been closed send to state to terminal

bit_set(STATUS, LATCH_FLAG);														// Clear the latch flag bit of the status byte

}

else {																					// Otherwise if button isn't indicated to be pressed

output_c(1);																		// Stop motor spinning

LCD_PRINT(8,3, "Closed");															// Print state of button on LCD in centre of line 3 continually to replace from LCD flush

if(bit_test(STATUS, LATCH_FLAG)) printf("Closed\r");								// If latch flag has not been set to indicate button has already been open send to state to terminal

bit_clear(STATUS, LATCH_FLAG);														// Set the latch flag bit of the status byte

}

}

void switchPosition4() {

unsigned LEDoutput = 0x80;																// Setting up variables for LED change direction indicator and byte to be outputted to LED port

LCD_PRINT(7,4, "Kitt Mode");															// Print 'Kitt Mode' on LCD in centre of line 4

while(input_state(PIN_A5)) {															// Loop while switch is in position 4

if(!(bit_test(STATUS, DIRECTION_FLAG)) && LEDoutput != 1) {							// If direction is shifting to the left and output byte is not reached b'00000001'

LEDoutput >>= 1;																// Bit shift output byte left by 1

output_b(LEDoutput);															// Output output byte to LEDs

delay_ms(200);

}

else if(bit_test(STATUS, DIRECTION_FLAG) && LEDoutput != 128) {						// If direction is shifting to the right and output byte is not reached b'10000000'

LEDoutput <<= 1;																// Bit shift output byte right by 1

output_b(LEDoutput);															// Output output byte to LEDs

delay_ms(200);

}

else {

BIT_INVERT(STATUS, DIRECTION_FLAG);												// Inverting direction flag of status byte with macro defined in header file

}

}

}

Comments

2.. 20   3.. 30    4.. 40   5.. 10

***************************************

2..Your code has a number of structures and all work . Your statements indicate what most lines do 68%

3.. Your program flows nicely 75%

4.. Your program achieves the desired results. 75%

5.. Your program is nicely laid out and well indented but could have been remarked better. 70%